TRAJECTORY MAKER

A Trajectory Simulation Tool

����������������������������

ALGORITHM SUPPLEMENT

ORBIT HORIZONS

94 Promenade

Irvine, California 92715

U.S.A.

COPYRIGHT NOTICE

This document and software package  consisting of the Trajectory Maker

and Trajectory Scape computer programs  are copyrighted (C) 1991, 1992

by H.B. Reynolds, President, ORBIT HORIZONS.  All rights are reserved.

No  part   of  this   publication  may   be  reproduced,  transmitted,

transcribed, stored  in any retrieval  system, or  translated into any

language by any means without  the express written permission of ORBIT

HORIZONS, 94 Promenade, Irvine California 92715, USA.

FIRST EDITION/FIRST PRINTING

July 1992

ii

LIMITED WARRANTY

THIS  ALGORITHM  SUPPLEMENT,  THE  USER'S  GUIDE,  AND  THE  PROGRAMS,

"TRAJECTORY MAKER" AND  "TRAJECTORY SCAPE" ARE  SOLD "AS IS",  WITHOUT

WARRANTY  AS  TO  THEIR PERFORMANCE,  OR  FITNESS  FOR  ANY PARTICULAR

PURPOSE.  THE  ENTIRE RISK  AS TO THE  RESULTS AND  PERFORMANCE OF THE

PROGRAMS IS ASSUMED BY THE USER.

HOWEVER, ORBIT HORIZONS WARRANTS THE MAGNETIC DISKETTE(S) ON WHICH THE

PROGRAM IS RECORDED  TO BE FREE  FROM DEFECTS IN  MATERIALS AND FAULTY

WORKMANSHIP UNDER NORMAL USE FOR A PERIOD OF NINETY DAYS FROM THE DATE

OF PURCHASE.   IF DURING  THIS NINETY  DAY PERIOD  THE DISKETTE SHOULD

BECOME  DEFECTIVE,  IT  MAY  BE  RETURNED  TO  ORBIT  HORIZONS  FOR  A

REPLACEMENT WITHOUT CHARGE, PROVIDED YOU SEND PROOF OF PURCHASE OF THE

PROGRAM.

YOUR SOLE AND EXCLUSIVE  REMEDY IN THE EVENT  OF A DEFECT IS EXPRESSLY

LIMITED TO REPLACEMENT OF THE  DISKETTE AS PROVIDED ABOVE.  IF FAILURE

OF  A DISKETTE  HAS RESULTED  FROM ACCIDENT  OR ABUSE,  ORBIT HORIZONS

SHALL HAVE NO  RESPONSIBILITY TO REPLACE THE  DISKETTE UNDER THE TERMS

OF THIS LIMITED WARRANTY.

IN NO  EVENT SHALL ORBIT  HORIZONS BE  LIABLE TO YOU  FOR ANY DAMAGES,

INCLUDING BUT NOT LIMITED TO ANY  LOST PROFITS, LOST SAVINGS, OR OTHER

CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OR INABILITY TO USE THESE

PROGRAMS AND  ACCOMPANYING MATERIALS EVEN  IF ORBIT  HORIZONS HAS BEEN

ADVISED OF THE  POSSIBILITY OF SUCH  DAMAGES, OR FOR  ANY CLAIM BY ANY

OTHER PARTY.

SOME STATES DO NOT ALLOW THE  LIMITATION OR EXCLUSION OF LIABILITY FOR

INCIDENTAL  OR  CONSEQUENTIAL  DAMAGES,  SO  THE  ABOVE  LIMITATION OR

EXCLUSION  MAY NOT  APPLY TO  YOU.  THIS  WARRANTY GIVES  YOU SPECIFIC

LEGAL RIGHTS, AND YOU MAY ALSO HAVE OTHER RIGHTS WHICH VARY FROM STATE

TO STATE.

THIS WARRANTY SHALL BE CONSTRUED,  INTERPRETED AND GOVERNED BY LAWS OF

THE STATE OF CALIFORNIA.

YOU AGREE TO THESE TERMS BY YOUR DECISION TO USE THIS SOFTWARE.

iii

iii

TABLE OF CONTENTS

1  INTRODUCTION ...................................... 1-1

2  EQUATIONS OF MOTION ............................... 2-1

3  GRAVITY PERTURBATION MODEL ........................ 3-1

4  DRAG PERTURBATION MODEL ........................... 4-1

4.1  Density Profile ............................. 4-4

4.2  Wind Profile ............................... 4-14

5  NUMERICAL INTEGRATION SCHEME ...................... 5-1

6  COORDINATE SYSTEMS AND TRANSFORMATIONS ............ 6-1

6.1  Time Considerations ......................... 6-1

6.2  Orbital Elements ............................ 6-2

6.3  Geospherical Inertial Coordinates ........... 6-6

6.4  Earth-Fixed-Geocentric Coordinates .......... 6-6

6.5  Local Topocentric Coordinates ............... 6-7

6.6  Common Radar Coordinates ................... 6-10

6.7  Geodetic Coordinates ....................... 6-10

7 BIBLIOGRAPHY ....................................... 7-1

LIST OF TABLES

Table                                                Page

3-1  WGS-72 Earth Constants ........................ 3-1

3-2  Smithsonian Zonal Harmonic Coefficients ....... 3-2

4-1  Drag Coefficient vs. Mach Number .............. 4-3

4-2  Molecular Scale Temperature ................... 4-4

4-3  Atmospheric Density Profile ................... 4-6

5-1  Shanks 8-12 Numerical Integration Constants ... 5-3

iv

LIST OF FIGURES

Figure                                                Page

3-1  Magnitude of Vector Differences Between the

23rd and 2nd, 3rd, 4th and 5th Order Gravity

Models ......................................... 3-7

3-2  Magnitude of Vector Differences Between the

23rd and 6th, 7th, 8th and 9th Order Gravity

Models ......................................... 3-8

3-3  Magnitude of Vector Differences Between the

23rd and 10th, 11th, 12th and 13th Order

Gravity Models ................................. 3-9

3-4  Magnitude of Vector Differences Between the

23rd and 14th, 15th, 16th and 17th Order Gravity

Models ........................................ 3-10

3-5  Magnitude of Vector Differences Between the

23rd and 18th, 19th, 20th and 21st Order

Gravity Models ................................ 3-11

3-6  Magnitude of Vector Differences Between the

23rd and 19th, 20th, 21st and 22nd Order

Gravity Models ................................ 3-12

3-7  Magnitude of Vector Differences Between the

23rd and 2nd, 4th, 6th and 8th Order (Even)

Gravity Models ................................ 3-13

3-8  Magnitude of Vector Differences Between the

23rd and 3rd, 5th, 7th and 9th Order (Odd)

Gravity Models ................................ 3-14

3-9  Magnitude of Vector Differences Between the

23rd and 2nd, 6th, 10th and 14th Order

Gravity Models ................................ 3-15

3-10 Magnitude of Vector Differences Between the

23rd and 5th, 10th, 15th and 20th Order

Gravity Models ................................ 3-16

4-1  1976 U. S. Standard Atmosphere Density

Profile ........................................ 4-7

4-2  Abbreviated 1976 U. S. Standard Atmosphere

Density Profile ................................ 4-8

v

LIST OF FIGURES

Figure                                                Page

4-3  Trajectory Maker Density Profile Model ......... 4-9

4-4  Cubic Spline Interpolation Error for 21 and

48 Point Tables ............................... 4-10

4-5  Cubic Spline Interpolation Error for 21 and

48 Point Tables ............................... 4-11

4-6  Cubic Spline and Linear Interpolation Error

for 48 Point Table ............................ 4-12

4-7  Cubic Spline Interpolation Error for 48 Point

Table ......................................... 4-13

4-8  East/North Wind velocity Components

(Example 1) ................................... 4-15

4-9  East/North Wind velocity Components

(Example 2) ................................... 4-16

5-1  Numerical Integration Error .................... 5-6

vi

Trajectory Maker                              Introduction

Algorithm Supplement                                  1-1

����������������������������������������������������������

1.  INTRODUCTION

The Trajectory Maker/Trajectory Scape software system is a

state of the art simulation and design tool for use on IBM

compatible personal computers.  Within minutes it predicts

precision trajectory ephemerides and displays their ground

traces on world map projections.

The shapes  of various  orbit ground  traces can  be quite

bizarre, unexpected and difficult  to visualize because of

two motion  processes; namely  the Earth's  rotation about

its axis, coupled with the  motion of the orbiting object.

With  Trajectory  Maker,  visualization  of  this  complex

interaction is a breeze.

But Trajectory Maker  gives much more  than this graphics'

visualization capability.  It  also creates high-precision

numerical  output  data,  in  several  formats,  for those

wanting more comprehensive analysis capabilities.

Major  features of  the  software and  how  to use  it are

described in detail in the User's Guide.  The User's Guide

also  contains  some  historical  background  material,  a

glossary of  terms, definitions  and illustrations  to aid

the user with the external interface to the system.

Transparent  to the  user is  an underlying  foundation of

sophisticated mathematical algorithms  and techniques that

are  employed  by  the  system  in  order  to  perform the

relevant  numerical  computations,   in  an  accurate  and

expedient manner.

The user of  the system need  not necessarily be concerned

with  its  inner workings;  however,  an  understanding of

theory is sometimes helpful in understanding its practice.

This paper presents a detailed description of mathematical

algorithms and  techniques used  by the  Trajectory Maker/

Trajectory Scape software system.

In  addition to  presenting the  mathematical core  of the

software system, the  merits and limitations  of the algo-

rithms  are  discussed.  Illustrations  are  provided that

will aid in selecting various  aspects of the models, such

as truncation error, integration  step-size, etc., for the

particular application under consideration.

The equations  of motion  that govern  the trajectories of

Trajectory Maker                              Introduction

Algorithm Supplement                                  1-2

����������������������������������������������������������

Earth  orbiting  satellites  and  ballistic  missiles  are

summarized.

The force models include perturbation effects due to Earth

oblateness, aerodynamic drag and atmospheric winds.  These

forces are developed and investigated.

Explicit  closed-form  expressions  for  the gravitational

acceleration components  are derived  for an axi-symmetric

oblate  Earth  model.   The  model  contains  an arbitrary

number of zonal harmonic coefficients.  Effects of various

orders of the coefficients  on certain mission ephemerides

are investigated.

The  aerodynamic drag  force  is defined,  the atmospheric

density profile is  modeled as a  function of altitude and

its relative  error is analyzed.   An innovative algorithm

for computing altitude is presented that is very fast, yet

produces highly accurate values.

A numerical scheme for integrating the equations of motion

is presented, relevant coordinate  systems are defined and

algorithms for  transforming the various  input and output

parameters are summarized.

All physical constants used by the software are included.

The  algorithms  presented herein  support  the trajectory

mechanics  functions  in  the  program  source  code  with

examples that demonstrate their integrity.  A bibliography

is  provided  for  algorithm  extensions,  map  projection

related processes and routine mathematical processes.

Trajectory Maker                       Equations of Motion

Algorithm Supplement                                  2-1

����������������������������������������������������������

2.  EQUATIONS OF MOTION

The equations  of motion for  propagating trajectories are

modeled  in rectangular,  geocentric  inertial coordinates

with fundamental  plane coplanar with  the Earth's equator

and principle axis in the direction of the vernal equinox.

Let x, y,  z denote these  coordinates with z-axis coinci-

dent with the Earth's  spin-axis, positive northwards; the

x-axis in the equatorial  plane, positive in the direction

of  the  vernal  equinox;  and  the  y-axis  lying  in the

equatorial  plane  in such  a  manner that  the  system is

right-handed.

Further, let  i, j, k  be unit  vectors along the  x, y, z

axes, respectively.  Then, the position  of a point mass m

may be represented in vector form

r = xi + yj + zk

and according  to  Newton's  second law,  the differential

equations of motion may be written

d2r

m ��� =  � F

dt2

where � F is the sum  of the external forces acting on the

mass m.

In this paper the forces  due to the Earth's gravitational

potential  function   and  its   atmosphere  are  treated.

Letting G and D denote these  forces per unit mass, we may

write

d2r

��� = G + D

dt2

By  substitution  of  variables,   this  equation  may  be

decomposed  into  the following  first  order differential

equations,

dr

�� = v

dt

Trajectory Maker                       Equations of Motion

Algorithm Supplement                                  2-2

����������������������������������������������������������

dv

�� = G + D

dt

where,  using  Newtonian   differentiation,  the  velocity

vector is

.    .    .

v = xi + yj + zk

Expressed in component form, these two vector differential

equations become

dx   .            dy   .            dz   .

�� = x ,          �� = y  ,         �� = z

dt                dt                dt

.                 .                 .

dx                dy                dz

�� = Gx + Dx ,    �� = Gy + Dy ,    �� = Gz + Dz

dt                dt                dt

A  solution to  these  six differential  equations  may be

obtained,  once the  components  of the  gravity  and drag

acceleration components  are expressed  as known functions

of x, y,  z.  A time  series solution is  called an ephem-

eris.  The actual path of the mass m is its trajectory.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-1

����������������������������������������������������������

3.  GRAVITY PERTURBATION MODEL

We suppose that the Earth's  figure is an oblate spheroid,

and let P be point at a distance r from its center of mass

and let �'  be the declination of  P.  Then, the potential

at P may  be expressed as a  series of spherical harmonics

of the form

� ��     �                        Ŀ

V = - �� � 1 -  �  Jn ( R/r)n  Pn(sin �') �

r ��    n=2                       ��

where � is the Earth's  gravitational mass constant, R its

equatorial radius, Jn the  zonal harmonic coefficients and

Pn the associated  Legendre polynomials of  order n.  This

form of  the Earth's  potential function  is known  as the

Vinti potential function [1].

The gravitational  mass constant  and Earth  radii used in

this  paper are  from the  WGS-72  constants [3]  shown in

Table 3-1.

Table 3-1.  WGS-72 Earth Constants

�������������������������������������������������͸

�   �      English          �      Metric         �

�������������������������������������������������Ĵ

� � � 62750.16789 nm3/sec2  � 398600.50 km3/sec2  �

�������������������������������������������������Ĵ

� R �  3443.91739 nm        �   6378.135 km       �

�������������������������������������������������Ĵ

� � �  0.7292115147 (10)-4  rad/sec               �

�������������������������������������������������Ĵ

� f �  1/298.26  Dimensionless                    �

���������������������������������������������������

The  harmonic  coefficients are  the  Smithsonian  1973 II

coefficients [2] tabulated in Table 3-2.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-2

����������������������������������������������������������

Table 3-2.  Smithsonian Zonal Harmonic Coefficients

�������������������������������������������������͸

�    J2 = 1082.636e-6    �    J13 =  -0.336e-6    �

�    J3 =   -2.540e-6    �    J14 =   0.101e-6    �

�    J4 =   -1.619e-6    �    J15 =   0.104e-6    �

�    J5 =   -0.230e-6    �    J16 =   0.043e-6    �

�    J6 =    0.552e-6    �    J17 =  -0.227e-6    �

�    J7 =   -0.345e-6    �    J18 =  -0.077e-6    �

�    J8 =   -0.204e-6    �    J19 =   0.083e-6    �

�    J9 =   -0.162e-6    �    J20 =  -0.108e-6    �

�    J10 =  -0.232e-6    �    J21 =  -0.070e-6    �

�    J11 =   0.317e-6    �    J22 =   0.075e-6    �

�    J12 =  -0.196e-6    �    J23 =   0.111e-6    �

���������������������������������������������������

Now, the distance  from the Earth's center  to the mass m,

coincident with the point P,  is the magnitude of r, given

by

r = �(x2 + y2 + z2)

and the sine of its declination is

sin �' = z/r

Since  the gravitational  force  is conservative,  the net

gravitational  acceleration G  acting on  the mass  is the

negative gradient of V

G = -   V

which can be decomposed, by differentiating, into the form

G = -(�/r3)r -   U

The first term in  this expression represents the dominant

attraction the  Earth would  exhibit if  it were  a sphere

comprised  of  homogeneous concentric  shells.   This term

accounts for the non-perturbed conic motion of m.

The function U  is the disturbing  function whose gradient

represents  the  gravity  perturbation  acceleration which

causes  deviations  from  the  conic  motion  of  m.  This

function is given by

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-3

����������������������������������������������������������

�

U = �  �  Jn Rn r-(n+1) Pn(sin �')

n=2

It follows from the  gradient operator that the components

of the gravitational perturbation acceleration are

�Gx � -   U/  x = (�x)/r3 � Jn(R/r)n fn(�')

�Gy � -   U/  y = (�y)/r3 � Jn(R/r)n fn(�')

�Gz � -   U/  z = (�z)/r3 � Jn(R/r)n gn(�')

where

fn(sin �') = (n + 1)Pn(sin �') + sin � Pn'(sin �')

gn(sin �') = (n + 1)Pn(sin �') - cos � cot � Pn'(sin �')

Whittaker  and  Watson  [5]   derive  the  following  four

recurrence formulas for the complex variable u:

Pn+1'(u) - uPn'(u) = (n + 1)Pn(u)                  (1)

(u2 - 1)Pn'(u) = nuPn(u) - nPn-1(u)                (2)

(n + 1)Pn+1(u) - (2n + 1)uPn(u) + nPn-1(u) = 0     (3)

Pn+1'(u) - Pn-1'(u) = (2n + 1)Pn(u)                (4)

Using the first recurrence formula yields

fn(sin �') = Pn+1'(sin �')

and the second and third formulas yield

gn(sin �') = (n + 1)Pn+1(sin �')/sin �'

Therefore, the components of the perturbation acceleration

become

�Gx = (�x)/r3 � Jn(R/r)n Pn+1'(sin �')

�Gy = (�y)/r3 � Jn(R/r)n Pn+1'(sin �')

�Gz = (�)/r2  � Jn(R/r)n(n + 1)Pn+1(sin �')

In order to numerically  evaluate these expressions it is,

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-4

����������������������������������������������������������

of  course, necessary  to limit  the  series' to  a finite

number of  terms.  Suppose, then,  that the  degree of the

polynomials in R/r is specified, say  M � 2.  Then, with a

shift of indices we have

M-1

�Gx = (�x)/r3 (R/r)2  �  ai(R/r)i-1

i=1

M-1

�Gy = (�y)/r3 (R/r)2  �  ai(R/r)i-1

i=1

M-1

�Gz = (�)/r2  (R/r)2  �  bi(R/r)i-1

i=1

where

ai = Ji+1 Pi+2'(sin �')

bi = Ji+1(i + 2)Pi+2(sin �')

The Legendre polynomials  and their derivatives  for n � 3

may  be respectively  computed from  the third  and fourth

recurrence formulas once it is noted that [5]

P1(u) = u

P2(u) = 1/2 (3u2 - 1)

and consequently

P1'(u) = 1

P2'(u) = 3u

In order to illustrate the effects of various combinations

of gravity  harmonic coefficients,  we consider  a nominal

Defense Meteorological Satellite Program mission.

Its state  vector, at  orbit insertion,  is represented in

the following Earth-centered-inertial coordinates:

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-5

����������������������������������������������������������

.

x =   442.151588 nm      x =  0.512255 nm/sec

.

y =  1387.396376 nm      y = -3.732104 nm/sec

.

z = -3611.173591 nm      z = -1.373147 nm/sec

The  satellite's  near circular  orbit  is sun-synchronous

with altitude of about 450 nm.

The orbit ephemerides were propagated from orbit insertion

for a duration of 25000 seconds, in increments of 100 sec.

This step-size is optimal, in this case, since any smaller

step  does not  significantly  improve the  solution.  The

trajectory ephemeris spans eight orbits.

Various gravity  models with reduced  orders (less coeffi-

cients)  were compared  with  the model  of  highest order

having the 22 coefficients listed in Table 2 above.

The state vector  of the highest  order model was compared

with the lower order  models by differencing their respec-

tive  position   vector  components,   and  computing  the

magnitude  of  the resulting  difference  vector.   In the

figures the difference between a trajectory generated with

a gravity model of order M and one of order N is indicated

in the legend by the symbol DJMJN.

The first group of the  figures, namely, Figures 3-1, 3-2,

3-3, 3-4, 3-5, 3-6, illustrate differences in ephemerides,

sequentially beginning with the  J2 coefficient and ending

with coefficients up to J23.   It is seen that this family

of curves has  a secular growth over  the time span, while

exhibiting a  tendency towards  convergence; however, that

is not to say that  any given order gravity model produces

more accurate trajectories than a lower order model, as

for  example, comparison  of the  sixth and  seventh order

models  in   Figure  3-2.   Figure   3-3  illustrates  the

beginning  of  the  loss of  data  integrity  as round-off

accumulates, becoming  more pronounced in  Figures 3-4 and

3-5.

The scales in  the above group of  figures vary across the

figures by  two orders of  magnitude.  In  order to illus-

trate variations across scales, the next group of figures,

namely,  Figures 3-7,  3-8, 3-9,  and 3-10  were composed.

Not all differences are illustrated, but enough to get the

idea.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-6

����������������������������������������������������������

The  gravitational effects  on  other orbit  parameters is

given in detail in Reynolds [4].

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-7

����������������������������������������������������������

Figure 3-1.  Magnitude of Vector Differences Between the

23rd and 2nd, 3rd, 4th and 5th Order Gravity

Models.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-8

����������������������������������������������������������

Figure 3-2.  Magnitude of Vector Differences Between the

23rd and 6th, 7th, 8th and 9th Order Gravity

Models.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-9

����������������������������������������������������������

Figure 3-3.  Magnitude of Vector Differences Between the

23rd and 10th, 11th, 12th and 13th Order

Gravity Models.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-10

����������������������������������������������������������

Figure 3-4.   Magnitude of Vector  Differences Between the

23rd  and  14th, 15th,  16th  and  17th Order

Gravity Models.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-11

����������������������������������������������������������

Figure 3-5.   Magnitude of Vector  Differences Between the

23rd  and  18th, 19th,  20th  and  21st Order

Gravity Models.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-12

����������������������������������������������������������

Figure 3-6.   Magnitude of Vector  Differences Between the

23rd  and  19th, 20th,  21st  and  22nd Order

Gravity Models.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-13

����������������������������������������������������������

Figure 3-7.   Magnitude of Vector  Differences Between the

23rd and 2nd,  4th, 6th and  8th Order (Even)

Gravity Models.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-14

����������������������������������������������������������

Figure 3-8.   Magnitude of Vector  Differences Between the

23rd and  3rd, 5th,  7th and  9th Order (Odd)

Gravity Models.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-15

����������������������������������������������������������

Figure 3-9.   Magnitude of Vector  Differences Between the

23rd  and  2nd,  6th,  10th  and  14th  Order

Gravity Models.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-16

����������������������������������������������������������

Figure 3-10.  Magnitude of  Vector Differences Between the

23rd  and  5th, 10th,  15th  and  20th Order

Gravity Models.

Trajectory Maker                             Gravity Model

Algorithm Supplement                                  3-17

����������������������������������������������������������

References

1.  Anon., Space Flight Handbooks, Volume I, NASA SP-33

Part 1, Office of Scientific and Technical Inform-

ation, NASA, Washington D. C., 1963.

2.  Gaposchkin, F. M., ed., 1973 Smithsonian Standard

Earth (III), Smithsonian Astrophysical Observatory,

Special Report 353, Cambridge, Ma., November 28, 1973.

3.  Hieb, H., Western Test Range Geodetic Coordinate

Manual, Part 1, Federal Electric Corporation, Feb.

1980.

4.  Reynolds, H. B., High-Order Gravitational Perturbation

Effects On DMSP Extended Mission Ephemerides, OAO

Corporation, Technical Operating Report, El Segundo,

Ca., Jan. 3, 1985.

5.  Whittaker, F. and G. Watson, A Course in Modern

Analysis, Cambridge University Press, Fourth Ed. 1963.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-1

����������������������������������������������������������

4.  DRAG PERTURBATION MODEL

The  aerodynamic drag  force  acting on  an  object varies

jointly with  the atmospheric  density �,  the object drag

coefficient CD,  the projected  cross sectional  area A of

the object (normal to the  velocity vector), the square of

its  speed relative  to  the rotating  atmosphere  va, and

inversely  with its  mass  m.  The  drag  force acts  in a

direction opposite that of the relative velocity vector.

In vector form this force per unit mass may be written

CD A

D = - � ����  va va

2m

justifiably, since the object  imparts a velocity of order

va  to a  mass of  air A�va  in each  second; the  rate of

change of momentum, equal to the drag force on the object,

is thus A�va�.  The empirical drag coefficient is based on

such  things  as  the object's  shape,  altitude  and Mach

number.

Taking into account local  wind conditions with the vector

q, the velocity relative to the rotating air mass is

va = v - � x r - q

where �  = �k is  the Earth's rotation  rate vector having

magnitude 0.7292115147 (10)-4  rad/sec.

It is usual to define a ballistic coefficient � by

m

� = ����

CD A

and write the drag force as

1

D = -  ��� � va va

2 �

which in component form,  after expanding the above vector

product, becomes

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-2

����������������������������������������������������������

.

Dx = -�va[(x + �y) - q1]/(2�)

.

Dy = -�va[(y - �x) - q2]/(2�)

.

Dz = -�va[ z - q3 ]/(2�)

where

.                  .                 .

va = �{[(x + �y) - q1]2 + [(y - �x) - q2]2 + (z - q3)2}

and  q1,  q2,  q3  denote  the  wind  velocity  components

relative to the rotating atmosphere.

Ballistic  coefficients  have  an  intuitive  appeal since

their units,  measured in  kg/m�,  resemble density units.

When working with English units, the ballistic coefficient

is usually expressed in mean  sea level weight rather than

mass, that is lb/ft�.   The following are typical examples

that may be encountered:

o A space based radar antenna: 0.144, to 0.448 lb/ft�

o Space Shuttle Discovery: 42 lb/ft�

o Ballistic missiles: 25, 75, 2150 lb/ft�

When  computing  a  ballistic  coefficients  for  orbiting

objects,  the  parameter  with  the  most  uncertainty  is

usually the drag coefficient.  Although this is not always

the  case.  For  example, Reynolds  [2] considers  a space

based radar system  having a rotating  antenna whose shape

is an arbitrary surface of revolution.

Wertz  [5] suggests  that  in the  absence  of aposteriori

knowledge of the  drag coefficient that a  value of 2.0 be

used.   Waison [4]  reports that  it should  be consistent

with the value  used to derive  the atmospheric model, and

that for the 1959  ARDC model one should  use 2.0; for the

1964 Jacchia model, a value of 2.2 should be used.

For reentry vehicles, the drag coefficient may be provided

as  a function  of Mach  number  M, as  in the  Table 4-1,

together with the vehicle's mass and effective area.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-3

����������������������������������������������������������

Table 4-1.  Drag Coefficient vs. Mach Number

Mass = 35.8365 slugs,   Area = 19.8 ft�

�����������������������������������������������������Ŀ

� M  � 0   � .25 � .50 � .75 � 1.0 � 1.25 � 1.5 � 100 �

�����������������������������������������������������Ĵ

� CD � .38 � .40 � .44 � .55 � .72 � .76  � .77 � .77 �

�������������������������������������������������������

Mach number  is function of  the speed of  sound, which in

turn is a function of temperature.  The expression adopted

for the speed of sound in Reference [1] is

Cs = �(  R* TM/M0)

where   is the  ratio of specific heat  of air at constant

pressure to that  at constant volume, and  taken to be 1.4

(dimensionless) exactly, R*  the gas constant  taken to be

8314.32  N.m/(kmol.K),  M0  the  sea-level  mean molecular

weight  of  the  mixture of  gasses  taken  to  be 28.9644

kg/kmol,  and  TM  is  the  molecular  scale  temperature,

modeled as a linear, segmental arc.

Putting all these things together results in

Cs = 20.04680276 �TM    (meters/sec)

and the Mach number is

M = va/Cs

The  concept of  speed of  sound progressively  looses its

range  of applicability  at  high altitudes  and  for this

reason the values  of speed of sound  are not computed for

heights above 86 km.

Table 4-2 is extracted from Table  9 of the U. S. Standard

1976 abbreviated tables.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-4

����������������������������������������������������������

Table 4-2

Molecular Scale Temperature

����������������������������������͸

� Geometric     � Molecular Scale  �

� Altitude (km) � Temperature (K)  �

����������������������������������Ĵ

�     0.0000    �     288.150      �

�    11.0190    �     216.650      �

�    20.0631    �     216.650      �

�    32.1619    �     288.650      �

�    47.3500    �     270.650      �

�    51.4124    �     270.650      �

�    71.8019    �     214.650      �

�    86.0000    �     186.946      �

������������������������������������

Since the U. S. STandard  models the temperature as linear

segments, it is only  necessary to interpolate linearly in

this  table  to  obtain  TM  for  arbitrary  altitude, and

substitute the result  into the above  equation to compute

the speed of sound.

4.1  Atmospheric Density Profile

Another complication  is that  density varies  in a fairly

irregular  manner  with factors  such  as  solar activity,

yearly  seasons, time  of  day, geographical  location and

altitude.   Fortunately,   extensive  research   has  been

performed  by various  agencies to  define models  for the

principle atmospheric parameters.

The 1976  U. S. Standard  Atmosphere [1]  is an idealized,

steady state representation of the Earth's atmosphere from

its surface  to 1000 km,  as it  is assumed to  exist in a

period of moderate solar activity.  It consists of compre-

hensive  tables  (approximately  1000  points)  of various

atmospheric parameters, including  density, tabulated as a

function of altitude.  The  density profile is illustrated

in Figure 4-1 on a common logarithm scale.

Since 1000 pairs is a lot  of data to handle, subsets were

extracted and modeled  with the goal  of obtaining a rela-

tively small subset that would yield sufficient accuracy.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-5

����������������������������������������������������������

In effort to obtain  the smoothest possible representation

for density, cubic spline functions were used to model the

common logarithm of the density profile, with the realiza-

tion  that,   during  orbit   propagation,  density  would

necessarily  be  obtained from  the  logarithm  profile by

exponentiation.

Several  subsets were  extracted  from the  Standard Atmo-

sphere, starting  with the  abbreviated tables, consisting

of  21 pairs,  from the   Standard Atmosphere  [1].  These

points are illustrated in Figure 4-2.

The final set selected for the model, herein, was obtained

by judiciously  selecting points from  the idealized model

until the percent relative error in density was reduced to

less than 1%.  The resulting set consisted of 48 altitude-

density pairs illustrated in  Figure 4-3.  Note that these

data are not equally spaced.  However,  besides being very

fast, Thompson's  algorithm [3]  accounts for  this situa-

tion.

Figure 4-4 illustrates the percent relative error obtained

when  using  these  two  density  profiles  to  model  the

idealized atmosphere.  The dashed  curve shows that in the

critical altitude  region (less  than 200  km) for reentry

trajectories, the 21 point set  produces errors as high as

6%.  On the  other hand, for  the 48 point  set the errors

are less  than 1%.  This  is shown more  clearly in Figure

4-5.

Figure  4-6 compares  linear interpolation  (dashed curve)

with spline interpolation (solid  curve).  It is seen that

linear interpolation  produces errors  as high  as 4%.  It

should be emphasized  that these error  curves are density

errors rather than the logarithm of the density errors.

Figure 4-7  isolates the error  for the 48-point  set on a

magnified scale.   The figure shows  that the  error to be

less than 1% on the entire altitude range.  It is this set

that  comprises  the  density  profile  used  herein.  The

actual values for this set are tabulated in Table 4-3.

This study also found that the endpoint derivatives played

a  significant  role  towards  achieving  the  1% accuracy

criterion.   Thus, rather  than computing  the derivatives

with  values from  Table  4-3, the  following  values were

computed  using  a  finer  resolution  from  data  in  the

comprehensive table:

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-6

����������������������������������������������������������

(log �(0))' = -0.041934

(log �(1000))' = -0.001834

where we  recall that the  splines were fit  to the common

logarithm of the density values.

Table 4-3.  Atmospheric Density Profile

��������������������������������������������������͸

� Altitude   Density     � Altitude   Density      �

�  (km)      (kg/m3)     � (km)       (kg/m3)      �

��������������������������������������������������Ĵ

�   0       1.2250       �    90      3.416e-6     �

�   2       1.0066       �   100      5.604e-7     �

�   4       8.1935e-1    �   110      9.708e-8     �

�   6       6.6011e-1    �   120      2.222e-8     �

�   8       5.2579e-1    �   130      8.152e-9     �

�   10      4.1351e-1    �   140      3.831e-9     �

�   12      3.1194e-1    �   150      2.076e-9     �

�   14      2.2786e-1    �   160      1.233e-9     �

�   16      1.6647e-1    �   170      7.815e-10    �

�   18      1.2165e-1    �   180      5.194e-10    �

�   20      8.8910e-2    �   190      3.581e-10    �

�   25      4.0084e-2    �   200      2.541e-10    �

�   30      1.8410e-2    �   220      1.367e-10    �

�   35      8.4634e-3    �   240      7.858e-11    �

�   40      3.9957e-3    �   260      4.742e-11    �

�   45      1.9663e-3    �   280      2.971e-11    �

�   50      1.0269e-3    �   300      1.916e-11    �

�   45      1.9663e-3    �   400      2.802e-12    �

�   60      3.0968e-4    �   500      5.215e-13    �

�   65      1.6321e-4    �   600      1.137e-13    �

�   70      8.2829e-5    �   700      3.069e-14    �

�   75      3.9921e-5    �   800      1.136e-14    �

�   80      1.8458e-5    �   900      5.759e-15    �

�   85      8.2196e-6    �  1000      3.561e-15    �

����������������������������������������������������

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-7

����������������������������������������������������������

Fig. 4-1.  1976 U. S. Standard Atmosphere Density Profile.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-8

����������������������������������������������������������

Fig. 4-2.  Abbreviated 1976 U. S. Standard Atmosphere

Density Profile.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-9

����������������������������������������������������������

Fig. 4-3.  Trajectory Maker Density Profile Model.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-10

����������������������������������������������������������

Fig. 4-4.  Cubic Spline Interpolation Error for the 21 and

48 Point Tables.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-11

����������������������������������������������������������

Fig. 4-5.  Cubic Spline Interpolation Error for the 21 and

48 Point Tables.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-12

����������������������������������������������������������

Fig. 4-6.  Cubic Spline and Linear Interpolation Error

for the 48 Point Table.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-13

����������������������������������������������������������

Fig.  4-7.  Cubic  Spline Interpolation  Error for  the 48

Point Table.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-14

����������������������������������������������������������

4.2  Wind Profile

Local atmospheric winds are typically supplied in the form

of a table which contains the wind velocity magnitude q(h)

and direction �(h)  as functions of  altitude.  The direc-

tion of the wind is defined as the true bearing from which

it is blowing.   The east, north and  up components of the

wind velocity are, respectively,

qe = -q(h) sin �(h)

qn = -q(h) cos �(h)

qu = 0

For the purpose of illustration,  a couple of typical wind

profiles from the Lompoc region are illustrated in Figures

4-8  and   4-9.   The  symbols   represent  observed  wind

velocities that were mapped into east and north components

by the above equations.   Cubic spline functions were used

to model these  data and several  values were computed and

plotted at  intermediate values  of altitude.   Two things

are noteworthy - that  splines provide a smooth represent-

ation of the data, and that Lompoc is a windy place.

The above components can  be transformed into Earth-fixed-

geocentric coordinates with the aid of a rotation matrix R

which  shall  be  defined,  subsequently,  when coordinate

systems and transformations are treated.  At that time, an

innovative algorithm  for computing altitude  will also be

developed.  For now, it suffices to write

��  Ŀ      ��  Ŀ

� q1 �      � qe �

� q2 � =  R � qn �

� q3 �      � qu �

��  ��      ��  ��

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-15

����������������������������������������������������������

Fig. 4-8.  East-North Wind Velocity Components, Example 1.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-16

����������������������������������������������������������

Fig. 4-9.  East-North Wind Velocity Components, Example 2.

Trajectory Maker                                Drag Model

Algorithm Supplement                                  4-17

����������������������������������������������������������

References

1.  Menzner, et. al., Defining Constants, Equations,

and Abbreviated Tables of the 1975 U. S. Standard

Atmosphere, National Aeronautics and Space

Administration, Technical Report NASA TR R-459,

Washington D. C., May 1976.

2.  Reynolds, H. B., Atmospheric Drag Effects on a Space

Based Radar Antenna, TRW Technical Memorandum,

January, 1991.

3.  Thompson, R. F., Spline Interpolation on  a Digital

Computer, Goddard Space Flight Center, Technical

Report X-692-70-261, Greenbelt, Md., July 1970.

4.  Waison, G. R., Prediction of Orbital Decay Rates Due

to Atmospheric Drag, TRW Program Technical Report,

3150-6020-RO-000, January 7, 1966.

5.  Wertz, J. R., ed., Spacecraft Attitude Determination

and Control, D. Reidel Publishing Company, Boston,

1985.

Trajectory Maker                        Integration Scheme

Algorithm Supplement                                  5-1

����������������������������������������������������������

5.  NUMERICAL INTEGRATION SCHEME

Substituting  the   atmospheric  drag   and  gravitational

perturbation acceleration  components, into  the component

form of the  first order vector  differential equations of

motion, results  in the  following explicit  system of six

first-order ordinary differential equations:

dx   .

�� = x

dt

dy   .

�� = y

dt

dz   .

�� = z

dt

.

dx     �x

�� = - �� + �Gx + Dx

dt     r3

.

dy     �y

�� = - �� + �Gy + Dy

dt     r3

.

dz     �z

�� = - �� + �Gz + Dz

dt     r3

If  the  perturbation  components  are  set  to  zero, the

resulting equations are clearly recognizable as describing

classical two-body motion.

By defining the system state as the vector

. . .

x = [ x y z x y z ]T

where the superscript T  denotes matrix transposition, the

differential  equations   may  now   be  brought   into  a

computational convenient vector form

dx

�� = f(x)

dt

Trajectory Maker                        Integration Scheme

Algorithm Supplement                                  5-2

����������������������������������������������������������

where f represents the vector-valued function of the state

vector defined by the  right-hand-side of the differential

equations.

The differential  equations of motion  comprise an initial

value system,  which may be  written in  more generally in

the vector form including time,

dx

�� = f(t, x),   x(t0) = x0

dt

.  .  .

where x = (x1, x2, x3, x4, x5, x6) � (x, y, z, x, y, z).

A numerical  scheme for integrating  initial value systems

that  is  of  very  high-precision,  generally  stable and

relatively  easy  to implement  is  Shanks'  explicit 8-12

formulation  for  solutions to  differential  equations by

evaluations of functions [1].

This method is based on the classical Runge-Kutta formulas

found in most textbooks on numerical analysis.

The 8-12 means that the order of the integration scheme is

eight (that  is, the truncation  error is  order nine) and

twelve  evaluations (that  is,  stages) of  the right-hand

side of the differential equations are performed.

This scheme is a fixed  step integrator which is preferred

for  trajectory  animation   graphics.   For  example,  an

orbiting object may move farther  in its present time step

than its preceding  time step. To  an observer, the object

appears to move faster.

An  advantage of  employing  a high-order  scheme  is that

accuracy  may be  maintained  in considerable  fewer steps

than the classical formulations.

Given the state x = x(t) of the system at time t, then the

state of the system at t + �t is given by

x(t + �t) = x(t) + �x

where

�x � ( 41 k1 + 216 k6 + 272 k7 + 27 k8 +  27 k9

+ 36 k10 + 180 k11 + 41 k12 ) / 840 + O(�t9)

Trajectory Maker                        Integration Scheme

Algorithm Supplement                                  5-3

����������������������������������������������������������

Using the Einstein  summation convention with  � = 1, ...,

12 and � � �, the coefficients may be defined by

k� = f(t + t� �t, x + s�� k� ) �t

The  non-zero  constants  t�  and  s��  appearing  in this

expression are defined in Table 5-1 below.

Table 5-1. Shanks' 8-12 Numerical Integration Constants.

��������������������������������������������������������͸

� t2 = 1/9     t3 = 1/6   t4 = 1/4    t5 = 1/10          �

� t6 = 1/6     t7 = 1/2   t8 = 2/3    t9 = 1/3           �

� t10= 5/6     t11= 5/6   t12= 1                         �

��������������������������������������������������������Ĵ

� s11 = 1/9                                              �

� s21 = 1/24         s22 = 1/8                           �

� s31 = 1/16         s33 = 3/16                          �

� s41 = 29/500       s43 = 33/500      s44 = -3/125      �

� s51 = 11/324       s54 = 1/243       s55 = 125/972     �

��������������������������������������������������������Ĵ

� s61 = -7/12        s64 = 19/9        s65 = 125/36      �

� s66 = -9/2                                             �

��������������������������������������������������������Ĵ

� s71 = -10/81       s74 = -32/243     s75 = 125/243     �

� s77 = 11/27                                            �

��������������������������������������������������������Ĵ

� s81 = 1175/324     s84 = -32/3        s85 = -3125/162  �

� s86 = 26           s87 = 121/162      s88 = -1/12      �

��������������������������������������������������������Ĵ

� s91 = 293/324      s94 = -71/27       s95 = -1375/324  �

� s96 = 51/9         s97 = -59/162      s98 = 1/2        �

� s99 = 1                                                �

��������������������������������������������������������Ĵ

� s101 = 1303/1620   s104 = -71/27      s105 = -1375/324 �

� s106 = 37/6        s107 = 103/162     s1010= 1/10      �

��������������������������������������������������������Ĵ

� s111 = -955/492    s114 = 2560/369    s115 = 8125/738  �

� s116 = -612/41     s117 = 7/82        s118 = -27/164   �

� s119 = -18/41      s1110= -12/41      s1111=  30/41    �

����������������������������������������������������������

Then, given the initial state x0 of the system at time t0,

the time increment �t and  the final time tf, the solution

may  be  generated  (or  propagated)  with  the  following

recursion scheme:

Trajectory Maker                        Integration Scheme

Algorithm Supplement                                  5-4

����������������������������������������������������������

x(tm) = x(t0 + m �t);   m = 0, 1, ..., (tf - t0)/�t

A  numerical  scheme   that  integrates  the  differential

equations of motion  directly, such as  this one, is known

as a Cowell integration scheme.  The scheme doesn't really

care  what kind  of  orbit it  is  generating, and  it can

handle   deviations   from   classical   two-body  motion.

Moreover,  the user  has  complete control  concerning its

accuracy with his choice of propagation step-size.

Figure  5-1  illustrates  the   common  logarithm  of  the

relative  error  in  Shanks'  8-12  numerical  integration

scheme.

The error is based on a 200 nm unperturbed orbit.  In this

instance, the  Earth is  modeled as  a sphere  so that, at

least theoretically,  the altitude  is a  constant 200 nm.

The  computed altitude  will, of  course, differ  from the

true altitude  depending on the  integration step-size and

the number of integration steps.

The  figure illustrates  the error  as  a function  of the

final time  in days of  86400 seconds.  (Equivalent to the

duration of the orbit since the initial time was zero.)

The relative error is shown because of its connection with

the number  of significant  figures and  independence from

any particular units  employed.  From elementary numerical

analysis, it  is known that  if the relative  error in any

number  is not  greater  than 0.5  X  10-N, the  number is

certainly correct to N significant figures.

It is  seen, for example,  that if an  orbit is propagated

with a step size of 300 seconds (5 minutes) for a duration

of  7 days  (2016 steps),  the  logarithm of  the relative

error is about -5.5.  Since

log(0.5 X 10-N) � -(0.301 + N)

and  -5.5  <  -5.301, the  computation  is  accurate  to 5

significant figures, which is  equivalent to about 0.01 nm

for a 200.00 nm orbit.

It is emphasized that the results obtained from the figure

are  based on  an unperturbed  orbit.  However,  they will

generally be valid for gravity perturbed orbits as well.

Trajectory Maker                        Integration Scheme

Algorithm Supplement                                  5-5

����������������������������������������������������������

Things  are  quite  different  for  objects  entering  the

Earth's atmosphere.  Such an object  experiences a violent

transient in  its acceleration  profile that  can tax most

any integrator's numerical integrity.

Several factors are  involved. The speed  of the object is

squared in the drag force  equation, the density varies by

orders of  magnitude becoming large  in the  100 km region

and the drag force is sensitive to the ballistic coeffi-

cient. (Small ballistic coefficients result in larger drag

forces than large ballistic coefficients.)

Trajectory Maker                        Integration Scheme

Algorithm Supplement                                  5-6

����������������������������������������������������������

Figure 5-1.  Numerical Integration Error

Trajectory Maker                        Integration Scheme

Algorithm Supplement                                  5-7

����������������������������������������������������������

References

1.  Shanks, B. F., Solutions of Differential Equations by

Evaluations of Functions, Math. Comp. 20 (1966) pp.

21-38.

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-1

����������������������������������������������������������

6.  COORDINATE SYSTEMS AND TRANSFORMATIONS

We have seen  that in order to  propagate the equations of

motion  with  a  Cowell integration  scheme,  all  that is

required  is  an  initial state  vector  given  in  Earth-

centered-inertial coordinates,

.  .  .

[x, y, z, x, y, z]T

at some epoch  t0.  Rectangular coordinates  such as these

are  difficult  to visualize.   Nevertheless,  the program

should  include the  option for   specifying input  in the

form of ECI coordinates, because these coordinates will be

known for  many applications; in  particular for ballistic

missile trajectories.

However, there  exist other coordinate  systems which will

simplify  our  visualization  process  when  it  comes  to

describing  an  initial trajectory  with  input  data, and

interpreting various characteristics with output data.

6.1  Time Considerations

A  fundamental  parameter  that  is  needed  to  define an

inertial reference system  is the Greenwich  hour angle of

the  vernal equinox  at  epoch or  orbit  insertion.  This

angle is  equivalent to  Greenwich sidereal  time at epoch

and serves to fix a satellite ground trace relative to the

rotating earth.

We suppose that  the initial time,  or epoch, is specified

by the year,  I, month, J,  day, K, and  universal time in

hours, minutes, and seconds  (past midnight).  These units

may  be converted  to the  Julian  date JD,  measured from

January 0.0 by the clever expression in Wertz [4],

JD = K - 32075 + 1461*( I + 4800 + (J - 14)/12)/4

+ 367*( J -    2 - (J - 14)/12*12)/12

- 3*((I + 4900 + (J - 14)/12)/100)/4

- 0.50

where  I,  J,  K  are  defined  to  be  integers  and  the

arithmetic  is  performed  in   integer  mode.   That  is,

floating point  numbers are truncated  at each  stage of a

computation, such as done by a computer language such as C

or FORTRAN.

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-2

����������������������������������������������������������

The right ascension of the Sun is obtained using Newcomb's

equation [1]

Ru = 279.6910 + 36000.7689 T + 0.0004 T2   (degrees)

where T  is the number  of Julian centuries  of 36525 days

elapsed since noon Greenwich mean time on January 0, 1900.

This value is computed from

T = (JD - JD0)/36525

where JD0  is the Julian  day from Greenwich  mean noon on

January 0.5, 1900 which is 2,415,020.0.

Universal time in degrees is

UT = 15*(hours + minutes/60 + seconds/3600)

So that the Greenwich sidereal time is

GST = Ru + UT - 180   (degrees)

6.2  Orbital Elements

A  convenient method  for defining  an  orbit is  with its

orbital elements.  These elements describe its size, shape

and orientation.   They are:  semimajor  axis a, eccentri-

city e,  inclination i, longitude of the ascending node �,

argument of  perigee w,  and some  time when  the orbiting

object passes perigee �.

This latter element is also  equivalent to mean anomaly M,

but  when  specifying non-elliptical  orbits,  it  is more

convenient to specify  time past perigee  and compute mean

anomaly from the formula

M = n�

where n is the mean motion given by

n = (� a3)1/2

This parameter  is the  same as  2�/P for  elliptic orbits

having period P.

It is a  known from two-body  theory that orbital elements

at an  instant in  time, in  particular at  the epoch, are

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-3

����������������������������������������������������������

integration  constants  of the  differential  equations of

motion,  and   these  constants  are   equivalent  to  the

instantaneous   rectangular   coordinates   by   a   given

coordinate  transformation.  This  transformation  is from

Roy [3].

A geocentric, inertial coordinate system is defined in the

orbit plane  with principle axis  x* along  the major axis

positive  in the  direction  of perigee,  another  axis z*

normal to  the orbit  plane in  the direction  of positive

angular momentum and  a third axis y*  defined so that the

system is right handed.

From the orientation angles �, w, i, the direction cosines

of the  x*, y*,  z* two  planar axes  relative to  the ECI

system may be obtained.  They are

l1 = cos � cos w - sin � sin w cos i

m1 = sin � cos w + cos � sin w cos i

n1 = sin w sin i

l2 = -cos � cos w - sin � sin w cos i

m2 = -sin � cos w + cos � sin w cos i

n2 = cos w sin i

These  cosines are  independent  of the  specific  type of

orbit be it elliptic, hyperbolic or parabolic.

Elliptical Case

���������������

The ECI state vector for the elliptical case is given by

x = al1 cos E + bl2 sin E - ael1

y = am1 cos E + bm2 sin E - aem1

z = an1 cos E + bn2 sin E - aen1

.

x = (bl2 cos E - al1 sin E)(na/r)

.

y = (bm2 cos E - am1 sin E)(na/r)

.

z = (bn2 cos E - an1 sin E)(na/r)

where b is the orbit's semiminor axis given by

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-4

����������������������������������������������������������

b = a(1 - e2)1/2

and r is objects' radius vector magnitude given by

r = a(1 - e cos E)

The  parameter   E  is  the   object's  eccentric  anomaly

obtainable from Kepler equation

E - e sin E = M

by employing Newton's method for finding roots.

Hyperbolic Case

���������������

The ECI state vector for the hyperbolic case is given by

x = -al1 cosh F + bl2 sinh F + ael1

y = -am1 cosh F + bm2 sinh F + aem1

z = -an1 cosh F + bn2 sinh F + aen1

.

x = (bl2 cosh F - al1 sinh F)(na/r)

.

y = (bm2 cosh F - am1 sinh F)(na/r)

.

z = (bn2 cosh F - an1 sinh F)(na/r)

where b is the orbit's semiminor axis given by

b = a(e2 - 1)1/2

and r is objects' radius vector magnitude given by

r = a(F cosh F - 1)

The parameter F is the objects' hyperbolic areal parameter

obtainable from the hyperbolic analog of Kepler's equation

F sinh F - F = M

also, by employing Newton's method for finding roots.

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-5

����������������������������������������������������������

Parabolic Case

���������������

The ECI state vector for the parabolic case is given by

x = l1r x* + l2r y*

y = m1r x* + m2r y*

z = n1r x* + n2r y*

.

x = l1 x* + l2 y*

.

y = m1 x* + m2 y*

.

z = n1 x* + n2 y*

where

x* = �(�p) ( sin f* cos f* /p - sin f* /r)

y* = �(�p) ( sin f* sin f* /p - cos f* /r)

For the parabolic  case, the semimajor  axis a is defined,

herein,  as the  object's perigee  distance and  the conic

parameter is computed from

p = 2a

The parameter f*  is the object's true  anomaly and may be

found by solving Barker's cubic equation

_

D + 1/3 D3 = 2n(t - �)

where

D = tan (f*/2)

and

_

n2 p3 = �

Details of the solution are given in Roy [3].

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-6

����������������������������������������������������������

6.3  Geospherical Inertial Coordinates

Geospherical    inertial    coordinates    are   spherical

coordinates  having their  origin  at the  Earth's center.

These  coordinates consist  of an  objects range  r, right

ascension � and declination  �.  Declination is equivalent

to geocentric latitude, until now, denoted by �'.

These coordinates are defined by

r = �(x2 + y2 + z2)

� = arcsin(z/r)

� = arctan(y/x)

In addition, the following flight parameters are computed:

Inertial velocity

.    .    .

v = �(x2 + y2 + z2)

Flight path angle

� = arcsin(x'/v)

Velocity azimuth angle

� = arctan(y'/z')

where

��  Ŀ   ��                                   Ŀ �� Ŀ

� x' �   �  cos � cos �    cos � sin �   sin � � � x �

�    �   �                                     � �   �

� y' � = �    -sin �          cos �        0   � � y �

�    �   �                                     � �   �

� z' �   � -sin � cos �   -sin � sin �   cos � � � z �

��  ��   ��                                   �� �� ��

All the remaining coordinates used for the software system

are relative to the rotating Earth.

6.4  Earth-Fixed-Geocentric Coordinates

Earth-fixed-geocentric  (EFG) coordinates  have  origin at

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-7

����������������������������������������������������������

the Earth's  geocenter, fundamental  plane coincident with

the equatorial plane  and principle axis  in the direction

of the Greenwich meridian.   This coordinate system, being

fixed  relative  to  the  Earth  is  actually  a  rotating

coordinate system.

Letting e, f, g denote  these coordinates, the e-axis lies

in the equatorial  plane and is  positive in the direction

of Greenwich;  the g-axis  is coincident  with the Earth's

polar axis, i.e.,  its spin axis; and  the f-axis lying in

the equatorial plane such that the system is right-handed.

This coordinate system is related to the inertial frame by

the transformation

e =  x cos(gst) + y sin(gst)

f = -x sin(gst) + y cos(gst)

g =  z

where gst is the current  Greenwich sidereal time.  If gha

is the Greenwich sidereal time at epoch, then

gst= gha + �t  (mod 360)

where t is the time past epoch.

6.5  Local Topocentric Coordinates

Local topocentric  coordinates (UVW)  have origin  at some

local site  on the surface  of the  Earth, and fundamental

plane tangent to the surface  at the site.  This reference

system is also a rotating coordinate system.

Letting u, v, w denote these coordinates, where the w-axis

is  normal  to the  tangent  plane at  the  site, positive

upwards;  the  principle  u-axis is  lies  in  the tangent

plane, positive at  in an azimuthal  direction   ; and the

v-axis lies in  the tangent plane such  that the system is

right-handed.

It is shown in O'Connor  [2], that the transformation from

earth-fixed-geocentric  coordinates  to  local topocentric

coordinates is given by the transformation

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-8

����������������������������������������������������������

�� Ŀ       ��      Ŀ

� u �       � e - e0 �

� v � =  ABC� f - f0 �

� w �       � g - g0 �

�� ��       ��      ��

where the rotation matrices are

��                    Ŀ

�  sin      cos      0 �

A =   � -cos      sin      0 �

�    0        0      1 �

��                    ��

��                    Ŀ

� 1      0        0    �

B =   � 0    sin �    cos �  �

� 0   -cos �    sin �  �

��                    ��

��                    Ŀ

� -sin      cos      0 �

C =   � -cos     -sin      0 �

�    0        0      1 �

��                    ��

where �,   denote the latitude  and longitude of the site,

respectively, and e0, f0, g0, are its EFG coordinates.

These latter coordinates may be computed directly from

��                     Ŀ

�        a              �

e0 = � ����������������  + h � cos � cos *

� �(1 - e2 sin2 �)      �

��                     ��

��                     Ŀ

�        a              �

f0 = � ����������������  + h � cos � sin *

� �(1 - e2 sin2 �)      �

��                     ��

��                     Ŀ

�    a(1 - e2)          �

g0 = � ����������������  + h � sin �

� �(1 - e2 sin2 �)      �

��                     ��

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-9

����������������������������������������������������������

If the azimuth angle is 90�, this system is referred to as

the  East-North-Up coordinate  system.   In this  case the

azimuthal matrix is the unit matrix

��           Ŀ

� 1    0    0 �

A =  � 0    1    0 �

� 0    0    1 �

��           ��

and the transformation becomes

��  Ŀ      ��      Ŀ

� ue �      � e - e0 �

� vn � =  BC� f - f0 �

� wu �      � g - g0 �

��  ��      ��      ��

where

��                                      Ŀ

�        -sin            cos        0    �

BC=   �  -sin � cos     -sin � sin        0    �

�   cos � cos      cos � sin      sin �  �

��                                      ��

By differentiating  and solving for  the e,  f, g velocity

components, we find that

.  .  .                .  .  .

[ e  f  g ]T =   (BC)T [ u  v  w ]T

These  components   may  be   identified  with   the  wind

components in Section 4.  That is  R= (BC)T so that

�   Ŀ      ��  Ŀ

� q1 �      � qe �

� q2 � =  R � qn �

� q3 �      � qu �

�   ��      ��  ��

Carrying out the matrix operations, we have

q1 = -[qe sin   + qn cos * sin �]

q2 =  [qe cos   - qn sin * sin �]

q3 =  qn cos �

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-10

����������������������������������������������������������

which are  the inertial wind  velocity components required

for the equations of motion.

6.6  Common Radar Coordinates

Radar coordinates  (RAE) are  used for  measurements of at

some particular geographic location or site.  They consist

of an object's  range, azimuth and  elevation.  Range R is

the distance from the site to the object; azimuth A is the

angle between  the object  and the  radar site's meridian,

measured  clockwise  from   the  northern  direction;  and

elevation  E  is the  angle  above or  below  the horizon.

These coordinates are given by the following expressions

R = �(ue2 +vn2 + wu2)

A = arctan(ue/vn)

E = arcsin(wn/R)

where the subscripts e, n, u denote the east, north and up

components, respectively.

In  addition  range rate  is  obtained  by differentiating

range.  We have,

.      .      .       .

R = (ueue + vnvn +  wuwu)/R

6.7  Geodetic Coordinates and Computation of Altitude

Altitude of  an orbiting  object is  its height  above the

Earth's surface.  If the Earth  were a sphere of radius of

radius a, then the  altitude of an object  at a distance r

from the geocenter would be

h = r - a

But the equatorial and polar radii differ by nearly 12 nm;

The  polar  radius  being  the  smaller  of  the  two.   A

parameter  commonly used  for measuring  the shape  of the

Earth is its flattening defined by

f = (a - b)/a

where,  in  this  section, a   and  b  denote  the Earth's

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-11

����������������������������������������������������������

equatorial and  polar radii,  respectively.  Flattening is

usually provided  as a  reciprocal, for  example the value

used herein, is the WGS-72 value of 1/298.26.

An improved altitude model is

h = r - a(1 - f sin �')

where,  as  before  �',  denotes  the  declination  of the

object.  The  declination   is  equivalent  to  geocentric

latitude.

Both  of  these  expressions are  used  by  the trajectory

software, but  are principally  used for  estimates in the

computer graphics.

A great deal of effort was expended in order to accurately

model the density profile, so we should expect an accurate

altitude model from which density is obtained during orbit

propagation.

An altitude model that will  achieve any desired degree of

precision that is also extremely fast; indeed the one used

in this paper, is an unpublished algorithm conceived by L.

H. Ivy.   A by-product of  his algorithm  are the trigono-

metric functions  required to construct  the geocentric to

topocentric rotation matrix, R sited in previous sections.

Consider  an  instantaneous  cross-section  of  the  Earth

defined by  the plane  containing its  polar axis  and the

mass m.   Define a two-dimensional  coordinate system with

u-axis formed by  the intersection of  this plane with the

equatorial  plane, and  v-axis  coincident with  the polar

axis.

The cross-section is bounded  by an ellipse whose equation

is

u2   v2

�� +  �� = 1

a2   b2

Since the eccentricity of the ellipse is given by

b2 = a2(1 - e2)

the its equation may be written

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-12

����������������������������������������������������������

u2(1 - e2) + v2 = a2(1 - e2)

Now, the tangent of the geodetic  latitude � of a point on

the ellipse is the negative reciprocal of the slope of the

line tangent to the ellipse  at that point.  It follows by

differentiation that

1      v

tan � =  �������� ���

(1 - e2)  u

Consequently,

a cos �

u =  ����������������

�(1 - e2 sin2 �)

a(1 - e2) sin �

v =  ����������������

�(1 - e2 sin2 �)

These coordinates  are the coordinates  of a  point on the

ellipse whose geodetic latitude is �.

On  the  other hand,  the  coordinates  of a  point  at an

altitude h whose geodetic latitude is � are

uh= u + h cos �

vh= v + h sin �

The geocentric latitude �' of this point is given by

��                                Ŀ

�                   e2             �

tan �' = � 1 -  ��������������������������� � tan �

�      1 + (h/a) �(1 - e2 sin2 � ) �

��                                ��

One can also find the following expression for altitude

h= uh cos �  + vh sin � - a�(1 - e2 sin2 �)

These two equations form the basis of Ivy's algorithm.

In essence,  the algorithm  is an  iterative process which

uses these equations starting  with geocentric latitude as

an estimate for geodetic latitude.

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-13

����������������������������������������������������������

To complete the algorithm, Ivy  defines a miss distance as

the  distance between  the  current estimate  for geodetic

latitude �i and the line normal to the reference ellipsoid

corresponding to the true geodetic coordinates.

Let ui, vi be  a point on the  ellipsoid whose latitude is

�i.  Then the equation of the line normal to the ellipsoid

at this point, expressed in normal form, is

(v - vi)cos �i - (u - ui)sin �i = 0

The distance from the line to the point u, v is

p = � (u - ui)sin �i - (v - vi)cos �i �

Using (3) and (4) with u = ui, v = vi, we have the

convergence criterion,

�                          ae2sin �i cos �i �

p = � uh sin �i - vh cos �i - ����������������� �

�                          �(1 - e2 sin2 �) �

The algorithm can now be summarized.

Algorithm Summary

�����������������

1. Input the coordinates of the object: x, y, z

2. Compute:  �(x2 + y2)

3. Compute:  tan �' = z/�(x2 + y2)

4. Replace:  tan � = tan �'

5. Compute:  cos2 � = 1/(1 + tan2 �)

6. Compute:  sin2 � = 1 - cos2 �

7. Compute:  rdcl = �(1 - e2 sin2 �)

8. Compute:  cos � = �cos2 �

9. Compute:  sin � = cos � tan �

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-14

����������������������������������������������������������

10. Compute: h = x cos � + y sin � - a rdcl

11. Compute: p = �x sin � - y cos � - ae2sin � cos �/rdcl�

12. Test: If p is less than some error criterion, end

algorithm.  Otherwise,

13. Compute: D = 1 + (h/a) rdcl

14. Compute: tan � = tan � '/(1 - e2/D)

15. Repeat Steps 5 through Step 14.

Optional computations:

Compute geocentric and geodetic latitudes:

�' = arctan(tan �'),    � = arctan(tan �)

Compute right ascension:

cos � = x/�(x2 + y2),    sin � = y/�(x2 + y2)

� = arctan(sin � /cos �)

It is worthy  to note that  no trigonometric functions are

actually  computed  with  this  algorithm,  yet  it  makes

available the  sine and  cosine of  the geodetic latitude.

Thus  the  rotation matrix,  required  for  computing wind

velocity components, may readily  be constructed.  This is

a great savings in  computer resources because this matrix

must be computed for each integration step.

The algorithm was tested  with latitudes ranging from -90�

to 90�, and altitude ranging from -300 to 5 X 1012 feet at

each latitude.  Miss distances of 10-1, 10-4 and 10-5 feet

were used as  convergence criteria.  In  no case were more

than three iterations required, and  no more than two were

required for miss distances of 10-4 feet.

Trajectory Maker                        Coordinate Systems

Algorithm Supplement                                  6-15

����������������������������������������������������������

References

1.  Anon., United States Navel Observatory and Royal

Greenwich Observatory, Explanatory Supplement to the

American Ephemeris and Nautical Almanac, Her Majesty's

Stationary Office, London, 1961.

2.  O'Conner, J. J., Transformations Applicable to Missile

and Satellite Trajectory Computations, RCA

International Service Corporation, Technical Report

AFETR-TR-75_29, Patrick Air Force Base, Florida, July

1975.

3.  Roy,A. E.,  The Foundations of Astrodynamics, The

Macmillan Company, New York, 1965.

4.  Wertz, J. R., ed., Spacecraft Attitude Determination

and Control, D. Reidel Publishing Company, Boston,

1985.

Trajectory Maker                              Bibliography

Algorithms Supplement                                  7-1

����������������������������������������������������������

7.  BIBLIOGRAPHY

Abell, G.,   Exploration of  the Universe,  Holt, Reinhart

and Winston, New York, 1964.

Anon., Space Flight  Handbooks, Volume I,  NASA SP-33 Part

1, Office  of Scientific and  Technical Information, NASA,

Washington D. C., 1963.

Anon., United States Navel Observatory and Royal Greenwich

observatory,  Explanatory   Supplement  to   the  American

Ephemeris and  Nautical Almanac,  Her Majesty's Stationary

Office, London, 1961.

Anon., Orbital  Flight Handbook, Office  of Scientific and

Technical  Information,  National  Aeronautics  and  Space

Administration, Washington D.C., 1963.

Bates,  R.,  and  D. Mueller,  J.  White,  Fundamentals of

Astrodynamics, Dover Publications, Inc., New York, 1971.

Battin,  B.,   An  Introduction  to  the  Mathematics  and

Methods   of   Astrodynamics,    American   Institute   of

Aeronautics and Astronautics, Inc., New York, 1987.

Deetz,  C.  and  O. Adams,  Elements  of  Map Projections,

Greenwood Press, New York, 1969.

Felberg,  F., New  One-Step  Integration Methods  of High-

-Order  Accuracy  Applied to  Some  Problems  in Celestial

Mechanics, NASA Technical Report  NASA TR R-248, George C.

Marshall Space Flight Center, Huntsville, Alabama, October

1966, pp. 32-33.

Gaposchkin, F.  M., ed.,  1973 Smithsonian  Standard Earth

(III),  Smithsonian   Astrophysical  Observatory,  Special

Report 353, Cambridge, Massachusetts, November 28, 1973.

Hieb, H.,  Western Test Range  Geodetic Coordinate Manual,

Part 1, Federal Electric Corporation, Feb. 1980.

Ivy  L.  and H.  Reynolds,  On the  Sensitivity  of Impact

Prediction   to  Methods   of   Interpolating  Atmospheric

Density,  ITT Technical  Note, TN-76-1463,  Vandenberg Air

Force Base, 1976.

Maling,  D.,   Coordinate  Systems  and  Map  Projections,

George Phillip and Son Limited, London, 1973.

Trajectory Maker                              Bibliography

Algorithms Supplement                                  7-2

����������������������������������������������������������

Mc  Cuskey,  S.,   Introduction  to  Celestial  Mechanics,

Addison Wesley Publishing Company, Inc., Palo Alto,  1963.

Menzner,  et.  al.,  Defining  Constants,  Equations,  and

Abbreviated Tables of the  1975 U. S. Standard Atmosphere,

National Aeronautics  and Space  Administration, Technical

Report NASA TR R-459, Washington D. C., May 1976.

O'Connor, J. J., Transformations Applicable to Missile and

Satellite   Trajectory  Computations,   RCA  International

Service  Corporation,   Technical  Report  AFETR-TR-75-29,

Patrick Air Force Base, Florida, July 1975.

Pearson,  F.,  Map  Projections: Theory  and Applications,

CRC Press, Inc., Boca Raton, 1990.

Ratcliffe, J., A.,  An Introduction to  the Ionosphere and

Magnetosphere, Cambridge Univ. Press., 1972, p.31.

Ridpath, I.,  Longman Illustrated  Dictionary of Astronomy

and Astronautics, York Press, Essex, 1988.

Roy, R.,  The Foundations of  Astrodynamics, The Macmillan

Company, New York, 1965.

Shanks,  B.  F., Solutions  of  Differential  Equations by

evaluations of Functions, Math. Comp. 20 (1966) pp. 21-38.

Steers,  D.,   An  Introduction   to  the   Study  of  Map

Projections,  University  of  London  Press  Ltd., London,

1962.

Struick, D., Lectures  on Classical Differential Geometry,

Addison-Wesley Publishing Company, Inc., Reading, 1961.

Thompson,  R.  F.,  Spline   Interpolation  on  a  Digital

Computer,  Goddard Space  Flight Center,  Technical Report

X-692-70-261, Greenbelt, Md., July 1970.

Waison, G.  R., Prediction of  Orbital Decay  Rates Due to

Atmospheric Drag, TRW  Program Technical Report 3150-6020-

RO-000, January 7, 1966.

Wertz, J.  R., ed., Spacecraft  Attitude Determination and

Control, D. Reidel Publishing Company, Boston, 1985.

Whittaker, F. and G. Watson,  A Course in Modern Analysis,

Cambridge University Press, Fourth Ed. 1963.

#