{Version 0.2 - Requires Turbo Pascal version 5.5}

{ Copyright (C) 1989 by Alpac Systems

Finn J. R. Nielsen

1741 W. Orchid Lane

Phoenix, AZ 85021 }

Unit QCom; Interface uses Dos;

Type

String255    = string[255];

BaudRates    = (B110, B150, B300, B600, B1200, B2400,

B4800, B9600, B19200, B38400);

ParityBit    = (NoParity, Odd, reserved1, Even,

reserved2, Mark, reserved3, Space);

DataBits     = 5..8;

StopBits     = (One, One_Half, Two);

ProtocolType = (NoProtocol, XoffProtocol, DsrProtocol);

CommPortPtr  = ^CommPort;

TimeOutCmd   = (Continue, Terminate);

TimeOutHdlr  = Function (Var Msg: string255): TimeOutCmd;

TimeOutPtr   = ^TimeOutHdlr;

commConfig = record                           { configuration record format }

Baud:        BaudRates;

Parity:      ParityBit;

Bits:        DataBits;

Stop:        StopBits;

Protocol:    ProtocolType;

ReadTime:    word; { read timeout value (0 = no checking)    }

EndOfString: char; { char signalling end of input string     }

InTreshold,        { Xoff or DTR to suspend when this input

queue treshold is reached               }

Resume:      byte; { Xon or DTR resumption at this threshold }

end; { Thresholds as integer percentages of the queue size     }

{ -------------------------------------------------------------------------- }

{ The CommPort object is here to allow the application to examine data values

in the object. The application may examine "Count" values to determine if

any characters are ready in either of the two queues. The "CommFlags" also

contains useful information about the RS232 line status (see flag

definitions below). This object is not very extensible since existing

method definitions are not included in the TPU source. The CommPort object

is actually a storage compatible facsimile of the actual object used by the

TPU }

CommPort = object

iQueue, oQueue: pointer;   { circular queue pointers    }

iCount, oCount,            { queue character counts     }

iTail, oTail,              { queue tail indecies        }

iHead, oHead,              { queue head indecies        }

iSize, oSize: word;        { queue space char counts    }

lowLimit, highLimit: word; { calculated tresholds       }

DosIntVec: pointer;        { saved DOS interrupt vector }

CommFlags: byte;           { line status flags          }

ComPort:   byte;           { port number (1 or 2)       }

Settings:  commConfig;     { configuration record       }

Constructor Init(inSize, outSize: word; iPort: byte);

Destructor  Done; virtual;

end;

Const

{bitmasks used to test and set CommFlags in CommPort object}

sntXoff     = $01;    { Xoff was sent to correspondent      }

xmitOn      = $02;    { Xmitter on                          }

RcvXoff     = $04;    { Xoff was received                   }

Dsr         = $08;    { Data set ready flag                 }

Carrier     = $10;    { Carrier flag                        }

LineActive  = $18;    { DSR + Carrier                       }

ParityErr   = $20;    { Input parity error                  }

FrameErr    = $40;    { Input framing error                 }

OvrFlow     = $80;    { Input overflow - receiver or queue  }

LineErrs    = $E0;    { Parity + framing + overflow         }

noSntXoff   = $FE;

xmitOff     = $FD;

noRcvXoff   = $FB;

noDsr       = $F7;

noCarrier   = $EF;

noLineActive= $E7;

noParityErr = $DF;

noFRameErr  = $BF;

noOvrFlow   = $7F;

noLineErrs  = $1F;

{ -------------------------------------------------------------------------- }

{ DeflCfg contains the configuration that is automatically installed

when the CommPort object is initially created. It may also be used

by the application as a template for modified configuration settings }

DeflCfg : commConfig = ( Baud:B9600; Parity:NoParity;

Bits:8; Stop:One; Protocol:XoffProtocol;

ReadTime:0; EndOfString:#13;

InTreshold:75; Resume:25 );

Var

MaxPorts: byte;       { Maximum number of usable comm ports }

{ -------------------------------------------------------------------------- }

Function  Connect(Iprt: byte; iqSize, oqSize: word): CommPortPtr;

{ This function must be used before any of the other communications

procedures described below - or else the program will be halted with

an unflattering error message. Circular queues are allocated from the

heap. It is the applications responsibility to check if the requested

space is actually available.

Iprt:         Comm port, usually 1 or 2;

iqSize:       Input queue size in characters;

oqSize:       Output queue size in characters;

CommPortPtr:  Pointer to the dynamically allocated CommPort object;

}

{ -------------------------------------------------------------------------- }

Procedure SetConfig(IprtPtr: CommPortPtr; var Config: commConfig);

{ Changes the configuration of the specified port. Although configuration

information is directly accessible to the application, this information

should not be changed directly by the application. Instead, a copy of

the configuration information should be made in the applications memory

space. The application may then modify the information and change the

setting for the port by using "SetConfig". A constant "DeflCfg" has been

defined. It contains the default configuration settings.

}

{ -------------------------------------------------------------------------- }

Procedure PutChar(IprtPtr: CommPortPtr; Ch: char);

{ Outputs a single character (control characters are of course legitimate).

IprtPtr:      Comm port, usually 1 or 2;

}

{ -------------------------------------------------------------------------- }

Function  GetChar(IprtPtr: CommPortPtr; TimeOut: TimeOutPtr): char;

{ Returns a single character from the input queue. In case of input time-

out no data was received. Timeout may occur if the read operation cannot

complete in the time allotted by the value of "ReadTime" in the config-

uration record (specified in hundredths of a second).

IprtPtr:      Comm port usually 1 or 2;

TimeOut:      Input timeout handling routine;

}

{ -------------------------------------------------------------------------- }

Procedure PutString(IprtPtr: CommPortPtr; var Msg: string255);

{ Outputs a string of 255 characters or less. No characters are added to

the output message. The caller must add any carriage returns and line

feeds (or other control characters) required by the application.

IprtPtr:         Comm port usually 1 or 2;

}

{ -------------------------------------------------------------------------- }

Function  GetString(IprtPtr: CommPortPtr; TimeOut: TimeOutPtr): string255;

{ Returns a string of 255 characters or less. The string may be terminated

by the character specified in "EndOfString" in the configuration record,

if such a character was received. Otherwise, the string will be terminated

after 255 characters are received. In case of input timeout, any partial

string received is available to the timeout handler. Timeouts will occur

if the read operation cannot be finished in the time allotted by the value

of "ReadTime" in the configuration record. If "ReadTime" is zero or the

timeout pointer is NIL, no timeouts will occur.

IprtPtr:      Comm port usually 1 or 2;

TimeOut:      Input timeout handling routine;

}

Implementation