PrintGL  1.23

pen plotter emulator for IBM/DOS systems

(C) Copyright Ravitz Software Inc. 1990,1992

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

p-1

Contents -------------------------------------------------------------

Introduction ..................................................... 2

License and Registration                                           3

What Is New ...................................................... 4

Installation                                                       5

Operation ........................................................ 6

How To Get HP-GL Plotfiles From Your Graphics Software             6

Messages ......................................................... 7

Syntax                                                             8

Overview of Options .............................................. 9

/A - Location Option                                               9

/B - Tiling Option ............................................... 9

/C - Pen Color Option                                             10

/D - Output Destination Option .................................. 10

/F - Output Format Option                                         11

/I - Scaling Point Location Option .............................. 13

/J, /K - Printer Code Prefix, Suffix Options                      13

/L - Page Layout Option ......................................... 14

/M - Magnification Option                                         15

/N - Page Number Option ......................................... 15

/O - Origin and Orientation Option                                15

/P - Plotfile Option ............................................ 16

/R - Internal Resolution Option                                   16

/S - Pen Shading Option ......................................... 16

/T - Temporary File Option                                        16

/W - Pen Width Option ........................................... 17

/X - User Interface Options                                       17

/Y - Y/D Switch Setting Option .................................. 17

/Z - Chord Angle and Enhanced Font Option                         17

PrintGL Menu Interface (PMI) .................................... 18

PMI Menu Functions                                                18

PMI Input and Output Choices .................................... 19

PMI Plot Options                                                  19

PMI Pen Options ................................................. 19

PMI Page Layout Options                                           20

PMI Other Options ............................................... 20

Large Paper                                                       20

Tips On AutoCAD ................................................. 21

Useful Prefix and Suffix Codes                                    21

Display Output .................................................. 22

HP-GL Output                                                      23

PostScript Output ............................................... 23

GEM .IMG and ZSoft .PCX Bit Map Output                            24

Printer Output .................................................. 25

HP-GL Commands                                                    29

HP-GL Character Sets ............................................ 38

Compatibility and Technical Information                           40

User Support .................................................... 41

p-2

Introduction ---------------------------------------------------------

PrintGL is a shareware pen plotter emulator for IBM/DOS systems.  It

prints  or displays HP-GL (Hewlett-Packard Graphics Language - 7475,

17440 subset) plotfiles on most PC graphics devices. HP-GL is widely

supported by CAD,  drawing,  plotting,  and  other graphics programs

including  AutoCAD,  Microstation, Generic  CADD,  MathCAD,  Schema,

OrCAD, and SAS.  PrintGL will print on Epson and IBM 9 pin printers,

Epson, Fujitsu, IBM, NEC, and Toshiba 24 pin printers,  HP LaserJet,

DeskJet,  color  DeskJet,  PaintJet,  QuietJet,  and  plotters,  IBM

ExecJet, Quietwriter 2 and 3, LaserPrinter, and plotters, Canon LBP,

BJ,  and BJC printers, Kodak Color 4,  and PostScript  printers.  It

will also display plots with a CGA, EGA, VGA, enhanced VGA, or  HGC,

and output a ZSoft .PCX or GEM .IMG bit map file.

Even  if  your  graphics program supports  your  printer,  you  will

probably  find that PrintGL  is faster,  gives better print quality,

and/or  gives more  formatting flexibility.  PrintGL  uses  the best

graphics modes available for each printer that  it supports and uses

transfer  data compression  to improve print speed on many printers.

And it gives options  such as orientation, magnification,  position,

and pen  width, color, and  shading.  If  you need  printed graphics

output from personal software, you can output  HP-GL and use PrintGL

to do the printing.  This gives immediate support to a wide range of

printers.  And it lets you use the HP 17440 command  set while still

supporting lower function plotters.

PrintGL interprets all of  the HP 7475 and 7440/17440 (ColorPro with

GEC)  command  set  that  is  applicable  to  plotfiles  except  for

character set 8 (Katakana).  None of the digitize, output, or device

control commands (except plotter on  and off) are supported as these

are only used with direct control of  a plotter.  And the error mask

command is not supported.  In addition to the HP 7475 and 7440/17440

commands,  PrintGL handles paper feed, extra space, and label origin

commands and proportional fonts from the HP 7550 command set.

PrintGL  can be  run  from the DOS command line (and  .BAT files) or

from PrintGL Menu Interface (PMI). PMI is a menu driven program that

lets you choose PrintGL options, select a list of plotfiles, and run

PrintGL. This simplifies the selection of PrintGL's many options and

lets you  run multiple plotfiles without intervention.  PMI lets you

save three  configurations,  so  after initial  setup  you  can  use

PrintGL with minimal effort.

This program is produced by a member of the Association of Shareware

Professionals (ASP).  ASP wants to  make  sure  that  the  shareware

principle  works   for  you.   If  you   are  unable  to  resolve  a

shareware-related  problem  with  an ASP  member  by  contacting the

member directly, ASP may be able to help. The ASP Ombudsman can help

you resolve a  dispute or problem  with an  ASP member, but does not

provide technical support for members' products. Please write to the

ASP  Ombudsman at  545  Grover Road,  Muskegon, MI 49442  or send  a

CompuServe message via CompuServe Mail to ASP Ombudsman 70007,3536.

p-3

License and Registration ---------------------------------------------

PrintGL (including PrintGL Menu Interface) is provided as is.  There

are no warranties expressed or implied.

PrintGL is copyrighted by Ravitz Software Inc.  You may copy PrintGL

only for use under this license agreement.

PrintGL is distributed as  shareware.  You may  use PrintGL  without

charge on a trial basis to determine its suitability for you. If you

continue  to use it  after  your  evaluation, a $50  registration is

requested.  This registration  covers  use  by a  single person  (on

multiple computers) or  installation  on  a  single computer (may be

used by  a group of people).  There is  no cost  to use a later 1.**

version after registering any 1.** version.

You may distribute PrintGL  as a stand alone product if you keep the

entire   package   together,  unchanged,   including   this  license

explanation, and do not charge more than $10. You may not distribute

PrintGL as part of another product.

To  register your copy of PrintGL,  send your name, address, and $50

check ($40 before March 1992) to:

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

You will be sent a printed manual and current  disk  (specify 3.5 or

5.25). If you previously registered PrintGL, you may send $15 to get

a current disk and manual.

Multiuser licenses are $50 for the first copy (with disk and manual)

and thereafter  $40 per copy  with disk and manual or  $25  per copy

without disk or manual.

LIMITED TIME DISCOUNT **********************************************

If you  send your registration before March 1992, the prices are $40

for  the first copy, $30 for additional copies with manual/disk, and

$20 for additional licenses without manual/disk.

********************************************************************

For users outside the USA, please make checks payable in US dollars,

and except for  Canada,  please  use  a  check  from  a  US  bank if

possible. Don't forget to include your country in your address.

If you register through a business, be sure to include your name.

If  you are interested in using  PrintGL as a  printer  driver in  a

commercial  program, send a note.  Special  license  agreements  are

available and there is an overlayable Turbo Pascal 6.0 unit that can

receive HP-GL directly from the calling program.

p-4

What Is New ----------------------------------------------------------

For version 1.23:

/DX1,..,/DX4 provide xon/xoff serial port drive

/DH1,..,/DH4 provide hardware handshaking serial port drive

improved Canon BJ130 driver (/FJ)

/FR drives the Tektronix ColorQuick (untested)

PostScript output is revised

Reverse image is allowed on /F4, /FD, and /FL

For version 1.22:

tiling is supported (see /B)

supports the Canon BJC and HP DJ500C color inkjet printers

supports Fujitsu 24 pin printers (/FF)

supports some Epson 9 pin almost compatibles (/FW)

generates Encapsulated PostScript preview bit maps

faster page printer, HP-PCL, color 24 pin drivers

/FV+ defaults to VESA 800x600 mode

/F5* gives 360x360 graphics for the IBM 2390, 2391

/F1* gives 240x144 graphics for the IBM 2380, 2381

revised PMI can create BAT files

/XP causes a pause for loading paper

For version 1.20:

revised font, kerned proportional font, character set 5

enhanced font (/ZE) improves curve smoothness

supports the ES and LO HP-GL commands

supports color PostScript

For version 1.18:

scrolling on displayed plots (see Operation paragraph 2)

/FO[+|*][!][~] works with Toshiba 24 pin printers

/F6* gives 360x364 dpi on IBM Proprinter X24s

9, .10 .. .16 are allowed as line widths (9..16 pels)

A..I specify line widths 2..10 in 4/1016 inch units (.1 mm)

print size correction

For version 1.16:

HP QuietJet, ZSoft .PCX, Kodak Color 4 output formats

compression mode drivers for 24 pin printers

HP-PCL data compression modes are utilized

/J and /K prefix and suffix codes

For version 1.14:

HP-GL, PostScript, Canon LBP, Canon BJ, IBM ExecJet output

HP 7550 polygon commands, proportional fonts are supported

The latest version of PrintGL is available from the support bulletin

board at  606-268-0577  (1200/2400,N,8,1  24  hours/day,  XMODEM  or

YMODEM (1K XMODEM) download), packed in .ZIP format.

p-5

Installation ---------------------------------------------------------

The PrintGL package includes these files:

PRINTGL.EXE  program

PRINTGL.DOC  documentation

CHARSET.PLT  HP-GL character set plotfile

PMI.EXE      menu interface program

SAMPLE.PLT   sample plotfile

README.*     quick information

To  install PrintGL copy the distribution files to your working disk

or directory.

PRINTGL.EXE is a stand alone executable  file.  There  are no  extra

printer driver files or overlays.

PRINTGL.DOC is this manual.  You can print it with "COPY PRINTGL.DOC

PRN".  The HP-GL character sets may  be  printed  with  the  command

PRINTGL  CHARSET.PLT  /M1  /AO0,0 and a /F  option specific to  your

printer, and inserted in the manual.

PMI.EXE is the executable file for PrintGL Menu Interface. PMI needs

to  be  able  to  find itself (PMI.EXE) and  PRINTGL.EXE, so both of

these files should  be in a directory that is in  your DOS PATH.  If

you prefer, you  can set the  environment variable PRINTGLP to  be a

path that tells  PMI where to  find itself and  PRINTGL.EXE.  If you

have PRINTGLP defined, PMI ignores the PATH value.

SAMPLE.PLT is a sample plotfile to help you try out PrintGL.

The README.* files  are not needed  to  run PrintGL,  but  they must

accompany the rest of the files if you distribute the package.

p-6

Operation ------------------------------------------------------------

PrintGL reads a plotfile, does arc,  character, fill, etc.  to  line

conversion,  stores the data in  a  coordinate list, and  if needed,

puts the list on disk. It then reorients the data and determines the

plot size. Next the data is resized and relocated. For vector output

formats (HP-GL, PostScript) data is output from here. For bit mapped

devices  (most printers, displays, bit maps) the coordinate list  is

rasterized and output in swaths.  You are likely to  see the printer

print a swath and stop while PrintGL is processing the next swath.

When you display a plot, press Esc to return to the DOS text screen,

or any  of the four cursor movement  keys to scroll across the plot.

Press  S  to cycle  between full, half  (the  default), quarter, and

eighth  page  scrolling.  Ctrl-Break stops PrintGL at the next break

point. Ctrl-Break is ignored when PrintGL is waiting for user input.

PrintGL works within a printer's current margins  and starts at  the

current  vertical print  position.  If  you decline  the  form  feed

suboption  of the  /L option then PrintGL leaves the  printer at the

start of the  line directly below the plot.  All printing is done in

printer lines that cover the full height of the printhead.

PrintGL will not automatically change a printer's operating mode. On

multimode  printers, the  right  operating  mode must be  set before

running PrintGL or with the /J option.

Internally PrintGL  uses two byte  coordinates  on  a 1016 dots/inch

grid. This allows for plots with coordinates of over 30 inches - far

larger than an HP 7475 can handle. However, if you are using PrintGL

to emulate a large plotter, you may overflow the internal coordinate

system.  This condition  is  not checked, and  it will cause garbage

output.  You can change the internal resolution to 508 dots per inch

(giving 60 inch maximum plot sizes) with the /R option (/R508). This

only helps  store the  original plot  internally.  The  next step in

PrintGL converts  these  coordinates  to the  requested size  in the

resolution of the output device, and these coordinates must also fit

in the two byte coordinate list.

How To Get HP-GL Plotfiles From Your Graphics Software ---------------

To use PrintGL, you must be  able  to  output HP-GL files  from your

graphics application.  Most CAD, drawing, and graphing software lets

you do this.  Just tell the application to output your  picture to a

plotter.  In either the configuration process or the output process,

you will need to specify that:  you have an HP 7475 or HP 7440/17440

plotter;  and that you want the output to go to a file instead  of a

COM port (some programs name the file automatically and some ask you

for a name).  Once you have the file, you can print it with PrintGL,

either from a DOS shell or after exiting the application.

p-7

Messages -------------------------------------------------------------

PrintGL  displays a number of messages to  keep you informed of what

it is doing. The output looks like this.

Reading SAMPLE.PLT

Plotfile has 1 page(s)    Processing page 1    2283 data records

Overall magnification    0.9964

Print window   Horizontal   0.00   7.54    Vertical   0.00  10.14

Plot area      Horizontal   0.42   7.13    Vertical   0.61   9.54

Output format /FN    120x72 dots/inch (HxV)

Writing to device LPT3#

The first  line  is the plotfile  that is currently being processed.

The second line tells how many pages are in the plotfile, which page

is being processed, and the number of data points in that page.  The

number of  data points  is roughly  equivalent  to the number of pen

moves and  is  shown  to  give  you  an  indication  of  the  plot's

complexity.

The  overall magnification is  either  the  magnification  that  you

specified  with the /M option or the  effective  magnification after

fitting to the print  window with /MF.  The  print window line gives

the horizontal  and vertical coordinates of the print window, always

starting  at 0,0 and in inches.  The plot area  gives the horizontal

and vertical coordinates of a box that just covers the plotted data.

This uses the same coordinate system as the print window, so you can

tell where the plot is relative to the print window.

The  output format  and resolution  are  given and  then the  output

device is  noted (except for screen plots).  If three plane color is

being used, the word "color" is displayed.  If you have a black only

printer, but the output format allows  color (this is  common with 9

and 24 pin  printers), using color  will triple the PrintGL run time

with no improvement in print quality.

During  processing, PrintGL informs  you when  it is  processing and

writing data,  and you can watch the progress from 0% to 100%, along

with the elapsed time.

p-8

Syntax ---------------------------------------------------------------

The calling syntax of PrintGL is:

PRINTGL plotfile /option1 /option2 ...

Spaces between parameters  are  not required.  The  plotfile  may be

specified with  the /P option if it  is inconvenient to make it  the

first parameter.

If a given option is entered more  than once, only the last entry is

used.  And if  an  option  is invalid  (but  not  incorrect),  it is

ignored.  This  lets  you set  up a  batch  file with your preferred

options  but override them as desired.  For example,  create PHP.BAT

with the line:

PRINTGL /FL /M1 /P %1 %2 %3 %4 %5 %6 %7 %8 %9

Then  PHP can be called just  like PRINTGL  but  it  defaults to  HP

LaserJet format with magnification 1.

The DOS environment variable PRINTGL  is examined by PrintGL and  if

it  is  nonblank  then  it  is  put  in  front  of  the command line

parameters.  If you  put "SET PRINTGL=/FL/M1"  in your  AUTOEXEC.BAT

then  PrintGL will default to LaserJet format with  magnification 1.

This is ignored when running under PrintGL Menu Interface.

If you do not enter any parameters or if any parameter is incorrect,

PrintGL displays its option reminder screen.

Below, [  and ] enclose optional suboptions and | separates mutually

exclusive suboptions. The options and suboptions may be in any order

except  that  multiple  numeric suboptions are  ordered  and numeric

suboptions must follow character suboptions where the characters may

be digits.  "c"  refers to  a suboption character,  "x" and  "y"  to

suboption  numbers,  "i"  and "j" to suboption  integers, "b"  to  a

suboption byte, and "f" to a DOS file or device name.

For numbers (x, y), a decimal point  is allowed but is not required,

and scientific notation is not allowed.  "-" is allowed but not "+".

For  integer  input (i, j),  only  base ten  integers from -32768 to

32767 are allowed. "+" is not allowed. A byte (b) must be a base ten

integer from 0 to  255 or a hexadecimal  number prefixed with $ from

$00 to $FF.  Further range checking is done for most options to flag

unreasonable input.  If an option  has multiple  numeric suboptions,

these may be separated with commas or blanks.

********************************************************************

It  is very important to choose the right output format (/F option).

It is likely  that  the  default  output  format will  work on  your

printer, but it is equally likely  that it  will not produce optimal

results.  Please look carefully at the entire list of output formats

before  choosing one, and do not assume that because a  format works

that it is the best one to use.

********************************************************************

p-9

Overview of Options --------------------------------------------------

PrintGL's options fall into several categories.  The  first is input

and output choices.  The  input is chosen with /P, the plotfile, and

/N,  the  page number.  Output options  are  /F,  the output  format

(printer model), and /D, the destination port, device, or file.

The second category is the page layout, which is mostly contained in

the  /L  option.  This includes the  size and location  of the print

window (which defines the hard  clip  limits of  the  plot), and the

option to box the window, send a form feed, reverse the image color,

and use opaque lines. And tiling is controlled with /B.

The third category defines  how the  plot  looks  within  the  print

window.  /M,  the  magnification,  sets  the size  of  the plot.  /A

determines the location of the plot within the print window. /O sets

the  origin  and  orientation, and  is used to rotate or  mirror the

plot. /I sets the initial scaling points as if you had set them from

the front panel of the plotter.

The  fourth category defines the pens, /W for the width,  /C for the

color, and /S for the shade.

And  the last category  determines how PrintGL processes  the  plot.

This includes the temporary file  /T, the default and minimum  chord

angle  /Z,  the  plotter  Y/D   switch  position  /Y,  the  internal

resolution /R, user interface options /X, and  /J and /K let you set

up and reset the printer for special situations.

/A - Location Option -------------------------------------------------

/A[O][x,y] - location - default /A

The A option determines the location of the plot in the print window

by specifying a point on the plot that is to be aligned with a point

in the print window.  You can specify the plot alignment  point with

x,y  in  inches  from  the  plot's  origin  (before PrintGL  applies

magnification), or let it default to the plot center.  For the print

window alignment point, you can default to the center or specify the

origin with O.

/A      puts the center of the plot at the center of the print

/AO0,0  puts the plot 0,0 at the print window origin

/B - Tiling Option ---------------------------------------------------

/B[i,j,x] - tiling parameters - default /B

/B lets  you specify tiling  options that break  a large  plot  into

pages.  Each page is defined by the page layout option (/L). I and j

specify the number of tiles  (pages) horizontally and vertically.  X

specifies the overlap in inches.  Displayed plots are sized based on

the  tiling, but  only the first tile is displayed.  Use  the cursor

keys to scroll the plot.

/B2,2,.5  breaks the plot into 4 pages with .5 inch overlap

/B1,1,0   is the same as /B and produces no tiling

p-10

/C - Pen Color Option ------------------------------------------------

/C[O|T]c.. - color - default /CK

/C specifies the color of each pen. The colors are B for blue, C for

cyan,  G  for green, K  for black, M for magenta, R for  red,  W for

white, and Y for  yellow.  Unspecified  pens use the  last  selected

color (/CRGB is  the same as /CRGBBBBBB).  If the output format does

not allow color then nonwhite colors are changed to black.

Additional colors can be created with shade mixing. These colors use

shading patterns  to  mix  the  eight pure colors.  The format for a

mixed color is  .??  where each ?  represents any of  the eight pure

colors.  The first  color is printed  with the pen's shading pattern

(/S option)  and the  second  is printed  with  the  inverse shading

pattern.  If  the shade value is  0 (solid) it is replaced by 2 (one

pel checkerboard) which produces the most uniform mixed colors.

Mixed colors need line  widths  of at least  2 (4  for 2 pel shading

patterns) to look good, and they may have  visual artifacts on edges

near a 45 degree angle.  Multipass dot matrix formats may not handle

color mixing well.

By  default,  colors  are  ored  into  the  print,  so  they  appear

translucent.  You can switch to opaque  colors (overwritten into the

print) with the O suboption, and then back to translucent with the T

suboption.  Make this mode switch in front of the pen that you  want

to change. For PostScript devices and plotters, the color processing

is done by the device, not  PrintGL.  PostScript devices use  opaque

colors and plotters (with standard pens) use translucent colors.

/CK           sets pen 1..8 to black

/CKOR.MBT.KY  sets pen 1    to black

pen 2    to opaque red

pen 3    to opaque magenta on blue

pen 4..8 to translucent black on yellow

/D - Output Destination Option ---------------------------------------

/D[+]f - destination - default /D1

The D option specifies the output printer port, device, or file. The

+ suboption causes files to be appended rather  than overwritten (it

has no effect on device output). 1, 2, and 3 select the BIOS printer

routines.  X1, X2, X3, and X4 select xon/xoff controlled serial port

drive.  H1,  H2, H3, and H4 select hardware controlled  serial  port

drive. You can also use the DOS devices LPT1, COM1, etc.

PrintGL cannot set up the serial ports.  This is usually done in the

AUTOEXEC.BAT with MODE (probably MODE COM1:9600,N,8,1,P).

/D is ignored for display output.

/D2          sends output to the second parallel port via BIOS

/DLPT3       sends output to the DOS device LPT3

/D+TEMP.PRN  appends output to file TEMP.PRN

p-11

/F - Output Format Option --------------------------------------------

/Fc[-|+[b]|*][!][~][x,y] - output format - default /FN~

The F option specifies the output format. -, +, and * are resolution

modifiers,  !  selects a compression mode  driver, and  ~ turns  off

color processing. -, +, *, !, and ~ are allowed even where they have

no effect.  The display mode (b) lets  you choose a display mode for

the nonstandard EVGA  800x600  mode.  This  must  follow  the format

character and it may be entered in hex by prefixing it with $.

You may  enter the horizontal and vertical  resolution of the output

device in dots/inch.  This overrides  PrintGL's normal output device

resolution, but does not affect  the structure of the output  format

(-,+, and * generally do alter the output structure).  These numbers

must follow the format character and the display mode number.

In addition to letting you choose any resolution for a bit map, this

feature lets you compensate  for  print size error.  For  example, a

360x360 dpi printer consistently under indexes so that a 6 inch high

box  comes  out  5.94 inches.  Using  360,363.6 (6/5.94*360 = 363.6)

removes the error.

Drivers marked below with ! have a compression mode driver, selected

with "!".  This may be  very useful, but it depends on your printer.

These  drivers compress the  output data by breaking  bit map strips

with printhead positioning commands.  This  saves data  transmission

time, but some  printers spend more  time positioning the  printhead

than is saved by not sending the data.

A  number of  printers use  compression  mode by default.  It can be

turned off with � (ASCII 173).

Drivers marked below with ~  are  color devices.  Using ~ suppresses

color processing, substituting black for all colors except white.

/F1     ~ IBM 9 pin ....................................... 120x72

/F1+    ~ IBM 9 pin (2 pass)                                120x144

/F1*    ~ IBM 9 pin (4 pass) .............................. 240x144

/F2       IBM Quietwriter 2                                 240x240

/F3       IBM Quietwriter 3 ............................... 240x240

/F4-      IBM LaserPrinter (PPDS mode)                      150x150

/F4       IBM LaserPrinter (PPDS mode) .................... 300x300

/F5    !~ IBM 24 pin alternate 1                            180x180

/F5+   !~ IBM 24 pin alternate 1 (2 pass) ................. 360x180

/F5*   !~ IBM 24 pin alternate 1 (4 pass)                   360x360

/F6    !  IBM Proprinter X24 .............................. 180x182

/F6+   !  IBM Proprinter X24 (2 pass)                       360x182

/F6*   !  IBM Proprinter X24 (4 pass) ..................... 360x364

/F7-      Canon BJ (Proprinter mode), IBM ExecJet           180x180

/F7       Canon BJ (Proprinter mode), IBM ExecJet ......... 360x360

/F8       HP-GL                                            1016x1016

/F9    !~ NEC 24 pin ...................................... 180x180

/F9+   !~ NEC 24 pin (2 pass)                               360x180

/F9*   !~ NEC 24 pin (4 pass) ............................. 360x360

/FA-    ~ HP DeskJet 500C, Mannesmann Tally MT92C           150x150

/FA     ~ HP DeskJet 500C, Mannesmann Tally MT92C ......... 300x300

p-12

/FB-    ~ Canon BJC 800 (Epson LQ mode) ................... 180x180

/FB     ~ Canon BJC 800 (Epson LQ mode)                     360x360

/FC       CGA display ...................................... 62x25

/FD-      Canon LBP (ISO mode)                              150x150

/FD       Canon LBP (ISO mode) ............................ 300x300

/FE     ~ 128K EGA color display                             62x45

/FF    !~ Fujitsu 24 pin .................................. 180x180

/FF+   !~ Fujitsu 24 pin (2 pass)                           360x180

/FF*   !~ Fujitsu 24 pin (4 pass) ......................... 360x360

/FG     ~ GEM .IMG bit map                                  100x100

/FGi,j  ~ GEM .IMG bit map .................................. ixj

/FH       Hercules graphics card display                     70x45

/FI-      HP LaserJet 2P/3, DeskJet ....................... 150x150

/FI       HP LaserJet 2P/3, DeskJet                         300x300

/FJ-      Canon BJ (BJ130 mode) ........................... 180x180

/FJ       Canon BJ (BJ130 mode)                             360x360

/FK     ~ Kodak Diconix Color 4 ........................... 192x192

/FL-      HP LaserJet                                       150x150

/FL       HP LaserJet ..................................... 300x300

/FM       128K EGA monochrome display                        62x45

/FN     ~ Epson 9 pin ..................................... 120x72

/FN+    ~ Epson 9 pin (3 pass)                              120x216

/FN*    ~ Epson 9 pin (6 pass) ............................ 240x216

/FO    !~ Toshiba 24 pin                                    180x180

/FO+   !~ Toshiba 24 pin (2 pass) ......................... 360x180

/FO*   !~ Toshiba 24 pin (4 pass)                           360x360

/FP    !~ HP PaintJet ..................................... 180x180

/FQ       HP QuietJet (PCL mode)                            192x192

/FR     ~ Tektronix ColorQuick ............................ 216x216

/FS     ~ PostScript                                       1016x1016

/FT    !~ Epson 24 pin .................................... 180x180

/FT+   !~ Epson 24 pin (2 pass)                             360x180

/FT*   !~ Epson 24 pin (4 pass) ........................... 360x360

/FV     ~ VGA display                                        62x62

/FV+    ~ VESA 800x600 VGA display ......................... 78x78

/FV+b   ~ 800x600 VGA display - mode b                       78x78

/FW     ~ Epson 9 pin alternate ........................... 120x72

/FW+    ~ Epson 9 pin alternate (3 pass)                    120x216

/FW*    ~ Epson 9 pin alternate (6 pass) .................. 240x216

/FX    !~ IBM 24 pin alternate 2                            180x180

/FX+   !~ IBM 24 pin alternate 2 (2 pass) ................. 360x180

/FX*   !~ IBM 24 pin alternate 2 (4 pass)                   360x360

/FY       append Encapsulated PostScript preview bit map ... 72x72

/FYi,j    append Encapsulated PostScript preview bit map      ixj

/FZ     ~ ZSoft .PCX ...................................... 100x100

/FZi,j  ~ ZSoft .PCX                                          ixj

Most 9 pin  printers, including the IBM Graphics Printer,  work best

with  the /FN  drivers, even  if  they claim Proprinter rather  than

Epson compatibility.  9 pin printers with 1/144  inch indexing, such

as the IBM  Proprinter and 2380,  work better with the /F1  drivers.

And some not fully compatible 9 pin printers only work with /FW, the

Epson 9 pin alternate drivers.

Most   Proprinter  X24   compatible  printers,  including   the  IBM

Quickwriter and 2390, work best with /F5 or /FX. /F6 is only for the

IBM Proprinter X24, XL24, X24E, XL24E, and 24P.

p-13

/I - Scaling Point Location Option -----------------------------------

/IA|R|W[N] - IP point location - default /IA

The I option sets the initial scaling points  (P1, P2 - the  scaling

points may  be set  in the  HP-GL  file  with  the  IP  command).  A

(absolute) sets  the  points to the default for the paper  size (see

below).  R fits P1 and P2 to the print window with at least .25 inch

margins  and an x/y ratio of 10/7.2 (the  same ratio as  the HP 7475

with  small paper).  W fits P1 and P2 to the print window with a .25

inch margin. N makes the margin 0 for R or W and is ignored with A.

The  default scaling points depend on the print window  (/L  option)

with the implied paper size and the plot orientation (/O option).

paper    options            default scaling pts  with RO90 command

7440 A   /O1 /L 7.54,10.14  250,279 10250, 7479  279,250  7479,10250

7440 A4  /O1 /L 7.54,10.74  250,279 10250, 7479  279,250  7479,10250

7475 A4  /O1 /L 7.84,10.20  250,596 10250, 7796  154,244  7354,10244

7475 A4  /O1 /L 7.60,10.88  603,521 10603, 7721    0,610  7200,10610

7475 B   /O1 /L10.20,16.38  522,259 15722,10259  283,934 10283,16134

7475 A3  /O1 /L10.88,15.90  170,602 15370,10602  607,797 10607,15997

7440 A   /O2 /L10.14, 7.54  250,279 10250, 7479  279,250  7479,10250

7440 A4  /O2 /L10.74, 7.54  250,279 10250, 7479  279,250  7479,10250

7475 A   /O2 /L10.20, 7.84  250,596 10250, 7796  154,244  7354,10244

7475 A4  /O2 /L10.88, 7.60  603,521 10603, 7721    0,610  7200,10610

7475 B   /O2 /L16.38,10.20  522,259 15722,10259  283,934 10283,16134

7475 A3  /O2 /L15.90,10.88  170,602 15370,10602  607,797 10607,15997

anything else               250,279 10250, 7479  279,250  7479,10250

/O3 and /O4 gives the same default scaling points as /O1 and /O2.

/IA   uses the default scaling points

/IWN  sets the scaling points to fit the print window

/J, /K - Printer Code Prefix, Suffix Options -------------------------

/J[b,b,..] - printer code prefix - default /J

/K[b,b,..] - printer code suffix - default /K

/J lets you send codes to the printer before PrintGL's usual data is

sent and /K lets you  send codes after PrintGL's usual data is sent.

This lets  you  set up and reset the printer for special situations.

These are not needed in the normal use of PrintGL.

For  plotter  and  PostScript  output,  the   /J  codes  follow  the

initialization commands.

The  codes  are  specified  with  byte  values  (0..255  or $0..$FF)

separated with blanks or commas.  There is  a limit of 64 characters

for any option, so very long  sequences must  be handled outside  of

PrintGL.

/J27 79   sets skip perforation off  to allow plots  larger than one

page on most continuous form printers

/K10 10   sends two line feeds after the plot

p-14

/L - Page Layout Option ----------------------------------------------

/L[B][F][O][R][x,y[,x,y]] - page layout - default depends on printer

The L option defines the page layout.

B boxes the plot at the print window edge using pen 8.

F  causes a form feed after the print.  This is ignored for bit maps

and display output.

O sets pen overwrite mode.  By default, pen colors are ored into the

print. With the O suboption, they are overwritten into the print, so

the lines are opaque. For PostScript devices and plotters, the color

processing is done by the  device,  not PrintGL.  PostScript devices

act like pen overwrite mode is on and plotters act like it is off.

R reverses the colors on displays,  bit maps,  and output formats 4,

D,  and  L, (Canon,  HP, and  IBM laser  printers) and  is otherwise

ignored. This gives the equivalent of a photographic negative.

The first optional x and y are the print window width and height  in

inches.  These  define the hard clip limits.  These  values are  not

checked against the printer's capabilities.  If you specify  a print

window  that  exceeds the  output  device  limits,  the  results are

unknown.  You may need to reset the printer's margins or page length

for large plots (see Large Paper). The size needed for a given width

and height exceeds x and y by 8 pels to allow for line widths.

The  second  optional x and y are the left and top margins, measured

from the printer's left margin and  current vertical position to the

print window.  The  top margin is  modified  to match the  printer's

native indexing, but is accurate to .02 inch.  The margins for HP-GL

and PostScript are measured from the  device's  0,0 (left,  bottom).

Margins are ignored for display output and bit maps.

THE PRINT WINDOW PLUS THE MARGINS MUST FIT ON THE PAPER.

Each printer has a default page layout.  The default print window is

7.54x10.14 for printers  and bit maps  and  10.14x7.54 for HP-GL and

displays. These correspond to HP 7440 A size paper.  The margins are

printer specific to  compensate  for different  paper  handling.  In

evaluating user  options,  PrintGL processes  each  option and  when

finished reprocesses the  default page layout for the chosen printer

and then reprocesses the last  user /L option.  This procedure gives

good default values  so that your /L option can change the letter or

size suboptions and still have good margins.

/LB7.5,3,.5,0   makes the  print window 7.5 inches wide and 3 inches

high,  with a .5 inch left margin and no top margin,

boxes the window, and does not do a form feed

/LF7.54,10.14   sets window equivalent to HP 7440 with A  paper

/LF7.54,10.74   sets window equivalent to HP 7440 with A4 paper

/LF7.84,10.20   sets window equivalent to HP 7475 with A  paper

/LF7.60,10.88   sets window equivalent to HP 7475 with A4 paper

/LF10.20,16.38  sets window equivalent to HP 7475 with B  paper

/LF10.88,15.90  sets window equivalent to HP 7475 with A3 paper

p-15

/M - Magnification Option --------------------------------------------

/M[F]x - magnification - default /MF.9

The M option specifies the magnification.  F fits the  plot  to  the

print window size.  The default is /MF.9 which makes the plot 90% of

the largest size that will  fit in the print  window.  /M1 gives the

same size plot as an HP 7475 plotter.

/MF.9  fits the plot to the print window and shrinks it 10%

/M1    plots at the same size as an HP 7475

/M.5   plots at half the size of an HP 7475

/N - Page Number Option ----------------------------------------------

/N[i] - page number - default /N

/Ni says to print only page i of the plotfile. If you do not specify

i then all pages in the plot will be printed (this is the default).

/N   prints all pages in the plotfile

/N3  prints only page 3

/O - Origin and Orientation Option -----------------------------------

/O[1|2|3|4[L]] - origin, orientation - default /O

The O  option sets  the  print window  origin and  orientation.  The

origin is one of four corners  numbered  1..4 for upper  left, lower

left, lower right, and upper right, and implies a plot rotation. The

L suboption specifies a left handed coordinate system, which mirrors

the plot.

If no origin is specified then printers  use  /O1  and  displays and

plotters  use /O2.  This  best matches  the  plotter's  width/height

ratio.

Here are pictures of PrintGL's view of  the output media compared to

an HP 7475's view, to help with choosing a /O option.

HP 7475                                 PrintGL

display or

A size           B size                 printer        plotter

y��������Ŀ    0,0��������Ŀy         ul�����Ŀur   ul��������Ŀur

�        �       �        �           1�     �4     1�        �4

�        �       �        �            �     �       �        �

0,0����������x      �        �            �     �     ll����������lr

�        �            �     �      2          3

�        �          ll�������lr

�        �           2       3

x����������

/O1  puts the print window's origin in the upper left corner

/O2  puts the print window's origin in the lower left corner

p-16

/P - Plotfile Option -------------------------------------------------

/Pf - plotfile - no default

The plotfile may be specified as the first parameter with no prefix,

or  it may be specified anywhere in the option list with /P.  If you

use a  file name  mask with "*"  and/or  "?",  each matching file is

processed, but the first failure or user break stops all processing.

/R - Internal Resolution Option --------------------------------------

/Ri - internal resolution - default /R1016

/R sets the internal grid resolution  in dots/inch.  It is  normally

1016 and rarely needs  to be changed.  To fit very large plots  into

the   -32768..32767  coordinate  limit,  use   /R508.   Setting  the

resolution to a multiple  of the output device  resolution times the

magnification may yield a small improvement in print quality.

/S - Pen Shading Option ----------------------------------------------

/Sc.. - pen shading patterns - default /S0

/S  assigns a shading pattern to each pen.  Unspecified pens use the

last specified shade.  The shade values are 0..8.  0  is the default

solid line.  1..4 are 1 dot patterns (75%, 50%, 25%, 12.5%) and need

line widths of at least three to give good looking  lines.  5..8 are

2x2 dot patterns  (75%, 50%, 25%,  12.5%) and need line widths of at

least  five to give good looking  lines.  Some printers may wash out

shading because they have  a dot size  that is large relative to the

dot spacing.  This is often the  case  with  dot  matrix printers in

multipass modes.

1 ��������         2 ��������          3 � � � �          4 �   �

��������           ��������            � � � �              �   �

��������           ��������            � � � �            �   �

��������           ��������            � � � �              �   �

5 ��������         6 ��  ��            7 ��  ��           8 ��

��  ��               ��  ��

��������           ��  ��              ��  ��                 ��

��  ��               ��  ��

/S023  sets pen 1    to solid

pen 2    to 1 pel checkerboard

pen 3..8 to 1 pel dots

/T - Temporary File Option -------------------------------------------

/T[+]f - temporary file - default /TPRINTGL.TMP

PrintGL uses a temporary file when it does not have enough memory to

handle a plotfile.  You can use /T to put it on a RAM disk for extra

speed.  If  you  have 256K of free memory, it is likely that PrintGL

will  never  use  a  temporary file.  You  can  force  the use of  a

temporary file, if  the plot has  over 1024 data points, with +.  If

you use a RAM disk this may speed printing.

p-17

/W - Pen Width Option ------------------------------------------------

/Wc.. - pen widths - default /WA

The W option assigns pen line widths. Each width may be 0..9, .10 ..

.16 pels or A..I for 2..10 units of 4/1016 inch (.1 mm).  A..I gives

a  device  independent width, but it is an approximation, limited by

the device technology. Unassigned pens use the last assigned width.

For nonsquare matrix printers, PrintGL uses pen points optimized for

an h/v ratio of .67, 1, or 1.5.  Widths are figured in the direction

of higher resolution, and are not accurate in the other direction.

Line  widths  of over eight pels are slightly distorted at the print

window edge to fit within the four pel border.

/WA      sets pen 1..8 to .2 mm

/WB2.16  sets pen 1 to .3 mm, pen 2 to 2 pels, pens 3..8 to 16 pels

/X - User Interface Options ------------------------------------------

/X[C][P][S][F][U][H] - interface options - default /X

The X option sets various interface options. C causes a break when a

plot is clipped. P causes a pause before each printed page.

S suppresses  location and  size messages.  F  suppresses  location,

size, file name, and page number messages.  U suppresses unsupported

command and incorrect number messages. H suppresses the help text.

/XS  gives only the most important messages

/Y - Y/D Switch Setting Option ---------------------------------------

/YD|Y - Y/D switch setting - default /YD

The Y option sets the plotter's Y/D switch to D or Y.  Y starts with

the  plotter  off and responds to  esc.(,  esc.Y,  esc.), and  esc.Z

commands. D starts with the plotter on and ignores these commands.

/YD  puts the YD switch in the usual D position

/Z - Chord Angle and Enhanced Font Option ----------------------------

/Z[E]i,j - default, min chord angle, enhanced font - default /Z10,5

The Z option specifies  the default  and  minimum chord  angles  for

circles and arcs. The HP 7475 defaults to a chord angle of 5 with no

minimum.  Using chord  angles  of less than  10 returns little print

quality improvement  and  adds to  processing time.  The  default is

/Z10,5. For better emulation use /Z5,2.

The E suboption says to  use  the enhanced  font, which doubles  the

number of chords on curves in text,  improving the curve smoothness.

This is only effective on character heights above .5 cm.

/Z5,2  sets the default chord angle to 5 degrees, the minimum to 2

p-18

PrintGL Menu Interface (PMI) -----------------------------------------

PrintGL Menu Interface  (PMI) is a menu driven program that lets you

select PrintGL options, choose a list of plotfiles, and run PrintGL.

To run  PrintGL Menu  Interface, enter  PMI at the  DOS prompt,  and

press Enter. There are no command line options. The PMI main menu is

displayed at the top of the screen, and the bottom  of the screen is

used  differently for each main menu  option.  The  PMI main menu is

divided by column into six groups. Each group is described below.

When the cursor is on a  main menu  item, the current  value  of the

option is displayed below the main menu.  To change the value, press

Enter (or left mouse  button).  The options are modified with cursor

movement and typing numbers/names.  The Enter key returns you to the

main menu with the new  option values.  Esc (or  right mouse button)

returns you to the main menu with the option values unchanged.

If a main menu  item includes a capital  letter, that  item  may  be

accessed immediately by pressing the letter.

When a cursor bar has a notch, the value in the notch may be edited.

PMI Menu Functions ---------------------------------------------------

The first column of main menu options is dedicated to PMI functions.

These include  running  PrintGL,  switching  to alternate  settings,

exiting PMI,  saving  the current PMI settings, setting PMI options,

and creating a BAT file corresponding to the current PMI settings.

The "alt settings"  item rotates  between three  PMI configurations.

The current configuration name is on line 2.

The "save settings" item saves the current PMI  configurations.  All

of the current  PrintGL and PMI options  for  each configuration are

saved.  The plotfile list is not saved.  PMI saves its configuration

by writing to  PMI.EXE.  To  find PMI.EXE  it  looks in  the current

directory and  then  searches  the  DOS PATH, saving  to  the  first

PMI.EXE that it finds.

The  "menu options" item lets you  set the configuration name, mouse

sensitivity,  screen colors, up  to five initial plotfile masks, and

the name of the BAT file created  by the "creat bat" option.  Screen

colors are specified with the standard IBM BIOS attributes.

For color displays, the left hex  digit is the  background color and

the  right hex digit is the foreground color.  Bright colors are not

available in the background.

0 black       4 red           8 gray (bright)     C bright red

1 blue        5 magenta       9 bright blue       D bright magenta

2 green       6 brown         A bright green      E bright yellow

3 cyan        7 white         B bright cyan       F bright white

"Create bat" creates a  BAT  file  that  can  be  run  with "batfile

hpglfile /opt1 /opt2 ..".  The BAT file contains the options defined

by the current PMI settings and accepts additional options.

p-19

PMI Input and Output Choices -----------------------------------------

Column  two of the main menu  lets  you choose the plotfiles to plot

(the /P option  in  PrintGL),  the plot page number (/N), the output

destination (/D),  the output format  (/F), and modifications to the

output - compression mode, device resolution override and prefix and

suffix printer codes (/J,/K).

You may enter up to 99 plotfiles in the "plotfile" main menu option.

To  choose from a  list of files, enter a  mask name and press Enter

(the cursor must  be on the  mask).  PMI will then display a list of

corresponding  files.  Select  any number of the files by moving the

cursor  to the desired file and pressing Space  (or the middle mouse

button). If no files have been selected, then pressing Enter selects

the file under the cursor.  The selected files are added to the file

list at  or below  the  cursor.  Files that would overflow the  99th

position on the plotfile list are dropped.

When you run PrintGL, PMI runs PrintGL for each file on the plotfile

list.  Masks are ignored.  For any  plotfiles that PrintGL does  not

successfully process, PMI inserts an arrow in front of the  plotfile

name.  You  can view the results from the "plotfile" main menu item.

The arrow is not considered  part of the file name by PMI, so you do

not need to remove it to retry PrintGL.

Printers that  support  color  have  a  color  option ("clr:" on the

menu).  This lets you specify a  black/white printer even though you

may have set  up colors with the  "pen color"  menu.  Output formats

that do not support color do not have this option and always convert

the selected colors to black and white.

Bit map formats let you select the resolution in the "output format"

list, but it  will be overridden with  the "modify output" "override

h,v dpi" option.

PMI Plot Options -----------------------------------------------------

The third  column of  main  menu options  define  how the  plot will

appear on the  page.  The options include magnification (/M), origin

(/O), position (/A), and initial scaling points (/I).

Left handed (mirrored) orientations are not available from PMI.

PMI Pen Options ------------------------------------------------------

The  fourth  column   of   main  menu   options   covers   the   pen

characteristics: color  (/C),  shade (/S), and  width (/W).  Each of

these menus use the up and down cursor keys to select a pen, and the

left and right cursor keys to choose an attribute.

For  color  selection,  the  one character  colors (b,c,g,k,m,r,w,y)

represent  pure  colors,  and  the  two character  colors are  shade

pattern mixes.  The  shade pattern  mixed  colors  use  the  current

shading  option  for  that  pen,  unless the  shade  pattern is 100%

(solid), in which case a one pel checkerboard is used.

p-20

PMI Page Layout Options ----------------------------------------------

The  fifth column  of main menu options covers the page layout.  The

first four items are suboptions of  the /L  option - box, form feed,

print window, and left and top margins. Tiling options (/B) are also

here.

Reverse image is not available from PMI.

PMI Other Options ----------------------------------------------------

Options that  are generally set  once and then never  touched are in

column six.  These include the temporary file  name (/T), and  under

"other",  the  default  and  minimum chord  angle  (/Z), plotter D/Y

switch setting (/Y), internal resolution  (/R), pen  overwrite  mode

(opaque lines - /LO), and the pause to load paper option (/X).

Large Paper ----------------------------------------------------------

PrintGL  can handle printing on paper larger than  the usual 8.5x11,

but this takes some extra work. First you need to increase PrintGL's

print  window to  cover the large paper.  This  is done with  the /L

option or "print window" in PMI.  The  print window plus the margins

(/L or "print margins") must fit  on the paper.

And then you must make  sure that your printer recognizes the larger

paper height.  For  continuous  forms paper  this  is  best  done by

setting skip perforation to off,  either by  printer switch or  with

PrintGL prefix codes.  For Canon, Epson, Fujitsu, IBM, and NEC  line

printers (dot  matrix,  ink jet, thermal) use  /J27  79,  or "modify

output" "prefix codes" 27 79.

And for  some  printers you need to increase the right margin past 8

inches. For the Canon BJC this is done with /J27 81 114.

For cut sheet paper, on Canon,  Epson, Fujitsu,  IBM,  and  NEC line

printers use /J27 67 0 n, where n is the page length in inches.

For  page printers,  legal  size paper  may be specified with  these

printer  code  prefixes: HP-PCL printers /J27 38 108 51  65 (/K27 38

108 50 65 to reset  to 11 inch paper), Canon LBP printers /J27 91 51

50 59 59 112 (/K27  91 51 48 59 59 112  to  reset), IBM LaserPrinter

/J27 91 70 5 0 3 1 2 1 (/J27 91 70 5 0 3 1 1 1 to reset).

To get  a 10x16  print  window  on  11x17  paper in a  Canon, Epson,

Fujitsu, IBM, or NEC line  printer, use  these options:  /LF10 16 .4

.25 /J27  67 0 17.  You will probably need to adjust the margins (.4

.25) for best centering.

The  maximum  width of the  print  window  on  most narrow  carriage

printers is 8  - 8/(horizontal dpi).  For  300 dpi printers, this is

7.97. At 120 dpi printers it is 7.93. The maximum width of the print

window on most wide carriage printers is 13.6 - 8/(horizontal  dpi).

At 180 dpi this  is 13.55.  At 360 dpi  it is 13.57.  The horizontal

print margin should be set to 0 to get this width.

p-21

Tips On AutoCAD ------------------------------------------------------

There are many ways to set up AutoCAD for use with PrintGL.  Here is

a good way to start.

Configure  AutoCAD for an HP 7550 plotter.  Select output to a file.

Do not do any plotter calibration. Set a specific scale, do not fit.

Do not rotate.  Set  a plotter  window that is larger than  you will

ever  need,  30x30  for  example  (this  will  prevent  AutoCAD from

clipping the  plot).  Set  up your plotter  pen table so  that  each

screen color is assigned a different pen.

With  PrintGL  you can  rotate  (/O  or  "origin"),  magnify  (/M or

"magnification"), assign pens (/C,/S,/W or "pen color", "pen shade",

"pen  width"), and calibrate (/F  or  "modify output" "override  h,v

dpi").  Use /A or "position" "plot center at print window center" to

automatically center your plot in the print window.

Useful Prefix and Suffix Codes ---------------------------------------

HP-PCL printers

reset                  27 69

portrait mode          27 38 108 48 79

top margin 0           27 38 108 48 69

11 inch paper          27 38 108 50 65

14 inch paper          27 38 108 51 65

Canon, Epson, Fujitsu, IBM, NEC dot matrix and inkjet printers

11 paper               27 67 0 11

14 inch paper          27 67 0 14

17 inch paper          27 67 0 17

22 inch paper          27 67 0 22

skip perforation off   27 79

Canon LBP

cursor to top of page  27 91 48 100

11 inch paper          27 91 51 48 59 59 112

14 inch paper          27 91 51 50 59 59 112

full page memory mode  27 59 27 91 50 38 122

Diablo mode            27 58

IBM LaserPrinter

11 inch paper, tray 1  27 91 70 5 0 3 1 1 1

14 inch paper, tray 1  27 91 70 5 0 3 1 2 1

14 inch paper, manual  27 91 70 5 0 1 1 2 1

HP QuietJet

HP-PCL mode            27 37 65

DIP switch mode        27 37 64

p-22

Display Output -------------------------------------------------------

When  you display a plot, the cursor keys scroll the plot, S  cycles

between  full,  half   (the  default),  quarter,   and  eighth  page

scrolling, and Esc returns to DOS.

/FC drives a CGA, but does not give color.

The EGA drivers, /FE and /FM, require a 128K or 256K EGA.  /FC  will

work with  a 64K EGA.  /FM  is for an EGA card  with a digital (TTL)

monochrome monitor.

/FH supports the Hercules monochrome graphics card.

/FV works with any VGA card.  /FV+ requires a system with 800x600 16

color support.  By  default /FV+ assumes VESA support.  If  your VGA

card does not support the VESA standard then specify the mode number

that selects 800x600 16 color mode. Here are /FV+ options for common

VGA cards ($ means hex).

ATI    /FV+$54      Paradise     /FV+$58      Trident      /FV+$5B

Genoa  /FV+$29      Renaissance  /FV+$64      Video Seven  /FV+$16

For MCGAs, use the VGA driver without color - /FV~.

Display  output cannot be  redirected  to  a  file (the /D option is

ignored).

p-23

HP-GL Output ---------------------------------------------------------

/F8 outputs  low level HP-GL, converting all characters, arcs, fill,

and clips to vectors.  The output is HP 7440 compatible,  containing

only  IN, SP, IW, LT, PU, and PD commands.  PG is added if form feed

is  specified  (the  default).  If  you  decline the form  feed then

multiple plots may be appended to the same page.

PrintGL uses its width parameter (/W) as a pen selector, so for each

pen in the  original HP-GL, you may specify a  new pen in the output

HP-GL.  The default /W  option is /W12345678 (for /F8).  Shading and

color are ignored.

PostScript Output ----------------------------------------------------

/FS outputs printable Encapsulated  PostScript without  preview  bit

map.  Characters, arcs, fill, and clips are converted to vectors, so

PrintGL is not a general purpose HP-GL to PostScript converter.

PrintGL's  pen color  and shading parameters  are mapped  into  gray

levels  or  RGB  colors.  PostScript  overwrites  as  it  draws,  so

overlapping lines appear as opaque colors.

If  no  form feed is  specified  then  the showpage  command is  not

included the PostScript, and more PostScript may be appended  to the

same page.

/FY appends a TIFF preview bit map to a PostScript file.  To use it,

create  a  PostScript file with  PrintGL and then run PrintGL again,

specifying  the  same  options  except  /FY  instead  of  /FS.   The

destination  file is appended  by default.  /FY assumes a PostScript

file, generated by  PrintGL,  is  being  appended.  You may set  the

resolution - the default is 72x72 dpi. Here is an example:

PRINTGL sample.plt /Dsample.eps/LB6,6/O2/FS

PRINTGL sample.plt /Dsample.eps/LB6,6/O2/FY

p-24

GEM .IMG and ZSoft .PCX Bit Map Output -------------------------------

Multiple pages plots, and tiled plots do not work well with  bit map

output  because the bit map formats have no facilities for  multiple

pages in one file.

For  both bit map formats, 1 is the background and  0 the foreground

for b/w  files.  1,1,1 is  the background  for color  files, with  0

representing  the absence of blue, green, or red.  The bit usage may

be reversed with /LR.

The bit map is  the size specified by the /L option  plus eight pels

in each direction.  The  width is extended to a  multiple of sixteen

pels.

/FG creates GEM .IMG format files.  The header is composed of 8 word

values, high byte first:

1

8

number of color planes - 1 or 3

repeated data pattern length - 1

pel width in microns - round(25400 / horizontal dots/inch)

pel height in microns - round(25400 / vertical dots/inch)

scan line width in pels

number of scan lines

The scan lines follow, in top to bottom order. For color plots, each

each complete  scan line  consists  of a blue,  green,  and red scan

line. Data for the scan lines are composed of these commands:

$00 n b           - n bytes of repeated bit mapped data

$80 n b1 b2 .. bn - n bytes of bit mapped data

/FZ gives .PCX output with  the  following format.  The  file starts

with a header composed of 128 byte values:

10,2,1,1,0,0,0,0,

max_x,max_y,                              (2 bytes each, in pels )

dpi_x,dpi_y,                              (2 bytes each          )

0,0,0,    0,0,191,  0,191,0,  0,191,191,  (color palette         )

191,0,0,191,0,191,191,127,0,191,191,191,

0,0,0,    0,0,255,  0,255,0,  0,255,255,

255,0,0,255,0,255,255,255,0,255,255,255,

0,

color_planes,                             (1 or 3                )

width                                     (2 bytes, in bytes     )

1,0,..,0

The scan  lines, top  to  bottom,  follow.  For  color  plots,  each

complete scan  line consists of a blue,  green, and red  scan  line.

Each scan line is composed of these data commands:

$00..$BF    - 1 byte of bit mapped data

$C1 b       - 1 byte of bit mapped data

$C2..$FF b  - 2..63 bytes of repeated bit mapped data

p-25

Printer Output -------------------------------------------------------

Canon BJ and BJC Printers, IBM ExecJet

For  the BJ130e,  BJ10e, BJ10ex,  BJ300,  BJ330,  and  ExecJet  in

Proprinter mode, use /F7.  This is the fastest way to  drive these

printers at 360 dpi.

/FJ gives 360 dpi graphics for any Canon BJ printer in BJ130 mode.

The printer is set to 10 pitch text when finished.

You  can use /FT* with the BJ or ExecJet printers in Epson LQ mode

to get 360 dpi print, but it is very slow.

These printers  have an image  density option (switch  1-9 on  the

BJ130,  1-4  on  the  BJ300,  BJ330,  and  ExecJet).  Low  density

eliminates  every second pel.  To get the full 360 dpi resolution,

use high density.  For the BJ300, BJ330, and ExecJet, also be sure

that the front panel FONT HQ (high quality) light is on.

For Canon BJC printers, use /FF in Epson LQ mode. This driver uses

a graphics mode that is not supported by the BJ printers.

Canon LBP Printers

/FD  works  with any Canon LBP printer in ISO mode.  At 300 dpi it

takes up to one  meg  of  printer memory to print a full  page  of

graphics. At 150 dpi (/FD-) 512K handles a full page.

It  may help to set the printer memory mode with /J27 59 27 91  50

38 122.

If the printer is in Diablo mode, PrintGL switches to ISO mode. To

switch back to Diablo mode on completion, use /K27 58.

Epson, IBM 9 Pin Printers

/FN, /FW, and /F1 cover a wide range of Epson/IBM compatible 9 pin

dot matrix printers.  You need a cyan/magenta/yellow ribbon to get

color prints. The * drivers use 1/240 graphics.  /FN+, /FN*, /FW+,

and /FW*  use  1/216 indexing and do three  vertically interleaved

passes. /F1+ and  /F1*  use 1/144  indexing  and do two vertically

interleaved passes. /FW resets the line feed distance to 1/6 inch.

For  Epson printers, the IBM  Graphics Printer, and other printers

with 1/216 indexing hardware, /FN is preferred. For some not quite

compatible  printers,  /FW works.  For IBM  Proprinters and  other

printers  with 1/144 indexing hardware, /F1 gives better  results.

Some Proprinters do not handle 1/240 graphics correctly.

These drivers use the following escape codes:

esc 3  set n/216 line feeds for all /FW

esc J  index n/216 (n=2 for 1/144 index) for all /FN and /F1

esc L  graphics command for /FN, /FN+, /FW, /FW+, /F1, and /F1+

esc Z  graphics command for /FN*, /FW*, and /F1*

esc r  set ribbon color (only if color is specified)

p-26

Epson, Fujitsu, NEC, Toshiba 24 Pin Printers

/FT  covers  all  Epson LQ printers and  many  compatible printers

(Panasonic in LQ mode and others).  /FF works with  Fujitsu 24 pin

printers in Fujitsu mode.  /F9 works with NEC 24 pin printers. /FO

works  with Toshiba 24  pin printers.  /FO  resets the  line  feed

distance to 1/6 inch on completion.

For all of these drivers, you need a cyan/magenta/yellow ribbon to

get color prints. The + and * drivers use 1/360 graphics mode, and

the  * drivers  use 1/360 (1/120  for /FO*) indexing and reset the

line feed distance to 1/6 inch on completion.  A clean paper  path

with  equal  tension  on  each side  is  needed  for  good 360x360

graphics.

Many 24 pin printers are not capable of 360x360 graphics and a few

cannot handle 360x180 graphics.

The compression mode drivers (!  suboption)  may or  may not be an

improvement with these printers.

The /FT drivers use the following printer escape codes:

esc $    absolute tab

esc * '  graphics command for /FT

esc * (  graphics command for /FT+, /FT*

esc +    set n/360 indexing for /FT*

esc 2    set 1/6 indexing for /FT*

esc J    index n/180 for /FT, /FT+

esc r    set ribbon color (only if color is specified)

/FT does  not work with IBM Proprinter  X24s in alternate graphics

mode because it uses the esc $ command.

HP DeskJet, Mannesmann Tally MT92C

/FI works with any black/white DeskJet in portrait mode.

/FA  drives  the  DeskJet  500C and  MT92C,  and any color 300 dpi

HP-PCL printer that supports data transfer mode 2.

HP LaserJet

/FL works with any LaserJet  compatible printer in portrait  mode.

Portrait mode  may be set with /J 27 38 108 48 79 or you can reset

the printer with /J 27 69.

/FI  works with the LaserJet 2P and 3, and any LaserJet compatible

printer that supports data transfer mode 2.

LaserJets with under one meg of memory may not be able to handle a

full page of graphics.  /FL uses less printer  memory than /FI, so

it may be preferred on LaserJet 2P and 3s with limited memory.

p-27

HP PaintJet

/FP drives PaintJet  printers.  PaintJet  printers  are  sold with

either a serial or parallel interface.  The  parallel interface is

faster.

The  compression mode  driver (/FP!)  uses  data  transfer  mode 2

(instead of 1) and works only with a PaintJet XL.

HP QuietJet

/FQ works with QuietJets  in HP-PCL mode.  /J27  37 65  /K27 37 64

sets HP-PCL  mode and  resets  to  the DIP switch selected mode on

completion.

IBM LaserPrinter

/F4 drives any LaserPrinter in PPDS (native)  mode.  /FL will work

if the LaserPrinter is in HP LaserJet mode, but this may be slower

than  PPDS mode.  At  300 dpi it takes up to  one meg  of  printer

memory  to print a full page of graphics.  At  150 dpi (/F4-) 512K

handles a full page.

IBM 24 Pin Printers

PrintGL has three drivers that  cover  Proprinter  X24  compatible

printers. The printers should be in native mode (not AGM).

The first driver, /F6, is for all IBM Proprinter X24s and the 24P.

These printers do  1/144 inch indexing, and using them at 180  dpi

results  in horizontal  white streaks  every  2/3  inch.  The  /F6

drivers  compensate  for  this by indexing  19/144 inch  per line,

giving  182 dpi.  To work properly, the printer must be on a  1/72

boundary when the graphics are printed.  If you keep the line feed

distance  a multiple of 1/72 (1/6, 1/8, 1/9) then this will always

be the case.

The second X24 compatible driver is /F5 and  is referred to as X24

alternate 1.  This is for printers that are  X24 compatible except

that they  do 1/180 inch indexing.  This is probably the case with

most  X24 compatible printers, including the  IBM  Quickwriter and

IBM 2390  and 2391.  /F5* works only with printers that can handle

1/360  indexing,   including  the  2390  and  2391   but  not  the

Quickwriter.

The third  X24 compatible  driver is /FX and is referred to as X24

alternate  2.  This is  the same  as  /F5 except that the vertical

units command uses  a  different  format  that is not strictly IBM

compatible.  This driver works  with Panasonic 24 pin  printers in

X24 mode.

The compression mode drivers (!  suboption)  may or  may not be an

improvement with these printers.

Proprinter X24 and XL24 printers are very slow with graphics.  The

X24E and XL24E are significantly faster.

p-28

IBM Quietwriter 2 and 3

/F2 and /F3 drive  the Quietwriter 2 and 3.  Do not use  1 for pen

widths because  Quietwriters do not  print consistent 1 pel lines.

These  drivers  reset  the  line  feed distance to  1/6  inch upon

completion.

Kodak Diconix Color 4

/FK drives the Color 4.

Tektronix ColorQuick

/FR drives the ColorQuick.

p-29

HP-GL Commands -------------------------------------------------------

PrintGL's HP-GL specification is taken from the  programming manuals

for the HP 7475A, ColorPro (7440/17440), and 7550A.  The 7550 manual

is  the  best reference for HP-GL - HP  manual 07550-90001.  PrintGL

supports  the 7475, and 7440/17440 command sets except for character

set 8 (Katakana), digitize commands, output commands, the error mask

command, and some device control  commands.  Supported commands  are

listed below.  Commands that are not applicable to printing, such as

pen velocity, are  considered supported  but are completely ignored.

Commands that are not supported are flagged and then ignored.

In  addition,  PG,  AF,  ES,  LO, character set 5, and  proportional

spaced  fonts, from the 7550, are  supported.  And  fifteen pens are

allowed instead of eight.

In reading a plotfile, PrintGL ignores carriage returns, line feeds,

and nulls  except where  they may be valid characters (LB,  SM,  DT,

esc.), and ASCII EOF (026) is interpreted as an end of file mark.

AA x_center , y_center , arc_angle [, chord_angle]

arc absolute

AA moves the pen through an arc, starting at the current position,

with the pen in the current up/down state.

AF

advance full page (7550 extension)

AF starts a new plot. PrintGL starts every page with the pen up at

0,0.

AP

automatic pen pickup - IGNORED

AR x_center_delta , y_center_delta , arc_angle [, chord_angle]

arc relative

AR moves the pen through an arc, starting at the current position,

with the pen in the current up/down state.

CA character_set

designate alternate character set

See HP-GL Character Sets for a list of character sets.

CI radius [, chord_angle]

circle

CI draws a  circle centered at the current position.  The  current

up/down state and position are unchanged by a CI command.

p-30

CP [spaces , lines]

character plot

CP  moves  the  pen by character spaces (+ is  along the text) and

lines  (+  is  up),  using  the current up/down  state.  Using  no

parameters is the same as a carriage return/line feed.

CS character_set

designate standard character set

See HP-GL Character Sets for a list of character sets.

DF

default values

This sets default values for the following commands:  CA,  CS, DR,

DT, ES, FT, IW, LO, LT, PA, PT, SC, SM, SL, SR, SS, TL.

DI [run , rise]

absolute character direction

DI sets the baseline direction for labels.  The default is 1,0.

DR [run , rise]

relative character direction

DI  sets the baseline  direction  for labels.  Run  and rise are a

percentage of P2X-P1X and P2Y-P1Y. The default is 1,0.

DTt

define label terminator

The label terminator ends an LB command. The default is ASCII 003.

EA x_opposite_corner , y_opposite_corner

edge rectangle absolute

EA  draws a  rectangle defined  by  the  current position  and the

parameters. The pen position and up/down state are unchanged.

EC

enable cutter - IGNORED

EP

edge polygon

EP draws  the  current  polygon  edge.  Any  pen  up/down used  in

creating the polygon is respected in drawing the edge.

p-31

ER x_opposite_corner_delta , y_opposite_corner_delta

edge rectangle relative

ER draws a rectangle with corners defined  by the current position

and the command  parameters.  The current pen position and up/down

state are unchanged.

ES [ space [, line]]

extra space (7550 extension)

ES adds to the space between characters and  lines.  The units are

in character cells (1.5*width, 2*height).  The allowed range is -1

to 1. -.3333 causes characters to touch.

EW radius , start_angle , sweep_angle [, chord_angle]

edge wedge

EW draws a wedge centered at the current position. The current pen

position and up/down state are unchanged.

FP

fill polygon

FP  fills  the  current  polygon.  Pen up/down  commands  used  in

creating the polygon are ignored.

There is a limit of 2000 fill line segments per polygon.

FT [type [, spacing [, angle]]]

fill type

These are  the  fill types.  Type  2  is identical to  type 1  for

PrintGL.

1  solid lines spaced at half the pen thickness (see PT command)

2  unidirectional solid lines spaced at half the pen thickness

3  lines at the specified spacing

4  cross hatched lines at the specified spacing

5  type ignored

A spacing  parameter  of 0 sets  the  spacing  to 1%  of the P1-P2

diagonal. Spacing is in x axis units (this is only relevant if the

axes have different scaling).  PrintGL supports any  integer angle

for fill. Cross hatched lines are at angle + 90.

IN

initialize

p-32

IP [p1x , p1y [, p2x , p2y]]

sets scaling points P1 and P2

IP sets the scaling points (P1 and P2) in plotter units. P1 and P2

are used  by a number  of  commands  to help with size independent

plots.

IW [x_lower_left , y_lower_left , x_upper_right , y_upper_right]

input window

IW sets a  clipping window in plotter units.  With  no parameters,

the clipping window is turned off.

LB..text..terminator

label

Labels use  the current character set, based on CA, CS, SA, and SS

commands. See HP-GL Character Sets for font specifics.

LO [ position_number ]

label origin (7550 extension)

LO sets the label origin.  The label origin relative to the  label

is  shown in the diagram below.  For positions 4..9 and 14..19 the

label buffer is limited to 150 characters. The default is 1.

13                     16                      19

3......  .......  ...6...  .......  ......9

.     .  .     .  .     .  .     .  .     .

12 2     .  .     .  .  5  .  .     .  .     8 18

.     .  .     .  .     .  .     .  .     .

1......  .......  ...4...  .......  ......7

11                     14                      17

LT [pattern_number [, pattern_length]]

line type

PrintGL's line type  patterns are close to, but not  identical  to

HP's.  Line  type patterns  are dynamically  limited to a  minimum

length of 4 pels for types 1 and 2 and 16 pels for other types.

no parameter  ������������������������������������������������

0             dots at endpoints only

1             �               �               �

2             ��������        ��������        ��������

3             ������������    ������������    ������������

4             ������������  � ������������  � ������������  �

5             ����������  ��  ����������  ��  ����������  ��

6             ������  ��  ��  ������  ��  ��  ������  ��  ��

Pattern  length  is a  percentage  of the  P1-P2  diagonal with  a

default of 4%.

p-33

PA [x , y] [, x , y] ...

plot absolute

PA sets absolute mode and optionally moves the  pen in the current

up/down state.

PD [x , y] [, x , y] ...

pen down

PD sets  the pen  down and optionally  moves  it with  the current

absolute/relative mode.

PG [i]

new page (7550 extension)

PG starts  a new  plot.  The parameter is ignored.  PrintGL starts

every page with the pen up at 0,0.

PM i

polygon mode

PM 0 starts polygon mode.  The current position is the first point

in the polygon.

PM 1 starts  a new  subpolygon.  The  position after the next  pen

movement is the first point in the subpolygon.  The first pen move

of a subpolygon is done with the pen up, regardless of the current

up/down status.

PM 2 ends polygon mode.

PM  1 and PM 2 close  the current subpolygon with  the current pen

up/down state. If used without first starting polygon mode (PM 0),

PrintGL adds an implied PM 0 before processing the PM 1 or PM 2.

The following  commands are accepted in polygon  mode: PM, PA, PR,

PU, PD,  AA,  AR,  CI  (circles  are  automatically  prefixed  and

suffixed by PM 1), and IN (ends polygon mode).

The polygon buffer is limited to 500 data points.

PR [x_delta , y_delta] [, x_delta , y_delta] ...

plot relative

PR sets relative mode and optionally moves the  pen in the current

up/down state.

PS

paper size - IGNORED

PrintGL ignores PS. The paper size must be set with the /L option.

p-34

PT [thickness]

pen thickness

PT specifies twice the line spacing used in solid fills and may be

from 0 to 5.0 mm. It is reset to .3 mm with each SP command.

PU [x,y] [,x,y] ...

pen up

PU  raises  the  pen  and  optionally  moves it with  the  current

absolute/relative mode.

RA x_opposite_corner , y_opposite_corner

shade rectangle absolute

RA  fills a rectangle with corners defined by the current position

and the command parameters.  The current pen  position and up/down

state are unchanged. The rectangle edge is not drawn.

RO [angle]

rotated coordinate system

RO or RO0 sets up a normal coordinate system. RO90, the only other

allowed  value, sets up a system that is rotated  90 degrees.  The

type of rotation depends on the paper size.  This command does not

change the scaling points, so an IP command is useful after an RO.

The HP 7475 specifies two different meanings for RO90 depending on

the  paper  size (A/A4 or B/A3).  PrintGL  decides  which  type of

rotation to do based on the sum of  the  print  window  width  and

height.  If this sum  is  greater than 24 inches  then large paper

rotation is used. Otherwise, small paper rotation is used.

Small paper rotation:              Large paper rotation:

RO 0             RO 90             RO 0             RO 90

y��������Ŀ   0,0��������Ŀy     0,0��������Ŀy    y��������Ŀ

�        �      �        �         �        �      �        �

�        �      �        �         �        �      �        �

0,0����������x    x����������         �        �      �        �

�        �      �        �

�        �      �        �

�        �      �        �

x����������   0,0����������x

RR x_opposite_corner_delta , y_opposite_corner_delta

shade rectangle relative

RR  fills a rectangle with corners defined by the current position

and the command parameters.  The current pen  position and up/down

state are unchanged. The rectangle edge is not drawn.

p-35

SA

select alternate character set

SA specifies the alternate character set as current. The alternate

character set is chosen with CA.

SC [xmin , xmax , ymin , ymax]

scaling

SC  sets user units for subsequent drawing.  The units are defined

by  user values for P1 (xmin,ymin)  and  P2 (xmax,ymax).  Using no

parameters  turns  scaling off, and  subsequent drawing is done in

plotter units (1/1016 inch).

SI [width , height]

character size absolute

SI sets the capital letter  box  width and height  in centimeters.

Letter  spacing  is  1.5  *  width and line spacing is 2 * height.

Using no parameters with B or A3 paper is the  same as SI.285,.375

and any other paper is the same as SI.187,.269.

SL [tan_angle]

character slant

SL sets the character slant.  The parameter is  interpreted as the

tangent  of  the angle from vertical.  Using no parameter sets the

slant to 0 degrees.

SM[c]

symbol mode

SM  sets symbol  mode, which centers  the symbol mode character at

any plotted point.  Any character from ASCII 033 to 127 except ";"

sets symbol mode. Any other character turns symbol mode off.

SP pen_number

select pen

SP selects a pen.  Using 0 or no parameter stores the pen  without

selecting a new pen.  This is usually done at the  end of a  plot.

PrintGL ignores any drawing with pen 0.

SR [width , height]

character size relative

SR sets the capital letter box width and height as a percentage of

P2X-P1X  and P2Y-P1Y.  Letter  spacing is 1.5  *  width  and  line

spacing  is  2  *  height.  Using no  parameters  is the  same  as

SR.75,1.5.

p-36

SS

select standard character set

SS specifies the standard character  set as current.  The standard

character set is chosen with CS.

TL [positive_length [, negative length]]

tick length

TL sets the tick lengths  used by XT and YT.  The  parameters  are

lengths above  and  below the current position as a  percentage of

P2Y-P1Y for x-axis ticks and  a percentage of  P2X-P1X  for y-axis

ticks.

UC [[pen_control ,] x_delta , y_delta] ...

user character

UC specifies  a user defined character.  The pen control number is

99 for pen down  or -99  for pen  up.  Pen movements are relative,

with x_delta in  units  of 1/4 the character  width and y_delta in

units of 1/8 the character height.

VA

adaptive velocity - IGNORED

VN

normal velocity - IGNORED

VS

pen velocity - IGNORED

WG radius , start_angle , sweep_angle [, chord_angle]

shade wedge

WG fills a wedge centered at the current position. The current pen

position and up/down  state are unchanged.  The wedge  edge is not

drawn.

XT

x-axis tick

XT draws a vertical line, with length specified by the TL command,

at  the current position.  The pen position  and up/down state are

unchanged.

p-37

YT

y-axis tick

YT draws  a  horizontal  line,  with length  specified  by  the TL

command, at the  current position.  The  pen position  and up/down

state are unchanged.

esc.(

esc.Y

plotter on

Esc.(  and esc.Y turn the plotter on.  This is only relevant  with

the /YY option.

esc.)

esc.Z

plotter off

Esc.) and esc.Z turn the plotter off. They are only effective with

the /YY option.  The plotter on  and off commands let you  include

comments  or  data  to  pass through  to  another  device  in  the

plotfile. These are flagged as unsupported commands if /YD is set,

as an indication that you should set /YY.

esc.@, esc.H, esc.I, esc.M, esc.N, esc.R

RS-232 device control commands - IGNORED

p-38

HP-GL Character Sets -------------------------------------------------

PrintGL  supports all of the character sets that  are included  with

the HP 7475 and 7440/17440 except Katakana, and adds character set 5

and  the  proportional  spaced  fonts   from  the  7550.   PrintGL's

characters are similar to but not identical to HP's.

0  ANSI ASCII

1  9825 character set

2  French/German

3  Scandinavian

4  Spanish/Latin American

5  Special Symbols (7550 extension)

6  JIS ASCII

7  Roman Extensions

8  Katakana - NOT SUPPORTED BY PRINTGL

9  ISO IRV (International Reference Version)

30  ISO Swedish

31  ISO Swedish For Names

32  ISO Norway Version 1

33  ISO German

34  ISO French

35  ISO United Kingdom

36  ISO Italian

37  ISO Spanish

38  ISO Portuguese

39  ISO Norway Version 2

The character sets specify a font for codes 033..127. Codes 000..032

and 128..255 are independent of the character set and are all no ops

except these.

003  default label terminator

008  backspace

009  half backspace

010  line feed

011  reverse line feed

013  carriage return

014  select alternate character set

015  select standard character set

032  space

Each of  the  character sets  is available as a proportional font by

adding 10 to the set number.  The proportional  fonts  are kerned to

improve  appearance (the character spacing is adjusted to compensate

for  character shape).  Use the LO  command to simplify working with

proportional fonts. To improve the appearance of large text, use the

ES command to reduce the character spacing.

Codes 65 to 81 in character set 5 are symbols designed to be used in

symbol mode (SM command) or as one character labels.  When used in a

label (LB command) these characters  are centered at the current pen

position and leave the pen position unchanged.

p-39

The  HP-GL characters cannot  be  printed  with  the  standard ASCII

character set.  The file CHARSET.PLT  is an HP-GL file  that can  be

used  to  print a  replacement for this page, showing  the character

sets.  Use the  command  PRINTGL CHARSET.PLT  /M1 /AO0,0 with  a  /F

option specific to your printer to do this.

p-40

Compatibility and Technical Information ------------------------------

PrintGL will run on any IBM compatible MS-DOS computer. It will work

in a  region as  small as  112K but  may  need  more memory for high

resolution  or wide prints.  144K is enough for the worst case black

only  prints  - 13.5 inches wide, 360 dpi.  Color output requires an

additional 32K or more for high resolution or wide  prints.  256K is

enough for the worst  case.  A larger region usually makes things go

faster by allowing larger  buffers  and averting the use  of a spill

file.

By default, PrintGL uses the  BIOS printer interrupt to send data to

a  printer.  The D1..D4 and X1..X4 drivers send data directly to the

serial  port - addresses hex 3f8,2f8,3e8,  and  2e8.  If  a  file or

device  is specified  for output then DOS 2.0 file handling is used.

Messages are written to the  screen  with standard  DOS CON  output.

Graphics screen modes (except  Hercules) are  set up with  BIOS, and

graphics screen updates are done by writing  directly to the display

buffer.

PMI will run on any IBM compatible MS-DOS computer.  It will work in

a  region as small as 192K.  When running  PrintGL,  all but 48K  is

given to PrintGL. More memory may allow PrintGL to run more quickly.

PMI runs PrintGL via a DOS shell.  It searches the current directory

and then the DOS PATH, calling the first copy of PRINTGL.EXE that it

finds.  When saving the current settings, PMI makes the  same search

for PMI.EXE.

PMI  can use any text or graphics display  mode that allows at least

80 columns and 25 lines of text.  In text modes, PMI writes directly

to  the  display  buffer, and  in  graphics modes, BIOS  is  used to

display text.  When under DESQview, PMI should be started in a  text

mode, and it writes directly to the DV selected screen buffer.

PRINTGL.EXE may be compressed  with PKLITE, LZEXE, or other EXE file

compressors.  PMI  should  not  be  compressed because  it  rewrites

PMI.EXE when saving the user settings.

PrintGL and PMI  were  written  by  Cary Ravitz  and  compiled  with

Borland's Turbo Pascal 6.0 and Turbo Assembler 2.0.

p-41

User Support ---------------------------------------------------------

The  quickest  way to  resolve  problems with PrintGL  is to use the

support bulletin board.  The Ravitz Software support  bulletin board

is at:

606-268-0577  1200/2400,N,8,1  24 hours/day.

To ask a question (or make a comment or suggestion), first prepare a

file with the details of your question. Please be very specific, and

include  any related information  on  your  hardware,  software, and

operating environment.  Then call the bulletin board  and choose the

upload question  option.  XMODEM file  transfer protocol is required

for this.  Your question will be assigned  a number (such as 1015) -

remember this number. An answer file (with the assigned number) will

be posted, usually within 24 hours.

To  get an  answer,  call the  bulletin  board, choose the  download

option, then the answer option, and then enter  the  answer  number.

XMODEM is also required for downloading answers.

All question and answer files are public.

The  latest version of PrintGL is available from the bulletin board,

packed in .ZIP  format, and may be downloaded with  XMODEM or YMODEM

(1K XMODEM).

Anyone may use the bulletin board.

You may send questions to Compuserve - 70431,32  Cary Ravitz.

You may send questions by mail to:

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

Please  send a diskette (360K, 1.2M, 720K, or 1.44M,  720K is  best)

with your question so that example files  or a newer version of  the

software may  be  included  with  the response.  Return  postage  is

appreciated.