A $20 LOGIC ANALYZER

Yes that is right if you own a PC compatible computer that has a printer

port, you can have a logic analyzer for $20 (or less depending on what

parts you have lying around).  Our $20 logic analyzer features:

- 5 data channels.

- 8 K byte data buffer.

- 5 bit trigger with don't care states.

- maximum sample rate up to several hundred Khz.

- 5 sample rate selections.

- easy to used PC based software.

- timing and state display modes.

- can be used with a notebook PC as a portable logic analyzer.

Of course these features are not comparable to the latest offerings

from HP, but for thousands of dollars less you can own a very useful

tool.  The maximum sample is not fast enough to use on a typical

microprocessor, but it is fast enough to trouble shoot most I/O control

signals, TTL level serial and devices such as EPROM or GAL programmers.

Building the $20 Logic Analyzer

The only piece that needs to be built is an easy to make probe assembly.

It requires only eight components as listed in the parts list.  The

probe assembly is used to connect the five logic analyzer channels and a

ground path to the circuit being tested.  The assembly is detailed in

figure 1.  Figure 2 shows the connection list and figure 3 shows the pin

out of the male DB-25 connector as view from the rear (solder cup side).

Although only six conductors are actually used having a nine conductor

ribbon cable provides for a grounded conductor between each of the

signal channels.  This makes for a shield between signals and prevents

crosstalk between channels.  The three conductors that do not attach to

mini clips can be cut at the point where the ribbon cable is broken out.

Once the signals are separated there is no longer any concern about

crosstalk.  The 9 conductor ribbon cable can be made from any wider

ribbon cable by stripping some of the conductors off.  The 6 clips that

are connected at the end of the cable should be labeled as shown in

figures 1 and 2.  Once the assembly is complete, it is plugged into the

PC's parallel printer port for use.  The software can be configured to

use either LPT1 or LPT2.

Setting Up the $20 Logic Analyzer Software

The $20 Logic Analyzer is very easy to setup.  Copy the program

LOGIC$20.EXE onto your disk.  Then start the program by typing LOGIC$20

and then Enter at the DOS prompt.  The first time it is run the program

will indicate that it could not find the configuration file LOGIC$20.CFG

- this is OK.  As prompted press any key for the main menu to appear.

Select option F8 - Calibrate from the main menu.  This will create a

proper configuration file for the next time the program is run.  You

will be asked which printer port you wish to use.  Enter 1 or 2

corresponding to either LPT1 or LPT2.  The program will take a second to

determine the maximum sample rate that can be used on that PC.  This

information will automatically be stored in the configuration file.  The

calibration process will need repeated for each different PC the $20

Logic Analyzer is installed on.  That completes the setup.

Using the $20 Logic Analyzer

The $20 Logic Analyzer is easy to use.  Start the program as above.

Connect the mini clip labeled ground to the ground of the circuit being

tested.  Connect the other mini clips to the points of interest in the

circuit.  CAUTION - do NOT connect the clips to any signals that are

outside of the 0 to 5 Volt TTL range, it will damage your printer port.

The programs main menu will appears as in Figure 4.  Use option F1 to

select the trigger condition.  Only bits 0 to 4 are valid, bits 5 to 7

are ignored.  The trigger for each bit can be selected to be a 0, a 1 or

an X.  An X at a given channel position means that you do not care about

the state of that channel as part of the trigger condition.  All Xs

means that a trigger will occur immediately.  A trigger of XXX01XX1

means that a trigger will occur when channel 5 is 0 (TTL low), channel 4

is 1 (TTL high) and channel 0 is 1.  Data would still be acquired on

channels 1 and 2, but it would be ignored when checking for a trigger

condition.  Option F3 allows you to select the sample rate, the options

are the maximum sample (as determined from the calibration routine), and

one half, one quarter, one eighth and one sixteenth of the maximum

sample rate.  The choices are expressed as both the sample rate in Khz

and the sample period in microseconds.  Option F4 starts the data

acquisition, data will be acquired until 4096 samples after a trigger

condition occurs or until a key is pressed.  Option F5 will show the

acquired data in STATE format.  For each sample the state display shows

the clock cycle number relative to the trigger point.  The trigger point

is considered clock cycle zero, the negative numbers are the clock

cycles before the trigger and positive are after the trigger.  The next

number is the time in micro seconds relative to the trigger.  Then the

data is displayed first in hex and then in binary channel 0 the least

significant bit).  You can move forward or backward through the state

display by pressing Pg Up, Pg Dn, Home, End and the numeric pad 5 key

(this may only work with Num Lock active depending on your keyboard).

Pg Up and Pg Dn will move backward or forward one page at a time.  Home

will jump to the first data acquired and End to the last data acquired.

The 5 key will return you back to the trigger point.  A special feature

is the search capabilities, by pressing S you will see the search menu.

Specify the search pattern in the same manner as selecting a trigger

condition (1,0 and X), press return when finished.  Select F (forward

search) or R (reverse search).  The program will search for data that

matches your condition and display that screen of data.  This is a very

powerful feature because of the rather deep 8K data buffer (many

expensive logic analyzers have only 2K deep buffers).  Press ESC to exit

the state display.  Press F6 to enter the timing display (Note - this

requires a CGA or compatible display (EGA or VGA)).  The data will be

displayed in timing diagram format.  There is a cursor at the trigger

point and the state information from the cursor position is displayed at

the top of the screen.  The ESC, Pg Up, Pg Dn, Home, End, numeric 5 key

and the search option work the same way as with the state display.  In

addition the left and right arrow keys can move the cursor back and

forth through the data to display the state information for any point on

display.  F7 the file option on the menu allows you the save and restore

data and setup information (trigger word, clock rate etc.).  You can use

this to save data and view it at another time.  F8 is the calibration

mode described above and F9 gives program information.  Use option F10

to exit the program.

How the $20 Logic Analyzer Works

On most system designs there is a trade off between performing various

functions in hardware or using software.  Typically the software route

has a lower parts cost at the expense of lower performance.  For the 20$

logic analyzer the data acquisition is done in software with the

assistance of the standard hardware that is available on all PCs.  This

is why the cost is so low.  The only special hardware needed is the

inexpensive probe assembly.  The probe assembly plugs into the parallel

printer port of your PC, either LPT1 or LPT2 can be used.

The standard PC printer port has 5 input pins, these of course become

the five channels on the $20 Logic Analyzer.  The 5 inputs are all

present on an input port at either 379H (LPT1) or 279H (LPT2) so they

can be read simultaneously by the logic analyzer software.  The software

can check for a match with the trigger condition and then stop data

acquisition when appropriate.  An 8 K byte block of PC memory forms a

buffer for the data.

The sample rate time base is generated by stealing the PCs time of day

clock hardware during data acquisition. To understand how this works we

need to take a look back at the original PC design.

Since the original IBM PC, the PC has had a time of day clock.  The

original PC used an 8253 PIT (programmable interval timer) chip to

produce an interrupt about 18.2 times per second.  This interrupt kept

the time of day clock.  The 18.2 Hz interrupt rate may seem a little

strange, it also has it's origins in the original PC design.  The

original PC was designed so that it could use a TV as a monitor.  The TV

horizontal scan rate is based upon the 3.58 Mhz color burst frequency.

The 8088 CPU used in the original PC needed a clock with a 67 % duty

cycle, this required a master clock that is three times the CPU clocks

frequency.  The first 8088s had a maximum operating frequency of 5 Mhz.

In order to save an oscillator the PC used a 14.31 Mhz master clock,

this is four times the 3.58 Mhz color burst clock (Note - all values are

rounded at two decimal places) and the CPU would run at a clock rate of

4.77 Mhz (14.31/3).  The 8253 had a maximum clock rate of 2 Mhz so it

was feed a 1.19 Mhz clock, which is derived from the 4.77 Mhz clock

divided by 4.  The 8253 is a 16 bit programmable counter which was

programmed to divide the 1.19 Mhz input clock by it's maximum divisor of

65536, this produced the 18.2 Hz output which is used to generate the

time of day interrupt.  To this day all PCs have the equivalent of an

8253, driven with a 1.19 Mhz clock, incorporated into their chip sets to

keep compatibility with the original PC design.  If you look on your

PC's motherboard you will probably find a 14.31 Mhz oscillator somewhere

on the board.  It is needed to generate that all these oddball

frequencies.  So much for ancient PC history, the point is that all PCs

have a built in timer that can be used by the $20 logic analyzer for

it's sample rate time base.

The input clock to the 8253 is 1.19 Mhz, the minimum divisor the 8253

can handle is 2 this means the maximum interrupt rate the 8253 can

generate is 1.19/2 or 0.6 Mhz.  That is the maximum theoretical sample

rate of the $20 logic analyzer.  In reality the maximum interrupt rate

will be limited by the length of time the interrupt takes to execute, so

that faster PCs will have a faster maximum sample rate.  The first

version of the $20 Logic Analyzer is written totally in C.  I will

rewrite the interrupt routine in assembler for version 2.0, this should

reduce the interrupt processing time and increase the maximum sample

rate significantly for all PCs.

Wrapping Up

The $20 Logic Analyzer can be a very useful and educational tool.  As

with any you must be aware of it's limitations, a logic analyzer does

not record what occurs in between samples.  This means to be sure that

you are getting a valid picture of what is occurring in a circuit, a

logic analyzer must have a faster sample rate than the change rate in

the signals being examined.  If you need to work with faster circuits

then the $20 Logic Analyzer can handle, be sure to check out my low

cost 25 Mhz, 16 channel logic analyzer. Look for file RGBLOGIC.ZIP.

------------------------------------------------------------------------

Probe Assembly Connection List

DB-25 Pin	Ribbon Cable Conductor		Mini Clip Label

13		1				1

25		2				GROUND

12		3				2

24		4				No Connection

11		5				4

23		6				No Connection

10		7				3

22		8				No Connection

15		9				0

-----------------------------------------------------------------------

$20 Logic Analyzer Parts List

6 - Mini Clips (or Micro Clips).

1 - DB-25 Male connector with solder cup connections.

1 - 16 to 24 inch length of 9 conductor ribbon cable.

Note - these parts can be purchased from Radio Shack or many of the

suppliers in Nuts & Volts.

The software is a shareware program with a $10 registration fee. You can

get a copy from the IBMAPP section on Compuserve or in Don Lancaster's

PSRT on GENIE. Search using the keyword RGB to find the exact file

location within those sections. You may also purchase a registered copy

on disk for $10 from:

Robert G. Brown

30 Wicks Road

E. Northport NY 11731

At this time (12/1/93) users who register RGBLOG$20 will receive Rev 2.0.

----------------------------------------------------------------------------

LOGIC$20 License and Registration

LOGIC$20 is provided as is.  There are no warranties expressed or

implied.

LOGIC$20 is distributed as shareware. You may use LOGIC$20 without charge on a

trial basis to determine its suitability for you. If you continue to use it

after your evaluation, a $10 registration is requested. This registration

covers use by a single person (on multiple computers) or installation on a

single computer (may be used by a group of people).

You may distribute LOGIC$20 as a stand alone product if you keep the entire

package together, unchanged, and do not charge more than $10. You may not

distribute LOGIC$20 as part of another product.

To register LOGIC$20, send your name, address, and $10 check to:

RG Brown

30 Wicks Road

E. Northport NY 11731