This README file of DRLFX1.ZIP accompanies DIRLPFX1.NLM for Netware 386

v3.11. The following files are included for this revision:

DIRLPFX1.NLM      797      4-09-91  10:39a

PATCHMAN NLM     7421   4-12-91  10:23a

PATCHMAN DOC     4262   4-12-91   3:23p

PURPOSE:

DIRLPFX1 is a dynamic patch which is loaded at the file server console as a

NetWare Loadable Module.  This patch fixes a problem in NetWare 386 v3.11 that

results in an infinite loop when the DOS DIR command is used in a sub

directory where more than 65,535 files exist. This loop will also occur with

the FindFirst and FindNext DOS function calls.

DIRLPFX1 requires version 2.00b of PATCHMAN.NLM (or later) in order to load.

Unlike previous patches, the SERVER.EXE file is never modified by this patch.

DIRLPFX1.NLM patches the Operating System in memory, not on the disk.  This

means that DIRLPFX1.NLM must be loaded each time before any "fixes" take

effect. It is recommended that the command "LOAD DIRLPFX1" be placed in the

server's AUTOEXEC.NCF file to ensure that the patch is always in effect when

the server is initially brought up.

Since it is an NLM, DIRLPFX1 can be loaded and unloaded without ever having to

bring the server down. Once loaded, the Operating System is patched and ready

to go. Unloading DIRLPFX1 will restore the Operating System to its original

"un-patched" state.

Like all NLM-type patches, DIRLPFX1 requires the Patch Manager (PATCHMAN.NLM)

to be loaded before it can be loaded.  If PATCHMAN is not already loaded,

DIRLPFX1 will attempt to locate PATCHMAN and load it automatically if found.

PATCHMAN only needs to be loaded once.  All other patches can load once

PATCHMAN is loaded. PATCHMAN cannot be unloaded until all patches that rely

on it are first unloaded. Typing "Patches" at the system command line once

PATCHMAN is loaded will display all patches currently loaded on the file

server.

INSTALLATION:

To install DIRLPFX1, simply place the DIRLPFX1.NLM file in the same directory

as the other NLM files (SYS:SYSTEM or DOS partition), and type"LOAD DIRLPFX1".

Once the patch has loaded, all changes are in effect until DIRLPFX1 is

unloaded.

The changes made by this patch will be implemented in all future versions

NetWare 386. This patch and PATCHMAN.NLM will only operate on NetWare 386

V3.11. Future versions of NetWare 386 will require a version of PATCHMAN

specific to that release.

*****************************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to this

software patch, and specifically disclaims any express or implied warranties

of merchantability, title, or fitness for a particular purpose.

Novell's intentions for this software patch is to provide a temporary

work-around to the anomalies described in this file.  Such work-arounds are

typically addressed in future releases of NetWare.

Novell will not be responsible for any data loss that may result from

implementing this patch.  Novell strongly recommends a backup be made before

any patch is applied.  Technical support for this patch is provided at the

discretion of Novell.

*****************************************************************************