LAN+MODEM

Quick Start Installation

Remember: LAN+MODEM makes modem sharing as easy as printer sharing.

You can allow anyone on the LAN to access any modem on the LAN. The

person with the installed modem can continue working as others dial

in or out of the network using the shared modem(s).

1. MAKE A BACKUP COPY OF THE LAN+MODEM DISKETTE

2. DO A DIRECTORY OF THE LAN+MODEM DISKETTE

Enter DIR a:

The Serial number of your software will appear after the word

VOLUME. The following files should appear on the listing:

MCOM.EXE  This is a TSR (TERMINATE & STAY RESIDENT) background

program that designates the modems as shareable on the

network. MCOM is normally loaded in the AUTOEXEC.BAT file

of the PC with the modem that you want to share.

NETDEV.COM  This is a TSR program normally loaded onto a menuing

system or batch file of any other computer on the network

which needs to access, with third  party communications

packages, those modem(s) that  have been made shareable

by MCOM.EXE program.

README.DOC  Contains recent additions to the documentation made

since the last release.

HELP.DOC  If you're having problems, read this file for support

tips.

UCOM.EXE  This program is available as an optional module to

LAN+MODEM. UCOM is the CCC "communications" program that

is designed to work in conjunction with the MCOM program.

UTIL<DIR> A subdirectory: This subdirectory contains useful

utilities for enhancing modem performance over your

network. These utilities are also helpful with installing

a new release of your LAN operating system. They will

ensure your new operating system is as clean as the one

you had installed. The files within this UTILITIES

subdirectory are not normally used during your first

installation. The contents and descriptions of these

files can be found in the LAN+MODEM documentation.

3. COPY THE DISKETTE FILES ONTO THE FILE SERVER.

For demonstration purposes, unless otherwise stated, it will be

assumed throughout this quick start section that LAN+MODEM will be

installed in the F:\LANMODEM subdirectory. You can install the

LAN+MODEM software anywhere on the network.

Start by placing the LAN+MODEM diskette into drive A:. At the DOS

prompt, execute the following instructions:

F:

CD\

MD\LANMODEM

CD\LANMODEM

XCOPY A: *.*  /S

The software necessary to run LAN+MODEM is now installed in the

F:\LANMODEM subdirectory.

The utilities are now installed in the F:\LANMODEM\UTIL subdirectory.

4. DESIGNATING WHICH MODEMS ARE SHAREABLE ON YOUR LAN.

Load the MCOM.EXE file into the AUTOEXEC.BAT file of the workstations

with modems that are designated as shareable.

If you have two modems simply execute MCOM.EXE twice in the

AUTOEXEC.BAT file as follows:

MCOM

MCOM

One common use of LAN+MODEM is to install it on a computer that was

previously dedicated as a bulletin board or an E-Mail server. In that

case, if the Bulletin Board or E-Mail modem is on COM1 then run

MCOM.EXE followed by the # of the COM PORT on which your shareable

modem resides. For example, a Bulletin Board modem uses COM1 and you

want to put another shareable modem on that computer on COM2, then

put in your AUTOEXEC.BAT file the following command:

MCOM 2

5. NAMING MULTIPLE MODEMS WITH THE MCOM PROGRAM.

For naming the modem drivers LAN+MODEM uses the series COMM01-COMM20

for the 20 modem version (the maximum name number is based on the

version you have). When you have modems with different

characteristics and/or maximum speeds, you can select the name for

each modem driver with a command line option. For example, if you

have a 9600 BPS modem and a 2400 BPS modem and you want to access

them by name, you can name the COM PORT where each modem resides.

To do this, change the AUTOEXEC.BAT command to MCOM followed by

'COMM#' where # is a number from 01 to 20. As an example for the 9600

BPS modem:

MCOM COMM10

And for the 2400 BPS modem:

MCOM COMM11

To access the modems designated as COMM10 and COMM11, see Sections 6

and 7.

6. ACCESSING SHAREABLE MODEMS ON YOUR LAN WITH UCOM.EXE.

There are two ways to access the shareable modems. One is by running

the UCOM program which accesses the shareable modem. The second is by

running the NETDEV.COM program which allows you to run other

compatible communications programs.

To access a shareable modem with UCOM.EXE, just type in the command:

UCOM

You will access the first available shared modem on the network. In

the example in section 5, UCOM will first select the 9600 BPS modem,

and if it is busy, it will select the 2400 BPS modem.

To select a specific modem, such the 9600 BPS modem in section 5,

then name the modem in a command line by following UCOM with the

command "COMM#" as is used by the MCOM program. For example,

UCOM COMM10

To select the 2400 BPS modem in section 5, enter the UCOM command

followed by the command COMM11. for example:

UCOM COMM11

7. ACCESSING SHAREABLE MODEMS ON YOUR LAN WITH OTHER COMMUNICATIONS

PROGRAMS

To access a modem with a third party communications program that has

a network interface (known as an INT14H interface) you must use the

NETDEV.COM command at the workstation that needs to the modem. There

is no change in the MCOM.EXE setup.

Examples of programs that have an INT14H driver (or network driver)

are:

1. PROCOMM PLUS NETWORK VERSION    7.   CC:Mail

2. REFLECTION                      8. Westlaw-Westmate

3. CROSSTALK MARK IV               9. PC Anywhere

4. LEXIS/NEXIS                    10. Windows 3.0 Dynacom

5. CO/SESSION LAN                 11. DCS-EM320

6. SMARTERM                       12. Broadview-Pibterm

Details on setting up these communications programs to address our

NETDEV program is contained in a README file on the diskette and in

the manual.

The only difference between setting up a third party software package

and setting up the UCOM program is the additional need to "Open a

Channel to the Modem." This is done by following the NETDEV command

with an OPEN command. For example, to install PROCOMM PLUS NETWORK

VERSION you would use a batch file in your network menuing system:

1.  NETDEV               4.  NETDEV/CLOSE

2.  NETDEV/OPEN          5.  NETDEV/DISAB

3.  PCOMNET

This batch file would open the channel and find the first available

modem on the network that you designated as shareable with the MCOM

file. Then PROCOMM PLUS NETWORK VERSION would have full control of

the modem.

To access a specific "named" modem on the network, use the "START"

command. For example, to access the 9600 BPS modem designated as

COMM10 in section 5, run the command:

1.  NETDEV               4.  PCOMNET

2.  NETDEV/START=10      5.  NETDEV/CLOSE

3.  NETDEV/OPEN          6.  NETDEV/DISAB

Please Note: The above method of accessing modems with other

communications programs is valid for all compatible programs except

the CO/SESSION software. CO/SESSION has written an interface to

LAN+MODEM eliminating the need for the "OPEN" command after the

NETDEV command. Therefore a sample batch file for running CO/SESSION

looks like this:

NETDEV

Optionally, NETDEV/START=10

SUPPNET -I2

For more detailed information on LAN+MODEM refer to specific chapters

in the manual.

Cross Communications Company (303) 444-7799

LAN+MODEM is a registered trademark of

Cross Communications Company.