==============================================================================

OKI 900 - DEBUG MODE

==============================================================================

Here's some info on using the debug mode from the keypad.

Power the phone up.  Wait for PowerOn msg.  Hit 7 and 9 together.

Then hit Menu, Snd, End, Rcl, Sto, Clr.  Phone says good timing!

Debugger is now enabled, but phone works normally.  Hit 1 and 3

together to halt phone and enter debugger.  Everything on display

lights up.  Hit Clr, Clr till you get status display.

Now you can execute commands listed below.  For example to reboot phone

enter #, 0, 2, Snd.  Commands all start with # and end with Snd.  Some

take arguments.

You can use #25, to display memory in EEPROM, and I think once in

that command you can hit # and * to go up and down in memory, Clr to

exit.  Hex chars are entered as "*n", like *1=A, *2=B, etc.

SUSPEND     #01         Performs Initialization

RESTART     #02         Terminates the test mode

STATUS      #03         Shows current status of TRU

RESET       #04         Resets the autonomous timer

TURNAROUND  #05    ?    Returns Data Bytes following command to the Test Set.

INIT        #06         Initialize the TRU to following states:

Carrier Off, Attenuation - 0db, Receive Audio Muted

Transmit Audio Muted, Signalling tone off,

Autonomous timer reset, SAT off, and DTMF off

CARRIER ON  #07         Turns the carrier on

CARRIER OFF #08         Turns the carrier off

LOAD SYNTH  #09XXXX     Sets the synthesizer to channel XXXX

SET ATTN    #10X        Set the RF power attenuation to X

0=0db, 7=-28 db (in steps of -4db thru 7)

RXMUTE      #11         Mutes the receive audio

RXUNMUTE    #12         Unmutes the receive audio

TXMUTE      #13         Mutes the transmit audio

TXUNMUTE    #14         Unmutes the transmit audio

RESETOFF    #15         Discontinues resetting of autonomous timer

STON        #16         Transmits a continuous signalling tone

STOFF       #17         Stops transmission of signalling tone

SETUP       #18         Transmits a 5 word RCC message (fixed text pattern)

VOICE       #19         Transmits a 2 word (RCC) RVC message (fixed test pattern)

RCVSU       #20         Receives a 2 word FCC message (cancel with 0x38)

RCVVC       #21         Receives a 1 word (FCC) FVC message (cancel with 0x38)

SEND-NAM    #22         Returns the information contained in the NAM

VERSION     #23

SEND-SN     #24

MEM         #25XXXX     Displays the resident memory data at XX

00XX=in micro, XXXX=EEPROM

WSTS        #28         Count 1 word messages on CC, until TERMINATE

WSTV        #29         Count 1 word messages on VC, until TERMINATE

SATON       #32X        Enable the transmission of SAT X

0= 5970 Hz, 1=6000 Hz, 2=6030 Hz

SATOFF      #33         Disables the transmission of SAT

CDATA       #34<60>     Transmits 5 word RCC message (30 bytes)

HITNON      #35         Activates the 1150Hz tone to receive audio line

HITNOFF     #36         Deactivates the 1150Hz tone

LOTNON      #37         Activates the 770Hz tone to receive audio line

LOTNOFF     #38         Deactivates the 770Hz tone

DTMFON      #42XX       Enable the transmission of DTMF frequency XX[2]

DTMFOFF     #43         Disable the transmission of DTMF

?           #44

?           #45

?           #46

?           #47

?           #48

?           #51

-           #52<xx>

?           #53

-           #54XXXXZZ   Write HEX (ZZ) into ADDRESS $XXXX

if 00XXZZ then store #$YY in MicoRAM $XX

-           #56         Return Value stored in $BEBB

?           #60

?           #62

?           #63

RCVSU       #64         Receives a 2 word FCC message (duplicate of cmd #20)

COMP-ON     #65         Enable the compressor and expandor

COMP-OFF    #66         Disable the compressor and expandor

setvol      #67         X-Set volume (0-7) 0=max

SERIAL I/O  #683XX?     Mutes/Unmute Tx/Rx Audio Signal Enable

Disable the Compressor/Expandor, XX=commanded states.

CMD Compress Tx Mute Rx Mute

--- -------- ------- -------

40  on       unmuted unmuted

41  off      unmuted unmuted

42  on       muted   unmuted

43  off      muted   ummuted

44  on       unmuted muted

45  off      unmuted muted

46  on       muted   muted

47  off      muted   muted

?           #72          [pulls something, outputs 1 word!?!]

?           #73<arg>

Scans channels,...

#73 XXXX xxxx YY

XXXX = Start channels scan

xxxx = End   channels

yy  = Time

?           #74

-           #75         Enable Handsfree (disable spkr)

-           #76         Disable Handsfree (enable spkr)

-           #77         Turns on Loudspeaker near mic

-           #79

?           #80

?           #81

?           #84

?           #85