Current Video and Printer Drivers

=================================

VIDEO DRIVERS: as of January, 1992

=============

This is a current listing of the video drivers included with CompuShow. Modes

in (parentheses) are simulated.  Note that some modes may not be available on

your video adapter. For example:

-  16 color 1024x768 requires a 512K VGA,

- 256 color  640x480 requires a 512K VGA,

- 256 color 1024x768 reguires a   1M VGA, and

- 256 color modes on an 8514/A adapter require the memory expansion.

Also, VGA adapters  that are built-in  to a  system board or  bundled with  a

computer (unless you're getting a specific name-brand board), are  frequently

low-end (cheap) adapters, and probably don't have "Super-VGA" capabilities.

Finally, remember that all  IBM standard   graphics modes are  'built-in'  to

the program. If your graphics adapter  doesn't go beyond the standard  modes,

then you don't need a video driver.

Driver                          Notes

=====================================================================

A2=ATI Graphics Solution

320x200x16, 640x200x16

A3=Ahead Systems

800x600x16, 1024x768x16, 640x400x256, 640x480x256, (640x480x256),

800x600x256, 1024x768x256

A4=Amstrad PC1512

(320x200x16), 640x200x16

A5=ATI Vga Wonder(s)

800x600x16, 1024x768x16, 640x400x256, 640x480x256, 800x600x256,

1024x768x256, 640x480x32k

A6=AT&T 6300

640x400x2, 640x400x16

A7=AcuMos

800x600x16, 1024x768x16, 640x400x256, 640x480x256, 800x600x256

C1=Chips & Technologies         Cardinal

800x600x16, (320x400x256), 640x400x256, 640x480x256, (640x480x256),

800x600x256

C2=Compaq VGA

640x480x256

C3=Cirrus

640x400x16, 720x540x16, 800x600x16

D1=DGIS

(Available modes depend on the video adapter and DGIS interface)

[DRIVERS.DOC] Page 1

E1=Everex VGAs

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

800x600x256, 1024x768x256

G1=Genoa 6400

800x600x16, 1024x768x16, 512x512x256, 640x350x256, 640x400x256,

640x480x256, 800x600x256

H1=Hercules Graphics Station

640x400x256, 640x480x256, 800x600x256, 1024x768x256, 1024x768x256,

640x480x32k, 512x480x16m

[ The first 1024x768x256 (mode "V") is interlaced,

the second (mode "W") is non-interlaced ]

H2=Hercules InColor

720x348x16, (360x174x16)

I1=Generic CGA                  (any CGA)

160x100x16, (320x200x16)

I3=IBM PGC                      IBM Professional Graphics Controller,

Vermont Microsystems

(320x200x256), (320x240x256), (320x400x256), (320x480x256),

(640x400x256), 640x480x256

I4=IBM 8514/A                   Single monitor

I5=IBM 8514/A                   8514+VGA monitor

640x480x16, 1024x768x16, 640x480x256, 1024x768x256

[Load the 'Adapter Interface' (HDILOAD) in AUTOEXEC.BAT]

I7=IBM XGA

1024x768x16, 640x480x256, 1024x768x256, 640x480x64k

N1=NCR VGA

800x600x16, 1024x768x16, 640x400x256, 640x480x256, 800x600x256

O1=Oak Technologies             [see "CAUTION", below]

800x600x16, 1024x768x16, (320x480x256), 640x480x256, 800x600x256,

1024x768x256

P1=Paradise                     AST, Compaq, Dell

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

(640x480x256), 800x600x256

P2=Plantronics

320x200x16

S1=STB EM16+                    This driver is identical to T4, except for

one "non-standard" mode number

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

800x600x256, 1024x768x256

S2=STB Ergo + Sierra DAC

640x400x256, 640x480x256, 800x600x256, 1024x768x256, 320x200x32k,

640x480x32k, 800x600x32k

[DRIVERS.DOC] Page 2

T1=Tseng 3000                   Genoa, STB, Orchid

800x600x16, 1024x768x16, (320x350x256), (320x480x256), 640x350x256,

640x480x256, 800x600x256

T2=Tandy 1000SL/TL              "ETGA"

640x200x16, (320x200x16)

T4=Tseng 4000 series            Orchid ProDesigner II

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

800x600x256, 1024x768x256

T5=Trident 8900 series          Logix, ZyMOS

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

800x600x256, 1024x768x256

T6=Tseng 4000 series VGAs with Sierra DAC

640x400x256, 640x480x256, 800x600x256, 1024x768x256, 640x400x32k,

640x480x32k, 800x600x32k

V1=Video 7

800x600x16, 1024x768x16, (320x400x256), 640x400x256, 640x480x256,

800x600x256, 1024x768x256

V2=VBE (VESA Bios Extensions)

800x600x16, 1024x768x16, 640x400x256, 640x480x256, 800x600x256,

1024x768x256, 1280x1024x256, 640x480x32k, 800x600x32k, 640x480x64k,

640x480x16m

[Maximum of SEVEN of the above modes. Available modes depend BOTH on the

capabilities of the video adapter and the VESA Bios Extensions.]

CAUTION: Problems with Oak VGAs

-------------------------------

There seems to be a serious problem with the Oak BIOS, and if you try to use

the System Configuration program CSHOWI to test (or CSHOW to display) an Oak

mode which isn't available  on your VGA,  it will still  attempt to set  the

graphics mode,  and throw  the VGA  into a  configuration where  it will  no

longer do ANY extended graphics modes until you re-boot!

The only way around  this bug in the  Oak BIOS is to  select the Oak  driver

(perform one test in  CSHOWI and then ESCape  to skip the  rest) and set  up

VIDEO.DAT manually:

- With the Oak video driver selected, use CSHOW to display a graphic in

each video driver mode UNTIL YOU ENCOUNTER A MODE THAT DOESN'T WORK.

- Re-boot the computer and continue testing.

- Once you have a list of the modes which DO WORK, set up VIDEO.DAT with

the corresponding letters:

COPY CON VIDEO.DAT

STVW^Z              (^Z means hold down the CTRL key and type Z.)

[DRIVERS.DOC] Page 3

Printer Drivers: as of January, 1992

----------------

Note that the <Q>uality  print mode of MOST  COLOR PRINTERS and the  LASERJET

and DESKJET at 300dpi  requires MORE THAN 640k  of memory, as CompuShow  must

expand, rescale and dither the entire image to the 'virtual screen'  (VScreen

on the Shift-F1 box), before it starts to print. This means if you don't have

EMS or XMS memory, you'll be limited to the <F>ast print mode.

The following printer drivers are included with the enhanced program:

Color Printer Drivers

=============================================================================

IBM Color Printer         Vertical dot spacing is 84 dpi.

Single Density: esc,'K',n1,n2         Prints at 84 dots/inch.

Double Density: esc,'L',n1,n2         Prints at 168 dots/inch.

=============================================================================

Color 8 pin (EX-800)      GENERIC 8 pin color driver for 24/216" graphics

line spacing. Tested on a Star NX-1000 Rainbow and

Citizen MSP-50. Similar to EX-800, LX-800, FX-86e,

LX-86, LX-80.

Single Density: esc,'*',0,n1,n2       Prints at 60 dots/inch.

Dual Density:   esc,'*',1,n1,n2       Prints at 120 dots/inch.

=============================================================================

Color 8 pin (JX-80)       Identical to COLOR8A, except for 20/180" line

spacing. Similar to JX-80, LQ-1500, ALPS ALQ200.

Single Density: esc,'*',0,n1,n2       Prints at 60 dots/inch.

Dual Density:   esc,'*',1,n1,n2       Prints at 120 dots/inch.

=============================================================================

Color 24 pin (LQ-2550)    GENERIC 24 pin color printer for 24/216" graphics

line spacing. Similar to LQ-2550.

Single Density: esc,'*',0,n1,n2       Prints 8 pins at 60 dots/inch.

Dual Density:   esc,'*',33,n1,n2      Prints 24 pins at 120 dots/inch.

CRT III:        esc,'*',38,n1,n2      Prints 24 pins at 90 dots/inch.

=============================================================================

Okidata 290 series color  Tested on a model 292.

Single Density: esc,'K',n1,n2          Prints at 60 dots/inch.

Double density: esc,'L',n1,n2          Prints at 120 dots/inch.

[DRIVERS.DOC] Page 4

Monochrome Printer Drivers

=============================================================================

IBM/EPSON 8pin (n/216")   a GENERIC driver for 8 pin printers that specify

line spacing in multiples of 1/216"

Single Density: esc,'K',n1,n2         Prints at  60 dots/inch.

Double Density: esc,'L',n1,n2         Prints at 120 dots/inch.

=============================================================================

IBM/EPSON 8pin (n/180")   a GENERIC driver for 8 pin printers that specify

line spacing in multiples of 1/180"

Single Density: esc,'K',n1,n2         Prints at  60 dots/inch.

Double Density: esc,'L',n1,n2         Prints at 120 dots/inch.

=============================================================================

Citizen MSP 10/15/20/25   a GENERIC driver for 8 pin printers that support 72

and 144 dpi.

One-to-one      esc,'*',5,n1,n2       Prints at  72 dots/inch.

Two-to-one      esc,'*',7,n1,n2       Prints at 144 dots/inch.

=============================================================================

KX-P2624 (24 pin: LQ1050) for 24 pin Epson compatible printers.

CRT II:         esc,'*',6,n1,n2       Prints 8 pins at 90 dots/inch.

Double Density: esc,'*',33,n1,n2      Prints 24 pins at 120 dots/inch.

=============================================================================

KX-P2624 (ProPrntr XL24E) for the IBM Pro Printer XL24E and compatibles.

Double Density: esc,'[g',n1,n2,1      Prints 8 pins at 120 dots/inch.

Double Density: esc,'[g',n1,n2,9      Prints 24 pins at 120 dots/inch.

=============================================================================

HP DeskJet 75/300 dpi     prints slightly darker than the LaserJet driver.

75 dpi          esc,'*t75R'           Prints at 75 dots/inch.

300 dpi         esc,'*t300R'          Prints at 300 dots/inch.

=============================================================================

HP LaserJet 75/150 dpi    for Laser Jets with limited memory installed.

75 dpi          esc,'*t75R'           Prints at 75 dots/inch.

150 dpi         esc,'*t150R'          Prints at 150 dots/inch.

=============================================================================

HP LaserJet 75/300 dpi    for Laser Jets with adequate memory installed.

75 dpi          esc,'*t75R'           Prints at 75 dots/inch.

300 dpi         esc,'*t300R'          Prints at 300 dots/inch.

=============================================================================

HP Thinkjet [MDL 2225C]   supports 96 and 192 dpi.

Single Density: esc,'K',n1,n2         Prints at  96 dots/inch.

Double Density: esc,'L',n1,n2         Prints at 192 dots/inch.

=============================================================================

IBM Compact               has one graphics mode, at 70 dpi

Single Density: esc,'L',n1,n2         Prints at 70 dots/inch.

=============================================================================

NEC 8023                  May also work with the C.Itoh Prowriter.

Bit-Image:      esc,'S',a1,a2,a3,a4   Prints at 80 dots/inch.

=============================================================================

Okidata ML92/ML93         prints 7 pin graphics.

10 pitch:       30,3                  Prints at 60 dots/inch.

12 pitch:       28,3                  Prints at 72 dots/inch.

[DRIVERS.DOC] Page 5