BOOTSLOW -- slow down for self boot programs -- 1.00

Copyright (c) 1991 Alexander R. Pruss

May be freely distributed, modified and used provided that no copyright

messages are removed, and this sentence together with the next two are

always included in unmodified form.  If you like this program a lot, you are

invited to show your appreciation by making a tax-deductible donation to

the Pro-Life or Anti-Abortion movement, but are under no obligation, moral

or otherwise to do so (especially if you disagree with the goals of this

movement.)  There is no warranty; use at own risk.

There are many slowdown programs available, but this is the only

one I know of that can slow down a self-booting program, i.e. one

where you simply put the program disk in the computer and boot it.

Send comments, bug reports to:

pruss@unixg.ubc.ca  (InterNet)

pruss@ria           (UUCP)

PRUSS@UWOVAX        (Bitnet)

1. Preparation

For BOOTSLOW to boot all PC-"compatible" computers (some of which like

the AT&T 6300 having a brain-dead interrupt 19), some information must

be known to the program about the conditions on boot-up.  This

information may differ between systems and thus it must be newly

garnered on each system BOOTSLOW is to run on.  This information must

also be regenerated after every hardware or BIOS upgrade.

You will need a PC compatible system with at least two drives (one

floppy, and the others can be RAM or hard), with about 16Kb of free

memory.  Some of the utilities may crash in less memory.

1.1 WHIRR.COM: knowing which drive is which

You will have to know which DOS drive is which of your drives, and which

physical drive is which of your drives.

All the programs except for SLOW.COM make use of physical drives.  For

convenience, they are called A,B,C,etc., referring to BIOS drives

0,1,2,etc.  Note that since hard drives usually have numbers

greater than 127 this makes it impossible to access them.  Usually the

physical drive letter will be the same as the usual DOS drive letter,

but not always.  (Especially not if you have run ASSIGN, JOIN or SUBST,

and almost never for hard-drives.)

SLOW.COM uses logical drives, to let you boot special devices such as

hard-drives or bubble-drives.  Each logical device is also referred to

by a DOS drive number 0,1,2,etc. for convenience denoted as a,b,c,etc.

If you did not use ASSIGN, JOIN, or SUBST then it is most likely these

letters are the same as the "usual" DOS drive letters.

To find which physical or DOS drive letter refers to which of your

drives, run

whirr

at the DOS prompt.  You will get the display:

----

Disk Whirr Program.

(C) Copyright 1991 Alexander Pruss

Press an upper case letter to whirr a physical floppy, or a lower case letter

to whirr a DOS drive.

Press A,B,..., or a,b,c,..., or ESC to abort.

----

Pressing a capital letter A,B,C,etc. will quickly whirr the

corresponding physical drive.  You should be able to tell which drive it

was by watching your disk indicator lights.  Similarly, pressing a lower

case letter a,b,c,etc. will whirr (light up) the corresponding DOS

drive.  This is a nondestructive test.

Try pressing a,b,c,d and A,B,C,D.  If there is no disk in a drive, a

disk error will be signalled.  Now remember the physical and DOS letters

for each drive.  Press ESC to exit.  Note that as aforementioned, you

will likely be unable to address hard-drives by physical letters.

1.2 BOOTSEC.BIN, MKBOOT.COM, MKTABLE.COM, BOOT.ITB: knowing your

configuration

You will need to do this once at the beginning, and henceforth after

every BIOS or hardware upgrade.

1.2.1 Making a special boot disk

Now, to garner the system configuration data, prepare a blank formatted

disk.  (If it is not blank, the data will be lost.)  Insert this disk in

one of your physical drives, and type at the DOS prompt (while in a

directory containing the BOOTSLOW distribution):

mkboot

If all goes well, you will be prompted for a physical drive letter.

Press the letter corresponding to the drive you put your blank formatted

disk in.  The data (if any) on this disk will likely be destroyed, and

the file BOOTSEC.BIN will be installed on it.  If you have a disk error,

try using a different disk or drive.  You may instead press Ctrl-Break

to abort.

Your blank formatted disk has now become a special boot disk.

1.2.2 Booting it

Take this special boot disk and boot it.  (Put it in a drive that is

normally checked at boot-up and reboot the computer, say by pressing

Ctrl-Alt-Delete.)  If everything works, you will see on your screen:

----

Interrupt table recorder.

(C) Copyright 1991 Alexander Pruss

Choose physical floppy drive to save to: (A,B,etc.)

----

(Again, even more data on the disk will now be destroyed, so if the

special boot disk is not really blank and you realise you need the data,

quickly remove the disk, reboot your computer and run your favorite data

recovery program on it. It should not be too late.)

Press the physical drive letter the special boot disk is in.  If all

goes well, you will get the message:

----

Saved!

Remove this disk and reboot with Ctrl-Alt-Del.

-----

Remove the special boot disk and reboot the system.  The disk now

contains your systems initial interrupt table.

1.2.3 Grabbing information from the disk

After rebooting the system, go to your directory containing BOOTSLOW.

Insert the special boot disk again, and type

mktable

to get the message:

----

Interrupt table recorder.

(C) Copyright 1991 Alexander Pruss

Choose physical floppy drive to load data from: (A,B,etc.)

----

Press the physical drive letter where your special boot disk is.  Drives

should whirr, and the message "Done!" should appear.  You now have all

the necessary information saved in the file BOOT.ITB.  You can now

recycle your special boot disk.  I recommend you reformat it.

BOOTSLOW is now ready to slow games and other programs down.

2 Running SLOW.COM

SLOW.COM does all the work of slowing your computer down (after you

spent all the money on making it so fast...)  In summary, you type:

slow [d] [factor]

and press the DOS drive letter containing the disk you wish to boot.

You wust be in the directory containing the BOOT.ITB file.

Normally the slowdown does not occur during disk access, but the logic

behind the detection of disk access may at times fail with the effect

that slowdown does not occur at all even not during disk access

(I have yet to actullay meet such a case, though.)  In this case, (or

for some other reason), you can include the "d" flag which will make the

slowdown constant, even when the disk is being accessed.

factor is the slowdown factor, by default 200.  The larger it is, the

slower your computer is.

2.1 How to do it

Find the disk you wish to boot at a lower speed.  In the directory

containing BOOT.ITB, type:

slow

You will receive the message

----

Self-Booting Disk Slowdown.

(C) Copyright 1991 Alexander Pruss.

Choose DOS drive to boot: (A,B,C,etc.), ESC to abort.

----

Insert your booting disk (unless it is a hard-disk!) and press the DOS

drive letter of its drive.  (Not necessarily the same as the usual DOS

letter, nor the same as the physical letter.)  If thing are working you

will be told, "Booting..." and the disk will boot, with the computer

slowed down and robbed of 1Kb of memory.  You can boot your main

hard-disk too, so you can load DOS.  To return to the usual

configuration, reboot your system.

If you change your mind, you can hit ESC to abort.

You should look and see if the computer is slowed domn enough or too

much.  You con control the amount of slowdown by typing:

slow factor

instead of just:

slow

where factor is the slowdown factor.  The greater it is, the slower your

computer will be.  By default factor is 200.

If you are having problems, try including the "d" flag as in:

slow d 300

or

slow d.

Note that if you use SLOW.COM to boot DOS then you can run SLOW.COM

again.  The slowdown will NOT be cumulative, although the 1Kb of memory

use will be.

3 Testing

Not enough testing done.  Tested and seems to work perfectly with Jump

Man (on floppy) and DOS 3.3 on hard-drive.

4 Bugs

Not enough testing done.

Some programs may crash in less than 16Kb of free memory.

Source not tested on MASM or versions of TASM other than 2.0.

I cannot figure out how to get it to work in conjunction with SIMCGA's

self-boot handler.  But then, SIMCGA by itself should be a sufficient

slowdown!