VGADISK Version 1.0: 176K virtual disk in VGA memory    (c) 1991 Tommy Frandsen

-------------------------------------------------------------------------------

FEATURES:

- 176K virtual disk (RAM disk) using only VGA memory

- Is compatible with all text-only programs

- Easy installation and use

- Has been tested on many different machines

REQUIREMENTS:

- An IBM PC/XT/AT/PS2/Clone

- MS-DOS/PC-DOS 2.0 or higher

- A VGA graphics adaptor

SUPPLIED FILES:

- VGADISK.SYS: Device driver for the virtual disk

- VGADISK.EXE: Use when running graphics programs

- VGADISK.DOC: Documentation (this file)

INSTALLATION:

- Copy VGADISK.SYS into the root directory of your boot disk

- Insert the line "DEVICE=VGADISK.SYS" in CONFIG.SYS

- Reboot by pressing CTRL-ALT-DEL or by turning OFF/ON

- You should see a message containing the drive designation of

the created virtual disk (typically D: or E:)

- The virtual disk is now ready to be used as a normal DOS disk

COMPATIBILITY:

- VGADISK is compatible with any program that doesn't use graphics

- Graphics programs will destroy the contents of the virtual disk

- The virtual disk can be used like a normal (very fast) DOS disk

VGADISK.EXE:

- Allows you to restore the virtual disk after running graphics programs

- "VGADISK -F"    : Reformat the virtual disk

- "VGADISK -Sxxx" : Save an image of the virtual disk to file xxx

- "VGADISK -Lxxx" : Load an image of the virtual disk from file xxx

- Example (could be included in a BAT-file):

VGADISK -SC:\TMP (save the virtual disk to file C:\TMP)

FS4              (run Flight Simulator or a similar program)

VGADISK -LC:\TMP (load the virtual disk from file C:\TMP)

WARNINGS:

- If you fail to restore the virtual disk after running a graphics

program, the results of using the virtual disk are unpredictable.

- Although this program has been very troughfully tested, any usage

of VGADISK is totally at your own risk. I am not responsible for

any damage caused by the program, either direct or indirect, to

your hardware or software. If you can't accept this restriction,

please don't use VGADISK.

-------------------------------------------------------------------------------

You may freely copy and distribute this program. If you find VGADISK useful

and convenient, a donation of $5 would be appreciated. Please send check to:

Tommy Frandsen

Lovtoften 50

DK-2630 Taastrup

Denmark

-------------------------------------------------------------------------------