�����������������������������������������������������������

Vplus, The Ultimate Sound Digitiser.

Copyright (c) 1991, Andrew MacBeth

Programme Documentation

�����������������������������������������������������������

������������������������������������������������������������

Page One

Table Of Contents

�����������������

Table Of Contents     ......................... Page Two

Introduction          ......................... Page Three

Hardware Requirements ......................... Page Four

Programme Features    ......................... Page Five

Playing Samples       ......................... Page Six

Recording Samples     ......................... Page Seven

How To Sample    ......................... Page Seven

Sampling Tips    ......................... Page Eight

Script Files          ......................... Page Ten

Programme Support     ......................... Page Eleven

������������������������������������������������������������

Page Two

Introduction

������������

This programme is designed to make digitising, and playing

digitised samples simple.

Not everybody realises just how simple it is, to turn your

SoundBlaster card into a professional quality sampler. The

aim of this programme is to make available to you, the user,

a simple medium to record and to playback digitised samples.

This user manual will assist you with the use of the Vplus

programme. It will also help you to overcome a lot of

problems that are to be found with sampling. You will find

the helpful tips that are in this manual to be of great

help, when you are sampling your own files.

A Big problem with sampling, is the amount of space that it

takes up. Samples can become very large when you are

sampling sounds of any length. This introduces new problems

of its own, as your computers memory may not be large enough

to contain the whole sample.

To solve this, Vplus uses Disk Double Buffering. This means

that sounds are not stored in memory, but rather swapped in

an out in blocks when they are required.

This allows you to record, and play files of unlimited

length. Indeed, your only limit is the size of your hard

disk.

The quality of sampling is defined by the sampling rate. The

higher your sampling rate, the higher quality the sample.

However, high sampling rates mean high sample sizes. You

will have to trade�off size against quality for yourself.

This manual can be considered as a guide to using Vplus, and

a helpful guide in creating your own samples. Sampling

experts should ignore a lot of this manual, as it is

designed to help the novice.

������������������������������������������������������������

Page Three

Hardware Requirements

���������������������

Vplus requires hardware minimums. As you can appreciate,

sampling can take a lot of the computers processing. Because

of this, hardware minimums have to be defined.

To run this programme, without noticing any degradation of

sampling quality, you will need at least an IBM compatible

AT style computer. If you have a fast XT, then you may be

alright. A Lot of this rests on the type of controller card

your machine has. More on this later.

You will need at least 512K of ram. Although the programme

is mostly disk based, it still uses large buffers to store

data. This takes up a considerable amount of space.

Multi�Taskers are not recommended for sampling, unless you

have a very powerful machine, such as the 386 and 486 class

of IBM compatibles. If you attempt to sample on a slow

machine, you will notice that the programme wont run up to

its full capacity.

You will need a Hard Disk drive. This really goes without

saying. Floppy drives simply cannot keep up with the data

transfer rates that are demanded by Vplus.

Obviously, you will also need some sort of medium to get the

sound samples from an external source, into your

soundblaster card. This can take many forms, depending on

what external source you are taking sounds from. If you want

to sample directly from a microphone, then the microphone

with a 1/8" jack is all you will need.

If you want to sample from a CD player, then you will need

some sort of cable. Consult your sound output device for the

types available.

And you will need undying patience. Sampling properly takes

time. Very rarely do you get it right the first time around.

������������������������������������������������������������

Page Four

Programme Features

������������������

Vplus contains many features to assist you in sampling

sounds that you will not find in any other sampler.

These features include :

. Direct sampling of raw sounds to disk, allowing files

of unlimited length to be played, and sampled.

. Scripts can be created, to allow you to play samples

directly from DOS. This allows you to place sounds

within batch files, for presentation. This has the

bonus of Vplus being transparent to any programmes

that you decide to write. You can shell to Vplus

without the programmer having to worry himself

about how to play and record samples. Vplus

does not 'Identify' itself in any way, when

run in script mode.

. The programme is very simple to use. You dont need

to remember complicated command line parameters.

. Sliding windows are implemented in play mode. You can

tell how much has been played, and how much is to be

played as a percentage of the total file size. A

sliding bar is used to implement this.

. Sampling rates from 5,000 and 13,000 Khz are

available in steps of 200 Khz.

. Mouse support. Vplus can be accessed via the mouse

in some aspects of its operation. This makes life

a little easier.

. And last, but not least, the programme has helpful

and decorative 'Exploding windows' to make Vplus

easy, and enjoyable to use

������������������������������������������������������������

Page Five

Playing Samples

���������������

Playing VOC files, is simple with Vplus. Vplus can detect

the sampling rate of the file, and change itself to ensure

output is of the proper speed.

To play a file, follow these simple steps :

. Run Vplus

. Select [Play A VOC File] From the main menu

. You will now be presented with a list of files that

are available in the current directory. If the file

you want to play is not in the current directory, you

can move around the current logged drive and select

files from other directories. the [..] file listed

in the files list, is the directory lower down in

the hierarchical file structure than your current

directory.

NOTE: Directories are YELLOW entries, and playable

files are WHITE.

You can use the mouse to select files. Move the mouse

up and down to move the pointer, and the left mouse

button can be used to select files/directories.

. Once you select your file, output will commence.

. To Pause output, press any key, to resume playing

press any key again.

. You can abort the playing of the file, at any time by

hitting your [ESC] Key.

. Once output has finished, press any key to return

back to the main menu.

That's it! That is all you have to do, to play back files.

The playback of files, is the simple part. The hardest part

about digital sample usage, is the correct and proper

procedure of creating your own samples. I will deal with

this in the next section.

������������������������������������������������������������

Page Six

Recording Samples

�����������������

The recording of digitised samples is a very tricky process

to get just right.

If you are just starting out with Vplus, and sampling, then

the best way to learn is by trial and error. The more

experience you get with sampling, the better you will

become. Its just like riding a bike.

If you intend to sample from a CD player, then you have

chosen the easiest way to sample. If your CD has a Headphone

Jack built into it, with a volume control then you will find

the process of sampling relatively simple.

If you are going to be sampling from cassette, then get

ready for a lot of 'Mucking Around' to get it just right.

How To Sample

�������������

Sampling files with Vplus is straight forward. Follow these

steps.

. Connect a cable from your external sound device, into

the input socket of your Soundblaster

. Load Vplus

. Select 'RECORD A FILE' from the main menu

. You will now be presented with a prompt to enter the

filename. You need only enter a filename, without

the extension. Vplus will automatically add the

extension '.VOC'.

. Select the sampling rate that you require. The

higher the rate, the better the quality but the

higher the space required.

. If you are sure you want to continue, press 'Y'

when prompted. Sampling will now commence.

. When you are finished sampling, hit the [ESC] key.

You will then be returned to the main memory, and

sample will be stored on disk.

������������������������������������������������������������

Page Seven

Sampling Tips

�������������

This section contains information to help you make perfect

samples. Use this guide if you are having troubles getting

your samples just the way you like them.

Use an Optimum volume

���������������������

A very important issue when sampling files, is to make sure

that you have your input devices volume at an optimum level.

Ensure that the external device is turned up as loud as

possible, before you start getting 'Crackles' on your

samples. This will make sure that background noise is

reduced, and your sample is the only thing that can be

heard.

Use a decent cable

������������������

Make sure that the cable that you are using, is shielded,

and of high quality. If you dont, then you will find that

you get a high amount of background noise.

Get Treble and Bass adjustments correct

���������������������������������������

This is VERY important if you are sampling from sources

other than Compact Disk.

If the Bass is turned up too much on the sound device, then

you will get 'Muffled Samples'. Optimum samples are attained

when treble is high.

Even if you think the treble is too high, still TRY it, and

see how it turns out. The soundblaster adds a lot of its own

bass to samples, so it will probably still turn out alright.

������������������������������������������������������������

Page Eight

Adjust volume level during samples

����������������������������������

Some samples may have loud, and quiet sections. If this is

the case then you will have to make sure that optimum volume

is achieved at all times. To do this, you will have to

manually adjust the external devices volume as the sounds

change. This can be tricky to get just right, but without

it, your samples wont sound very good.

Make many tests of each sample

������������������������������

If your sample turned out 'Ok' then dont let that stop you

from taking the sample again. Do it as many times as you see

fit, adjusting sound output each time, until you get the

sample just right.

This is the most time consuming part of sampling, but it is

also the most important.

These are the main points that you will have to observe

while you are sampling your own files.

Remember that HUGE samples dont make the best samples. The

best samples are those that are interesting, make there

point, and impact the listener in some way.

������������������������������������������������������������

Page Nine

Script Files

������������

Vplus also has the capability of being able to play sound

samples from the DOS prompt.

This will allow you to add 'flair' to your batch files.

Imagine a computer that plays the tune from StarWars, each

time its booted up!

You can use this feature to add professional quality to your

demonstration programmes. Imagine a demo that 'Speaks' to

the user while he is viewing it!

Programmers can use Vplus as a shell, to play files in the

middle of their programmes!

Vplus does not display any internal information, or update

screen memory in any way. You dont even know its running,

apart from the fact that you can hear the sample!

Creating a script file is simple. All you have to do, is

load up your favourite text editor and create one in the

following format :

�������������������������

c:\samples\1.voc

c:\samples\test.voc

c:\sounds\demo\game.voc

������������������������

Simple isn't it? Each line of the file contains a filename,

That's it!

Vplus will go through your list, playing each file in turn.

If a file cannot be found, you will get a warning message.

The file you create, MUST be called 'VPLUS.SCR'.

To run Vplus in script mode, just add the command line

SCRIPT when you run Vplus, ie  VPLUS SCRIPT will load VPLUS

in script mode.

And That's it!

������������������������������������������������������������

Page Ten

Programme Support

�����������������

I hope you get a lot of pleasure with Vplus. Sampling sounds

can either be a fun hobby, or a demanding profession.

If you are having difficulties using VPlus, I urge you to

contact me with your questions.

I run a BBS, that is almost totally dedicated to the

SoundBlaster and its supported files. If I cant answer your

question, no doubt somebody will!

You can contact me, on my BBS, on the Telephone number

+61�9�295�3453. The Board can answer calls 24 hours a day,

and can support all baud rates up to and including 9600

Baud.

If you are not fortunate enough to own a modem, then you can

simply post me a letter!

You can post me, at the address :

Andrew MacBeth

PO Box 270

Mundaring, 6073

Perth, Western Australia.

Even if your not having troubles, I would still be

interested to hear from you!

Well that brings us to the end of the documents. I hope you

have a lot of fun sampling files, and I hope this programme

makes it a little easier for you!

Bye for now,

Andrew MacBeth . Perth, Western Australia.

18 July 1991.

������������������������������������������������������������

Page Eleven