Adding Communication ports

AC.SYS can be inserted into your CONFIG.SYS file to add non-standard

communication ports for use as COM1, COM2, COM3, or COM4.  If your serial card

uses a hardware port address which your BIOS ROM does not know then that card

will be unavailable to DOS,  The Weak Link  and  The $25 Network.  With this

program you can give the BIOS its proper address.  Serial card port addresses

are kept as follows:

word at 40h:0  = COM1

40h:2  = COM2

40h:4  = COM3

40h:6  = COM4

This program merely "pokes" a proper port address to the appropriate

memory address.  You must know your card's port address in order to use this

program.  Usually, a card will have several different address options which are

set via jumpers or DIP switches.  Your manual is usually a good place to find

this information but I have found several Taiwanese manuals with bad

information.  I have several boards which are almost identical but the later

versions have COM3 at 1F8h even though the manuals clearly state the COM3

address of the card is 2E8h.  Evidently the board was revised but not the

manual.

Once you find out your hardware's port address then add AC.SYS to your

CONFIG.SYS file (with the appropriate parameters) prior to The Weak Link or

The $25 Network. For example:

DEVICE=AC.SYS COM2=1F8 COM3=2E8

DEVICE=PM.SYS COM2                    <-- Weak Link

This will add COM2 and COM3.  Characters can be entered in upper or

lower case. You cannot add the ":" to the COM name and you cannot add a space

before or after the "=" sign.  The port address must be a hex number.  If the

port address you give is not found then an error message will be printed and

that parameter will be skipped.

Also note that you can use this program to expand the number of

available ports beyond COM4.  Immediately after the four communication port

addresses are kept 4 parallel port addresses.  If you are using only LPT1 then

you could use the LPT4 slot as COM8, LPT3 as COM7, and LPT2 as COM6.  It would

be best to use COM8 first, then COM7, etc.  But remember that all port

addresses must be different.  Also remember that a special DOS device driver

must be written in order for DOS to use anything above COM2.  These extended

ports are only available to programs which simply need to know port addresses,

such as The Weak Link and The $25 Network.

Don Jindra

Information Modes

P.O. Drawer F

Denton, Texas  76202

Ph. 817-387-3339