[                 D A N P L O T   A N D   D A N C A M                ]

DOCUMENTATION FILE

CAMPLOT.DOC

FOR

DANPLOT.COM v2.5 AND DANCAM.COM v2.5

THE TWO COMPANION

COMPUTER AIDED MANUFACTURING PROGRAMS

FOR USE WITH DANCAD3D v2.5

Copyright (c) 1989-1991 by Daniel H. Hudgins, All rights reserved.

Daniel H. Hudgins, 466 Diamond Street, San Francisco, CA 94114, USA

SEE ALSO THE FILE DANCAD3D.DOC ON THE  DANCAD3D  SHAREWARE  DISKS.

USE  THE PROGRAM PRINTDOC.EXE ON THE DANCAD3D SHAREWARE DISKS TO PRINT

OUT THIS FILE IN ORDER TO HAVE THE PRINT CENTERED PROPERLY.

You may not copy this DANCAM/DANPLOT disk, this documentation file

or  printed  copies  of  this documentation file (other that to make a

working copy for your own  personal  use)  without  obtaining  express

written  permission  form  the  author  of this disk,  i.e.  Daniel H.

Hudgins.  Under no conditions is this DANCAM/DANPLOT disk to  be  sold

for more than $10.  None of the files of the DANCAM & DANPLOT disk may

be altered, modified, deleted, or translated on copies of the DANCAM &

DANPLOT  disk that are to be sold,  shared,  or loaned without express

written permission from Daniel Hudgins.

The  latest  version  of DANCAM and DANPLOT come with the disk set

and manuals that are included with the DANCAD3D v2.5 user outfit.  The

DANCAD3D user order form is accessed from the opening menu that  comes

up  when  DANCAD3D v2.5 is run from DOS.  Additional documentation and

schematics for use with DANCAM  and  DANPLOT  are  supplied  with  the

DANCAD3D user outfit.

Please read all disclaimers and all warnings,  notices, and danger

statements before You use DANCAM or DANPLOT.  Since DANCAM and DANPLOT

control motors,  and other electro-mechanical  hardware,  the  use  of

DANCAM  or  DANPLOT can cause serious or fatal personal injury.  It is

assumed that the user of DANCAM and DANPLOT  understands  the  dangers

involved  with  working with power driven machinery,  and high voltage

circuitry,  and will take any and all safety precautions necessary  to

protect  himself  and others from injury.  Although DANCAM and DANPLOT

are being used successfully,  DANCAM  and  DANPLOT  may  not  function

properly  at  all  times and are provided AS IS for you to USE ONLY AT

YOUR OWN RISK.

[1]

[           D I S C L A I M E R   A N D   W A R N I N G              ]

DISCLAIMER

Please remember that DANCAM can DANPLOT are sold AS IS, please use

them ONLY AT YOUR OWN RISK.  Daniel H. Hudgins will not be responsible

or liable for any loss,  damage,  or injury of any kind.  Please  read

the  disclaimer that is in the DANCAD3D manual since it covers DANPLOT

and DANCAM as well.  Also read the disclaimer in the file DANCAD3D.DOC

on the DANCAD3D shareware disks,  as well as this disclaimer  and  the

warning below.

This  documentation  for DANCAM and DANPLOT may contain errors and

or  omissions.  Under  no  circumstances  or  conditions  will  Daniel

Hudgins be held accountable for any errors or omissions.

This  documentation  includes suggested connections for hooking up

devices to your computer's parallel port.  Daniel Hudgins will not and

shall not be held accountable or liable for any loss  or  damage  that

might   occur   to  your  computer  to  other  devices.   It  is  your

responsibility  to  check  and  verify  all  the  appropriateness   of

connections  to  your computer before turning on the power.  Verifying

the appropriateness  of  connections  may  require  you  to  read  and

understand more information than this DANCAM and DANPLOT documentation

contains.

Under  no  circumstances  are  DANCAM  or  DANPLOT  to  be used in

equipment that is  used  for  medical  purposes.  DANCAM  and  DANPLOT

should  not be used in equipment that could cause injury to persons or

loss of property if  the  equipment  or  software  failed  to  operate

properly.

The  use  of  DANCAM  or  DANPLOT  is disallowed if any laws might

require Daniel Hudgins to be in any way responsible or liable for  any

loss  or  injury.  DANCAM  and  DANPLOT  may  not satisfy occupational

safety laws in your state or country.

No license to third parties is expressed or implied.  No violation

of copyrights or patents held by  third  parties  is  implied  by  the

suggested  use  and connection of DANCAM and DANPLOT.  The user should

confirm that no copyrights or patents will be violated by his  use  of

DANCAM or DANPLOT.

The  use  of  DANCAM  or  DANPLOT indicates full acceptance of all

disclaimers and an understanding that DANCAM and DANPLOT are  only  to

be used at the users OWN RISK.

In  this documentation the what is said about using stepper motors

will also generally apply to servo motors and any other type of motors

that might be used.  It is a fact that any component can and should be

expected to fail,  therefore your safety depends on your not  relaying

of any particular component to protect you from harm.  To improve your

safety  from  harm  use  several stages of protection,  so that if any

single safety measure fails you have several others to back the failed

[2]

[           D I S C L A I M E R   A N D   W A R N I N G              ]

one up.  Always be more careful than you think you need to be!

WARNING

Power driven and or automated equipment can cause  serious  injury

or even death.  You must take serious efforts to protect yourself  and

anyone  else  near  such  equipment.  Do not place your hands or other

body parts near power driven equipment without first disconnecting the

power to the stepper motors (or other type of motors) in  such  a  way

that  the  motors  and  tool  cannot  harm you or others.  DANPLOT and

DANCAM are only to be used  by  people  that  understand  the  dangers

involved  and  have  been trained in the necessary safety precautions.

Be  aware  that  failure  of  electrical   connections   or   computer

hardware/software   related   faults   can   cause   motors  to  start

unexpectedly.  To help avoid injury caused by unexpected motor  starts

you  should  disconnect  the  motor  power at both the AC input of the

motor power supply and the DC output of the motor power supply  before

you  expose  yourself  or  others to the equipment.  Evaluation of the

completeness  of   any   safety   measures   is   the   users   (your)

responsibility.

DO  NOT  USE any TSR program that puts a clock on the screen while

the  DANCAM  or  DANPLOT  programs  are  in  use!   Do  not  use   any

multitasking,  or  print spooling programs while DANCAM or DANPLOT are

in use!  Such programs interfere with the  computers  CPU  timing  and

therefore make the motors not run smoothly.  Do not use the DOS screen

dump  command,  e.g.  [Shift]  and  [PrtSc],  while  running DANCAM or

DANPLOT since the parallel port is not connected to a printer and  the

motors or auxiliary relays may be accidentally turned on (the computer

may  also  lock  up since the screen print command will not be getting

the correct printer messages!)

If the computer re-boots because of a power failure,  your turning

the computer off,  your pressing [Ctrl] & [Alt] & [Del],  or any other

reason the voltage on the output pins of the parallel port may  change

and   activate   the  auxiliary  relays  and  other  equipment  in  an

inappropriate and possible harmful manner.  You should investigate the

default state of the parallel port pins after  a  computer  reboot  to

determine  if  you have the high/low state for the output optimized to

minimize any hazard.

[3]

[                         A B S T R A C T                            ]

ABSTRACT

DANCAM and DANPLOT are two utility programs that you  can  use  to

automatically  manufacture  parts.  DANCAM and DANPLOT run on your IBM

(tm) compatible personal computer  and  use  your  personal  computers

parallel  port to send motion control signals to almost any TTL signal

compatible steeper motor translator/driver module,  as well  as  using

the  parallel  port  to  drive all the necessary control relays and to

read all of the necessary home, limit and pause switches.  If you have

a desire to use servo motors in preference to stepper motors, a set of

plans for a special closed loop servo  motor  driver  compatible  with

DANCAM  v2.52 and DANPLOT v2.52 is included in the DANCAD3D v2.5H user

outfit, write to Daniel Hudgins for full information and pricing.

Specifically DANCAM and DANPLOT are designed to read drawing files

made with DANCAD3D.COM v2.5 and control  stepper  motors  movement  in

such a way as to follow the lines drawn with DANCAD3D.  You can attach

stepper motors to almost any 2 or 3 axis machine tool or plotting type

device's hand wheel (crank, knob) shafts to automate it.  Additionally

DANPLOT  can be used to drive 4 axis devices where the forth axis (the

"C" axis) keeps the cutting edge of a saber saw or knife blade pointed

into the direction of the tool motion.

DANCAM and DANPLOT offer  very  high  possitional  accuracy  (when

properly  installed)  and  can be used for applications requiring fine

accuracy to points spaced large distances (the actual accuracy of  the

cutting  tool  location  depends  on the quality of the device you are

automating,  e.g.  flexing in the tool post and play in the ways  will

cause  the  tool to be slightly misplaced,  if the error is repeatable

you can adjust the tool path file with DANCAD3D to compensate for  the

errors.)

Because  DANCAM  and  DANPLOT  are  general purpose Computer Aided

Manufacturing programs you can use them to automate most if not all of

the types of machine  tools  you  use.  Additionally  because  of  the

extreme  low  cost  and high repeatable accuracy of DANCAM and DANPLOT

you may become interested in automating tasks that are currently  done

with manual hand labor.

[4]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

ABOUT THE DANCAM AND DANPLOT DISK

The  DANCAM  and  DANPLOT  disk has several example files on it in

addition to the program files.  Please read the file FILES.DOC  for  a

description  of  the  files  on  this  disk.  Before  you  can use the

programs DANCAM or DANPLOT you will need to make a copy of this floppy

disk and then delete the *.DOC files on your disk working copy.

EXAMPLE: C>COPY A:*.* B:

C>DELETE B:*.DOC

The need to delete the *.DOC files is to make room for DANCAM  and

DANPLOT  to  save  their  configuration  files  i.e.   DANCAM.CFG  and

DANPLOT.CFG.  When you copy the configured versions  of  the  programs

remember to copy the *.CFG file along with the *.COM files.

If  you  want  to use DANCAM and DANPLOT from a harddisk just copy

them into an appropriate subdirectory, i.e. if DANCAD3D v2.5 is on the

harddisk just copy all the files into DANCAD3D subdirectory DC25.

EXAMPLE: C>COPY A:*.* C:\DC25\*.*

To print out this documentation file use the program  PRINTDOC.EXE

on  the  DANCAD3D  shareware  disks.  If your printer is on the serial

port use the DOS mode command to configure  your  serial  port  before

trying  to  print the file.  PRINTDOC.EXE will print the file centered

on the page (the leading spaces have been omitted from  this  file  to

save disk space.)

EXAMPLE: C>C:\DC25\PRINTDOC A:CAMPLOT.DOC PRN

C>C:\DC25\PRINTDOC A:CAMPLOT.DOC LPT2

C>C:\DC25\PRINTDOC A:CAMPLOT.DOC COM1

OVERVIEW OF HOW DANCAM AND DANPLOT WORK

DANPLOT  and  DANCAM  are  two  programs  that you can run on your

computer to manufacture parts.  YOU DO NOT NEED to understand or  have

any  background in CNC or "G" codes to use DANCAM and DANPLOT.  DANCAM

and DANPLOT work in a SIMPLE AND DIRECT MANNER,  they simply move  the

tool along lines and curves THAT YOU DRAW with DANCAD3D.  All you have

to  do  is  draw  on  DANCAD3D's  drawing screen a series of lines and

curves in the order you want the tool to trace out those  motions  and

DANCAM or DANPLOT will FOLLOW THE MOTIONS YOU HAVE DRAWN OUT.

To  use  DANCAM  and DANPLOT you simply go to a motor supply house

and purchase some stepper motors and driver modules (instructions  and

schmatics  for  low  cost  stepper  and servo motor driver modules and

power supplies you can build are included in the  DANCAD3D  v2.5  user

outfit.)  You  will  also need a power supply (some driver modules are

powered from the AC line and do not require a power supply) and  a  36

pin mating connector for your parallel port cable.  Connect the motors

[5]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

to  drive  your  machine's  lead screws with timing belts and pulleys.

Follow the connections shown in the drawing HOOKUP1.2D that  comes  on

the DANCAD3D v2.5 shareware disks and connect up the motors,  modules,

and relays to the mating 36  pin  connector  (note  that  use  of  the

information  in this file and the HOOKUP drawings is at your own risk,

Daniel Hudgins is not responsible for any damage or  loss  that  might

occur.) Connect the mating 36 pin connector to the printer end of your

parallel  port  printer  cable.  Run  DANCAM and/or DANPLOT and select

menu option 4.  from the DANCAM  or  DANPLOT  main  menu.  Answer  the

questions  in  the configuration menu so that the motors will move the

proper amount when the program is used.  Press [E]scape  to  exit  the

configuration section.  Try using main menu option 3. to manually move

the  motors.  Use a dial gage if you have one to confirm that the tool

is moving the proper amount (when you test the movement make the  tool

or  work-piece  move  0.25 inch two or three times in order to take up

the backlash in your machine,  and ignore  the  first  reading.)  Quit

DANCAM or DANPLOT.

Now  that  you have an automated machine tool or plotter hooked up

to your computer you can use DANCAD3D v2.5 to  make  some  ASCII  type

data  files  for  tool  paths.  Run  DANCAD3D  and  select  DRAW  from

DANCAD3D's main menu.  Draw the tool path.  Save the tool path element

as an ASCII type file (if you save the ASCII tool  path  to  a  floppy

drive  you  can  take the drawing from your home or office computer to

the computer in the shop and read the drawing from the shop computer's

floppy drive or copy the file to the shop computer's harddisk and read

(execute) the drawing from the  shop  computer's  harddisk  (a  single

computer  can  be  hooked  up to many machine tools to have many parts

made simultaneously, or each machine can have its own computer and the

tool path file can be run  separately  and  or  concurrently  on  each

machine tool.)) Quit DANCAD3D.

Turn  the  power  supply  for  the  automation  motors (DANCAM and

DANPLOT can control relays to turn things on,  but for safety  reasons

you  should  have a manual switch to control the master power supply.)

Run DANCAM or DANPLOT,  select menu option 1.,  and enter the name  of

the  tool  path,  e.g.  SOMEFILE.ASC  or  A:\SOMEFILE.ASC,  to execute

(execute means trace out the path drawn by moving the  tool  or  work-

piece relative to each other.) DANCAM or DANPLOT should then make your

part  for  you.  When the tool path is finished DANCAM or DANPLOT will

return the tool to home position and be ready  to  make  another  part

from  the  same tool path or another tool path file.  You can file the

floppy disk copy of the tool path so that if you ever need more parts,

or replacement parts,  all you have to do is shove the floppy disk  in

the  computer  and  enter the tool path name to make another identical

part.  If you have manufacturing stations around  the  world  you  can

send  tool  paths on floppy disks so that the remote stations can make

up parts on demand locally and  reduce  your  shipping  and  inventory

costs.

DOS  batch  files  can  be  used  to  operate  DANCAM  and DANPLOT

automatically to make several parts.  The machine  operator  DOES  NOT

NEED  TO  USE  ANY  OF THE MENUS in DANCAD3D,  DANCAM,  and DANPLOT to

[6]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

manufacture parts.  The machine operator does need to  understand  the

meaning  of  some of the messages that come on the screen (such as the

DOS PAUSE command telling him to press ANY-KEY when ready) or at least

know enough to alert a more knowledgeable operator when something goes

wrong.  The machine operator should also be told that the power driven

tool can move unexpectedly and hurt him,  therefore THE POWER  TO  THE

MOTORS  COILS SHOULD ALWAYS BE SWITCHED OFF BEFORE EXPOSING HIMSELF TO

THE EQUIPMENT.  Safety switches on the equipment access covers to turn

off the motor coil power are advisable to  help  reduce  injuries  (to

maintain  possitional  accuracy  the  logic  power to the motor driver

modules should not normally be switched off in the middle  of  a  tool

path  execution,  so when hooking up DANCAM and DANPLOT you should try

to wire up two separate power switches:  one for the motor coil power,

and the other for the motor driver module logic power.)

To summarize, using DANCAM and DANPLOT takes these steps:

PHASE ONE: HOOKING IT UP

1. Hook up automation motors and switches mechanically to machine.

2. Hoop up electrical connections to modules and switches.

3. Configure DANCAM and DANPLOT to fit your mechanical hook up.

4. Test the installation to make sure the motors are working.

PHASE TWO: COMPUTER AIDED DESIGN

1. Use DANCAD3D to draw tool paths for the tool paths needed.

2. Save the tool paths to floppy disks.

3. Write a  DOS  batch  file  to  control the manufacture of the parts

needed,  and save the batch  file  to  the  floppy  disk  with  the

corresponding tool path files.  Set up the AUTOEXEC.BAT on the shop

computer  so  that  the  user will be prompted to insert the floppy

disk with the tool path and batch file to  control  the  number  of

parts  to  make.  For  safety,  the  batch  file  should prompt the

operator to turn the motor power on  and  off  at  the  appropriate

points in the manufacturing cycle.

PHASE THREE: MACHINE OPERATION

1. Turn  power  on  to the shop computer.  The  AUTOEXEC.BAT file will

automatically  prompt  the  operator  to insert the tool path disk.

The AUTOEXEC.BAT file then runs the batch file  on  the  tool  path

floppy  disk  (in order to have the AUTOEXEC.BAT file run the batch

file of the floppy disk you would always use the same filename  for

the batch file on the floppy disk, e.g. GO.BAT.)

2. Operator inserts the stock and turns the motor power on.

3. Operators presses [Any-Key] to start making a part.

4. Operator is prompted to turn the power off, insert stock,  and turn

the  power  back  on  (it  is  important that a manual safety power

switch be used since automatic control of power may not be reliable

[7]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

(the manual control may not be  reliable  either,  but  the  manual

control of the power puts the responsibility for motor power on the

user/operator).)

5. The  manufacturing cycle continues under the direction of the batch

files  on  the  tool path floppy disks (i.e.  GO.BAT can prompt the

user to insert further floppy disks and run sub-batch files.)

Phase three is of course not necessary if you are going to be  the

operator  of the machine.  In the case that you are doing all the work

yourself you would save the data file on your harddisk  from  DANCAD3D

and then run DANCAM or DANPLOT, turn on the power for the machine, and

enter the filename of the tool path you saved to your harddisk.

[8]

[                  P R O G R A M   F E A T U R E S                   ]

PROGRAM FEATURES

DANCAM and DANPLOT both have several features that make setup  and

operation simple and relatively easy to accomplish:

* All commands displayed on screen.

* Menu driven configuration is adjustable to any units.

* Works with tool paths drawn in metric, inch, or other units.

* Motor testing utilities to check maximum motor speed under load.

* Jog menu lets you position tool manually and has position read out.

* Can be paused while making parts from keyboard or switch on port.

* Supports up to four auxiliary relays to control anything.

* Has automatic or manual home up to home switches.

* Limit switches can be used to protect against out of range movement.

* Can be operated from dos command line automatically.

* Can be automatically operated from DOS batch files.

* Uses user accessible DANCAD3D ASCII data file type.

* Overall scaling factor to adjust part size.

* Adjusts to any pitch lead screws or drive chain.

* High accuracy over large distances of tool travel.

* Works with any motor driver using step pulse and direction signals.

* Feed rates can be installed from rapid movement to very slow.

* Feed  rate  change,  auxiliary  relay  toggle,   and  pause  can  be

controlled automatically from line colors in the tool path file.

* Installable delay after relay toggle lets equipment settle down.

* Automatic backlash compensation when needed.

* DANCAM is fully three dimensional and can make any three axis shape.

* DANCAM and DANPLOT will run on almost any low cost PC compatible.

* Low cost simple connection to parallel port, LPT1, LPT2, OR LPT3.

When run on a 10 MHz 80286 computer DANCAM and DANPLOT can produce

about  3420  step  pulses per second which would run servo motors with

200 steps per revolution at a speed of about 1026  RPM.  Most  stepper

motors  have  a  reliable  top speed less than 200 RPM so when stepper

motors are used the reliable operation of the motors  limits  the  top

speed  obtainable  and  not  DANCAM  or  DANPLOT.  The  built  in rate

multiplier can be used to increase the step  angle  for  servo  drives

(i.e.  to  3.6  degrees) to get speeds of 2000 rpm or more since servo

motors are normally feed through a 6:1 or 8:1 reduction before driving

a 5 tpi lead screw,  giving about 300 RPM at the lead screw or a  feed

of 60 inches per minute.

[9]

[                      A P P L I C A T I O N S                       ]

APPLICATIONS FOR DANCAM AND DANPLOT

DANCAM  and  DANPLOT  are similar programs,  but each one has been

optimized  for  different  applications.   DANCAM  is   designed   for

applications  that require the tool or actuator to move between points

at any location in three dimensions.  DANPLOT is optimized for working

on flat material, e.g. sheet or plate, and will automatically lift the

tool or turn off the cutting flame or laser beam before moving to  the

next  point  in  the drawing file.  In other words DANCAM is used used

for applications like 3D milling, and DANPLOT is used for plotter like

work such as engraving and torch cutting.

Both DANPLOT and DANCAM can draw circles and curves of  any  type.

DANCAM  can  manufacture any part that can be produced on a three axis

machine tool.  The production of circles and  curves  takes  place  in

DANCAD3D  as  part of the preparation of the drawing of the tool path.

For complex contoured surfaces the tool path must define  points  that

are  more  closely  spaced  than for the tool paths of parts than have

flat surfaces.  In the event that the tool path is to complex  to  fit

entirely  in DANCAD3D's workspace at once,  the required tool path can

be drawn in sections,  and chained together for  use  with  DANCAM  or

DANPLOT.  DANCAM and DANPLOT can read tool path files of any length.

DANCAD3D  has  special  macro  commands  that can produce swept or

contoured surfaces.  The macro commands for DANCAD3D are explained  in

detail  in  the  DANCAD3D manuals and example disks that come with the

registered user outfit.

APPLICATIONS DANCAM CAN BE USED FOR

MAIN USES OF DANCAM

* Three axis fully Three-Dimensional vertical milling.

* Operation of lathes.

* Cutting foam and other materials with hot wires.

* Flame cutting where the torch height needs to be adjusted.

* Laser cutting where the laser height needs to be adjusted.

* Drilling holes to varying depths.

ADDITIONAL POSSIBLE USES FOR DANCAM

* Spray application where sprayer moves in three dimensions.

* Glue application where applicator moves in three dimensions.

* Welding, spot or arc.

* Parts insertion, screw insertion, or nailing.

* Automated warehouse loading and unloading.

* Three axis motion control photography.

* Automation of a motion picture animation stand.

* Dipping of parts for coating by use of a gripper.

* Processing film, PC boards, or micro chip wafers.

[10]

[                      A P P L I C A T I O N S                       ]

In addition to the linear  three  axis  applications  noted  above

DANCAM  can  be  used  to  have  one axis or more move in a non-linear

manner.  For instance the X axis could control a  rotating  table,  so

when  the  drawing for the motion of the X axis is drawn with DANCAD3D

the person drawing the tool path would use units of arc (i.e. 0 to 360

degrees) rather than units of distance (e.g.  0.001  inch  as  +/-  10

inches.)

For  lathes  the  third  axis in DANCAM can be used to control the

spindle speed by connecting a small stepper motor to the shaft  of  an

SCR speed control for the spindle motor.

THREE AXIS APPLICATIONS WHERE DANPLOT CAN BE USED

* Building a large pen plotter to plot drawings full life size.

* Engraving scales and lettering.

* Cutting fabric or leather from a pattern.

* Drilling printed circuit boards.

* Drilling holes to constant depth, slow down & fast up motion option.

* Flame cutting with automatic flame control on the Z axis.

* Laser cutting automatic laser on and off control on the Z axis.

* Automated routing of wood for cabinets and stair runners.

* Stamping or punching of sheet metal or other material.

* Glue, Grease, Potting compound, OR Oil application.

With  DANPLOT the third (Z) axis can be used in several ways:  you

can control a stepper motor,  you can control  a  relay,  or  you  can

control  a  solenoid.  The  normal  use of the stepper motor for the Z

axis would be to raise and lower the tool,  such that the tool is  not

in  contact  with  the  work-piece  when  the  tool  moves to the next

starting point if the line segments drawing in the file being executed

to not touch.  Additionally the Z axis stepper motor can  be  used  to

open  or  close  a valve or rheostat.  A relay on the Z axis direction

control line can be used to turn a laser or other electrical device on

or off.  A solenoid triggered by the Z axis could be used to stamp  or

punch the sheet material.

FOUR AXIS APPLICATIONS WHERE DANPLOT CAN BE USED

* Knife cutting of signs out of vinyl.

* Saw cutting of wood with a Saber, Band or Scroll saw.

* Cutting fabric or leather from a pattern.

* Broaching of key ways, groves, or gear teeth.

* Automating nibbling tools.

The  forth  or "C" axis (the c axis is rotation around the Z axis,

such that the cutting edge of the tool is parallel to the Z  axis)  in

DANPLOT  will rotate the cutting edge of a knife or recriprecating saw

so the cutting edge always faces into the direction of the motion  the

tool  as  the  tool  moves through the work-piece.  A tolerance on how

[11]

[                      A P P L I C A T I O N S                       ]

large a C axis angle change is required before the tool will  lift  is

available  to  speed  the  cutting  of curves and circles (normally in

knife cutting no harm comes if the knife rotates in the work-piece  as

circles  are  cut out,  but you will want (and DANPLOT will perform) a

tool removal,  tool rotation,  and tool  insertion  when  large  angle

changes (such as at a 90 degree corner) need to be made.

[12]

[                G E N E R A L   I N F O R M A T I O N               ]

GENERAL INFORMATION

To use DANPLOT and DANCAM you will need to create a text file that

contains information on the end points of line segments, such that the

line segments define the path you want your tool (or work-piece as the

motion  between  the  tool and work-piece is relative) to move through

space.  The easiest way to create the necessary file of  line  segment

data  is  to  use DANCAD3D v2.5 to draw the line segments in the order

you want them and in the direction you want  the  motion  to  proceed.

The  necessary  line  segment  data  file can also be made with a text

editor or by a data file conversion program  (to  convert  some  other

data filetype such as DXF, HPGL, or "G" code, into DANCAD3D v2.5 *.ASC

file format.) Information about the DANCAD3D ASCII data file format is

given in the DANCAD3D technical reference manual, but is not needed to

use  DANCAM  and  DANPLOT  since  DANCAD3D will create the proper file

format for you automatically.

DANPLOT and DANCAM are very similar programs but  have  each  have

special  features  to  make  them work better for their intended task.

When you  install  DANCAM  and  DANPLOT  by  using  the  configuration

utilities  accessed  from  DANCAM's  or DANPLOT's main menu you should

keep in mind the different applications that the programs are designed

for.

DANPLOT is designed for use for working on flat or sheet material.

The Z axis for DANPLOT has two states,  up or  on  and  down  or  off.

DANPLOT  can  be  used  to  build  large  plotters  for plotting large

drawings.  A "C" axis motor can  be  used  with  its  rotational  axis

parallel to the the Z axis in order to point the cutting tool into the

direction of the motion of the cut (i.e. for use with knifes and saber

saws.)  The Z (pen) axis motor can be replaced with a relay to control

(toggle) a laser or torch automatically (the Z axis direction  pin  on

the  port  connector  indicates  the state,  on or off.) Only the line

segments that will appear on the work-piece need to  be  in  the  data

file  since DANPLOT will automatically lift the tool clear (turns off)

of the work-piece when moving between the drawn line segments and when

the tool path is done.  Tool paths  for  use  with  DANPLOT  have  all

points in the Z = 0 plane.

DANCAM  is designed for use for working on 3D parts where the tool

moves in all three dimensions.  The Z axis in DANCAM is equal  to  and

interchangeable  with to the X axis and Y axis.  DANCAM is also useful

for 2 axis work where the full motion of the tool will be  drawn  out.

With DANCAM it is best to draw all motions out fully, including a safe

path  to the start of cutting,  controlled feed rate for the cuts (the

change in feed rate is accomplished by changing the line color in  the

tool  path  drawing),  and  a safe path from the final cut back to the

home position (you can draw every other line segment in the tool  path

for  DANCAM  to  reduce  the total number of line segments in the data

file,  if you keep in mind that DANCAM will always take  the  shortest

path through three dimensions to the next first point of the next line

segment.)

[13]

[                G E N E R A L   I N F O R M A T I O N               ]

When  making a tool path files for use with DANCAM or DANPLOT keep

the differences between the way the programs carry out  the  execution

of line segments in the drawing file in mind.

DANPLOT  will  read  two  types  of file (ASCII and Plotter driver

output) made with DANCAD3D.  You can draw a tool path as a set of line

segments or as a set of dots (holes to be drilled can be indicated  by

dots)  with  DANCAD3D  without  connecting the dots since DANPLOT will

automatically lift the tool before moving  to  the  next  point  (i.e.

DANPLOT works like a pen plotter, such that the pen will automatically

lift  and  move  to  the  next  line  segment without drawing unwanted

connecting line segments that  were  not  in  the  DRAWING  made  with

DANCAD3D.)  DANPLOT  can also read special plotter driver output files

made in DANCAD3D with the DANPLOT.PLT plotter driver that  accompanies

DANPLOT  on  this  disk.  The plotter driver DANPLOT.PLT would be used

for applications  such  as  engraving  that  require  DANCAD3D's  line

thickness  attribute  to be carried accurately to the finished result.

For  all  applications  other  than  engraving  use  DANCAD3D's  ASCII

filetype for tool paths that will be used with DANPLOT.

DANCAM works like DANPLOT except the movement of the tool is fully

3D,  with  all  3  axis  motors being simultaneously controlled by the

lines you draw with DANCAD3D.  DANCAM only  reads  the  DANCAD3D  v2.5

ASCII file format.

DANCAM  is  useful for most 3 axis tasks such as vertical milling.

You do however need to draw the tool path fully as a continuous series

of line segments.  If you skip a line segment  DANCAM  will  take  the

shortest  path  to  the  next  given  point  or  to  the home position

currently installed in DANCAM's configuration menu if there is no next

point to go to.  So when you use DANCAM you should always draw all the

movements you want the tool to make,  including the safe path clear of

the  work-piece  when  done (in DANPLOT to tool automatically lifts to

clear  the  work-piece  when  done  and  goes  home,  DANCAM  can  not

automatically  know what a safe return path is since dovetail mills or

other odd shape cutters may be in use (DANCAM will take  the  shortest

path  from  the  last point drawn to home when done,  so make sure the

last point in the tool path is clear of the work-piece!).)

HOW TO USE DANCAM AND DANPLOT

In general the use of DANCAM and DANPLOT follows these steps:

1. Build  or buy a machine to automate.  Generally you will  want  the

lead  screws that have about 5 threads per inch so that the stepper

motors 200 or 400 steps per shaft rotation will give  a  resolution

of  0.001  or  0.0005  of  an inch.  Chain or timing cable drive in

place of a lead screw  is  an  excellent  choice  for  drives  that

require  less  resolution  and  faster  feed  rates.  Always try to

minimize inertia in components that must be driven, i.e.  use small

[14]

[                G E N E R A L   I N F O R M A T I O N               ]

radius aluminum pulleys.

2. Install stepper motors  on  your  machine.   Use  timing  belts  or

flexible  shaft  (bellows)  couplings  between the motor shafts and

lead screws to reduce noise and lost steps caused by  vibration  in

the  motor  coupling.  Stepper  motors  get  hot  (80C  to  85C) in

operation so proper mounting would allow  the  conduction  of  heat

from  the  motor  and  circulation of cooling air.  Used or surplus

stepper motors can be purchased for a fraction of what  new  motors

cost.  In general stepper motors should have a rated holding torque

that is at least FOUR TIMES THE TORQUE that will be required at the

operating  RPM (the torque of stepper motors falls off as the motor

turns faster,  so that a motor  rated  at  500  inch/ounce  holding

torque  might  only  have  200  inch/ounce at 60 RPM,  and only 100

inch/ounce at 120 RPM,  with the torque falling completely to  zero

at  180  RPM.)  Using  stepper  motors  that are two small,  and or

running the motors faster than they can reliable go  is  the  major

cause  of  problems  people have with stepper motors.  Also stepper

motors need to be operated at 4 to 6 times their rated  voltage  to

give  satisfactory  results  (a constant current stepper translator

module or resistors in series with the motor windings are  required

to  prevent  the  motor  from  overheating  when  operated  at high

voltages, i.e. 48 to 96 volts.)

3. Buy  or build the stepper motor translator modules and  motor power

supplies (instructions for building the  electronics  required  are

included  in  the  DANCAD3D  v2.5 outfit's manuals.) Stepper motors

generally operate with two coils (phases) on so the current draw is

double the rated current,  e.g.  three motors rated at 2 Amps  will

require  a  power  supply  that  can deliver at least 12 amps.  The

stepper driver translator modules you will need are available  form

your  local  motor  distributor.  Most  stepper  motor manufactures

supply driver modules optimized for  their  own  brand  of  motors.

Surplus  or  used stepper motors can be purchased for a fraction of

what new motors cost,  e.g.  500 inch ounce motors can be found for

less  than  $75.  If  you want to use solid state relays to control

the spindle of your machine or other  auxiliary  device  buy  solid

state  relays  that are rated for input of 3 volts DC or less since

the parallel port cannot supply more than 3 volts (if more power is

needed than the parallel port can supply a TTL buffer chip such  as

the 74H07 and pull-up resistors (approximately 330 ohm) can be used

amplify  the signal.) You can build your own stepper driver modules

to drive three five amp motors for about $100 or less depending  on

how fancy you want to get.  Ready made translator modules cost from

$50 to $600 per axis depending on the power and speed required.

4. Use   the  diagram  HOOKUP1.2D  on  the  DANCAD3D v2.5 shareware or

program  disks  to  connect  the  stepper  motor  translator driver

modules to your computers parallel port.  If four auxiliary  relays

are  to be used a 5 volt 500ma power supply (about a $10 item) will

be required for the pull-up bias,  otherwise the auxiliary C and  D

outputs can be used to pull-up the home and limit switch inputs (X,

Y,  and  Z  home  switch  inputs  being  pulled  up through 10K ohm

[15]

[                G E N E R A L   I N F O R M A T I O N               ]

resistors to auxiliary C output,  and the pause/limit  input  being

pulled up through a 5K ohm resister to the auxiliary D output pin.)

5. Run   DANCAM  and  DANPLOT  and  install  the  values for speed and

resolution  that correspond to your machine.  Test the motors under

load to make sure they can keep up with  the  movements  they  will

need  to  make.  If the motors "louse steps" (that is slip and fail

to rotate to the proper position) under load  use  a  larger  Pulse

With  Factor  (p.w.f.)  to give the motor time to complete one step

before another is requested (if a stepper motor louses steppes  the

rotor will no longer have the proper rotational position (until the

motors  are  sent  home again which is a good reason for installing

the home switches.)) Reducing the motor current to 80% of the rated

current can sometimes reduce lost steps caused by resonance in  the

middle speed range.

6. Run DANCAD3D,   draw a tool path, and save the tool path drawing as

an ASCII data file (or a plotter driver output file if you need the

line width feature for DANPLOT.)

7. Run  DANCAM or DANPLOT,  select menu option 1,  and enter the  file

name of the ASCII (*.ASC)  tool  path  file  you  just  saved  from

DANCAD3D.  If  you  did  the  installation properly your first part

should come out as designed.  If  the  first  part  is  not  proper

adjust the installation and or the tool path drawing and try again.

Keep  in  mind that DANCAM and DANPLOT can run from the DOS command

line and therefore from DOS batch files.  Operators of equipment do

not need to use any of DANCAM or  DANPLOT's  menus  since  you  can

write  a  batch  file  to  read  the  data  file from a floppy disk

automatically.

[16]

[   D O S   C O M M A N D   L I N E   A N D   B A T C H   F I L E S  ]

EXAMPLE OF HOW TO RUN DANCAM OR DANPLOT FROM THE DOS COMMAND LINE

To  operate  DANCAM  or  DANPLOT  from the DOS command line simply

enter DANCAM or DANPLOT followed by a space and then the name  of  the

data  file  to  be  plotted and then followed by another space and the

number 1 (or 1 or 2  for  DANPLOT.)  The  number  following  the  data

filename  is  equilevent  to  the  menu option number from DANCAM's or

DANPLOT's main menu.

EXAMPLE: C>DANCAM A:DATAFILE.ASC 1

C>DANPLOT A:DATAFILE.ASC 1

If you want to have one of the other menu options come up from the

DOS command line you can use the dummy filename NUL.

EXAMPLE: C>DANCAM NUL 3

C>DANPLOT NUL 3

EXAMPLE OF BATCH FILES THAT AUTOMATICALLY RUN DANCAM OR DANPLOT

The following DOS batch file will make parts from data files saved

on floppy disks.  Note that all the data files should  have  the  same

name  (when  you  save  the  DOS  batch file you should always use the

filename extension *.BAT.)

EXAMPLE: :START

ECHO OFF

CLS

ECHO PUT TOOL PATH DISK IN DRIVE A:

PAUSE

DANCAM A:DATAFILE.ASC 1

GOTO :START

Since the floppy drives are somewhat slow you can get  faster  and

smoother  plots by coping the data file to a harddisk or RAM disk (the

delay for the floppy drive to come up to speed when data is being read

will cause the cutter to idle for a  moment  periodically,  a  problem

that  can  be  avoided by reading the data file from a harddisk or RAM

disk.)

EXAMPLE: :START

ECHO OFF

CLS

ECHO PUT TOOL PATH DISK IN DRIVE A:

PAUSE

COPY A:DATAFILE.ASC C:\DC25\*.*

DANCAM C:\DC25\DATAFILE.ASC 1

GOTO :START

You can have DANCAM automatically run after you quit  DANCAD3D  by

using  a DOS batch file.  To break a loop in a DOS batch file you need

[17]

[   D O S   C O M M A N D   L I N E   A N D   B A T C H   F I L E S  ]

to press [Control] & [C] while the batch file is operating.

EXAMPLE: :START

CLS

DANCAD3D

DANCAM

GOTO :START

[18]

[              H A R D W A R E   R E Q U I R E M E N T S             ]

HARDWARE REQUIREMENTS

Although DANCAD3D requires a fast computer and  harddisk,  DANPLOT

and DANCAM can be used with a less expensive system.  One advantage of

using  a  less  expensive  computer  in  the  shop  with the automated

equipment is that if the shop computer is damaged you will not have to

spend much to repair it.  In applications requiring higher motor shaft

speeds (i.e.  Greater than about 100 RPM) a 80286  replacement  mother

board  can  be put in an XT type case (replacement 80286 mother boards

only cost about $140 or less.) Since a 10MHz 80286 XT size  AT  mother

board  can  run  as much as 8 times as fast as a 4.7MHz 8088 XT mother

board it is very much worth the slight  added  cost  to  use  a  80286

mother board for DANCAM or DANPLOT.

DANCAM  and DANPLOT will work on almost any IBM (tm) compatible PC

that has one floppy drive and a IBM (tm) PC  standard  parallel  port.

No  special  hardware  is  required  for  the computer.  Many standard

stepper motor driver modules and solid state relays can  be  connected

directly to the parallel port.  The signals from the parallel port can

be  boosted  with  inexpensive parts to drive almost any other stepper

driver module or relay that requires more power  than  the  power  the

parallel port can supply.

COMPUTER: 100% IBM PC-XT or AT compatible.

PORTS:    One Parallel Port is required, i.e. LPT1, LPT2, or LPT3.

VIDEO:    Any 80 column by 25 line video text display.

MEMORY:   192KB RAM DOS system memory, 128KB or more free.

DRIVES:   1 floppy disk drive.  Reading the tool path data file from a

RAM  disk  can  avoid  periodic  short  interruptions due to

blocks of data being loaded from the data file.  If you have

640KB on the computer you are using for DANCAM or DANPLOT  a

RAM  disk  program  to  make  a 448KB RAM disk from your DOS

system memory and have enough left over to run  DANPLOT  and

DANCAM  (you  would  copy the ASCII tool path data file into

the RAM disk,  and have DANPLOT of DANCAM read the tool path

from the RAM disk drive.) A RAM disk is created by running a

small utility program before you run DANCAM or DANPLOT.  RAM

disk utility programs are available from ShareWare dealers.

CABLES:   You will  need a parallel port cable.  If you have a printer

connected  to  your  computer  you  can  use that cable.  To

connect the wires to the translator modules you will need  a

Centronics 36 pin connector that will mate with your printer

cable.  You  can  then  solder  wires  onto  the  Centronics

connector rather than your printer cable and that will allow

you to disconnect the  printer  cable  from  the  Centronics

connector  at  any  time  to  use  the printer cable on your

printer (reboot your computer after using DANCAM or  DANPLOT

[19]

[              H A R D W A R E   R E Q U I R E M E N T S             ]

before  you  use  your printer.) If you buy a printer switch

box make sure all of the needed  pins  are  connected  since

some cheaper switch boxes may not connect all of the pins.

The  wire going between the parallel port pins and the input

pins  of  the  translator  modules   should   be   shielded.

Microphone  or  video  coaxial  cable  can  be used with the

shields of the two cables being tied to  common  (one  cable

for  the  step  signal,  and  one  cable  for  the direction

signal.) Electro-Magnetic noise can be a problem because  of

a  high current that flows through the wires that connect to

the motor power supply and to the motors themselves.  Always

keep the step and direction cables as separated and far away

from the motor coil wires as you can.  Putting  a  2.2K  ohm

pull  up  resistor from the step and direction inputs of the

translator module to a positive five volt  regulated  supply

can help reduce the input impedance of the translator module

and therefore the voltage of induced electro-magnetic noise.

Opto  isolators,  or LC or RC low pass filters on the inputs

of  translator  modules  can  also  help  reduce   incorrect

triggering  of  the  translator modules since opto isolators

and filters reduce the input impedance.  To filter the  step

and  direction signals at the input of the translator module

connect a 1000  pf  capacitor  from  the  input  to  common,

connect a 2.2K ohm resistor from the input to +5 volts,  and

connect  a  100  ohm  resister  between  the  input  of  the

translator  and  the  parallel  port  signal.  If adequately

shielded and filtered the parallel port to translator  cable

can be as long as 50 feet.

Wire  for connecting the motors should be stranded and large

enough to handle the current required,  i.e.  14 to 16 gage.

The wires for the limit and home switches can be rather thin

(22  gage)  since  very  little  current  flows  through the

switches.

MODULES:  Stepper motor translator driver modules for stepper  motors.

Hundreds of different driver modules are available to  drive

different types of stepper motors.  You will want TTL signal

compatible  modules  that  have MOTOR STEP (pulse) and motor

DIRECTION (cw/ccw) inputs.  Since steeper motors need to  be

operated at voltages above what the motors are rated for the

stepper  motor  translator  you  purchase  should offer some

constant current or current  cutback  feature  to  keep  the

motors  from  burning  up  when  they  stop turning (stepper

motors get hottest when run  slowly  or  are  stopped.)  Two

types  of  translator  module that give good performance are

Bipolar chopper and Bilevel  Bipolar.  Bipolar  drives  give

more  torque  at medium speeds than Unipolar driver modules.

You should check that the stepper motors you  are  going  to

use  are  compatible  with  the driver type you want to use.

Regardless of the type of  module  selected  the  top  motor

speed will be less than the motor's best if the motor supply

[20]

[              H A R D W A R E   R E Q U I R E M E N T S             ]

voltage  is  less  than  five times the rated motor voltage,

e.g.  a motor rated at 5 volts will run well with a 25  volt

supply,  but  a  25 volt motor will need a supply voltage of

125 volts to run well.  The maximum  rated  voltage  of  the

motor  driver module of course limits the supply voltage you

can use.  Modules also have a  rated  range  of  motor  coil

current  they  can  handle  (the coil current for the motors

should never exceed the rated current!)

RELAYS:   Solid  state  relays  such  as  the  HAMLIN  model  7521D or

equivalent can be used to control auxiliary devices directly

form  the  parallel  port.  The  solid  state  relay  should

operate  from  a  3 volt or smaller signal.  Solenoid relays

can be controlled from  the  parallel  port  if  you  use  a

transistor  and  DC  power  supply (instructions for circuit

details of such advanced wiring are in the DANCAD3D outfit's

technical information.)

POWER:    External  power  supply.   You  will  need  a  power  supply

powerful enough to drive all  the  stepper  motors  you  are

using.  For medium duty tasks this would be 20 to 50 amps at

35  to  96  volts (stepper motors are run at voltages 4 to 6

times their rated voltage, so if you have motors rated at 10

volts you would want to have at least a 50 volt power supply

(how high the power supply  voltage  can  be  is  of  course

restricted  by the maximum voltage of the translator modules

you will be using.)) To determine the total current required

figure that two  coils  will  be  drawing  current  in  each

stepper  motor so if your motors are rated at 1 amp per coil

and you have 3 motors you will need 6 amps plus another 4 to

6 amps for "headroom" due to the dynamic nature of the motor

load.  To assure that the motors can  get  all  the  current

they need while stepping I recommend that you put a 20000 mf

capacitor  across  the  power  supply  input of each of your

motor driver modules.  A 5 volt 500 ma power supply may also

be required to bias any pull-up resistors required.

SWITCHES: Three Normally Open (N.O.) micro switches for home switches.

Six  Normally  Closed  (N.C.)  micro  switches   for   limit

switches.  One  Normally  Closed  (N.C.)  toggle  switch for

pause switch.  One Normally  Open  (N.O.)  push  button  for

bypass button.

RESISTOR: Five 2.2K ohm 1/4 watt,  for pull-up of  the  switch  inputs

(pull-up  resistors  connect  from  TTL logic IC's inputs or

outputs to a plus five volt power supply  (the  common  from

the  five  volt  power supply would of course connect to the

logic common or ground.)) Since the switches connect between

the input pins of the parallel port and common  the  pull-up

resistors  are  needed  to  insure  that  the input pins are

always at logic high (logic high is 2.8 to  5.0  volts.)  If

you do not want to use a special 5 volt supply for the limit

and  home  switch  inputs  you  can  use  10K  ohm  1/4 watt

[21]

[              H A R D W A R E   R E Q U I R E M E N T S             ]

resistors from the input pins to auxiliary outputs C  and  D

(of  course auxiliary C and D cannot then be used for relays

since they will need to be logic high all the time.)

HARDWARE: To mount the motors you will need flexible  shaft  couplings

(metal bellows type) or timing  belts  and  pulleys.  It  is

important  that  the coupling used on the stepper motor have

about 0.5 degree of give (elasticity) so that the motor  can

step easily and smoothly and without excessive vibration.

[22]

[                 H O O K   U P   C O N E C T I O N S                ]

HOOK UP CONNECTIONS TO THE PARALLEL PORT

The complete schmatic for the basic DANCAM and DANPLOT hook up  is

provided  in  drawing  file  HOOKUP1.2D on the DANCAD3D v2.5 shareware

program disks.  Printing out the HOOKUP1  drawing  will  make  reading

this  section  much more understandable.  Additional schmatic diagrams

are included in the DANCAD3D outfit.  This following information gives

all the basic connections for using DANCAM and DANPLOT.

DANPLOT and DANCAM use the same basic connections to the  pins  of

the parallel port.  The basic connections to the 36 pin printer end of

the Centronics parallel port cable are:

Pin No.

36 18

35 17

34 16

TIE TO COMMON - 33 15

AUXILIARY INPUT - 32 14

AUXILIARY RELAY "D" OUTPUT - 31 13 - Z AXIS HOME SWITCH INPUT

TIE TO COMMON - 30 12 - Y AXIS HOME SWITCH INPUT

TIE TO COMMON - 29 11 - X AXIS HOME SWITCH INPUT

TIE TO COMMON - 28 10 - LIMIT & PAUSE SWITCH INPUT

TIE TO COMMON - 27 09 - AUXILIARY RELAY "B" OUTPUT

TIE TO COMMON - 26 08 - Z AXIS DIRECTION OUTPUT

TIE TO COMMON - 25 07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 24 06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 23 05 - AUXILIARY RELAY "A" OUTPUT

TIE TO COMMON - 22 04 - Z AXIS STEP PULSE OUTPUT

TIE TO COMMON - 21 03 - Y AXIS STEP PULSE OUTPUT

TIE TO COMMON - 20 02 - X AXIS STEP PULSE OUTPUT

TIE TO COMMON - 19 01 - AUXILIARY RELAY "C" OUTPUT

All  of  the  input pins must be "pulled up" to logic high through

2.2K ohm 1/4 watt resistors to a +5 volt regulated power  supply  (you

should be able to find a +5 volt 500 milliampere power supply for less

than $10 (you can use three flashlight 1.5v dry cells in series to get

about  4.5  volts,  but  NEVER  use  more  than 5 volts or less than 3

volts.)) ALWAYS check that the MINUS (-) end of the five  volt  supply

connects  to the COMMON point,  since reversed connections will damage

your parallel port and possibly other parts of your computer.  If  you

do  not  want to bother with a +5 volt power supply and do not need to

use the auxiliary outputs  "C"  and  "D",  you  can  usually  use  the

auxiliary  outputs "C" and "D" to pull up the switch inputs,  as shown

here:

[23]

[                 H O O K   U P   C O N E C T I O N S                ]

Pin No.

36 18

35 17

34 16

TIE TO COMMON - 33 15

AUXILIARY INPUT - 32 14

PULL UP FOR PIN 10 & 32 - 31 13 - Z AXIS HOME SWITCH INPUT

TIE TO COMMON - 30 12 - Y AXIS HOME SWITCH INPUT

TIE TO COMMON - 29 11 - X AXIS HOME SWITCH INPUT

TIE TO COMMON - 28 10 - LIMIT & PAUSE SWITCH INPUT

TIE TO COMMON - 27 09 - AUXILIARY RELAY "B" OUTPUT

TIE TO COMMON - 26 08 - Z AXIS DIRECTION OUTPUT

TIE TO COMMON - 25 07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 24 06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 23 05 - AUXILIARY RELAY "A" OUTPUT

TIE TO COMMON - 22 04 - Z AXIS STEP PULSE OUTPUT

TIE TO COMMON - 21 03 - Y AXIS STEP PULSE OUTPUT

TIE TO COMMON - 20 02 - X AXIS STEP PULSE OUTPUT

TIE TO COMMON - 19 01 - PULL UP FOR PINS: 11, 12, & 13

The value of the pull up resistor will need to be increased to 10K

ohm when auxiliary "C" and "D" are used as the pull up source to avoid

over loading the parallel port.  When the auxiliary input on pin 32 is

not used a 4.7K ohm resistor can be connected between pin 31  and  pin

10 in place of a 10K ohm resistor.

The home switches are normally open (N.O.) and connect between the

home  switch  input pins and the common (ground) point.  The six limit

switches,  and pause  switch,  are  normally  closed  (N.C.)  and  are

connected  in  series between pin 10 and the common point.  The bypass

push button for  the  limit  switches  is  normally  open  (N.O.)  and

connects  from  pin  10  to  the  common  point.  If  one of the limit

switches opens (due to out-of-range travel, the motors will stop,  and

an  error  message  will  come  up  on  the computer screen) press the

[Control] and [X] keys on the keyboard and then press the limit switch

bypass button you  wired  from  pin  10  to  the  common  point.  When

installing  the  limit  switches at the home end of the travel be sure

that the limit switches become open several motor steps AFTER the home

switches have closed (otherwise the motors will stop before  the  home

position is reached!) The WINDOW command in DANCAD3D should be used to

clip  the  tool path before you save the tool path to an ASCII file to

avoid out-of-range motions.

DANPLOT can be hooked up in  some  additional  configurations.  In

order  to  use  the C axis the auxiliary relay "A" and "B" output pins

are used for the C axis step and direction signals,  and the auxiliary

input is used for the C axis home switch:

[24]

[                 H O O K   U P   C O N E C T I O N S                ]

Pin No.

36 18

35 17

34 16

TIE TO COMMON - 33 15

C AXIS HOME SWITCH INPUT - 32 14

AUXILIARY RELAY "D" OUTPUT - 31 13 - Z AXIS HOME SWITCH INPUT

TIE TO COMMON - 30 12 - Y AXIS HOME SWITCH INPUT

TIE TO COMMON - 29 11 - X AXIS HOME SWITCH INPUT

TIE TO COMMON - 28 10 - LIMIT & PAUSE SWITCH INPUT

TIE TO COMMON - 27 09 - C AXIS DIRECTION OUTPUT

TIE TO COMMON - 26 08 - Z AXIS DIRECTION OUTPUT

TIE TO COMMON - 25 07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 24 06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 23 05 - C AXIS STEP PULSE OUTPUT

TIE TO COMMON - 22 04 - Z AXIS STEP PULSE OUTPUT

TIE TO COMMON - 21 03 - Y AXIS STEP PULSE OUTPUT

TIE TO COMMON - 20 02 - X AXIS STEP PULSE OUTPUT

TIE TO COMMON - 19 01 - AUXILIARY RELAY "C" OUTPUT

You still have auxiliary outputs "C" and "D" available if you need

to  have  control  relays  and  use  a  +5 volt supply for the pull up

resistors on the switch inputs.  So the  C  axis  above  hook  up  can

connect to four motors and two relays.

Another  option  when  hooking  up  DANPLOT  is  to use the Z axis

direction bit to control a relay or solenoid  rather  than  a  stepper

motor.  This  option  gives you the possibility of two motors and five

relays:

Pin No.

36 18

35 17

34 16

TIE TO COMMON - 33 15

AUXILIARY INPUT - 32 14

AUXILIARY RELAY "D" OUTPUT - 31 13 - Z AXIS HOME SWITCH INPUT

TIE TO COMMON - 30 12 - Y AXIS HOME SWITCH INPUT

TIE TO COMMON - 29 11 - X AXIS HOME SWITCH INPUT

TIE TO COMMON - 28 10 - LIMIT & PAUSE SWITCH INPUT

TIE TO COMMON - 27 09 - AUXILIARY RELAY "B" OUTPUT

TIE TO COMMON - 26 08 - Z AXIS RELAY OUTPUT

TIE TO COMMON - 25 07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 24 06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 23 05 - AUXILIARY RELAY "A" OUTPUT

TIE TO COMMON - 22 04

TIE TO COMMON - 21 03 - Y AXIS STEP PULSE OUTPUT

TIE TO COMMON - 20 02 - X AXIS STEP PULSE OUTPUT

TIE TO COMMON - 19 01 - AUXILIARY RELAY "C" OUTPUT

[25]

[                 H O O K   U P   C O N E C T I O N S                ]

When using a relay on the Z axis you need to set the default state

for the Z axis direction bit and the rotation of the Z axis motion  so

that the relay is off before the tool path starts and is automatically

set off when the tool path ends, by using the configuration setup menu

option  number  4.  from  the DANPLOT and DANCAM main menus.  Normally

this means default to logic low and have the Z axis motion -10 to -200

steps.  The pulse width factor for the Z axis can be used  to  control

the  delay after the Z direction bit changes before the X and Y motors

start to turn (to allow for the time the Z axis relay/device takes  to

respond.)

You  can  of course use the C axis motor and a Z axis relay if you

want to (three relays and three motors.) You can  omit  the  home  and

limit switches if you wish,  since both DANCAM and DANPLOT can operate

without the switches connected.  The minimum hook up would be:

Pin No.

36 18

35 17

34 16

TIE TO COMMON - 33 15

32 14

31 13

TIE TO COMMON - 30 12

TIE TO COMMON - 29 11

TIE TO COMMON - 28 10 - TIE TO COMMON

TIE TO COMMON - 27 09

TIE TO COMMON - 26 08 - Z AXIS DIRECTION OUTPUT

TIE TO COMMON - 25 07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 24 06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 23 05

TIE TO COMMON - 22 04 - Z AXIS STEP PULSE OUTPUT

TIE TO COMMON - 21 03 - Y AXIS STEP PULSE OUTPUT

TIE TO COMMON - 20 02 - X AXIS STEP PULSE OUTPUT

TIE TO COMMON - 19 01

If you wish to connect directly to the pins on your  parallel  port

card  25  pin  connector rather than use the printer end of a parallel

port cable the connections are:

[26]

[                 H O O K   U P   C O N E C T I O N S                ]

Pin No.

13 - Z AXIS HOME SWITCH INPUT

TIE TO COMMON - 25

12 - Y AXIS HOME SWITCH INPUT

TIE TO COMMON - 24

11 - X AXIS HOME SWITCH INPUT

TIE TO COMMON - 23

10 - LIMIT & PAUSE SWITCH INPUT

TIE TO COMMON - 22

09 - AUXILIARY RELAY "B" OUTPUT

TIE TO COMMON - 21

08 - Z AXIS DIRECTION OUTPUT

TIE TO COMMON - 20

07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 19

06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 18

05 - AUXILIARY RELAY "A" OUTPUT

TIE TO COMMON - 17

04 - Z AXIS STEP PULSE OUTPUT

AUXILIARY RELAY "D" OUTPUT - 16

03 - Y AXIS STEP PULSE OUTPUT

AUXILIARY INPUT - 15

02 - X AXIS STEP PULSE OUTPUT

14

01 - AUXILIARY RELAY "C" OUTPUT

You can of course adjust connections on the 25  pin  connector  to

correspond to any of the alternate hook-ups, e.g.  use auxiliary input

on pin 15 for the C axis home switch,  and or use the auxiliary  relay

"D"  output on pin 16 to pull up pin 10.  As you have probably noticed

pins 1 through 13 have the same connections on both the parallel  port

25 pin connector and the 36 pin parallel printer cable connector.

[27]

[            C O N F I G U R E   A N D   I N S T A L L               ]

CONFIGURE AND INSTALL

DANCAM  and  DANPLOT send messages to the motor translator modules

in the form of what are called "steps".  The motor shaft  rotation  is

divided into fractions, usually 1.8 degrees per step, or 200 steps per

360  degrees (360/200=1.8).  Sometimes motors are run "half step" mode

where the step is divided in half,  e.g.  400 per  revolution  of  the

motor  shaft.  Micro  stepping  divides  the  motor  step into smaller

parts,  but does not usually offer higher actual possitional  accuracy

under  fluctuating loads since the motor shaft of a stepper motor will

typically exhibit 0.9 degrees of possitional error when  driven  under

full  load  from clockwise,  and then counter-clockwise rotation.  For

servo motors the shaft encoder defines the minimum rotational interval

or "step."

Note that the unit name inches is given in the configuration menus

in DANCAM and DANPLOT even though any units can be used so long as all

of the set up is done in the same units of measurement.  This was done

to avoid the confusion some users have about the meaning of  the  word

units.  You  will  also  have to set the overall scaling factor in the

configuration menu of DANCAM  and  DANPLOT  to  relate  to  the  units

scaling  factor used while drawing the tool path in DANCAD3D.  If this

gets confusing,  just set the global scale  in  DANCAD3D  at  240  and

configure  DANCAM and DANPLOT in steps per inch,  and set the DANCAM -

DANPLOT overall scaling factor to  1  (then  one  unit  in  DANCAD3D's

drawing  editor  will  move  the  tool  one  inch (this is the default

configuration.))

So since the motor shaft has finite positions that it can stop at,

the driven load will also have finite positions it can stop  at.  When

working  out  the  ratio  of the timing belt pulleys for the motor and

lead screw you should figure that one motor step should move the work-

piece relative to the tool by one half the smallest amount  necessary.

If  you  are  machining  to  0.001 of an inch and have a 10 pitch lead

screw then 1:1 coupling of the stepper  motor  will  give  you  0.0005

inches of movement per motor step, i.e.:

200 steps per revolution

--------------------------  =  2000 steps/inch  =  0.0005 inches/step

0.1 inch per revolution

Successfully  installing  DANCAM  or  DANPLOT  requires having the

motors  installed  to  give  the  coarsest  resolution  that  will  be

acceptable.  Excessively  fine resolution will limit the maximum speed

the tool can move (because the steps per second is constant.) Reducing

the resolution will increase the maximum speed the  tool  can  travel.

In  our example we can calculate the maximum tool feed speed in inches

per minute from the motor RPM, since with 1:1 drive the tool will move

0.1 inch per revolution, and 120 RPM would be a reliable maximum speed

for the motor shaft.

[28]

[            C O N F I G U R E   A N D   I N S T A L L               ]

120 revolutions per minute

--------------------------  =  12 inches per minute

10 revolutions per inch

Feed rates for other resolutions can be worked out for  the  motor

shaft speed of 120 RPM and 1:1 shaft coupling:

LEAD SCREW    FEED/REV   STEPS/INCH   INCH/STEP   INCHES/MINUTE

2.5 tpi       0.4"         500        .002           48

5.0 tpi       0.2"        1000        .001           24

10.0 tpi       0.1"        2000        .0005          12

20.0 tpi       0.05"       4000        .00025          6

Since  the torque of stepper motors falls off the faster the motor

turns you may only be able to get reliable operation at speeds  slower

than  60 RPM.  In general doubling the voltage used to drive a stepper

motor will increase the top speed by 50%,  i.e.  if a motor works well

at  60  RPM  at  24  volts  you  can  probably  get it up to 90 RPM by

increasing the voltage to  48  volts.  The  top  voltage  for  stepper

motors is about five times the rated voltage,  you will of course need

to limit the current to the rated current to  avoid  burning  out  the

motor (in the simplest scheme a series power resistor is used to limit

the  current  flowing  through the motor winding when the motor is not

turning to the rated current,  because the motor is an inductive  load

the  resistor  will allow the voltage to rise briefly at the beginning

of each motor step before the voltage drops back to the static  level,

thereby making the motor turn faster.)

When  building  a  plotter you can drive the pen holder by using a

timing belt driven directly by a pulley.  Small motors  can  generally

run at 240 RPM in half step mode to give 1600 (half) steps per second.

PULLEY DIAMETER    FEED/REV   STEPS/INCH   INCH/STEP   INCHES/SECOND

0.25"         0.785"      509.55      0.00196"     3.14159"

0.50"         1.570"      254.65      0.00392"     6.28318"

0.75"         2.356"      169.76      0.00589"     9.42477"

0.10"         3.141"      127.32      0.00785"    12.56636"

When  selecting  stepper motors to drive your equipment you should

use motors that have a rated holding torque AT LEAST  FOUR  TIMES  the

torque  that  will be required under load and at full speed.  Also use

the motor testing utilities built into DANCAM and DANPLOT  to  do  the

start  and  stop test to make sure that the spacing of the pulses sent

to the motor translator module are far enough apart  (a  larger  pulse

[29]

[            C O N F I G U R E   A N D   I N S T A L L               ]

width  factor will space the pulses wider apart) for the motor to keep

up with the step pulses (if the pulses are to fast the motor will  not

come  back  to the point it was at at the beginning of the motor test,

indicating unreliable operation.) The ramping should be set to 0  when

testing stepper motors.

I would like to warn you that stepper motor manufactures published

speed v.s. torque curves can be very optimistic, and are not generally

applicable to use with DANCAM or DANPLOT.  As a rule of thumb based of

my  experience  you can expect these values for maximum top speeds for

reliable error free operation under working load,  with the higher RPM

number being obtained by using the maximum voltage the motors can take

(when  the  voltage  is  more than six times the rated voltage stepper

motors may become unstable in the middle of the motors speed range, so

you are limited in how much the motors can be boosted):

MOTOR HOLDING TORQUE      MAXIMUM MOTOR SHAFT SPEED

20 -   40 in/oz              240 - 300 RPM

40 -  150 in/oz              120 - 240 RPM

150 -  400 in/oz               90 - 120 RPM

400 - 1500 in/oz               30 -  90 RPM

To configure DANPLOT and DANCAM to produce the  proper  number  of

steps  per  inch  of  travel simply run DANCAM or DANPLOT from DOS and

select mode #4 from the main menu that comes up.  Since stepper motors

typically can only step 300 to 1000 full steps per second (600 to 2000

half steps) the motor step delay of 0.5 ms to 3 ms is needed  to  keep

the  motors  working properly.  The minimum motor single step delay is

controlled a value called the pulse width factor (p.w.f.).  Each  axis

has  its  own  p.w.f  so  the  motors  can  run  as  fast  as possible

(allowable) when the maximum feed rate is requested.

When you install DANCAM and DANPLOT using their main  menu  option

#4,  be sure to work through the sub-menu options in order 1,2,  (sub-

menu of 2 options 1,2,3 for each axis) and then 3,4, and 5.

When you work through the menus keep in mind the number  of  steps

the motor shaft makes per revolution.  If you run a motor in half step

mode then multiply the number of motor steps by two.  For servo motors

the  number  of  steps  per revolution can be derived from the encoder

counts per revolution and any pulse rate multiplier used.

In the motor test sub-menu (#2  in  the  configuration  menu)  use

option  #1  to  find  the  smallest  p.w.f.  that  makes the motor run

smoothly under load,  use #2 (of #2 of #4) to check that the motor  is

not missing any steps (make the p.w.f.  larger if the motor is missing

steps and test again),  and use #3 (of #2 of #4) to test  the  RPM  if

needed.  When the p.w.f. has been configured for each axis press [Esc]

to  go  back  to  the  installation  sub-menu  and  answer  the  other

questions.  The ramping should be set to 0 for testing stepper motors.

[30]

[            C O N F I G U R E   A N D   I N S T A L L               ]

The questions in option #3 in the installation sub-menu should  be

used  to  install  the  scaling factors before you enter the feed rate

controlling delays with option #4.  In this  version  of  DANPLOT  and

DANCAM  the  feed  rate  is controlled by a delay between motor steps.

Since the p.w.f. keeps each motor working properly the feed rate delay

can vary from 0 to 65000000 micro seconds per step  depending  on  how

slow you want the feed.  To get different feeds,  enter the delays you

want in the feed rate table,  then make the  tool  path  drawing  with

DANCAD3D  so  that  the  line  segments  have the color attribute that

corresponds to the delay  set  in  the  feed  table.  The  line  color

attribute  is  set  while drawing in DANCAD3D v2.5 by pressing the [L]

key of the keyboard or by using the mouse to click on the with  [L]ine

style  option  of  the  [L]ines  sub-menu from the drawing editor root

menu.  Any step delay value can be assigned to any line color  number.

Set  all  the  delays  to  0  if  you want the tool to move as fast as

possible for all motion.

Another configuration menu  option  lets  you  assign  which  line

colors  will  turn  on  or  off  the  auxiliary  control relays.  Line

segments used to control the auxiliary relays can  have  starting  and

ending points at the same location (i.e. zero length.)

Option  #5  of  the  installation  sub-menu  asks  about  the home

switches.  Both DANPLOT and DANCAM let you install  home  switches  on

your  equipment  so  that  you can have the tool automatically home up

before  each  tool  path  file  is  read.   The  ability  to  home  up

automatically can be a great time saver.  You should note that without

the use of the switches to home up, an error of +/- 3 full motor steps

can occur when DANPLOT and DANCAM are first run since the motor rotors

will  jump  to the position dictated by the mechanical relationship of

the motor rotor and the state of the motor winding sequence  that  the

motor  controller  starts  with.  Most stepper motor controllers start

with the same coil (phase) energizition pattern, so you can loosen the

motor coupling,  reboot your computer,  re-run DANPLOT or  DANCAM  and

tighten  the  motor  coupling  with  the  tool  carrier  at  it's home

position.  Lousing,  rebooting the computer,  and tightening the motor

coupling  will  not  help  if  the motor controller initializes with a

random energizition of the motor windings,  in such a case  you  would

have  to  use the home switches or manually check to see if the motors

shafts have an incorrect position and use main menu option #3  to  get

the motor shafts into the proper starting orientation.

DANPLOT  has some additional special configurations.  The "C" axis

lets you mount a fourth motor with its shaft parallel to and  centered

on  the  Z axis such that a knife,  saw,  or broach will automatically

rotate to point the cutting  edge  into  the  motion  of  travel.  The

tolerance  on  the  C  axis  lets the tool stay down while cutting out

curves.  The manual up and down option lets you manually activate  the

Z  axis for use in automating a drill press with two motors (one for X

axis and one for Y axis.) In manual up/down  mode  the  work-piece  is

positioned  and then the computer is made to pause while you drill the

hole,  then you press a key on the keyboard and  the  work-piece  will

[31]

[            C O N F I G U R E   A N D   I N S T A L L               ]

move to the next position, and so on.

[32]

[          D R A W I N G   T O O L   P A T H   F I L E S             ]

DRAWING TOOL PATHS WITH CUTTER TOOL COMPENSATION

The  following  are the steps required to draw a tool path element

while being able to visually see  the  shape  of  the  cutter  on  the

computer  screen.  Please  read  the file DANCAD3D.DOC on the DANCAD3D

v2.5 program disks and read the menus in the DANCAD3D program  drawing

editor to learn more about what these steps mean.

NOTE:  You will need DANCAD3D v2.5H or later to draw cutter paths with

compensation for the cutter shape.

1.  Run the DANCAD3D program.  Press [Return] until you reach its main

menu.

2.  Select  [D]raw  from  the  DANCAD3D main menu.  Select the [F]ront

view.  If the drawing workspace is not empty  select  [I]nitialize

to clear the workspace.

3.  Select  [D]raw from the drawing editor root menu.  Draw an outline

of the shape of the finished work-piece you wish to make.  You can

make  the inside corners sharp if the radius will be determined by

the cutter shape.  You can make the shape outline in 3D if you are

going to do 3D work.  Remember that lathes are 2 axis devices,  so

the  tool path for the lathe will be 2D (i.e.  a profile.) You can

include an outline of the raw  material  in  the  outline  of  the

finished  piece to make visualization of the amount of material to

remove easier.  The outline of the work-piece can include outlines

of holding fixtures and other machine parts so that you can  check

for interference with the cutter.

4.  The next step is to make an outline for the cutting tool.  If  you

are  going  to  use  an end mill you can use the [P]olygon command

(try to keep the sides total to 36  or  less)  in  the  2D  shapes

drawing  editor  sub-menu.  For  other  shape cutters press [B] to

begin a new element (element  2)  and  draw  the  outline  of  the

cutter.  The center of the cutter outline element should be on the

cutter's  axis  of  rotation  if  the  cutter rotates.  The cutter

outline can be 3D and include the tool holder or tool post so that

you can visually check for interference  with  the  work-piece  or

other parts of the machine.

5.  To  draw the actual tool path element go into the drawing editor's

[E]lements  sub-menu  and  select  [D]rag.  Select [E]nter element

number and  enter  the  element  number  for  the  cutter  outline

(element  2.)  Select  the  element center as the reference point.

Press [Return] to start the drag command.  The cutter outline will

blink.  Press [B]egin element to begin the tool path element  (the

tool  path  will  be element number 3.) Move the cutter outline to

the home position with the [G]oto drawing command.  Use the  mouse

to  position the cutter outline near the edge of the raw material.

Press the left mouse button to draw the first line segment in  the

tool path.  Move the mouse for the path of the first cut and press

[33]

[          D R A W I N G   T O O L   P A T H   F I L E S             ]

the   left   mouse   button   again.   Continue  drawing  zig-zag,

concentric,  or spiral tool path until the blinking outline of the

cutter comes in contact with the outline of the final shape.  When

drawing  tool paths be sure that you only feed the amount that can

be safely taken off (so the tool will not stall  or  break!)  Also

remember that some cutters can only cut going one way,  so you may

have to retrace  without  feed  when  drawing  in  one  direction.

Pressing  [L]  while inside the [D]rag command will let you change

the drawing line color, and therefore set the feed rate, auxiliary

relays, and programed pause.

6.  When  the  tool path is finished press [Escape] to exit the [D]rag

command.  [Q]uit the drawing  editor  and  select  [F]iles  [S]ave

[A]SCII  from  the  main  menu.  Enter  the element number for the

element of the tool path (normally element 3) as  the  element  to

save.

7.  Make a trial part using DANPLOT or DANCAM.

8.  Measure the trial part.  Record any error in the final shape.

9.  Load  the  tool  path ASCII file into DANCAD3D and use the [P]ull-

Point  command  in  the  [L]ines sub-menu of the drawing editor to

alter the tool path line segments for the final cut,  in order  to

correct  the error.  Save the corrected tool path element again as

an ASCII file.

10. Repeat  steps  7  to  9  if  needed  until the finished part is in

tolerance.  Make the run of parts as needed.  Store the ASCII tool

path file on a floppy disk for future use.  You should  also  copy

the  DANPLOT.CFG or DANCAM.CFG file used to make the part onto the

floppy with the tool path so the tool path will match up  if  used

again.  You  should  also  file  notes about the spindle speed and

cutter seating used and also the location of  the  limit  switches

relative to the work-piece.

If you want to change the line color attribute for any of the line

segments  in  the tool path such as the line segment for the final cut

you can use the commands in DANCAD3D's  drawing  editor  [L]ines  sub-

menu.  The  conversion  of  the line color attributes in the tool path

data file to feed rates is done using the values for  the  step  delay

that  you  have  entered  into  the DANPLOT and DANCAM feed rate table

(option #4 in the DANPLOT and DANCAM installation sub-menu.)

You can also draw the bulk of the tool path in the drawing  editor

[D]raw  lines  sub-menu  and  then  go  into the [D]rag command in the

[E]lements sub-menu to align the cutter outline  to  the  final  shape

outline  for  the  last few cuts.  If you want to add line segments to

the tool path element that you started in the  [D]raw  lines  sub-menu

use  the  [D]rag  command  as  described  above in step 5.  but select

command [E]lastic line in place of [B]egin element.

When you need to use commands that create new elements such as the

[34]

[          D R A W I N G   T O O L   P A T H   F I L E S             ]

[P]olygon command,  or the [F]it-curve in DANCAD3D's,  you  can  unite

later  the  elements  created  into a single ASCII file by erasing the

finished part outline,  and the tool outline from the workspace,  then

use  element  0  as  the element to save rather than element number 3.

The [J]oin command in DANCAD3D's drawing editors [E]lements  menu  can

also be used to unite elements into a single tool path element.

[35]

[      R U N N I N G   T H E   C  A  M   D E M O   F I L E S         ]

RUNNING THE DEMO FILES FOR DANPLOT AND DANCAM

A  demo is provided for each program.  Enter DEMOPLOT.BAT from DOS

with the prompt indicating the drive and  directory  all  the  DANPLOT

files are in.  DEMOCAM.BAT does the same for DANCAM.

EXAMPLE: C>DANPLOT DANPLOT.ASC 1

You can interrupt the motion of the motors momentarily by pressing

the  [P]  key,  press [P] or [SpaceBar] to restart the motors.  If you

need to touch or otherwise expose yourself to the equipment while  the

motors  are  paused  from  the  keyboard you should also use the pause

switch wired in series with the limit switches (as  shown  on  drawing

HOOKUP1.ASC)  and  turn off the motor power (if your translator module

will re-initialize when the motor power is cut off  you  will  have  a

problem  (because  the motors may jump when the power is switched back

on), the circuit schmatics I provide with the DANCAD3D user outfit for

the stepper motor  translator  allow  the  translator  logic  to  stay

powered  while  the  motor  coil  (phase)  power is switched on or off

thereby avoiding any motor jumping.)

You can load the example *.ASC tool path files  into  DANCAD3D  to

look  at  how they were drawn.  DANCAD3D's main menu [P]review command

will let you rotate the display of the tool path's  line  segments  so

you can see the spatial relationships.

[36]

[      H E L P   W I T H   D A N C A M   A N D   D A N P L O T       ]

HELP AND FEEDBACK

If you have any questions or comments about DANCAM or DANPLOT feel

free to write me at:

ADDRESS: DAN HUDGINS

c/o DANCAD3D CNC

466 DIAMOND STREET

SAN FRANCISCO, CA  94114

The UNITED STATES of AMERICA

When you write PLEASE enclose some stamps for the replay  postage.

If  you  live  outside  the U.S.  you can get some postal vouchers for

first class international air mail postage from your post office.

More information and schematics for wiring  up  inexpensive  motor

driver  modules  is  included in the DANCAD3D user outfit.  I can help

you design custom hardware special applications.

If you need help in locating stepper motors,  drivers,  and  other

mechanical  and electronic components feel free to contact me for help

in locating just what you need.  I am always on the look out for parts

that are at the lowest cost.

If you would like to discuss something on the phone I can call you

collect if you send me a letter telling  me  what  time  to  call  and

asking  me  to call collect (sorry but collect calls cannot be made to

Germany.) I can call on evenings or weekends if  you  prefer,  but  be

sure to tell me what days and times I should call.

I  am  always  working  on  a  improvements  to  the  programs and

documentation.  If you register with me by sending one dollar and your

full name and address I will put your name on my  user  mailing  list.

If  you  are  using  DANCAM  or  DANPLOT  at all you will benefit from

communicating with me directly.  If you have a  machine  hooked  up  I

would  be  most  appreciative if you could send me a photo of what you

have built,  I always enjoy seeing  what  users  are  doing  with  the

programs.

Feel  free  to  send  me  your  "wish list" for DANCAD and DANPLOT

improvements.  Although "G" codes are not needed since DANCAD3D  makes

tool  paths  graphically  I  may  write a "G" code converter if enough

users ask for me to write one.  If you are  interested  in  "G"  codes

send me a "G" code file on a floppy disk so I can see the type of file

you need.

[37]