DTA 1.00 (18.11.94)                                                       DTA

NAME

DTA - Data Transfer Analysis

SYNOPSIS

Gives a very general and simple analysis of transfer speeds.

INPUTS

FILE - File to test transfer of data with (Make it a LARGE one!)

RESULTS

The following is an example of some of the useful information DTA

will display for you.

CPU: 68040/68882

Volume: DH1 (DCFFS)

Processing file "dl:LionKing.lha".

Read 946054 bytes / 923.9K in 0 seconds and ~432129 microseconds.

Data Transfer Analysis (DTA) = 2.1M/second (2189286 bytes/second).

The idea behind this very simple/crude program is to give you an

average idea of how well your machine/controller/drive are performing

together.  The whole "analysis" depends on the number of buffers and

what type of memory they are in, your drive/controller speed, your

system's CPU load, the maxtransfer and DMA settings, file system, ...

This program just samples the time, reads a LARGE file, and samples

the time again...

I wrote this to test the results of changing some settings with a

ZorroII (NonDMA) SCSI board with a friends A4000.  We tried changing

the maxtransfer, buffer memory type & size, and changing the DMA mask.

On the whole we got the best results that were opposite of the

suggested ones.  For example we set the DMA mask so we COULD DMA

into all our memory, which was surprising, BIG speed gain.  We

also kept "ANY" for the buffer memory type, CHIP/24bitDMA was

actually somewhat slower...  The big thing with his drive is the

maxtransfer.  For whatever reason it will not allow a very large

one, we got the best result with a 128kb = $200000, however I would

suggect trying the default and doing a "Copy BUF=0 <largefile> to RAM:"

BUF=0 sets the buffer size to that of the file you are copying.  If

your machine locks up then your drive requires a smaller maxtransfer.

This program was written for ourself to test some hardware.  If

you find it usefull then please use it.  If you think the results

are not all-together there then do not use it.  I would suggest

only using it to compare changes that you are making to a drive.

NOTES

This software, in its unmodified form and in its entirity, may be

freely distributed as long as no profits are gained.

THIS SOFTWARE IS PROVIDED BY IMAGINE SOFTWARE INC. "AS IS" AND ANY

EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR

PURPOSE ARE DISCLAIMED.  IN NO EVENT SHALL IMAGINE SOFTWARE INC. BE

LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR

CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF

SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR

BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY,

WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE

OR OTHERWISE) ARISING IN ANY WAY OUT OF THE USE OF THIS SOFTWARE,

EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

DTA 1.00 (18.11.94) Engineered by Trevor Andrews

Copyright 1994 Imagine Software, Inc.  All Rights Reserved.

Imagine Software, Inc.

1218 South 5th Street

Lafayette, IN  47905-1716