6-9

Opcodes    Instruction     Description

67 or nil  A2 (prefix)   3 Use 16-bit address (indexing) in next instruction

67 or nil  A4 (prefix)   3 Use 32-bit address (indexing) in next instruction

37         AAA             ASCII adjust AL (carry into AH) after addition

D5 0A      AAD             ASCII adjust before division (AX = 10*AH + AL)

D4 0A      AAM             ASCII adjust after multiply (AL/10: AH=Quo AL=Rem)

3F         AAS             ASCII adjust AL (borrow from AH) after subtraction

14 ib      ADC AL,ib       Add with carry immediate byte into AL

15 iv      ADC eAX,iv      Add with carry immediate vword into eAX

80 /2 ib   ADC eb,ib       Add with carry immediate byte into EA byte

10 /r      ADC eb,rb       Add with carry byte register into EA byte

83 /2 ib   ADC ev,ib       Add with carry immediate byte into EA vword

81 /2 iv   ADC ev,iv       Add with carry immediate vword into EA vword

11 /r      ADC ev,rv       Add with carry vword register into EA vword

12 /r      ADC rb,eb       Add with carry EA byte into byte register

13 /r      ADC rv,ev       Add with carry EA vword into vword register

04 ib      ADD AL,ib       Add immediate byte into AL

05 iv      ADD eAX,iv      Add immediate vword into eAX

80 /0 ib   ADD eb,ib       Add immediate byte into EA byte

00 /r      ADD eb,rb       Add byte register into EA byte

83 /0 ib   ADD ev,ib       Add immediate byte into EA vword

81 /0 iv   ADD ev,iv       Add immediate vword into EA vword

01 /r      ADD ev,rv       Add vword register into EA vword

02 /r      ADD rb,eb       Add EA byte into byte register

03 /r      ADD rv,ev       Add EA vword into vword register

0F 20      ADD4S         N Add CL nibbles BCD, DS:SI into ES:DI (CL even,NZ)

24 ib      AND AL,ib       Logical-AND immediate byte into AL

25 iv      AND eAX,iv      Logical-AND immediate vword into eAX

80 /4 ib   AND eb,ib       Logical-AND immediate byte into EA byte

20 /r      AND eb,rb       Logical-AND byte register into EA byte

83 /4 ib   AND ev,ib       Logical-AND immediate byte into EA vword

81 /4 iv   AND ev,iv       Logical-AND immediate vword into EA vword

21 /r      AND ev,rv       Logical-AND vword register into EA vword

22 /r      AND rb,eb       Logical-AND EA byte into byte register

23 /r      AND rv,ev       Logical-AND EA vword into vword register

63 /r      ARPL ew,rw    2 Adjust RPL of EA word not smaller than RPL of rw

62 /r      BOUND rv,m2v  2 INT 5 if rw not between 2 vwords at [m] inclusive

0F BC      BSF rv,ev     3 Set rv to lowest position of NZ bit in ev

0F BD      BSR rv,ev     3 Set rv to highest position of NZ bit in ev

0F C8+r    BSWAP rd      4 Swap bytes 1,4 and 2,3 of dword register

0F BA/4 ib BT rv/m,ib    3 Set Carry flag to bit # ib of array at rv/m

0F A3/r    BT rv/m,rv    3 Set Carry flag to bit # rv of array at rv/m

0F BA/7 ib BTC rv/m,ib   3 Set CF to, then compl bit ib of array at rv/m

0F BB/r    BTC rv/m,rv   3 Set CF to, then compl bit rv of array at rv/m

0F BA/6 ib BTR rv/m,     3 Set CF to, then reset bit ib of array at rv/m

0F B3/r    BTR rv/m,rv   3 Set CF to, then reset bit rv of array at rv/m

0F BA/5 ib BTS rv/m,ib   3 Set CF to, then set bit ib of array at rv/m

0F AB/r    BTS rv/m,rv   3 Set CF to, then set bit rv of array at rv/m

6-10

9A cp      CALL cp         Call far segment, immediate 4- or 6-byte address

E8 cv      CALL cv         Call near, offset relative to next instruction

FF /3      CALL ep         Call far segment, address at EA memory location

FF /2      CALL ev         Call near, offset absolute at EA vword

0F FF ib   CALL80 ib     N Call 8080-emulation code at INT number ib

98         CBW             Convert byte into word (AH = top bit of AL)

99         CDQ           3 Convert dword to qword (EDX = top bit of EAX)

F8         CLC             Clear carry flag

FC         CLD             Clear direction flag so SI and DI will increment

FA         CLI             Clear interrupt enable flag; interrupts disabled

0F 12/0    CLRBIT eb,CL  N Clear bit CL of eb

0F 13/0    CLRBIT ew,CL  N Clear bit CL of ew

0F 1A/0 ib CLRBIT eb,ib  N Clear bit ib of eb

0F 1B/0 ib CLRBIT ew,ib  N Clear bit ib of ew

0F 06      CLTS          2 Clear task switched flag

F5         CMC             Complement carry flag

3C ib      CMP AL,ib       Subtract immediate byte from AL for flags only

3D iv      CMP eAX,iv      Subtract immediate vword from eAX for flags only

80 /7 ib   CMP eb,ib       Subtract immediate byte from EA byte for flags only

38 /r      CMP eb,rb       Subtract byte register from EA byte for flags only

83 /7 ib   CMP ev,ib       Subtract immediate byte from EA vword for flags only

81 /7 iv   CMP ev,iv       Subtract immediate vword from EA vword, flags only

39 /r      CMP ev,rv       Subtract vword register from EA vword for flags only

3A /r      CMP rb,eb       Subtract EA byte from byte register for flags only

3B /r      CMP rv,ev       Subtract EA vword from vword register for flags only

0F 26      CMP4S         N Compare CL nibbles BCD, DS:SI - ES:DI (CL even,NZ)

A6         CMPS mb,mb      Compare bytes [SI] - ES:[DI], advance SI,DI

A7         CMPS mv,mv      Compare vwords [SI] - ES:[DI], advance SI,DI

A6         CMPSB           Compare bytes DS:[SI] - ES:[DI], advance SI,DI

A7         CMPSD           Compare dwords DS:[SI] - ES:[DI], advance SI,DI

A7         CMPSW           Compare words DS:[SI] - ES:[DI], advance SI,DI

0F C7 /1   CMPX8 mq      5 If EDXEAX=mq then mq:=ECXEBX, else EAXEDX:=mq

0F B0 /r   CMPXCHG eb,rb 4 If AL=eb then set eb to rb, else set AL to eb

0F B1 /r   CMPXCHG ev,rv 4 If eAX=ev then set ev to rv, else set eAX to ev

0F A2      CPUID         5 If EAX=1 set EDXEAX to CPU identification values

99         CWD             Convert word to doubleword (DX = top bit of AX)

98         CWDE          3 Sign-extend word AX to doubleword EAX

2E         CS (prefix)     Use CS segment for the following memory reference

27         DAA             Decimal adjust AL after addition

2F         DAS             Decimal adjust AL after subtraction

FE /1      DEC eb          Decrement EA byte by 1

FF /1      DEC ev          Decrement EA vword by 1

48+rv      DEC rv          Decrement vword register by 1

F6 /6      DIV eb          Unsigned divide AX by EA byte (AL=Quo AH=Rem)

F7 /6      DIV ev          Unsigned divide eDXeAX by EA vword (eAX=Quo eDX=Rem)

3E         DS (prefix)     Use DS segment for the following memory reference

6-11

C8 iw 00   ENTER iw,0    1 Make stack frame, iw bytes local storage, 0 levels

C8 iw 01   ENTER iw,1    1 Make stack frame, iw bytes local storage, 1 level

C8 iw ib   ENTER iw,ib   1 Make stack frame, iw bytes local storage, ib levels

26         ES (prefix)     Use ES segment for the following memory reference

F(any)          Floating point set is in Chapter 7

F4         HLT             Halt

F6 /7      IDIV eb         Signed divide AX by EA byte (AL=Quo AH=Rem)

F7 /7      IDIV ev         Signed divide eDXeAX by EA vword (eAX=Quo eDX=Rem)

F6 /5      IMUL eb         Signed multiply (AX = AL * EA byte)

F7 /5      IMUL ev         Signed multiply (eDXeAX = eAX * EA vword)

0F AF /r   IMUL rv,ev    3 Signed multiply ev into rv

6B /r ib   IMUL rv,ib    1 Signed multiply imm byte into vword register

69 /r iv   IMUL rv,iv    1 Signed multiply imm vword into vword register

69 /r iv   IMUL rv,ev,iv 1 Signed multiply (rv = EA vword * imm vword)

6B /r ib   IMUL rv,ev,ib 1 Signed multiply (rv = EA vword * imm byte)

E4 ib      IN AL,ib        Input byte from immediate port into AL

EC         IN AL,DX        Input byte from port DX into AL

E5 ib      IN eAX,ib       Input vword from immediate port into eAX

ED         IN eAX,DX       Input vword from port DX into eAX

FE /0      INC eb          Increment EA byte by 1

FF /0      INC ev          Increment EA vword by 1

40+rv      INC rv          Increment vword register by 1

6C         INS eb,DX     1 Input byte from port DX into [DI], advance DI

6D         INS ev,DX     1 Input vword from port DX into [DI], advance DI

6C         INSB          1 Input byte from port DX into ES:[DI], advance DI

6D         INSD          3 Input dword from port DX into ES:[DI], advance DI

6D         INSW          1 Input vword from port DX into ES:[DI], advance DI

CC         INT 3           Interrupt 3 (trap to debugger) (far call, with flags

CD ib      INT ib          Interrupt numbered by immediate byte    pushed first)

CE         INTO            Interrupt 4 if overflow flag is 1

0F 08      INVD          4 Invalidate the Data Cache without writing

0F 01 /7   INVLPG m      4 Invalidate the TLB Entry that points to m

CF         IRET            Interrupt return (far return and pop flags)

CF         IRETD         3 Interrupt return (pop EIP, ECS, Eflags)

77 cb      JA cb           Jump short if above (CF=0 and ZF=0)    above=UNSIGNED

73 cb      JAE cb          Jump short if above or equal (CF=0)

72 cb      JB cb           Jump short if below (CF=1)             below=UNSIGNED

76 cb      JBE cb          Jump short if below or equal (CF=1 or ZF=1)

72 cb      JC cb           Jump short if carry (CF=1)

E3 cb      JCXZ cb         Jump short if CX register is zero

74 cb      JE cb           Jump short if equal (ZF=1)

E3 cb      JECXZ cb      3 Jump short if ECX register is zero

7F cb      JG cb           Jump short if greater (ZF=0 and SF=OF) greater=SIGNED

7D cb      JGE cb          Jump short if greater or equal (SF=OF)

7C cb      JL cb           Jump short if less (SF>OF)               less=SIGNED

7E cb      JLE cb          Jump short if less or equal (ZF=1 or SF>OF)

6-12

EB cb      JMP cb          Jump short (signed byte relative to next instruction)

EA cp      JMP cp          Jump far (4- or 6-byte immediate address)

E9 cv      JMP cv          Jump near (vword offset relative to next instruction)

0F 8n cv   Jcond LONG cv 3 Jump, if condition, to offset >127 away

FF /4      JMP ev          Jump near to EA vword (absolute offset)

FF /5      JMP md          Jump far (4-byte address in memory doubleword)

76 cb      JNA cb          Jump short if not above (CF=1 or ZF=1)

72 cb      JNAE cb         Jump short if not above or equal (CF=1)

73 cb      JNB cb          Jump short if not below (CF=0)

77 cb      JNBE cb         Jump short if not below or equal (CF=0 and ZF=0)

73 cb      JNC cb          Jump short if not carry (CF=0)

75 cb      JNE cb          Jump short if not equal (ZF=0)

7E cb      JNG cb          Jump short if not greater (ZF=1 or SF>OF)

7C cb      JNGE cb         Jump short if not greater or equal (SF>OF)

7D cb      JNL cb          Jump short if not less (SF=OF)

7F cb      JNLE cb         Jump short if not less or equal (ZF=0 and SF=OF)

71 cb      JNO cb          Jump short if not overflow (OF=0)

7B cb      JNP cb          Jump short if not parity (PF=0)

79 cb      JNS cb          Jump short if not sign (SF=0)

75 cb      JNZ cb          Jump short if not zero (ZF=0)

70 cb      JO cb           Jump short if overflow (OF=1)

7A cb      JP cb           Jump short if parity (PF=1)

7A cb      JPE cb          Jump short if parity even (PF=1)

7B cb      JPO cb          Jump short if parity odd (PF=0)

78 cb      JS cb           Jump short if sign (SF=1)

74 cb      JZ cb           Jump short if zero (ZF=1)

9F         LAHF            Load: AH = flags  SF ZF xx AF xx PF xx CF

0F 02 /r   LAR rv,ew     2 Load: high(rw) = Access Rights byte, selector ew

C5 /r      LDS rv,ep       Load EA pointer into DS and vword register

8D /r      LEA rv,m        Calculate EA offset given by m, place in rv

C9         LEAVE         1 Set SP to BP, then POP BP (reverses previous ENTER)

C4 /r      LES rv,ep       Load EA pointer into ES and vword register

0F B4 /r   LFS rv,ep     3 Load EA pointer into FS and vword register

0F 01 /2   LGDT m        2 Load 6 bytes at m into Global Descriptor Table reg

0F B5 /r   LGS rv,ep     3 Load EA pointer into GS and vword register

0F 01 /3   LIDT m        2 Load 6 bytes into Interrupt Descriptor Table reg

0F 00 /2   LLDT ew       2 Load selector ew into Local Descriptor Table reg

0F 01 /6   LMSW ew       2 Load EA word into Machine Status Word

F0         LOCK (prefix)   Assert BUSLOCK signal for the next instruction

0F 33/r    LODBITS rb,rb N Load AX with DS:SI,bit rb (incr. SI,rb), rb+1 bits

0F 3B/0 ib LODBITS rb,ib N Load AX with DS:SI,bit rb (incr. SI,rb), ib+1 bits

AC         LODS mb         Load byte [SI] into AL, advance SI

AD         LODS mv         Load vword [SI] into eAX, advance SI

AC         LODSB           Load byte [SI] into AL, advance SI

AD         LODSD           Load dword [SI] into EAX, advance SI

AD         LODSW           Load word [SI] into AX, advance SI

6-13

E2 cb      LOOP cb         noflags DEC CX; jump short if CX>0

E1 cb      LOOPE cb        noflags DEC CX; jump short if CX>0 and equal (ZF=1)

E0 cb      LOOPNE cb       noflags DEC CX; jump short if CX>0 and not equal

E0 cb      LOOPNZ cb       noflags DEC CX; jump short if CX>0 and ZF=0

E1 cb      LOOPZ cb        noflags DEC CX; jump short if CX>0 and zero (ZF=1)

0F 03 /r   LSL rv,ev     2 Load: rv = Segment Limit, selector ew

0F B2 /r   LSS rv,ep     3 Load EA pointer into SS and vword register

0F 00 /3   LTR ew        2 Load EA word into Task Register

A0 iv      MOV AL,xb       Move byte variable (offset iv) into AL

A1 iv      MOV eAX,xv      Move vword variable (offset iv) into eAX

0F 22 /4   MOV CR4,rd    5 Move rd into control register 4

0F 22 /n   MOV CRn,rd    3 Move rd into control register n (=0,2, or 3)

0F 23 /n   MOV DRn,rd    3 Move rd into debug register n (=0,1,2,3)

0F 23 /n   MOV DRn,rd    3 Move rd into debug register n (=6,7)

0F 26 /n   MOV TRn,rd    3 Move rd into test register TRn (=6,7)

8E /3      MOV DS,mw       Move memory word into DS

8E /3      MOV DS,rw       Move word register into DS

C6 /0 ib   MOV eb,ib       Move immediate byte into EA byte

88 /r      MOV eb,rb       Move byte register into EA byte

8E /0      MOV ES,mw       Move memory word into ES

8E /0      MOV ES,rw       Move word register into ES

8C /1      MOV ew,CS       Move CS into EA word

8C /3      MOV ew,DS       Move DS into EA word

C7 /0 iv   MOV ev,iv       Move immediate vword into EA vword

8C /0      MOV ew,ES       Move ES into EA word

89 /r      MOV ev,rv       Move vword register into EA vword

8C /2      MOV ew,SS       Move SS into EA word

B0+rb ib   MOV rb,ib       Move immediate byte into byte register

8A /r      MOV rb,eb       Move EA byte into byte register

0F 20 /4   MOV rd,CR4    5 Move control register 4 into rd

0F 20 /n   MOV rd,CRn    3 Move control register n (=0,2, or 3) into rd

0F 21 /n   MOV rd,DRn    3 Move debug register n (=0,1,2,3) into rd

0F 21 /n   MOV rd,DRn    3 Move debug register n (=6,7) into rd

0F 24 /n   MOV rd,TRn    3 Move test register TRn (=6,7) into rd

B8+rw iv   MOV rv,iv       Move immediate vword into vword register

8B /r      MOV rv,ev       Move EA vword into vword register

8E /2      MOV SS,mw       Move memory word into SS

8E /2      MOV SS,rw       Move word register into SS

A2 iv      MOV xb,AL       Move AL into byte variable (offset iv)

A3 iv      MOV xv,eAX      Move eAX into vword register (offset iv)

A4         MOVS mb,mb      Move byte [SI] to ES:[DI], advance SI,DI

A5         MOVS mv,mv      Move vword [SI] to ES:[DI], advance SI,DI

A4         MOVSB           Move byte DS:[SI] to ES:[DI], advance SI,DI

A5         MOVSD         3 Move dword DS:[SI] to ES:[DI], advance SI,DI

A5         MOVSW           Move word DS:[SI] to ES:[DI], advance SI,DI

6-14

0F BF /r   MOVSX rd,ew   3 Move word to dword, with sign-extend

0F BE /r   MOVSX rv,eb   3 Move byte to vword, with sign-extend

0F B7 /r   MOVZX rd,ew   3 Move word to dword, with zero-extend

0F B6 /r   MOVZX rv,eb   3 Move byte to vword, with zero-extend

F6 /4      MUL eb          Unsigned multiply (AX = AL * EA byte)

F7 /4      MUL ev          Unsigned multiply (eDXeAX = eAX * EA vword)

F6 /3      NEG eb          Two's complement negate EA byte

F7 /3      NEG ev          Two's complement negate EA vword

NIL (prefix)    Special "do-nothing" opcode assembles no code

90         NOP             No Operation

F6 /2      NOT eb          Reverse each bit of EA byte

F7 /2      NOT ev          Reverse each bit of EA word

0F 16/0    NOTBIT eb,CL  N  Complement bit CL of eb

0F 17/0    NOTBIT ew,CL  N  Complement bit CL of ew

0F 1E/0 ib NOTBIT eb,ib  N Complement bit ib of eb

0F 1F/0 ib NOTBIT ew,ib  N Complement bit ib of ew

66 or nil  O2 (prefix)   3 Use 16-bit data operand in the next instruction

66 or nil  O4 (prefix)   3 Use 32-bit data operand in the next instruction

0C ib      OR AL,ib        Logical-OR immediate byte into AL

0D iv      OR eAX,iv       Logical-OR immediate word into eAX

80 /1 ib   OR eb,ib        Logical-OR immediate byte into EA byte

08 /r      OR eb,rb        Logical-OR byte register into EA byte

83 /1 ib   OR ev,ib        Logical-OR immediate byte into EA word

81 /1 iv   OR ev,iv        Logical-OR immediate word into EA word

09 /r      OR ev,rv        Logical-OR word register into EA word

0A /r      OR rb,eb        Logical-OR EA byte into byte register

0B /r      OR rv,ev        Logical-OR EA word into word register

E6 ib      OUT ib,AL       Output byte AL to immediate port number ib

E7 ib      OUT ib,eAX      Output word eAX to immediate port number ib

EE         OUT DX,AL       Output byte AL to port number DX

EF         OUT DX,eAX      Output word eAX to port number DX

6E         OUTS DX,eb    1 Output byte [SI] to port number DX, advance SI

6F         OUTS DX,ev    1 Output word [SI] to port number DX, advance SI

6E         OUTSB         1 Output byte DS:[SI] to port number DX, advance SI

6F         OUTSD         3 Output dword DS:[SI] to port number DX, advance SI

6F         OUTSW         1 Output word DS:[SI] to port number DX, advance SI

1F         POP DS          Set DS to top of stack, increment SP by 2

07         POP ES          Set ES to top of stack, increment SP by 2

0F A1      POP FS        3 Set FS to top of stack, increment SP by 2

0F A9      POP GS        3 Set GS to top of stack, increment SP by 2

8F /0      POP mv          Set memory word to top of stack, increment SP by 2

58+rw      POP rv          Set word register to top of stack, increment SP by 2

17         POP SS          Set SS to top of stack, increment SP by 2

61         POPA          1 Pop DI,SI,BP,SP,BX,DX,CX,AX (SP value is ignored)

61         POPAD         3 Pop EDI,ESI,EBP,x,EBX,EDX,ECX,EAX (ESP ign.)

9D         POPF            Set flags register to top of stack, increment SP by 2

9D         POPFD         3 Set eflags reg to top of stack, incr SP by 2

6-15

0E         PUSH CS         Set [SP-2] to CS, then decrement SP by 2

1E         PUSH DS         Set [SP-2] to DS, then decrement SP by 2

06         PUSH ES         Set [SP-2] to ES, then decrement SP by 2

0F A0      PUSH FS       3 Set [SP-2] to FS, then decrement SP by 2

0F A8      PUSH GS       3 Set [SP-2] to GS, then decrement SP by 2

6A ib      PUSH ib       1 Push sign-extended immediate byte

68 iv      PUSH iv       1 Set [SP-2] to immediate word, then decrement SP by 2

FF /6      PUSH mv         Set [SP-2] to memory word, then decrement SP by 2

50+rw      PUSH rv         Set [SP-2] to word register, then decrement SP by 2

16         PUSH SS         Set [SP-2] to SS, then decrement SP by 2

60         PUSHA         1 Push AX,CX,DX,BX,original SP,BP,SI,DI

60         PUSHAD        3 Push EAX,ECX,EDX,EBX,original ESP,EBP,ESI,EDI

9C         PUSHF           Set [SP-2] to flags register, then decrement SP by 2

9C         PUSHFD        3 Set [SP-4] to eflags reg, then decr SP by 4

D0 /2      RCL eb,1        Rotate 9-bit quantity (CF, EA byte) left once

D2 /2      RCL eb,CL       Rotate 9-bit quantity (CF, EA byte) left CL times

C0 /2 ib   RCL eb,ib     1 Rotate 9-bit quantity (CF, EA byte) left ib times

D1 /2      RCL ev,1        Rotate v+1-bit quantity (CF, EA word) left once

D3 /2      RCL ev,CL       Rotate v+1-bit quantity (CF, EA word) left CL times

C1 /2 ib   RCL ev,ib     1 Rotate v+1-bit quantity (CF, EA word) left ib times

D0 /3      RCR eb,1        Rotate 9-bit quantity (CF, EA byte) right once

D2 /3      RCR eb,CL       Rotate 9-bit quantity (CF, EA byte) right CL times

C0 /3 ib   RCR eb,ib     1 Rotate 9-bit quantity (CF, EA byte) right ib times

D1 /3      RCR ev,1        Rotate v+1-bit quantity (CF, EA word) right once

D3 /3      RCR ev,CL       Rotate v+1-bit quantity (CF, EA word) right CL times

C1 /3 ib   RCR ev,ib     1 Rotate v+1-bit quantity (CF, EA word) right ib times

0F 32      RDMSR         5 Read Model Specific Reg #ECX to EDXEAX

0F 31      RDTSC         5 Read Time Stamp Counter to EDXEAX

F3         REP (prefix)    Repeat following MOVS,LODS,STOS,INS, or OUTS CX times

65         REPC (prefix) N Repeat following CMPS or SCAS CX times or until CF=0

F3         REPE (prefix)   Repeat following CMPS or SCAS CX times or until ZF=0

64         REPNC (prfix) N Repeat following CMPS or SCAS CX times or until CF=1

F2         REPNE (prfix)   Repeat following CMPS or SCAS CX times or until ZF=1

F2         REPNZ (prfix)   Repeat following CMPS or SCAS CX times or until ZF=1

F3         REPZ (prefix)   Repeat following CMPS or SCAS CX times or until ZF=0

CB         RETF            Return to far caller (pop offset, then seg)

C3         RET             Return to near caller (pop offset only)

CA iw      RETF iw         RET (far), pop offset, seg, iw bytes

C2 iw      RET iw          RET (near), pop offset, iw bytes pushed before Call

D0 /0      ROL eb,1        Rotate 8-bit EA byte left once

D2 /0      ROL eb,CL       Rotate 8-bit EA byte left CL times

C0 /0 ib   ROL eb,ib     1 Rotate 8-bit EA byte left ib times

D1 /0      ROL ev,1        Rotate 16- or 32-bit EA vword left once

D3 /0      ROL ev,CL       Rotate 16- or 32-bit EA vword left CL times

C1 /0 ib   ROL ev,ib     1 Rotate 16 or 32-bit EA vword left ib times

0F 28/0    ROL4 eb       N Rotate nibbles: Heb=Leb   HAL,Leb=LAL  LAL=Heb

6-16

D0 /1      ROR eb,1        Rotate 8-bit EA byte right once

D2 /1      ROR eb,CL       Rotate 8-bit EA byte right CL times

C0 /1 ib   ROR eb,ib     1 Rotate 8-bit EA byte right ib times

D1 /1      ROR ev,1        Rotate 16- or 32-bit EA vword right once

D3 /1      ROR ev,CL       Rotate 16- or 32-bit EA vword right CL times

C1 /1 ib   ROR ev,ib     1 Rotate 16- or 32-bit EA vword right ib times

0F 2A/0    ROR4 eb       N Rotate nibbles: Leb=Heb   Heb=LAL  AL=eb

0F AA      RSM           5 Resume from System Management mode

9E         SAHF            Store AH into flags  SF ZF xx AF xx PF xx CF

D0 /4      SAL eb,1        Multiply EA byte by 2, once

D2 /4      SAL eb,CL       Multiply EA byte by 2, CL times

C0 /4 ib   SAL eb,ib     1 Multiply EA byte by 2, ib times

D1 /4      SAL ev,1        Multiply EA vword by 2, once

D3 /4      SAL ev,CL       Multiply EA vword by 2, CL times

C1 /4 ib   SAL ev,ib     1 Multiply EA vword by 2, ib times

D0 /7      SAR eb,1        Signed divide EA byte by 2, once

D2 /7      SAR eb,CL       Signed divide EA byte by 2, CL times

C0 /7 ib   SAR eb,ib     1 Signed divide EA byte by 2, ib times

D1 /7      SAR ev,1        Signed divide EA vword by 2, once

D3 /7      SAR ev,CL       Signed divide EA vword by 2, CL times

C1 /7 ib   SAR ev,ib     1 Signed divide EA vword by 2, ib times

1C ib      SBB AL,ib       Subtract with borrow immediate byte from AL

1D iv      SBB eAX,iv      Subtract with borrow immediate word from eAX

80 /3 ib   SBB eb,ib       Subtract with borrow immediate byte from EA byte

18 /r      SBB eb,rb       Subtract with borrow byte register from EA byte

83 /3 ib   SBB ev,ib       Subtract with borrow immediate byte from EA word

81 /3 iv   SBB ev,iv       Subtract with borrow immediate word from EA word

19 /r      SBB ev,rv       Subtract with borrow word register from EA word

1A /r      SBB rb,eb       Subtract with borrow EA byte from byte register

1B /r      SBB rv,ev       Subtract with borrow EA word from word register

AE         SCAS mb         Compare bytes AL - ES:[DI], advance DI

AF         SCAS mv         Compare vwords eAX - ES:[DI], advance DI

AE         SCASB           Compare bytes AL - ES:[DI], advance DI

AF         SCASD           Compare dwords EAX - ES:[DI], advance DI

AF         SCASW           Compare words AX - ES:[DI], advance DI

0F 14/0    SETBIT eb,CL  N Set bit CL of eb

0F 15/0    SETBIT ew,CL  N Set bit CL of ew

0F 1C/0 ib SETBIT eb,ib  N Set bit ib of eb

0F 1D/0 ib SETBIT ew,ib  N Set bit ib of ew

0F 9n /r   SETcond eb    3 Set eb byte to 1 if condition, 0 if not

0F 01 /0   SGDT m        2 Store 6-byte Global Descriptor Table register to M

D0 /4      SHL eb,1        Multiply EA byte by 2, once

D2 /4      SHL eb,CL       Multiply EA byte by 2, CL times

C0 /4 ib   SHL eb,ib     1 Multiply EA byte by 2, ib times

D1 /4      SHL ev,1        Multiply EA word by 2, once

D3 /4      SHL ev,CL       Multiply EA word by 2, CL times

C1 /4 ib   SHL ev,ib     1 Multiply EA word by 2, ib times

0F A5/r    SHLD ev,rv,CL 3 Set ev to high of ((ev,rv) SHL CL)

0F A4/r ib SHLD ev,rv,ib 3 Set ev to high of ((ev,rv) SHL ib)

6-17

D0 /5      SHR eb,1        Unsigned divide EA byte by 2, once

D2 /5      SHR eb,CL       Unsigned divide EA byte by 2, CL times

C0 /5 ib   SHR eb,ib     1 Unsigned divide EA byte by 2, ib times

D1 /5      SHR ev,1        Unsigned divide EA word by 2, once

D3 /5      SHR ev,CL       Unsigned divide EA word by 2, CL times

C1 /5 ib   SHR ev,ib     1 Unsigned divide EA word by 2, ib times

0F AD/r    SHRD ev,rv,CL 3 Set ev to low of ((rv,ev) SHR CL)

0F AC/r ib SHRD ev,rv,ib 3 Set ev to low of ((rv,ev) SHR ib)

0F 01 /1   SIDT m        2 Store 6-byte Interrupt Descriptor Table register to M

0F 00 /0   SLDT ew       2 Store Local Descriptor Table register to EA word

0F 01 /4   SMSW ew       2 Store Machine Status Word to EA word

36         SS              Use SS segment for the following memory reference

F9         STC             Set carry flag

FD         STD             Set direction flag so SI and DI will decrement

FB         STI             Set interrupt enable flag, interrupts enabled

0F 31/r    STOBITS rb,rb N Store AX to ES:DI,bit rb (incr. DI,rb), rb+1 bits

0F 39/0 ib STOBITS rb,ib N Store AX to ES:DI,bit rb (incr. DI,rb), ib+1 bits

AA         STOS mb         Store AL to byte [DI], advance DI

AB         STOS mv         Store eAX to word [DI], advance DI

AA         STOSB           Store AL to byte ES:[DI], advance DI

AB         STOSD           Store EAX to dword ES:[DI], advance DI

AB         STOSW           Store AX to word ES:[DI], advance DI

0F 00 /1   STR ew        2 Store Task Register to EA word

2C ib      SUB AL,ib       Subtract immediate byte from AL

2D iv      SUB eAX,iv      Subtract immediate word from eAX

80 /5 ib   SUB eb,ib       Subtract immediate byte from EA byte

28 /r      SUB eb,rb       Subtract byte register from EA byte

83 /5 ib   SUB ev,ib       Subtract immediate byte from EA word

81 /5 iv   SUB ev,iv       Subtract immediate word from EA word

29 /r      SUB ev,rv       Subtract word register from EA word

2A /r      SUB rb,eb       Subtract EA byte from byte register

2B /r      SUB rv,ev       Subtract EA word from word register

0F 22      SUB4S         N Sub CL nibbles BCD, DS:SI - ES:DI (CL even,NZ)

A8 ib      TEST AL,ib      AND immediate byte into AL for flags only

A9 iv      TEST eAX,iv     AND immediate word into eAX for flags only

F6 /0 ib   TEST eb,ib      AND immediate byte into EA byte for flags only

84 /r      TEST eb,rb      AND byte register into EA byte for flags only

F7 /0 iv   TEST ev,iv      AND immediate word into EA word for flags only

85 /r      TEST ev,rv      AND word register into EA word for flags only

84 /r      TEST rb,eb      AND EA byte into byte register for flags only

85 /r      TEST rv,ev      AND EA word into word register for flags only

0F 10/0    TESTBIT eb,CL N Test bit CL of eb, set Z flag

0F 11/0    TESTBIT ev,CL N Test bit CL of ew, set Z flag

0F 18/0 ib TESTBIT eb,ib N Test bit ib of eb, set Z flag

0F 19/0 ib TESTBIT ew,ib N Test bit ib of ew, set Z flag

6-18

0F 00 /4   VERR ew       2 Set ZF=1 if segment can be read, selector ew

0F 00 /5   VERW ew       2 Set ZF=1 if segment can be written to, selector ew

9B         WAIT            Wait until BUSY pin is inactive (HIGH)

0F 09      WBINVD        4 Write Back and Invalidate the Data Cache

0F 30      WRMSR         5 Write EDXEAX to Model Specific Reg #ECX

0F C0 /r   XADD eb,rb    4 Exchange eb with rb then add into new eb

0F C1 /r   XADD ev,rv    4 Exchange ev with rv then add into new ev

9r         XCHG eAX,rv     Exchange word register with eAX

86 /r      XCHG eb,rb      Exchange byte register with EA byte

87 /r      XCHG ev,rv      Exchange word register with EA word

86 /r      XCHG rb,eb      Exchange EA byte with byte register

9r         XCHG rv,eAX     Exchange  with word register

87 /r      XCHG rv,ev      Exchange EA word with word register

D7         XLAT mb         Set AL to memory byte [BX + unsigned AL]

D7         XLATB           Set AL to memory byte DS:[BX + unsigned AL]

34 ib      XOR AL,ib       Exclusive-OR immediate byte into AL

35 iv      XOR eAX,iv      Exclusive-OR immediate word into eAX

80 /6 ib   XOR eb,ib       Exclusive-OR immediate byte into EA byte

30 /r      XOR eb,rb       Exclusive-OR byte register into EA byte

83 /6 ib   XOR ev,ib       Exclusive-OR immediate byte into EA word

81 /6 iv   XOR ev,iv       Exclusive-OR immediate word into EA word

31 /r      XOR ev,rv       Exclusive-OR word register into EA word

32 /r      XOR rb,eb       Exclusive-OR EA byte into byte register

33 /r      XOR rv,ev       Exclusive-OR EA word into word register

"N" next to the instruction description means that instruction works only

on NEC chips.  A digit x means that instruction works only on the x86

or later processor.  See the note just before the chart.