GRAPHICS.DOC -- Graphic Plotting Procedures

===========================================

From `BLUEBOOK of ASSEMBLY ROUTINES for the IBM PC & XT'

by Christopher L. Morgan

Copyright (C) 1984 by The Waite Group, Inc.

Purpose: GRAPHICS.ASM contains routines to perform fundamental plotting jobs

on the IBM PC. These include point plotting, line drawing, character

drawing, and filling of areas.  GRAPHICS.DOC describes the routines.

Contents:

---------

CLS		--  Clear the screen

GET_COLOR	--  Get the color of a point on the med res color screen

GMSG_OUT	--  Plot a string

PAINT	--  Fill an area with specified color

RCHAR	--  Plot a raster character

SCHAR	--  Plot a stroke character

SET_BOX	--  Fill a rectangular box with specified color

SET_LIN	--  Draw a line

SET_PT	--  Plot a point

XOR_BOX	--  Fill a rectangular box using XOR

XOR_PT	--  Plot a point using XOR

_____________________________________________________________________________

Primitive Functions

-------------------

CLS -- Clear the graphics screen.

Function: This routine clears the color graphics screen.

Input: None.

Output: Just to the screen.

Registers used: Unmodified; AX & CX are saved, then restored.

Segments referenced: Upon entry, ES must point to screen RAM at B8000H.

Routines called: None.

-----------------------------------------------------------------------------

SET_PT -- Plot a point on the medium resolution color screen.

Function: This routine plots a point on the medium resolution color graphics

screen. The pixel at the specified location is given a specified color,

over-writing the old color.

Input: Upon entry:

X-coordinate (0-319) of the point is in SI

Y-coordinate (0-199) of the point is in DI

color (0-3) is in DX.

Output: Just to the screen.

Registers used: Unmodified; SI,DI, & DX are used for input.

Segments referenced: Upon entry, ES must point to video RAM at B8000, and

DS must point to a data segment containing the following look-up table

of rotated color masks:

CTABLE	DW	0003FH,0403FH,0803FH,0C03FH

DW	000CFH,010CFH,020CFH,030CFH

DW	000F3H,004F3H,008F3H,00CF3H

DW	000FCH,001FCH,002FCH,003FCH

Routines called: None.

Note: No bounds checking is performed.  Programmer must ensure the

coordinates and color are in their proper ranges.

-----------------------------------------------------------------------------

XOR_PT -- Plot a point using XOR onto the med res color screen.

Function: This routine plots a point on the med res screen using the

`exclusive or' operation. The pixel at the specified location is colored

with a color obtained by `exclusive oring' its original color with a

specified color.  This function is useful for making cursors.

Input: Upon entry:

X-coordinate (0-319) of the point is in SI

Y-coordinate (0-199) of the point is in DI

color (0-3) is in DX.

Output: Just to the screen.

Registers used: Unmodified; SI,DI, & DX are used for input.

Segments referenced: Upon entry, ES must point to video RAM at B8000, and

DS must point to a data segment containing the following look-up table

of rotated color masks:

CTABLE	DW	0003FH,0403FH,0803FH,0C03FH

DW	000CFH,010CFH,020CFH,030CFH

DW	000F3H,004F3H,008F3H,00CF3H

DW	000FCH,001FCH,002FCH,003FCH

Routines called: None.

Note: No bounds checking is performed.  Programmer must ensure the

coordinates and color are in their proper ranges.

-----------------------------------------------------------------------------

GET_COLOR  -- Get color of a point on med res screen.

Function: This routine returns the color of a specified point on the med

res color screen.  The color is returned in AL.

Input: Upon entry:

X-coordinate (0-319) of the specified point is in SI

Y-coordinate (0-199) of the specified point is in DI.

Output: Upon exit, AL contains the color (0-3) of the pixel at the

specified location.

Registers used: Only AX is modified.  SI & DI are used for input and AL is

used for output.

Segments referenced: Upon entry, ES must point to the video RAM at B8000H.

Routines called: None.

Note: No bounds checking is performed.  Programmer must ensure the

coordinates and color are in their proper ranges.

-----------------------------------------------------------------------------

SET_BOX -- Fill a rectangular box.

Function: This routine fills a rectangular box on the color graphics screen

with a specified color.

Input: Upon entry:

X-coordinate of upper left corner is in X1

Y-coordinate of upper left corner is in Y1

X-coordinate of lower right corner is in X2

Y-coordinate of lower right corner is in Y2

Color of the rectangle is in bits 0 & 1 of COLOR.

Output: Just to the screen.

Registers used: Unmodified; SI, DI, DX, BX, CX, & AX are save & restored.

Segments referenced: Upon entry, ES must point to video RAM at B8000, and

DS must point to a data segment containing the following look-up table

for color masks:

XTABLE	DW	0FFC0H,0FFF0H,0FFFCH,0FFFFH

DW	03FC0H,03FF0H,03FFCH,03FFFH

DW	00FC0H,00FF0H,00FFCH,00FFFH

DW	003C0H,003F0H,003FCH,003FFH

Routines called: None.

Note: No bounds checking is performed.  Programmer must ensure the

coordinates are in their proper range and order; i.e., the following

must be true:

0 <= X1 <= X2 <= 319

0 <= Y1 <= Y2 <= 199.

-----------------------------------------------------------------------------

XOR_BOX -- Fill a rectangular box using XOR.

Function: This routine fills a rectangular box in the color graphics screen

with a given color using the `exclusive or' operation. Each pixel in the

rectangle is colored with a color obtained by `exclusive oring' its

original color with a specified color.  This function is useful for

making cursors.

Input: Upon entry:

X-coordinate of upper left corner is in X1

Y-coordinate of upper left corner is in Y1

X-coordinate of lower right corner is in X2

Y-coordinate of lower right corner is in Y2

Color of the rectangle is in bits 0 & 1 of COLOR.

Output: Just to the screen.

Registers used: Unmodified; SI, DI, DX, BX, CX, & AX are saved & restored.

Segments referenced: Upon entry, ES must point to video RAM at B8000, and

DS must point to a data segment containing the following look-up table

for color masks:

XTABLE	DW	0FFC0H,0FFF0H,0FFFCH,0FFFFH

DW	03FC0H,03FF0H,03FFCH,03FFFH

DW	00FC0H,00FF0H,00FFCH,00FFFH

DW	003C0H,003F0H,003FCH,003FFH

Routines called: None.

Note: No bounds checking is performed.  Programmer must ensure the

coordinates are in their proper range and order; i.e., the following

must be true:

0 <= X1 <= X2 <= 319

0 <= Y1 <= Y2 <= 199.

-----------------------------------------------------------------------------

Second Level Functions (use Primitive Functions)

----------------------

SET_LIN -- Draw a line.

Function: This routine draws a line from (X1,Y1) to (X2,Y2) in the specified

color.  It uses Bresenham's algorithm.

Input: Upon entry:

X-coordinate of starting point is in X1

Y-coordinate of starting point is in Y1

X-coordinate of ending point is in X2

Y-coordinate of ending point is in Y2

Color of line is in COLOR.

Output: Just to the screen.

Registers used: Unmodified; BX, CX, DX, SI, DI, & AX are saved & restored.

Segments referenced: Upon entry, ES must point to video RAM at B8000H, and

DS must point to a data segment used by the SET_PT plotting routine.

Routines called: SET_PT.

Note: No bounds checking is performed.  Programmer must ensure the

coordinates are in their proper range; i.e., the following

must be true:

0 <= X1 & X2 <= 319

0 <= Y1 & Y2 <= 199

0 <= COLOR <= 3.

-----------------------------------------------------------------------------

SCHAR -- Plot a stroke character.

Function: This routine plots a stroke character. It uses a stroke character

table in which each character is stored as a series of strokes.  The

programmer must create this stroke character table according to specific

rules. Each stroke is stored as three bytes.  The first byte contains a

code as follows:

1AH = end of strokes

`U' = pen Up, move to new current position,

but don't draw

`D' = pen Down, draw a stroke from old to

new current position.

The second byte contains the local X-coordinate of the new current

position, and the third byte contains the local Y-coordinate of the new

current position. These local coordinates are relative to the upper left

corner of the character cell. At the beginning of the stroke table is an

address table for the locations of the strokes for each of the characters.

Input: Upon entry:

ASCII code character is in AL

X-coord of upper left corner of char cell is in X0

Y-coord of upper left corner of char cell is in Y0

Horizontal magnitude is in XMAGN

Vertical magnitude is in YMAGN

Color of character is in COLOR.

Output: Just to the screen.

Registers used: Unmodified; SI, CX, & AX are saved & restored.

Segments referenced: Upon entry, ES must point to video RAM at B8000H, and

DS must point to the data segment used by the point and line-drawing

routines. This data segment must also contain the table of stroke chars.

Routines called: SET_LIN.

Note: No bounds checking is performed. Unpredictable results happen if the

horizontal or vertical magnitude is too large. A string of raster

characters can be printed using the GMSG_OUT routine.

-----------------------------------------------------------------------------

RCHAR  -- Plot a raster character.

Function: This routine plots a raster character. It uses the raster

character table in the IBM BIOS ROM. Only ASCII codes 0 through 127

are supported.

Input: Upon entry:

ASCII code character is in AL

X-coord of upper left corner of char cell is in X0

Y-coord of upper left corner of char cell is in Y0

Horizontal magnitude is in XMAGN

Vertical magnitude is in YMAGN

Color of character is in COLOR.

Output: Just to the screen

Registers used: Unmodified; SI, DX, CX, & AX are saved & restored.

Segments referenced: Upon entry, ES must point to video RAM at B8000H, and

DS must point to the data segment used by the box-fill routine.

Routines called: SET_BOX.

Note: No bounds checking is performed. Unpredictable results happen if the

horizontal or vertical magnitude is too large. A string of raster

characters can be printed using the GMSG_OUT routine.

-----------------------------------------------------------------------------

GMSG_OUT  -- Print a string on the graphics screen.

Function: This routine prints a message on the graphics screen, using the

SCHAR or RCHAR routines. The message terminates in a zero.

Input: Upon entry:

Address of message is in SI

X-coord of upper left corner of string is in XMSG

Y-coord of upper left corner of string is in YMSG

Horizontal magnitude of characters is in XMAGN

Vertical magnitude of characters is in YMAGN

Color of characters is in COLOR

Choice of fonts (0=stroke,1=raster) is in FONT.

Output: Just to the screen

Registers used: Unmodified.

Segments referenced: Upon entry, ES must point to video RAM at B8000H, and

DS must point to the data segment used by the point-plotting, box-filling,

line-drawing, and stroke character routines.

Routines called: RCHAR & SCHAR

Note: No bounds checking is performed. Unpredictable results happen if the

horizontal or vertical magnitude is too large.

-----------------------------------------------------------------------------

PAINT -- Fill an area on the screen with color.

Function: This routine fills an area on the graphics screen with a specified

color. It begins `painting' at a `seed' position, filling a region bounded

by a `boundary' color.

Input: Upon entry:

X-coordinate of seed is in SI

Y-coordinate of seed is in DI

Paint color is in the low byte of COLOR

Boundary color is in the high byte of COLOR.

Output: Just to the screen.

Registers used: Unmodified.

Segments referenced: Upon entry, ES must point to video RAM at B8000H, and

DS must point to the data segment used by the point-plotting & get-color

routines.

Routines called: SET_PT & GET_COLOR

Note: The region must be completely surrounded by a boundary drawn in the

boundary color. Any paint color in the region can obstruct the filling

process, acting just like a boundary. This algorithm uses its own stack.

If the region is too complex, this stack will overflow.  There is no

check for stack overflow, but one could easily be added.

-----------------------------------------------------------------------------

_____________________________________________________________________________

>>>>> Physical EOF GRAPHICS.DOC <<<<<

####################################################################################################################