.R:Q

.H:

.H:

.H:...UASM - Universal Cross Assembler...

.H:

.F:

.F:...Custom Computer Consultants        $$$                  10 April 1986...

.F:

UASM - Universal Cross Assembler

8051

6805

Z8

10 April 1986

#

##1. INTRODUCTION

#General Description

UASM  is  a  program which facilitates the writing of assemblers for single

chip microcomputers.  It has only the rudimentary features of decoding  the

instruction  set  and a few pseudo operations.  It is intended for programs

that can be contained in a single  file  which  is  typical  of  the  short

programs written for these types of processors.  In particular there are no

provisions  for  macros  or include files.  The performance when generating

both an object file and a listing is about 700 statements per  minute  when

using a hard disk.

#Program Execution

UASM has several different names depending on which processor it has been

built for.  It is invoked by entering the name of the program ( asm05 for

the 6805 version) followed by the name of the source file followed by the

option switches.  The allowable options are at the present time are :

o       creates an object output file from the name of the source file

and the extension ".HEX".

l       creates a listing file from the name of the source file and the

extension ".LST".

n       suppress the listing function.

If the l and the n options are both absent then the  listing  goes  to  the

terminal.  If  no source file is specified then the input is taken from the

terminal and ouput is written to the terminal.  The  assembler  is  entered

directly  into  pass  2 with no symbol table.  This feature was mainly used

for program debugging and has not been removed.

#Program Operation

UASM is a two pass assembler.  On the first pass it reads  statements  from

the  input file and builds a symbol table.  On the second pass it reads the

statements from the input file and creates the object output file  and  the

listing  file  using  the  symbol table information it created in the first

pass.  At the end of the listing file the content of the  symbol  table  is

added.  The  object ouput is in Intel Hex format with a maximum of 34 bytes

to a record.  When displayed on  an  80  column  screen  the  records  will

exactly  fill  on  line.  The  listing  file is formatted with 55 statement

lines and four header lines per page and assumes 11 inch pages and 6  lines

per inch.  This leaves a combined top and bottom margin of seven lines.

#Files

The UASM system was written in the C language and consists of the following

set of files :

STDIO.H         Standard I/O Header File

UASM.H          UASM header file; same for all versions

UASM.C          Source code for the part common to all the

versions.

51SYM.C         Symbols and processing routines for the 8051.

6805SYM.C       Symbols and processing routines for the 6805.

Z8SYM.C         Symbols and processing routines for the Z8.

ASM51.EXE       8051 assembler execution file.

ASM05.EXE       6805 assembler execution file.

ASMZ8.EXE       Z8 assembler execution file.

51VAL.MAC       8051 validation/example file.

6805VAL.MAC     6805 validation/example file.

Z8VAL.MAC       Z8 validation/example file.

U-ASM.DOC       This File

#

##2. Assembler Conventions

#Statement Format

The  primary  component  in  an assembly language program is the statement,

which  consists  of  an  instruction  and  its  operands.  The  instruction

specifies  and  action  to be taken and the operands specify the data to be

acted upon.  An assembly language statement can include four fields:

Statement Labels

Instructions or Pseudo Operations

Operands

Comments

The statement label and comments are optional.  The statement may have zero

or more operands depending on the instruction.  The fields in  a  statement

are  separated  from  each  other  by  one  or more delimiters.  Delimiters

include spaces, tabs, commas, semi-colons, colons, newlines.  To accomodate

various programming styles the rules on delimiters  are  very  relaxed.  In

general  blanks separate fields and commas separate operands.  Colons after

labels are optional and delimiters for the comment field are also optional.

This can lead to problems with evaluating expressions if the first word  in

a comment happens to be a recognizeable symbol.  Therefore use of a comment

delimiter  (";")  is  strongly  recommended.  Comments may occupy an entire

line when the first character is either a semi-colon (";") or  an  asterisk

("*").

#Program Labels and Identifiers

A  program  label  is any identifier that begins in the first position of a

line.  An identifier is any string of alphanumeric characters  whose  first

character  is  alphabetic.  A  program label may be followed by an optional

colon.  A character is alphbetic if it is either upper case or lower  case.

A  character  is  alphanumeric  if  it  is  either alphabetic or numeric or

underscore.

upper case =  { A B C D E F G H I J K L M N O P Q R S T U V W X Y Z }

lower case =  { a b c d e f g h i j k l m n o p q r s t u v w x y z }

numeric = { 0 1 2 3 4 5 6 7 8 9 }

alphabetic = { upper case } or { lower case }

alphanumeric = { alphabetic } or { numeric } or { _ }

identifier = { alphabetic }  followed by one or more { alphanumeric }

program label = identifier in first position of a line

Each time an identifier is used it must be written in exactly the same  way

or  it  will  be considered a different identifier.  Identifiers may be any

length but only 31 characters are significant.  The following are valid and

unique identifiers:

begin                   BEGIN                   bEgIn

label12                 LABEL12                 lAbEl12

group1:                 GROUP1:                 gRoUp1:

In addition to their statement labeling function identifiers also represent

constants and variable names.  They may even define a value for the symbols

which  represent  instructions and pseudo operations,  and no conflict will

result.

#Instructions

The instruction field contains a mnemonic symbol which specifies one of the

processors instructions or a pseudo operation.  A pseudo  operation  is  an

instruction  to the assembler to perform some action.  Instructions must be

preceeded and followed by one or more spaces  to  separate  them  from  the

label   and  operand  fields.   Pseudo  operations  are  described  in  the

appendicies

#Operands

Depending on the instruction there can  be  zero,  one  or  more  operands.

Multiple operands are separated from each other by commas.  Operands supply

the  information  the  instruction  needs  to  carry out its operation.  An

operand can be :

Immediate data

The address of a location from which data is to be taken

(source address)

The address of a location where data is to be put(destination

address)

The address of a program location to which program control is

to be passed.

A condition code, used to direct the flow of program control

Each of the different assemblers has specific rules on the ordering of the

operands.  See the specific appendix for details.

#Comments

Comments may occur by themselves on a line or be placed after the  operands

field.  When  they occur by themselves on a line the first character of the

line must be a semi-colon (";") or an asterisk ("*").  When they  are  part

of  a  line  they  may follow the operand field with or without a specdific

delimiter other than blanks.  The use of  a  delimiter  is  recommended  if

there  exists  the  possibility  of  evaluating  part  of the comment as an

expression.

#Constants

Numeric  constants  may  be  expressed  in  binary,   octal,   decimal  and

hexidecimal.  Decimal  is  the  default  radix  for  evaluation  of numeric

constants.  The default radix may be overridden by the use of a  suffix  on

the  end  of  the  number.  The  suffix is always a #lower case# letter.

The following are valid suffixes:

b  -  binary

q  -

o  -  octal

d  -  decimal

h  -

x  -  hexidecimal

The convention of using lower case letters for the radix specifier requires

that  hexidecimal  digits for 10 through 15 be designated by the #upper case#

letters A through F.

A character constant is specified by  enclosing  the  character  in  single

quotes.

#Data Variables

A  data  variable  is  a memory location which contains a variable piece of

data.  There can be separate memory areas for different kinds of data.  The

data variable name can be associated with registers, bits, internal memory,

and external memory.

#Location Counter

The  value  of the location counter is represented by the dollar sign.  The

dollar sign may be freely used in expressions.

#Memory Segments#

In the single chip microprocessors  served  by  this  assembler  there  are

multiple address spaces.  These separate address spaces are called segments

and  are  indicated to the assembler by appropriate pseudo operations.  The

available segments and pseudo operations are shown in the following table:

Pseudo

#Operation       8051            6805            Z8

CSEG            Yes             Yes             Yes

DSEG            Yes             Yes             Yes

BSEG            Yes             No              No

RSEG            No              No              Yes

XSEG            Yes             No              Yes

#NOTE :

##Object output will be placed only in the code and external segments.

#Expressions and Operators

Expressions are formed by combining constants and data variable names  with

arithmetic  and  logical  operators.  Evaluation  is  left  to  right  with

precedence.  The operators are grouped according to the following table:

HIGHEST        (  )

+  -  ~

*  /  %

.  +  -

<  >

&

^

LOWEST         |

#Parentheses  ( )#

Parentheses are used to force the evaluation of the sub expression  within.

They  alter  the  normal left to right evaluation of an expression.  In the

following expression:

A - ( B + C )

the sub-expression B + C is evaluated and then  added  to  A.  Without  the

parentheses the evaluation would be:

( A - B ) + C

#Unary Operators  +  -  ~#

Three  unary  operators  are provided:  unary plus and minus along with the

logical NOT operator ( ~ ) have the highest precedence.

#Arithmetic Operators  +  -  *  /  .  %#

The arithmetic operators  are  the  familiar  plus,  minus,  multiply,  and

divide.   These  operations  are  performed  on  16  bit  two's  complement

integers.  The integer division  operation  will  truncate  any  fractional

result.

The  period is used as a synonym for plus.  It occurs primarily in the 8051

assembler for specifying a bit field.

The percent is the modulus operator.  It divides the left  operand  by  the

right operand and produces the remainder.

#Shift Operators  <  >#

The  left  and  right  angle  brackets  are  used  for left and right shift

operations respectively.  The operand on the left is shifted the number  of

places specified by the operand on the right.

#Logical Operators  &  |  ^#

The  logical  operators  perform  the indicated boolean operation on 16 bit

unsigned integers.

The ampersand ( & ) performs a logical AND

The bar ( | ) performs a logical OR

The caret ( ^ ) performs a logical EXCLUSIVE OR

#

##3.  Pseudo Operations

#ASCII - Define a String of ASCII Characters#

This operation takes a string of ASCII enclosed within any convenient  pair

of  delimiters  and assembles one byte per character in the string.  If the

trailing delimiter is omitted then the end of line character is assumed  to

terminate the string.

Examples:

ascii   /String in Slashes/

ascii   "String in Double Quotes"

ascii   'String in Single Quotes'

#BIT - Define Symbol as a Bit Address#

This  operation  defines  the  label to the left of the pseudo operation to

have the value of the expression on the right.  This  pseudo  operation  is

valid only in the 8051 assembler.

Example:

irq     bit     0EAx

defines the symbol irq and assigns it the hexidecimal value EA.

This operation is similar in function to the EQU and REG pseudo operations.

#BSEG - Define Bit Segment#

This  operation  takes  no  operands.  It  tells  the  assembler to set the

location counter to the current value for  the  bit  segment.  The  initial

values  for  the  bit  segment  depend  on the particular assembler and are

currently set to the following values :

8051 :           0

6805 :           Not Implementd

Z8   :           Not Implementd -- See RSEG

Refer to the descriptions of CSEG, RSEG, DSEG,  and XSEG.  Each time one of

the  segment  pseudo  operations  is  invoked it saves the current location

counter and retrieves the indicated one.  Segments may be freely  mixed  in

the source file.

#CSEG - Define Code Segment#

This  operation  takes  no  operands.  It  tells  the  assembler to set the

location counter to the current value for the  code  segment.  The  initial

values  for  the  code  segment  depend on the particular assembler and are

currently set to the following values :

8051 :           0

6805 :         80x

Z8   :           0

Refer to the descriptions of BSEG, RSEG, DSEG,  and XSEG.  Each time one of

the  segment  pseudo  operations  is  invoked it saves the current location

counter and retrieves the indicated one.  Segments may be freely  mixed  in

the source file.

#DB - Define Byte#

This  pseudo  operation  takes one or more expressions separated by commas,

evaluates the expressions,  converts the results to bytes,  and places them

in sequential locations.

Example:

db      93x,a+4,22

#DS - Define Storage#

This  pseudo  operation  takes  a  single operand and adds the value to the

current location counter.  The effect is to reserve that many bytes without

assigning a value to them.  The value zero ( 0 ) may  be  used  to  provide

duplicate definitions of the same symbol.

Example:

ds      14      ; reserve 14 bytes

table   ds      0       ; define table to have the value of the current

item    ds      1       ; location counter.  Define item to have the

word    ds      2       ; same value as table.  Define word = table + 1

#DSEG - Define Data Segment#

This  operation  takes  no  operands.  It  tells  the  assembler to set the

location counter to the current value for the  data  segment.  The  initial

values  for  the  data  segment  depend on the particular assembler and are

currently set to the following values :

8051 :          32

6805 :           0

Z8   :           0

Refer to the descriptions of BSEG, RSEG, CSEG,  and XSEG.  Each time one of

the  segment  pseudo  operations  is  invoked it saves the current location

counter and retrieves the indicated one.  Segments may be freely  mixed  in

the source file.

#DW - Define Word#

This  pseudo operation is similar to DB except tha each expression produces

a two byte word value which is placed in memory with the  most  significant

byte  first  and the least significant byte last.  Multiple operands may be

specified separated by commas.

Example :

dw      137x    ; defines the word with value 137 hex.

#EQU - Equate

This operation defines the label to the left of  the  pseudo  operation  to

have the value of the expression on the right.

Example:

irq     equ     85

defines the symbol irq and assigns it the decimal value 85

This operation is similar in function to the BIT and REG pseudo operations.

#ORG - Origin

This  pseudo  operation  takes  a  single  operand which is assigned to the

location counter for the current segment.

Example :

cseg

org     200x    ; set the code segment location counter to

; hexidecimal 200.

#PAGE - New Page

This pseudo operation causes the listing of the following lines to begin on

a new page.  It take no operands.

#RADIX - Set Radix

This pseudo operation sets the default radix for  number  conversion.  This

radix  is  used  for all number conversions unless explicitly overridden by

the radix suffix character.  Standard values for radix are 2,  8,  10,  and

16.  Other  values are possible and use the upper case letters to stand for

the digits 10 through whatever.

Example :

radix   16d     ; specify hexidecimal

radix   8d      ; specify octal

The default radix for UASM is ten.

#REG - Define Symbol as a Working Register#

This operation defines the label to the left of  the  pseudo  operation  to

have  the  value  of the expression on the right.  This pseudo operation is

valid only in the Z8 assembler.

Example:

irq     reg     10

defines the symbol irq and assigns it the decimal value 10.

This operation is similar in function to the EQU and BIT pseudo operations.

#RSEG - Define Working Register Segment#

This operation takes no  operands.  It  tells  the  assembler  to  set  the

location  counter  to  the  current  value  for  the register segment.  The

initial values for the register segment depend on the particular  assembler

and are currently set to the following values :

8051 :           Not implemented -- See BSEG

6805 :           Not Implementd

Z8   :           0

Refer to the descriptions of CSEG,  BSEG, DSEG, and XSEG.  Each time one of

the segment pseudo operations is invoked  it  saves  the  current  location

counter  and  retrieves the indicated one.  Segments may be freely mixed in

the source file.

#XSEG - Define External Segment#

This operation takes no  operands.  It  tells  the  assembler  to  set  the

location  counter  to  the  current  value  for  the external segment.  The

initial values for the external segment depend on the particular  assembler

and are currently set to the following values :

8051 :           16384

6805 :           Not Implementd

Z8   :           4096

Refer to the descriptions of CSEG,  RSEG, DSEG, and BSEG.  Each time one of

the segment pseudo operations is invoked  it  saves  the  current  location

counter  and  retrieves the indicated one.  Segments may be freely mixed in

the source file.