FASTHST 2.2 (Formerly 19200HST)

By Joseph Sheppard

Sysop, The Ledge PCBoard (818) 352-3620

WHAT IT DOES:

-------------

FASTHST is a utility program written with Turbo Pascal 5.5 using

routines from Prokit by Samuel Smith.

FASTHST is released as a free program.  No donations are requested for

it's use.  The authors are not responsible for any damage that might be

caused by the use of this program.  Please use this program at your own

risk.

The purpose of FASTHST is to aid owners of USRobotics Courier HST

modems in setting up for optimum speed.  It will work with either

older HST's that open at 19200 baud, or the newer ones that open at

38,400 baud (1440 HST's).

This program is designed for callers only.  Although I run my BBS with

the settings from FASTHST, I would recommend that Sysops follow the

instructions for their specific BBS package on modem settings.

PCBoard sysops have a similar program called PCBMODEM that

should be used instead of this program.

This program will allow you to lock your serial port at 19200 or

38400 depending on the model HST you have.  This setting may

only be effective on 286 and 386 systems.  Some multi-tasking software

may not be able to handle this setting.  If you find that you are

having trouble with aborted downloads after running this program then

you should probably go into your terminal program and type AT&F&W

followed by the ENTER key to reset the modem to it's factory settings.

Using these settings along with protocols designed for error correcting

modems such as Ymodem-G, Xmodem-1K-G or Zmodem MobyTurbo I have

achieved file transfer speeds in the 1100 - 1150 cps range on a regular

basis using both 286 and 386 systems with the old modem HST open at

19200 bps.  With the new 1440 HST open at 38,400 I have achieved speeds

in excess of 1700 cps using Ymodem-G and Zmodem Mobyturbo.

HOW IT DOES IT:

---------------

FASTHST starts by giving the suggested dipswitch settings, which are

as follows:

1   2   3   4   5   6   7   8   9   10

--  --  --  --  --  --  --  --  --  --

UP  UP  DN  DN  DN  UP  UP  DN  UP  UP

These settings will do the following to your modem:

1 UP - DTR is controlled by software

2 UP - Send verbal result codes

3 DN - Result codes are displayed

4 DN - Local echo of commands is suppressed

5 DN - Auto Answer Suppressed

6 UP - Do not force carrier detect

7 UP - Single phone line connection (RJ-11)

8 DN - AT command set enabled

9 UP - Hang up when escape code is sent

10 UP- Load from NRAM in power up

Next, it will ask you if you want to disable the "Data Compression"

(MNP Level 5) feature.  If you disable this feature, then you will

have better performance when transferring compressed files (ZIP,

ARC, LZH etc).  If you leave data compression enabled, then you will

be able to transfer text files faster, but compressed files will go

at a slower rate.

If you will be using your HST mainly for downloading ZIP or ARC

files from BBS system, it is recommended that you disable data

compression by answering Yes to this prompt.

You will then be asked weather you want to disable the modem

speaker. Because FASTHST programms the modem to send detailed status

messages to the terminal program, you may want to rely on these

silet messages.  If you choose to have the speaker on, then it will

be on from the time you dial, until connection is made.

It then will reset your modems internal NRAM settings to the original

factory defaults.

Next it sends the following settings to the modem's non-volitile

memory:

ATS7=60E0M0X7

S7=60 - Number of seconds modem waits for carrier

E0 - Local echo off

M0 - Speaker Off (can be overidden by putting M1 in your modem program's

initialization string)

X7 - Result code option (OK, CONNECT, RING, NO CARRIER, ERROR, CONNECT,

NO DIAL TONE, BUSY, NO ANSWER, RINGING, VOICE)

Once this string is received it sends the following additional

settings:

AT&H3&R2&S1&B1&Kx&W

&H1 - Hardware flow control

&R2 - Received data output to terminal on RTS high

&S1 - Modem controls DSR

&B1 - Lock port at speed setting (19200 or 38400)

&Kx - Disable or enable data compression depending on how you

answered the data compression prompt (x=0 or 1).

&W -  Write these settings to non-volitile RAM (NRAM).

After running this program, you should go into the setup portion of

your modem program (Qmodem, Procomm or whatever) and change the modem

initialization string to ATH0 followed by whatever character that

program uses to send a carriage return (Qmodem uses a { Procomm uses a

!).

TIPS 'N' TRICKS:

----------------

If you want to overide any of the NRAM settings that I have written in

the program, you can either do by placing the replacement commands in

the modem initialization string, or by going into the terminal mode of

your modem software and typing those commands followed by AT&W.

It is highly recommended that you use a high performance protocol

driver such as DSZ by Chuck Foresberg.  The registered version of DSZ

includes YMODEM-G and Xmodem-1K-G protocols which should yield the

highest possible CPS ratings under this setup.  The current versions of

many popular modem programs have built-in versions of Xmodem-1K-G

(called Ymodem-G in some versions) which will yield excellent speed,

but won't allow batch mode downloading that true Ymodem-G from the DSZ

program features.

Zmodem MobyTurbo will give 1000-1700 cps on properly configured

systems. Zmodem is featured in non-registered versions of DSZ that are

available on most BBS systems.

Avoid using Xmodem, Ymodem, WXmodem, Kermit, Jmodem, Bimodem or other

protocols that rely on software error checking when you are connected

to another HST at the fixed 19200 bps rate.  These protocols will give

you between 400-900 cps.  One reason I wrote this program was that I

see people night after night calling with HST modems improperly

configured and getting transfer speeds of 400 cps.

When running with your port locked at top speed, you must go into

the setup area of your modem communication program and change the

port opening speed to your top speed (19200 or 38400).  You should

also change the call-out speeds in your dialing directory to the top

speed.  Finally, make sure that your modem program is set to NOT

reset the baud rate at redial.

SUPPORT:

--------

As stated above, this is a free program.  You will probably use it

once, then throw it away and forget all about me.  That's fine, but at

least when you call my BBS at 38400 you won't be getting 400 cps!

Updates to FASTHST are always available on my BBS, The Ledge PCBoard,

which is listed at the top of this file.  Although you can't download

from my file directory on the first call, you will be able to open the

SUPPORT door and download any of the programs that I've written, or any

third party programs (such as DSZxxxx.ARC) that are mentioned in this

file.  My support door DOES have Xmodem-1K-G, Ymodem-G, Zmodem

MobyTurbo and Super8k-G so if you are looking for a place to

test...come on!