The Microcom MNP Library

(Microsoft C Version)

PROGRAMMER'S MANUAL

Version 1.0

December 15, 1987

Microcom, Inc.

1400 Providence Highway

Norwood, MA 02062

The Microcom MNP Library and MNP are trademarks of Microcom, Inc.

Microsoft and MS-DOS are trademarks of Microsoft Corporation.

IBM is a registered trademark of International Business Machines.

This manual is not subject to copyright and may be freely copied.

Questions relating to the Microcom MNP Library should be directed to

Microcom at:

Microcom, Inc.

15303 Ventura Blvd., Suite 900

Sherman Oaks, CA 91403

(818)986-4212

1. INTRODUCTION

The Microcom MNP Library is a set of subroutines which implement the

stream mode of the link protocol in Microcom Networking Protocol (MNP).

This mode of the MNP link protocol is appropriate for interworking with MNP

error-correcting modems or with other software implementations which use

the Microcom MNP Library or other compatible software.

This version of the library provides a set of C and assembly

language routines which are designed to be easily integrated into an

application program written in C and compiled using the Microsoft C

compiler (version 4.0).  Through the use of these routines, an

application is able to perform error-free data communications over a

physical-connection, such as one established on the public switched

telephone network.

These MNP Library subroutines are especially for the IBM Personal

Computer family (and compatibles) under PCDOS (MS-DOS).  Hardware

facilities for asynchronous communications are required.

2. LIBRARY MODULES

The Microcom MNP LIbrary is supplied in one of two forms, object

library only and object library with source.  The source version is de-

scribed in more detail in a file ('readme.doc') supplied as part of the

source version distribution.

In the case of the object library only form, the distribution diskette

contains one file, mnp.lib, which includes the various object modules

which make up the MNP Library.

When building the executable form of a particular application, the

mnp.lib file should be specified if the application calls any MNP sub-

routine described in this document.  The manner in which library modules

are specified for inclusion varies according to the linkage editor

employed in building the application program run module.

3. LINK MANAGEMENT

In general, the MNP link follows the life of the normal physical-

connection.  Immediately after the physical-connection is made (e.g. after

a modem first reports carrier detected), the application must attempt the

establishment of the Link via the MNPCONNECT subroutine.  MNP-capable

modems which answer the incoming telephone call, for example, must receive

the Link establishment sequence within 4 seconds of physical-connection

establishment.  If a link is not attempted within this period, such modems

may fall back to a normal connection and it will not be possible to provide

a reliable connection on that particular call.

Link termination, performed via the MNPDISCONNECT call, should

immediately precede termination of the physical-connection, i.e. right

before hanging up the telephone.  Note that the Link is active for the

entire duration of the physical-connection.

During the data phase of the Link (i.e. after establishment but before

termination), the Link must be kept running by allowing the Link code to

execute periodically.  Any of the data phase calls, namely MNPSEND,

MNPRECEIVE, MNPSTATUS, and MNPBREAK, will accomplish this.  One of these

routines must be called every 250ms or so which means that the Link code

gets a chance to run about 4 times per second.  The absolute interval is

not critical and it may be possible to reduce the frequency as well and

still get reasonable performance.  Polling the Link code, however, must

occur with a predictable frequency otherwise the Link may be lost.  When

the Link is lost, an MNP error-correcting modem may clear the call.

When a Link is in progress, all line I/O is performed via the

appropriate MNP call rather than by using other C mechanisms.  It

may be the case, however, that the com port opened elsewhere in the

application, must remain open.

Data sent and received on the Link will be "chunked" depending on the

number of bytes which happen to be sent in a single data message.  The

maximum amount in a single message is 64 bytes.  This "chunking" is not

noticeable to the sender but can be perceived by the data receiver.

Strings of data may be split over more than one data message and some

messages may have to be sent more than once when there is noise on the

connection.  Retransmission can be seen by the application as an

interruption, possibly in the middle of some data string.  The application

should be examined to see if there are any cases involving timing

dependencies which could be adversely effected by this kind of

irregularity in the data flow.

4. LINK INTERFACE SUBROUTINES

4.1  Link Establishment - MNPCONNECT

MNPCONNECT is used to establish a Link.  It is called after the

physical-connection has been established.  It can take up to 5 seconds

for an MNPCONNECT call to complete.

MNPCONNECT has the following parameters:

RATE - an integer which indicates the speed of the

physical-connection, as follows:

1 = 110 bps

2 = 300 bps

4 = 1200 bps

5 = 2400 bps

[Note: Other speeds are not supported in this

version of the Microcom MNP Library.]

FORMAT - an integer which indicates the data format,

as follows:

0 = 8 data bits, no parity

1 = 7 data bits, even parity

2 = 7 data bits, odd parity

3 = 7 data bits, mark parity

4 = 7 data bits, space parity

[Note: In this Library version, once the Link

is established, it is not possible to change

format.]

PORT -  an integer which indicates the communications

port, as follows:

1 = COM1

2 = COM2

[Note: This Library version does not support

other comm ports.  Also, COM2 cannot be speci-

fied if no COM1 is present in the system.]

MODE - an integer which indicates the role in link

establishment, as follows:

0 = link initiator (caller)

1 = link accepter (answerer)

The function returns an integer value, as follows:

RETCODE - an integer returned to the caller which

indicates the result of the link

establishment attempt, as follows:

0 = success (link established)

-64 - failure, no additional info

-65 - failure, timeout (no remote response)

-66 - failure, physical-connection lost

(carrier lost)

-70 - failure, incompatible MNP

-72 - failure, remote protocol error

[Note: -70 and -72 should never occur.]

MNPCONNECT is called as follows:

RETCODE = MNPCONNECT(RATE,FORMAT,PORT,MODE);

4.2  Link Termination - MNPDISCONNECT

MNPDISCONNECT has two functions, to send the appropriate protocol

message to the other side to indicate Link termination and to reset the PC

interrupt environment.  MNPDISCONNECT must always be called if MNPCONNECT

had been called previously.  This is so even for establishment failure or

for loss of the physical-connection (i.e. carrier lost).

MNPDISCONNECT has no parameters and returns no value.  It is

called as follows:

MNPDISCONNECT();

4.3  Send Data - MNPSEND

MNPSEND is called to copy data to be sent on the Link from the

application into the MNP transmit buffer.

MNPSEND has parameters as follows:

SNDBUF - a pointer to the user char buffer from which

transmit data is to be copied.

BUFLEN - an integer which indicates the maximum

number of data bytes which can be copied

from the user buffer

The function returns an integer value, as follows:

RETCODE - an integer returned to the caller which

indicates the number of bytes copied

or a Link error condition, as follows:

positive int = number of bytes copied

0=no bytes copied

-64 = link terminated, retransmission limit

exceeded

-65 = link terminated, unable to send

-66 = link terminated, carrier lost

-67 = link terminated, remote disconnected

MNPSEND is called as follows:

RETCODE = MNPSEND(&SNDBUF,BUFLEN)

4.4  Receive Data - MNPRECEIVE

MNPRECEIVE is called to copy data received on the Link into the

application.  MNPRECEIVE transfers all available data up to the size of the

application buffer.  MNPRECEIVE returns immediately if no data is

available.

MNPRECEIVE has the following parameters:

RCVBUF - a pointer to the user char buffer into which

received data is to be copied. This buffer must

be at least as long as the value of BUFLEN.

BUFLEN - an integer which indicates the maximum

number of data bytes which can be copied

into the user buffer

The function returns an integer value, as follows:

RETCODE - an integer returned to the caller which

indicates the number of bytes copied

or a Link error condition, as follows:

positive int = number of bytes copied

0=no bytes available.

-64 = link terminated.  retransmission limit

exceeded

-65 = link terminated, unable to send

-66 = link terminated,  carrier lost

-67 = link terminated, remote disconnected

MNPRECEIVE is called as follows:

RETCODE =  MNPRECEIVE(&RCVBUF,BUFLEN);

4.5  Link Status - MNPSTATUS

MNPSTATUS is called to return to the application the status of the

Link and to read parameters which indicate the availability of received

data or the ability to enqueue data for transmission.

MNPSTATUS has a structure as its parameter, with members

as follows:

PSTATUS - an integer which indicates the condition of

the physical-connection, as follows:

0 = not connected (carrier not present)

1 = connected (carrier present)

LSTATUS - an integer which indicates the condition of

Link, as follows:

0 = link not established (i.e. link

terminated)

1 = link established

SCOUNT - an integer which indicates the number of

bytes free in the Link send buffer.  This is

the number of bytes which can be completely

copied to the Link transmit code via an

MNPSEND call.

RCOUNT - an integer which indicates the number of

received data bytes which are available to

be copied into the application via MNPRECEIVE

ALLSENT - an integer which indicates whether or not

all data passed to the Link code has been

successfully sent and acknowledged, as

follows:

0 = all data not sent

1 = all data sent

MNPSTATUS is called as follows:

MNPSTATUS(&STATUS_BLOCK);

4.6  Send Signal - MNPBREAK

MNPBREAK is called to send a signal to the remote DTE.  When the

remote side is an MNP error-correcting modem, an MNPBREAK call causes the

remote modem to send a break signal (continuous spacing) to its attached

DTE.

MNPBREAK employs the MNP Link Protocol's reliable attention

mechanism in 'destructive mode'.  That is, when MNPBREAK is called, the

Link is reset and any data pending transmission on the link is discarded as

is any received data which has not been read via the MNPRECEIVE subroutine.

Data may also be discarded at the remote side of the Link.

MNPBREAK has no parameters but returns an integer value,

as follows:

RETCODE - an integer returned to the caller which

indicates the result of the MNPBREAK call

or a Link error condition, as follows:

0 = break initiated

-64 = link terminated, retransmission limit

exceeded

-65 = link terminated, unable to send

-66 = link terminated, carrier lost

-67 = link terminated, remote disconnected

-75 = break not supported (remote did not

negotiate attention support for this

Link)

-76 = previous break still in progress

MNPBREAK is called as follows:

RETCODE = MNPBREAK();

[Note that the MNP Library does not support application

notification of the receipt of the MNP attention message and

hence it is assumed that MNPBREAK will only be  employed when the

remote side of the connection is an MNP error-correcting

modem.]