To: Whom it may concern

From: Lauren A. Colby, 10 E. 4th St., Frederick, MD. 21701

November 26, 1990

Re: Avoiding interrupt conflicts between serial ports

In an earlier memo, I presented a simple basic terminal

program to shut off the interrupts in a serial port and use

polling instead of interrupts, to operate the port. There was an

error in the comments in that program (SANSINT). The terms "LSB"

and "MSB" were reversed. The program still works; the comments

were simply incorrect.

I have worked on the program and made some improvements.

With the new version (SANSINT2), the program looks for a

configuration file (SANS.PRO). If it doesn't find the file, it

asks you for the port address, baud rate, etc., and lets you

configure the port, any way you want. At my home, I have a

machine with add-on ports which use the IBM PS/2 standard. In

that standard (if you can call it a "standard"), COM3 is decimal

12832; COM4 is decimal 12840. I'm controlling a PAKRATT ham radio

interface on COM4, at 2400 BAUDS, and everything works just fine,

even though I have a mouse on COM1 and a modem on COM2!

I'm giving you the files SANSINT2.BAS, and SANSINT2.EXE.

When you run SANSINT2.EXE for the first time, it will create

SANS.PRO. Be careful to answer the questions correctly; there is

no error trapping. Also, SANSINT2.BAS is written in Quick Basic,

so you'll need Quick Basic if you want to make modifications.

While on the subject of interrupt conflicts, I ran into a

curious problem at my law office, which is worth mentioning. We

have two virtually identical 286 machines, used by the

secretaries. I installed Seiko smart label printers on both

machines. Both printers were installed on COM1.

One of the machines worked perfectly from the start. With

the other machine, however, the machine would "lock up",

intermittently, when labels were printed. The machine that locked

up had a modem installed on COM2. The machine that worked OK had

no modem.

I called Seiko tech support and was told that the only

solution was to remove the modem; according to the technician,

"ports have a tendency to 'spread out' and create conflicts". I

didn't believe that! I had a hunch that the problem had something

to do with the fact that COM1 uses IRQ4, which has a lower

priority that IRQ3, which is normally used by COM2, and that if I

could just reconfigure the devices to put the printer on COM2 and

modem on COM1, everything would work OK.

In the course of doing the reconfiguring, I discovered that

the serial card which was installed would not work properly on

COM2 and I had to replace the card. With the new card installed

and the printer moved to COM2 and the modem reset to COM1,

everything has worked just fine. It's been six months since the

job was done, and there have been no more lockups. I can't tell

you whether the fix was the new card or the swap in port

assignments. However, because Seiko tech support has apparently

received many complaints about this problem, and there can't

possibly be that many bad cards, I'd guess that simply swapping

ports will work, in most cases.

Incidentally, I don't mean to be critical of Seiko. Their

smart label printers are the greatest product since sliced bread.

If, however, they'd re-write their software to bypass the

interrupts, it would greatly simplify installations. The same is

true for the writers of mouse software and communications

software. At one time, when machines were slow and memory very

limited, the interrupts were useful to cut down on system

overhead. They really aren't needed any more except, possibly for

high speed applications.