HDINFO - Hard Disk Information, Version 1.02

(c) Michael B. Smith, 11/30/1987

I. Introduction

HDINFO is a collection of all the information I could get my hands on about 224

different hard disk models.  It was gathered because hard disks rarely come

with any information about them, but it is often necessary to know detailed

specifications for them in order to get them working.

To make searches through this file easier, I have included a database program

called CARDFILE.  CARDFILE is a little pop-up, index-card file program that was

published in PC Magazine.  It is very simple, and was only designed for use as

a phone directory, but it is enough to allow you to browse through the file,

and to search for specific keywords.  I chose it for two reasons: (1) it is the

smallest dB program I could find, and (2) it is not a commercial program, so it

can be distributed freely with the hard disk information file.

Here is a list of all the files that should be included in HDINFO.ARC:

HDINFO       - The data file, in CARDFILE format

HDINFO.DOC   - These instructions that you are reading now

CARDFILE.COM - The database program for looking at HDINFO

CARDFILE.DOC - Documentation for CARDFILE.COM

(both courtesy of PC Magazine)

These files may also be included.  They contain the same information that

HDINFO does, but they are formatted for use with different database programs.

HDINFO.DBF     dBASE II format

HDINFO.DB3     dBASE III format

HDINFO.DIF     DIF format

HDINFO.ASC     (specially formatted)

The dBASE and DIF files have not actually been tested with a database program;

I was only able to create them by using Lotus 1-2-3 v2.01's Translate utility.

HDINFO.ASC is set up to allow the data to be imported or translated easily into

most any other database program or format that you might want to use.  For

example, the whole file can be read into Lotus 1-2-3 as labels, simply by

typing these commands:

/ File Import Numbers HDINFO.ASC [Return]

Each line in HDINFO.ASC is 161 characters long, and contains all of the data

for one particular model of hard disk.  Each item of data on a line is enclosed

in quotes (""), and they are all separated by commas.  The specific locations

of each field on the line is listed below.

Column(s)    Length      Field

---------    ------      --------------------------------

2-20           19        Brand

24-35          12        Model

39-43           5        Capacity (MB)

47-49           3        Access time (ms)

53-55           3        # of heads

59-63           5        # of cylinders

67-71           5        Write precomp cylinder

75-79           5        Reduced write current cylinder

83-87           5        Step pulse rate (microseconds)

91-95           5        Landing Zone cylinder

99-104          6        Hard disk "Type" when used on AT

105-106         2        Power use (in Watts)

113             1        Is drive RLL-compatible? (Y/N)

117             1        Does drive have auto-park? (Y/N)

121             1        Is it a 3.5-inch drive? (Y/N)

125             1        Is drive half-height? (Y/N)

129-160        32        Notes

[End of line]

II. How to Use It

To look through HDINFO, type "CARDFILE HDINFO" at the DOS prompt.  The

following message will appear:

C>cardfile hdinfo

Cardfile 1.0 (c) 1987 Ziff Communications Co.

Hotkey is ALT-RIGHT SHIFT

C>

You can now pop up CARDFILE at any time by holding down the Alt key and the

Right Shift key together.  You will see something like this:

������������������������������������������Ŀ

� Name   ATASI              3033           �

� Addr   28?   MB, 30  ms, 5   hds,   W    �

�        645   cyl, WPC:320  , RWC:        �

�        SPR:      �s, LZ:     , RLL:N     �

� Phone  ATT:2*    , AP: , 3�":N, �ht:N    �

� Note   SPR is in range of 2.5 to 500.    �

� ���������������������������������������� �

� F1-New F2-Ed F3-Del F4-Sch F5-Sv F6-Dial �

��������������������������������������������

These arcane abbreviations had to be used because CARDFILE only holds short

records (6 lines of 32 characters).  Use the key below to interpret their

meaning.

������������������������������������������Ŀ

� Name   [HD brand]         [HD model]     �

� Addr   _____ MB, ___ ms, ___ hds, __W    �

�        _____ cyl, WPC:____ , RWC:___     �

�        SPR:_____ �s, LZ:____ , RLL:_     �

� Phone  ATT:_____ , AP:_, 3�":_, �ht:_    �

� Note   [<==== Comments go here ====>]    �

� ���������������������������������������� �

� F1-New F2-Ed F3-Del F4-Sch F5-Sv F6-Dial �

��������������������������������������������

At the top, on the "Name" line, is the hard disk brand and model.  The headings

on the next four lines are abbreviated as follows:

1)   ___ MB  = Size of hard disk in megabytes

2)   ___ ms  = Average access time, in milliseconds

3)   ___ hds = Number of read/write heads

4)   ___W    = Power, in Watts

5)   ___ cyl = Number of cylinders

6)   WPC:___ = Write precomp cylinder

7)   RWC:___ = Reduced write current

8)   SPR:___ = Step pulse rate

9)   LZ:____ = Landing zone

10)  RLL:_   = Can it be used with RLL? (Y/N)

11)  ATT:___ = Hard disk type (when used with an AT)

12)  AP:_    = Does the hard disk have auto-park? (Y/N)

13)  3�":_   = Is it a 3.5-inch disk? (Y/N)

14)  �ht:_   = Is it a half-height drive? (Y/N)

15)  Note    = A special note about that drive

A more detailed explanation of these terms is given in the next section, "III.

Reference".

On most records, there are many entries that are blank.  This is because I

could not find that information in my sources.  When I gather more info, an

update for HDINFO will be released.

If there is a question mark by an entry, it is because I am not positive that

it is correct.  Unfortunately, dealers often give conflicting or incorrect

information, and the only specs I could definitely rely on were those obtained

directly from the manufacturer.

To browse through HDINFO, use the PgUp and PgDn keys to move up and down.  To

search for information on a specific brand or model, hit the F4 key to search.

The message "Find:" will appear on the bottom line of the window.  Then type in

the brand name, model number, or some other key word to look for.  CARDFILE

will find the first record that has the key word(s) you chose.  If that record

is not the one you were looking for, you can hit RETURN to continue the search,

or hit ESC to exit the search, and then use the PgUp and PgDn keys to look

around.

[For more detailed instructions on using CARDFILE, read CARDFILE.DOC.]

If you are not sure about the brand or model of your drive, take it out of the

computer and look at it.  There is usually a sticker on the back or side that

gives the manufacturer's name and/or the model number.  Sometimes the

manufacturer's name is different from the brand name the drive was sold under.

IBM itself doesn't have any drives under its name in HDINFO, because all the

drives were really made by Seagate or some other company.

III. Reference

Here are more detailed explanations of the specifications given in HDINFO.

1) Size in Megabytes

This is a fairly rough estimate.  A megabyte is really 1K x 1K, or 1024 x 1024,

or 1,048,576 bytes, but dealers (and even manufacturers) often count a hard

disk's size by millions of bytes (1,000,000).  This means they are exaggerating

by about 5%.  Also, the sizes given are often rounded off a bit.  Many 39

megabyte drives are sold as 40 megs, and 32 meg drives are sold as 30's.  So

keep in mind that there is a margin of error of 1 or 2 megabytes, and more on

really large drives.

Remember also that this is the FORMATTED capacity.  The sector numbering, etc.

that is written during formatting actually uses a large chunk of the space on

the hard disk -- easily 5-10 megabytes!!  The "formatted capacity" is how much

space is really left once the drive is ready to use.  Beware of advertisements

that don't specifically say "formatted capacity".  They may mean the

UNformatted capacity, which is a full 20-25% more than you will really get to

use.

Also note that the size given here is for MFM formatting (the way drives are

normally formatted), even if the drive can be used with RLL formatting (see 10

below).  To find the drive's size with RLL formatting, multiply the number by

1.5.

Last, but not least, remember that PC- & MS-DOS 2.x can't handle hard disks

larger than 16 megabytes, and even DOS 3.x can't handle ones larger than 32

megabytes.  There is software available, however, that will get around this.

PCs Limited includes such software with the large hard drives they sell, and

most dealers offer Disk Manager (by Ontrack Systems) or Speedstor (by Storage

Dimensions) for this purpose with their large drives.  Both sell for about

$50-$70.  I personally recommend SpeedStor.  The address for Storage

Dimensions is given in Section IV below.

2) Average access time

This is only one of the many ways hard disks' speed are measured.  It is given

as the "maximum average" access time, in milliseconds.  It seems to be a

reliable way to compare hard disk's speeds, but remember that dealers don't

always have correct information.  For comparison, an IBM XT hard disk is rated

at 80 ms, and an IBM AT drive is rated at 40 ms.

3) Number of read/write heads

Hard disk drives usually have several different disks ("platters") inside them,

and a read/write head on *both* sides of each platter.  When a drive has an odd

number of heads, it is usually because there is extra head and platter-side

used only for positioning data.

4) Power, in Watts

This is the typical power the drive uses in Watts.  The smaller the number, the

less drain the disk puts on your computer's power supply.  Unfortunately, I

only have this information for a few models.

5) Number of cylinders

Data is written on a disk in rings, with the first ring at the outer edge, and

the last near the center.  In a hard disk drive (which actually has several

disks inside it), a cylinder is defined as a particular ring on ALL disks.

That is, cylinder 0 in a drive with 4 read/write heads is the outer ring on the

disk read by head 0, AND the outer ring on the disk read by head 1, and so on

with heads 2 and 3.  When formatting a disk with DOS 3.2 or 3.3, you can see a

running display of which head and which cylinder is currently being formatted.

Obviously, more cylinders on a disk means more data can be stored.  Small-

capacity drives usually have about 300 cylinders, and large one will have 1024

or more.

There are often a number of "extra" cylinders near the middle of the disk that

don't get used for data.  These may be used to make up for bad tracks, or to

provide a "landing zone" (see 9 below).  That is why different sources often

give a different number of cylinders for a hard disk; they are counting on a

different number of cylinders being set aside as extras.

6) Write Precomp Cylinder

I don't know exactly what this term means, but apparently some hard disks (not

all) need to adjust the timing of reads and writes when using a certain part of

the disk.  The Write Precomp Cylinder is the cylinder at which write

precompensation needs to start, and it is used on all tracks after that, up to

the end of the disk.  The WP cylinder is usually at the middle of the disk

(e.g. a WPC of 307 on a 615-cylinder disk).

Note that a WPC of 0 is not the same as a WPC of "None", and neither is the

same as a blank for that setting.  WPC 0 means use cylinder 0, and WPC "None"

means there is no write precomp used anywhere on that drive.  A blank means

that I don't know what WPC to use.

This setting doesn't seem to be too important, since I've seen plenty of disks

work fine with the wrong WPC set.  If it's possible, however, you should try to

use the correct setting.

7) Reduced Write Current

Again, I'm not sure exactly what this means, but it seems that some drives

reduce the amount of current used when writing to a certain part of the disk.

The RWC cylinder is the cylinder at which this starts.  It usually is at the

end of the disk, e.g. cylinder 1024 on a 1024-cylinder disk.

8) Step Pulse Rate

This measurement has something to do with the timing of the drive, given in

microseconds (�s or us).  It is often given as a range, e.g. 5 to 3500

microseconds.  Using the proper SPR setting will allow a hard disk to give the

best performance.

9) Landing Zone

The landing zone is the cylinder where the heads are moved to when "parked".

This spot is away from the cylinders used for data, so that if the heads bump

around while the drive is being moved, they don't damage any data-holding

sectors.  It is usually the last cylinder on the disk, e.g. cylinder 640 on a

640-cylinder disk.

Sometimes the LZ cylinder may seem to be PAST the end of the disk, e.g.

cylinder 615 on a 612-cylinder disk.  That is because 612 isn't really the last

cylinder; there are actually three more that are not used for data, and 615 is

the REAL end of the disk.

10) Can the disk be used with RLL?

Hard disks are normally formatted using MFM (Modified Frequency Modulation).

IBM invented a different type of coding called RLL (Run-length Limited), which

allows much more data to be stored in the same space.  The 2,7 RLL encoding

used by most things allows 50% more data to be stored.  I've seen reports of

some drive systems with ERLL (Enhanced RLL?), which stores twice as much data

as MFM encoding does.

To use RLL formatting, you need two things: (1) A hard disk controller board

that uses RLL encoding, and (2) your hard disk needs to have a high-quality

surface.  Most of the old oxide-coating hard disk surfaces can't reliably store

data at the higher density.  This setting is "N" if the drive can't handle RLL,

and "Y" if it can.  Note that this applies only to the common 2,7 RLL; it may

not be true of ERLL or other formats.

11) Hard disk type (when used on an AT)

This number is the AT hard disk "type" that best fits that particular drive.

It is selected from the 15 types available on an IBM AT; some clones may have

extra type settings available that fit the drive better.  An asterisk ("*")

means that not all of the drive's capacity can be used with that type setting.

A more detailed explanation of drive types is given below.

When running the SETUP program on an AT-type computer, you are asked to enter a

number for the hard disk's type.  On an IBM AT, this type is a number from 1 to

15, and it specifies certain characteristics for the hard disk as shown below:

| Write-   |

| Cylin- |       | Precomp  | Landing |

Type | ders   | Heads | Cylinder | Zone    | Capacity

----------------------------------------------------

1     306       4      128        305        10 meg

2     615       4      300        615        21 meg

3     615       6      300        615        31 meg

4     940       8      512        940        64 meg

5     940       6      512        940        48 meg

6     615       4      None       615        21 meg

7     462       8      256        511        31 meg

8     733       5      None       733        31 meg

9     900      15      None       901       115 meg

10    820       3      None       820        21 meg

11    855       5      None       855        36 meg

12    855       7      None       855        51 meg

13    306       8      128        319        21 meg

14    733       7      None       733        44 meg

15    ---       -      ----       ---        ------

You must find the type number for the set of characteristics that fits your

hard disk.  If none of the type settings match exactly, it is possible to use

one with different specifications.  It won't hurt anything to have the Write-

Precomp Cylinder and Landing Zone set improperly; it is the heads and cylinders

setting that is important.  You want to use as many heads and cylinders as

possible, because that increases the amount of data that can be stored on the

drive.  On the other hand, you cannot use a type setting that specifies more

heads or cylinders than the drive really has.

For example, the ATASI 3046 hard disk has 640 cylinders, 7 heads and a

formatted capacity of around 40 megabytes.  None of the type settings match

these specifications.  We can use a setting that doesn't match, as long as it

doesn't use more cylinders than the ATASI really has (640), and as long as it

doesn't use more heads than actually exist (7).  The setting that uses the most

heads & cylinders, but does not exceed the number actually present, is 3 (615

cylinders, 6 heads).  That is the best type setting to use, though it

unfortunately only makes use of 31 megabytes of the 40-meg space that is there.

When the type setting given doesn't allow all of the drive's capacity to be

used (as in the example above), the type number has an asterisk ("*") next to

it.

I believe that Type 15 is meant to be a catch-all for drives that don't match

the other 14 types, but I don't know how that works.  There are some programs

that will allow drives of non-standard type to work, with all of their real

capacity available.  Two notable ones are Disk Manager, by Ontrack Systems,

and SpeedStor, by Storage Dimensions.  Of these two, I would definitely

recommend SpeedStor.  It generally sells for $50-$70, and most places that

sell hard disks will have it.  Storage Dimensions' address is given below in

Section IV.

12) Does the drive have auto-park?

When a hard disk is running, it is spinning at about 300 rpm, and the

read/write heads are "floating" on a cushion of air a very tiny distance above

it.  When the power is turned off, the heads can drag along the disk as it

slows down, scraping the magnetic surface.  This is obviously not good for a

disk in the long run.  A drive with "auto-park" automatically moves the heads

to an unused place on the disk as soon as the power cuts off, so they don't

damage an area that holds data.  This unused area is referred to as the

"Landing Zone" (see (9) above).

Parking the heads is also important when the drive is going to be moved around.

The heads could easily bump the surface when the drive is moved, and it is best

if they knock against an area that does NOT have data.  For this reason, auto-

park is especially good to have with drives that are in a portable computer.

Parking a drive can be done with software.  You could run the PARK program

before powering-down, and it would work just as well.  The advantages of auto-

park are that it is more convenient, and it works even for a "surprise" power-

down, e.g. if there is a sudden blackout in your area, or someone knocks the

power cord loose.

13) Is it a 3.5-inch drive?

Most hard disk drives contain 5.25-inch "platters", that are the same size as a

normal 5.25-inch floppy disk.  The whole hard drive is the same width as a

standard floppy drive, i.e. 5.75 inches.  3.5-inch hard drives have just

recently become popular; they are the same width as a 3.5-inch floppy drive,

like the ones on the IBM PS/2 computers.  This smaller size is particularly

useful in laptop computers and "hardcards".  Some of the larger portable

computers, such as the Compaq II, use 3.5-inch drives even though they could

fit a 5.25-inch drive.  The extra space available is used for shock-mounts to

protect the drive.

14) Is it a half-height drive?

A "full height" drive is 3.25 inches high, as large as the old IBM floppy

drives.  A "half-height" drive is no more than 1.63 inches high, or one half

the size of a full-height drive.  Nearly all 3.5-inch drives are half-height.

15) Notes

This line is used for an extra information about the drive.  In some cases,

there isn't enough room to give Step Pulse Rate range next to "SPR:", so it is

given on this line.  If the drive uses a special interface, such as SCSI

or ESDI interfaces, that fact is noted on this line.

Sometimes conflicting information was available on a particular drive.  In

this case, that fact may be noted on this line, and the source may be given.

For example, "DTC: ....." means that the DTC (Data Technology Corp.) listed

the drive as having the following characteristics.

IV. Sources

Most of the information in the HDINFO file was obtained from the sources listed

below.  Note, however, that no copyrighted material was reproduced here.

ACB-2070A User's Manual 401404-00B (with Errata sheet 401403-00B), April

1986, Adaptec, Inc., 580 Cottonwood Drive, Milpitas, CA 95035.

Atasi OEM Manual, Models 3033 and 3046, Atasi Corporation, 2075 Zanker

Rd., San Jose, CA 95131.

"Boardwalk" HDC-1000 Hard Disk Controller manual, Microtek Inc., 11878

Market Street, Livonia, MI 48150.

Centan Hard Disk Controller manual, Distribution Plus, 7309-C Grove Road,

Frederick, Maryland.

DTC 5150CI Disk Controller with BXD7 Driver Installation Guide, DTC Part

Number 09-10354, Revision 0A, 06-JAN-87, Data Technology Corporation.

DTC 5150CR Disk Controller with BXD7 Driver Installation Guide, DTC Part

Number 09-00342-1, Revision A, 08-APR-87, Data Technology Corporation.

DTC 5150CX Disk Controller Installation Guide (Preliminary version), DTC

Part Number 09-00150, Revision 01, 10-JAN-86, Data Technology Corporation.

DTC 5150CX Disk Controller with BXD Driver Installation Guide, DTC Part

Number 09-00150, Data Technology Corporation.

MCT-HDC Winchester Disk Controller manual, Feb. 5, 1987.

Miniscribe Corporation Installation Guide for Miniscribe Models: 6032,

6053 and 6085, Miniscribe Part Number 1083, Revision A, April 15, 1987.

Seagate Universal Installation Handbook, Seagate Publication: 36042-001,

27 April 87, Seagate Technology, 920 Disc Drive, Scotts Valley, CA 95066-4544.

SpeedStor software, version 4.02a, March 1987, Storage Dimensions, 981

University Avenue, Los Gatos, CA 95030.

TL226/TL240/TL326/TL340 Installation and Integration Guide, Revision 4,

Sept. 3, 1985, Tulin Corporation, 2393 Qume Drive, San Jose, CA 95131.

V. Copyright / Registration

This document, HDINFO.DOC, is copyrighted material.  It may be distributed

freely, as long as:

(1) it is not modified in any way,

(2) it is distributed in its entirety,

and   (3) no fee is charged for copying it, distributing, or using it.

The information that is in the file HDINFO (and of course in HDINFO.DBF,

HDINFO.DB3, HDINFO.DIF and HDINFO.ASC) is considered to be in the public

domain.

CARDFILE.COM and CARDFILE.DOC are copyrighted by Ziff Communications Co.

(publishers of PC Magazine), and they distribute it for free, but with the

following rules for its use:

"You may make copies for others (including placement on noncommercial

electronic bulletin boards), as long as no charge is involved.  However,

making copies for any commercial purpose is prohibited."

No registration fee or contribution is asked for HDINFO.  The latest version of

these files can be obtained by sending $5 to Michael B. Smith, 733 Sligo Ave.,

Apt. 607, Silver Spring, MD 20910, (301) 588-5935.