PARK -- A new routine to park the heads of one or both fixed disks

The heads of fixed disks should be left in the parked position

whenever the drive is not in use, so that if the machine is jarred

or the power is dropped, the heads will land on an unused portion

of the platter.  IBM provides a way to do this, but only in

the DIAGNOSTICS diskette (under PREPARE SYSTEM FOR RELOCATION).

Several BBSs have had PARK routines available for download, but

universally they handle only one hard disk and universally the

cylinder number at which the disk heads should be parked is hard-coded

in the program.

This PARK.COM routine determines, for either or both hard disks,

where to park the heads (by trying ever-higher cylinders until it

finds a cylinder number to which the disk cannot move), and leaves

the heads there.  It does it separately for the first and second

drives, so that it is usable even in systems that have two dissimilar

hard disks.

An earlier version did the seeks one cylinder at a time; the current

version first moves 100 cylinders at a time, then 10 cylinders at

a time, and finally one cylinder at a time.  On a Seagate 225 (612

cylinders, 4 heads, 20 megabytes), this reduced the time from an

intolerable half-a-minute to less than 1/2 second.

The files are:  PARK.DOC       this DOC file

PARK.ASM       assembler language source

PARK.COM       the executable module, obtained

by processing PARK.ASM with the

MASM, LINK, and EXE2BIN programs.

Carl Hayes (SYSOP, NN5I BBS, 214-960-2730)