Sparta PCBoard Hard Drive Conference

(201) 729-5377 or (201) 729-7056

Auto-config Formatting Procedures

Western Digital WD 1002S-WX2

The following information is taken directly from the Western Digital

Installation Guide for the WD 1002S-WX2 HD controller. For clarity

I have inserted my own notes on some portions of the procedures that

WD has lightly touched.

Step  	Instructions

1.	Correctly install or remove jumper on SW1-5. This is the

uppermost jumper on SW1.

Note: This jumper tells the BIOS to read the jumpers 1-4

on SW1 for the corresponding table to use or to

accept keyboard input for the various parameters.

2.  	At the A Prompt, insert the PC-DOS Supplemental Diskette;

load and run the PC-DOS debug utility by typing DEBUG.

A>DEBUG c/r

3.  	At the debug prompt, initiate the WD format program by

typing the following command line. The debug prompt is the

hyphen "-".

-g=c800:5 c/r

4.  	The WD format program will display the following message:

WX2 Format Revision T.8 (c) Copyright Western digital Corp. 1985

Key in drive no and interleave as follows: d ii where

d = relative no (0-1)

ii = interleave factor (1-16)

Enter d and ii separated by a space and followed with a Return

Example:

0 03 c/r

NOTE: Drive C: = 0, Drive D: = 1. An interleave factor of 3 is

standard. If formatting two drives, this option must be run

twice; once with d=0 and again with d=1.

5.	The following message will be displayed:

Key in disk characteristics as follows: ccc h rrr ppp ee oo

where

ccc = total number of cylinders (1-4 digits)

h   = number of heads (1-2 digits)

rrr = starting reduced write cylinder (1-4 digits)

ppp = write precomp cylinder (1-4 digits)

ee  = max correctable error burst (1-2 digits)

oo  = CCB option byte (1 hex digit)

Enter each value separated by a space and follow entry with

a return

Example:

306 4 128 128 11 05 c/r

6.	CPU Response (did you know cpu's respond?)

Are you virtually configuring the drive - Answer Y/N

Enter a "Y" and c/r for yes or "N" and c/r for no.

Sysop note: All this is asking is "Do you want to split

the drive into more than one volume"?

7. 	If Yes, the following message is displayed.

Key in virtual drive size list as vvvv

where vvvv = virtual drive size (1-4 digits)

Enter two cylinder numbers, followed by a space and

c/r

Note: The sum of these two numbers cannot exceed the

maximum number of available cylinders on your hard disk.

Example:

153 153 c/r

If No, continue to next step

8.	The following message will be displayed.

Press "y" to begin formatting drive d with interleave ii.

Note: d = drive number and ii = interleave factor.

Type "Y" followed by a c/r

9.	System Responses:

If any key other than "Y" is typed, the program displays

the following message and returns to DOS.

CPU response:

Nothing done exit

A>

If an error occurs while formatting, the program will immediately

terminate, display the following error message, and return to DOS.

XX is the hexidecimal S-WX2 BIOS completion code.

CPU response:

Error----completion code XX

A>

If "Y" is typed, formatting is initiated. If there are no

resulting errors, the program displays the following message.

CPU response:

Do you want to format bad tracks - answer Y/N

Respond with appropriate Y or N followed by a c/r

The user is prompted to enter, via the keyboard, a bad track list.

This list should be provided by the drive manufacturer. However, due

to DOS limitations, this procedure is not remommended.

Sysop note: Let the sucker format the bad tracks and then use Norton's

DT immediately after partitioning and high level format

is performed to pick up any bad track information and mark

them appropriately in the FAT.

Because of the logical addressing used by DOS, marking an entire track

bad will result in more than one logical address being marked bad. As

DOS can only accept a limited number of defects, a drive with excessive

defects may cause the format program to termnate with an error. The

displayed error is typically "TRACK 00 BAD - DRIVE UNUSABLE".

As a recommended alternative, execution of the DOS utility program

FORMAT should correctly locate and deallocate all media defects.

10.	If Yes, the following message is displayed.

Key in bad track list as follows: ccc h ...(repeat)

where

ccc = bad track cylinder number (1-4 digits)

h   = bad track head number (1-2 digits)

Type in the cylinder and head numbers for the bad tracks,

separate them with spaces, and follow with a c/r

Example:

160 1 161 1 304 3 223 4 223 2 c/r

The bad track message will be displayed again. To

terminate bad gtrack entry, type "N" followed by a c/r

11.	The following message is displayed.

Format successful - Returning to system

If a second drive is to be formatted, repeat steps 4 through 13 with

d = 1, otherwise continue.

RUNNING THE FDISK UTILITY

The FDISK utility partitions the drive for DOS. Please refer to the

PC-DOS operating manual for detailed instructions regarding this utility.

12.	Load and run the FDISK utility to set up the disk partitions.

13.   	CPU Response:

IBM Personal Computer

Fixed disk Setup Program Version 2.00

Copyright IBM Corp. 1983

FDISK Options

Choose one of the following:

1. Create DOS Partition

2. Change Active Partition

3. Delete DOS Partition

4. Display Partition Data

[5. Select Next Fixed Disk Drive]

Enter Choice:

Note: When two disk drives are installed, the system will

automatically add a fifth choice on the menu:

"5. Select Next Fixed Disk Drive". The first drive is

partitioned using choice number 1. Then, the second drive

is partitioned starting with choice number 5. All subsequent

steps are the same for both drives.

Select "Create DOS Partition" and enter a 1 followed by a c/r.

14.	The following message will be displayed:

Create Dos Partition

Do you wish to use the entire fixed

disk for DOS (Y/N)...........?

Type a Y followed by a c/r

15. 	CPU response:

System will now reboot

Insert DOS diskette in Drive A:

Press any key when ready..

RUNNING THE FORMAT UTILITY

16.	Load and run the PC-DOS utility program FORMAT to write the

DOS structure and syustem files to the disk. At the A prompt,

type FORMAT C: /S/V followed by a c/r

17. 	The CPU will respond with:

Press any key to begin formatting C:

18. 	The System will respond with:

Formatting....Format Complete

system transferred

Volume Label(11 Characters, ENTER for none)?

19.	Either enter the optional volume name and press c/r or just

press c/r

20.	When the FORMAT program is finished, the hard disk is ready to be

booted from. It will contain the file COMMAND.COM and the two

system files which are hidden.

Note: This is known as the "high level" format process. If a

second drive, or Drive D:, is to be formatted, repeat steps

12 through 19. When running FDISK on the second drive, enter

choice [5] first, then continue as before.

21,  	To verify that the hard disk will now boot, open the door of the

floppy drive and do a warm boot by simultaneously pressing CTRL,

ALT, and DEL (We call this the three finger salute). The system

BIOS first examines Drive A for a system diskette. If it doesn't

find one, then it will try to boot from the C drive, the hard

disk. If the hard disk boot is successful, the system will assert

the date, time, and C> prompts in order. Other files may now be

copied to the hard disk.

Sysop Note: I am going to throw some technical mumbo jumbo in here just

to make you feel comfortable with the info you are typing into the

questions asked by WD's program.

The HD controller provides a system interface to the hardware and onboard

BIOS through direct and indirect i/o. Direct access is accomplished

by reading and writing to four I/O ports. Indirect access is achieved

by executing pre-written I/O subroutines contained in the BIOS on the

controller.

Commands are issued by the CPU to the controller over the host buss

using a pre-defined protocol (known as ST506/412). The CPU issues

a command sequence by selecting the controller. The controller accepts

the selection, and requests the appropriate command bytes. Upon command

completion, the controller issues completion status to indicate whether

the command was successfully completed or terminated because of an error.

Direct access operations are controlled through four I/O ports (assigned

default addresses 320H, 321H, 322H, and 323H). Each port is assigned one

Read register and one Write register as follows:

Registers

Port		Read	     		Write

320H		Data IN			Data OUT

321H		Status			Reset (function)

322H		Configuration		Select (function)

323H		N/A			Mask

The CPU specified the operation or command to be executed by the controller

by sending a 6 or 10 byte Command Descriptor Block (CDB). The CCB we

were asked for earlier in the WD formatter routine is called the Command

Control Byte and is the last byte in the CDB. It specifies the error

recovery procedure during the command execution.

The ECC is comparable to CRC on a floppy drive. It is known as Error

Correction Code and is set on or off by one of the bits in Byte 5 of

the CDB.

My fingers are tired at this point and I am not going to get into

a further detailed discussion of each bit and its values in the

CDB. If you wish to obtain further information, give us a call at

Sparta PCBoard, join the HD conference, and fire away at us. We

may not have the answer, but we can usually get it.

Richard Driggers

SysOp - Sparta PCBoard

-eof-

##########################