Program: OKMODE.COM Copyright 1988 by Keith P. Graham

Description: Video Mode selector. Selects or displays video

modes, for use on multiple monitor systems or systems

where mode command will not change modes.

Operation: Enter: OKMODE mode where mode is a number in hex

from 00 to ff. Some common modes are:

0 - 40 column b&w

1 - 40 column cga-mono

2 - 80 column cga-b&w

3 - 80 column cga-color

4 - 320x200 cga color

5 - 320x200 cga b&w

6 - 640x200 cga b&w

7 - monochrome text

f - EGA monchrome graphics

10 - EGA 640x350 16 color

11 - VGA monochrome graphics

12 - VGA 16 color graphics

13 - VGA 256 color Graphics

I am including this with OPTIKS because some systems start up in

mode 7. (Enter OKMODE with no parameters to find current mode).

OPTIKS assumes that mode 7 means you have a mono monitor which

might not be the case. If you start OPTIKS and get a blank screen

leave OPTIKS by pressing F10 and then the Y key. Enter OKMODE 2

and then try OPTIKS again.

OKMODE.COM can be used in place of the MODE command for changing

screen types. OKMODE does not look at the system switches and

takes your word for the mode you would like. It can be used to

switch between 2 monitors, display current mode and to reset

modes on the new multi-function extended ega/vga cards.

If OKMODE does not change the mode, it is the card's bios looking

at switches. Try entering a mode such as EGA mode 10 and then

changing modes from there. My ATI card can only be fooled into

changing to mode 7 if I do it from an EGA or VGA graphics mode.