RAMTEST                       Page 1

WARNING

-------

Executing  this  program WILL destroy the contents of memory, in-

cluding RAM disks!

Now that you've been warned, I think you will find  this  program

useful,  not  only  in identifying an existing fault, but also as

part of a periodic maintainence program. This  program  has  been

developed  as an exhaustive test of an IBM PC's system RAM. It is

much more comprehensive than the PC Power On Self Test (POST) and

delivers diagnostic messages which are much easier to understand.

There are two different versions of the test. RAMTEST.COM is  the

test  which  should be used with the IBM PC and compatibles. Note

that it requires a fair amount of hardware compatibility to  work

properly.  Users  of  computers  which  do not make use of parity

checked memory (such as the Eagle  PC)  or  who  experience  dif-

ficulty  during PASS 2 of the test should install the patch given

in the Hardware Requirements section.

RAMTESTA.COM is the version to be  used  on  the  IBM  PCAT.  The

documentation  for  both versions is identical. For more informa-

tion on the required hardware compatibility, see page 3  of  this

document.

To start the test, just type RAMTEST at the DOS prompt. There are

no additional parameters.

This test is best run from a floppy disk which has been formatted

using  the  /S  option and contains no other RAM-based utilities.

Some of the commonly available memory-resident utilities (for ex-

ample, SideKick from BORLAND) 'take command' of  the  system  and

will cause RAMTEST to crash.

RAMTEST  functions  in several distinct phases. After signing-on,

RAMTEST starts searching for the top of contiguous  memory.  This

search  is  performed  by examining RAM and not by consulting the

system configuration switches. Once the top of memory  is  found,

the  actual testing portion of the program is relocated and test-

ing begins.

RAM is tested beginning with address 0:500 and  continues  up  to

but  not including the top 32k of the contiguous RAM below screen

memory. These limits were chosen for the  following  reasons.  1)

BIOS  uses RAM below 0:500 for scratch area and RAMTEST uses BIOS

exclusively for screen I/O. 2) The testing program needs a  place

to live undisturbed.

RAM  is  tested  in 32k blocks. The pattern for the test is taken

from the BIOS ROMs. Randomness is added by use of iteration count

to modify the data. During PASS 1, each RAM location is  modified

and  then  verified.  This  insures that each location can be ac-

curately written to and read from. During PASS 2,  each  location

Copyright (c) 1986  Digital Mechanics, Inc.

RAMTEST                       Page 2

is  re-read  and  compared  to it's reference byte in ROM and the

parity bit is checked. This part of the  test  insures  that  the

parity  generating  and  checking  circuitry  is functioning cor-

rectly, that the address decoding is working  properly  and  that

there are no locations in RAM shorted to other locations.

Should  no  errors occur during the two passes, RAMTEST announces

the successful completion of the test and prompts you to turn off

the computer to stop the test. If not stopped,  however,  RAMTEST

will  start  over,  using  a different set of patterns each time.

Since some memory errors do not show  up  until  after  hours  of

operating  time,  if you're trying to track down a failure, it is

suggested that you let RAMTEST run awhile, perhaps overnight.

Should an error occur, RAMTEST stops immediately, displays a mes-

sage identifying the error, and halts. Note that the only way  to

exit  this condition is to turn the computer off. A typical error

message is:

Memory contents failure.

Address = 5000:0000

Data is: 1B      Data should be: 1F

Turn power off, wait 5 seconds, and turn power back on.

The above example shows that the failure occured on an  expansion

board  (System  memory extends to 0:FFFF on 64k motherboards, and

to 3000:FFFF on 256k motherboards. The presence of 4116 RAM chips

indicates a 64k motherboard.). It also shows that  bit  1  was  0

when  it  should have been 1. In cases like this, the RAM chip in

the bit 1 position is frequently at fault. The RAM chips are  the

most delicate components in the memory system, and the easiest to

test for defects. This is because the RAM chips are socketed (ex-

cept  for the first 16k or 64k) and the rest of the chips such as

address decoding and data buffers are rarely, if ever, socketed.

The last two pages of this document diagram the location  of  the

RAM  chips  on  the motherboard for both 64k systems and the 256k

systems. Refer to the documentation that came  with  your  add-in

memory board for instructions on locating and replacing RAM chips

that  make up the memory above 1000:0 (64k motherboard) or 4000:0

(256k motherboard).

The failure codes which are analyzed at  the  top  of  each  page

refer  to  the  code which the PC prints when an error is encoun-

tered during POST. If this occurs, the code will be printed,  the

speaker  will  beep,  the  screen will go blank, and the computer

will have to be turned off. This will all happen in the  span  of

about one second, so watch carefully to see the error code.

Copyright (c) 1986  Digital Mechanics, Inc.

RAMTEST                       Page 3

RAMTEST  is provided at no charge to the user. Feel free to share

it with your friends, but please do not give  away  altered  ver-

sions  or  versions  which have had the copyright notice removed.

The essence of 'user-supported' software is to  provide  personal

computer users with quality software without high prices, and yet

to  provide  incentive for programmers to continue to develop new

products. If you find this program useful,  a  contribution  ($10

suggested)  would be greatly appreciated. Any suggestions or com-

ments you may have are also welcome.

Jim Weisz

Digital Mechanics, Inc.

P.O. Box 1356

Lombard, Il.  60148

HARDWARE REQUIREMENTS

PC        NMI Enable/Disable port = 0A0H

RAM parity check control port = 61H  (bits 4 & 5   00xx0000)

Parity status port = 62H  (bits 7 & 8   xx000000)

Note:  Users of the Eagle PC and other compatible computers which do not

use parity memory need to make the following patch using DEBUG.

The patch disables parity checking which causes problems on the

Eagle.  (Maybe the Sperry PC also)

A>DEBUG RAMTEST.COM

-F 1F2 1F9 90

-F 21A 222 90

-W

Writing 06A4 bytes

-Q

A>

PCAT      NMI Enable/Disable port = 70H

RAM parity check control port = 61H  (bits 2 & 3   0000xx00)

Parity status port = 61H  (bits 7 & 8   xx000000)

Copyright (c) 1986  Digital Mechanics, Inc.

RAMTEST                       Page 4

MEMORY FAILURE CODE ANALYSIS:  64k motherboard

SAMPLE FAILURE CODE:    4402 201

|||| |||

|||| +++---- SYSTEM CODE - INDICATES MEMORY FAILURE

||++-------- FAILED BIT - 00 INDICATES PARITY

|+---------- FAILED 16K BLOCK - 0, 4, 8, OR C

+----------- FAILED 64K SEGMENT

SYSTEM BOARD  (Lower left corner when viewed from the front)

SEGMENT = 0000

BIT                             FIRST 2 DIGITS OF

ADDR            P       0   1   2   3   4   5   6   7   ERROR CODE

0000 - 3FFF     *       *   *   *   *   *   *   *   *   00  (This row is

soldered in.)

4000 - 7FFF     *       *   *   *   *   *   *   *   *   00

8000 - BFFF     *       *   *   *   *   *   *   *   *   00

C000 - FFFF     *       *   *   *   *   *   *   *   *   00

SECOND 2

DIGITS OF       00      01  02  04  08  10  20  40  80

ERROR CODE

Copyright (c) 1986  Digital Mechanics, Inc.

RAMTEST                       Page 5

MEMORY FAILURE CODE ANALYSIS:  256k motherboard

SAMPLE FAILURE CODE:    4402 201

|||| |||

|||| +++---- SYSTEM CODE - INDICATES MEMORY FAILURE

||++-------- FAILED BIT - 00 INDICATES PARITY

|+---------- FAILED 16K BLOCK - 0, 4, 8, OR C

+----------- FAILED 64K SEGMENT

SYSTEM BOARD  (Lower left corner when viewed from the front)

BIT                             FIRST 2 DIGITS OF

SEGMENT         P       0   1   2   3   4   5   6   7   ERROR CODE

0000           *       *   *   *   *   *   *   *   *   0x  (This row is

soldered in.)

1000           *       *   *   *   *   *   *   *   *   1x

2000           *       *   *   *   *   *   *   *   *   2x

3000           *       *   *   *   *   *   *   *   *   3x

SECOND 2

DIGITS OF       00      01  02  04  08  10  20  40  80

ERROR CODE

Copyright (c) 1986  Digital Mechanics, Inc.

Version 2 addendum

Version 2 of RAMTEST corrects an error which occurs when the test is

run on a compatible machine.  Briefly, the pattern which was written

to memory came from the top 32k of the IBM PC ROMs.  Compatibles

almost invariably do not have that much ROM, so the pattern was being

generated from non-existant memory.  On the second pass, a different

value was obtained and resulted in a memory contents error.  This has

been corrected by using a random number generator instead.

The memory contents error message has been changed slightly so as to

be more understandable.  The new format is:

Memory contents failure.

Address = 5000:0000

76543210   Bit #

Data is:        00011011

Data should be: 00011111

Turn power off, wait 5 seconds, and turn power back on.

As you can see, the offending bit is more easily recognized by

comparing the two data patterns.  In this example, the RAM chip

occupying the bit 2 position is probably bad.  As a note, if more

than one bit is different, the RAM chips are probably not at fault,

although it is possible.

To further explain the failure address, the following chart contains

the addresses contained in the first 4 rows of memory on 64k, 256k,

and popular 640k motherboards.

64k                         256k                        640k

0000:0000 - 0000:3FFF       0000:0000 - 0000:FFFF       0000:0000 - 3000:FFFF

0000:4000 - 0000:7FFF       1000:0000 - 1000:FFFF       4000:0000 - 7000:FFFF

0000:8000 - 0000:BFFF       2000:0000 - 2000:FFFF       8000:0000 - 8000:FFFF

0000:C000 - 0000:FFFF       3000:0000 - 3000:FFFF       9000:0000 - 9000:FFFF

I hope this additional information is helpful.  With any luck,

'Version 3' of this program will have much better diagnostics and may

even be published.  Thanks for your support.