Digital Sound Sample Playback and Manipulation System

-----------------------------------------------------

Version 2.0        Rowan McKenzie  28/3/89

(C) Copyright 1989 by Rowan McKenzie

You may copy these files  or use the source  code  only

for non-profit purposes. Please contact me if you wish to use any

part of the package for commercial purposes.

I accept  no responsibility for any  injury  or  damage

caused by the use of any of these files.

SAMPLER/PWMSAMPL  is true public domain. In other words, I ask for  no

payment, registration fee, donations.... But please distribute these  files

in  the form you found them. Also I would appreciate acknowledgment in  any

programs which use sections of my source code.

Introduction

------------

SAMPLER    and    PWMSAMPL    are    packages    for    playing    and

manipulating  digitised  sound  samples.  SAMPLER  expects  an  8  bit  D/A

converter  to be available on a parallel printer port, while PWMSAMPL  will

play  sounds on the PC speaker using pulse width modulation. Of course  the

quality  of sound will be much better for the D/A version, however the  PWM

version requires no additional hardware.

Support is provided for playing Pianoman .MUS files as songs using the

current sampled sound.

Hardware Requirements

---------------------

PWMSAMPL requires:

IBM XT/AT or close compatible

Hercules, CGA or EGA graphics card

SAMPLER requires in addition to the above:

An  8  bit D/A converter connected to  a  parallel

printer port

If  the sampling feature is to be used, a  fast  8

bit a/d converter is required (although the source

code  will  probably  need to  modified  for  your

setup).

Software Requirements

---------------------

MSDOS Ver 2.00 or later

Pianoman  program  and support files  (PCblue  Vol

216, PCsig Vol 279) if the song mode is to used

Background

----------

SAMPLER  and  PWMSAMPL were derived from an A/D,  D/A  sound  sampling

system I wrote for an old CPM computer system.

The  pulse width modulation code uses the 8253 timer chip in one  shot

mode  to  drive  the PC speaker with variable length pulses at  a  rate  of

about 20kHz. Varying the pulse length produces the same result  as  varying

the  voltage on a linearly controlled speaker. Unfortunately, the  software

overhead for this could mean that slow XT systems may not be fast enough to

run  PWMSAMPL.  Also  the sound volume is quite low (it's  limited  by  the

difference  between the shortest and longest pulses used, which is  limited

by  the  sample rate, which is 20kHz for reasonable quality), so  you  will

need to listen in a quiet room, or else build a D/A converter for SAMPLER !

If  you  are interested in building a D/A converter to hear  the  much

better sound quality of SAMPLER, a standard R-2R resistor ladder (R ~  10k)

into an amplifier of reasonably high input impedance (or low impedance  via

a  100k resistor) will be quite adequate. Alternatively, an active  circuit

using a chip such as the DAC0800 with its own power supply could be  built.

The circuit shown below worked adequately for me.

D7 --- 20k ---+--- 200k ----->  to amplifier

|

10k

|

D6 --- 20k ---+

|

10k

|

D5 --- 20k ---+

p             |

r

i           etc ...

n

t             |

e            10k

r             |

D0 --- 20k ---+

|

20k

|

=============== gnd

To  utilise  the sampling feature, a fast (at least  20kHz  conversion

rate) A/D converter must be available. My converter was designed to begin a

conversion following a CPU access. As long as samples are taken at  regular

intervals,  this  technique  minimises  I/O  overheads.  The  converter  is

addressed  at the same location as the D/A converter to simplify the  code.

If your converter doesn't meet these specifications (probably won't),  then

the  assembler code will require modification. SAMPLER is written in  Turbo

Pascal Version 5, and the font and driver units can only be used with  that

compiler.

Using SAMPLER / PWMSAMPL

------------------------

Version  2.0 of SAMPLER now uses mouse control rather than  cumbersome

keyboard commands. It is therefore necessary to have an MSMOUSE (R)  driver

installed  before  starting the program. Most mouse packages come  with  an

MSMOUSE  compatible driver and either two or three button mouses will  work

fine.  If a mouse is unavailable, the cursor keys provide mouse  emulation,

although this makes controlling SAMPLER more difficult.

If  you  are using a mouse, generally the left  mouse  button  selects

objects  while the right button cancels. The exception to this is for  wave

drawing where the right button enables drawing. Dialog boxes which  contain

a  Continue prompt expect you to click on the box containing  the  Continue

prompt,  although  the  right mouse button will have  the  same  effect  by

canceling the prompt. Also, pressing Enter in this situation will cause the

program  to continue. You will notice that while a sound is being  replayed

no mouse activity is possible. It was necessary to disable mouse interrupts

while producing sounds to avoid overloading the CPU. However, mouse control

can  be temporarily restored during sounds by holding down the right  shift

key.

If no mouse is available, the arrow keys provide coarse mouse control,

and the left shift key together with the arrow keys provides fine  control.

The  left  button  is emulated with the Alt key, and the  right  button  is

replaced by the Ctrl key. For compatibility with my other mouse  controlled

programs,  the  Caps lock key performs the same function as  Ctrl,  however

because a custom keyboard handler intercepts keyboard information, the caps

lock key can become confused. If the Caps Lock LED ever comes on, it may be

necessary to press Caps lock again before the program will continue.

When  SAMPLER  is  first  started  by  typing  SAMPLER  or   PWMSAMPL,

information   will  be  displayed  about  the  default  settings   of   the

configuration  file. When the default sound file has loaded, a  dialog  box

containing  all current settings will appear. Normally this is  ignored  by

clicking on the continue box (or canceling with the right button or Enter),

however, you may wish to alter an option before starting.

Once  you  are  past the initial dialog box, a  plot  of  the  current

waveform  will appear, together with several other objects. At the  top  of

the  screen  appears a group of menu headings. Clicking on a  heading  will

bring  a  sub-menu  down.  All SAMPLER functions now  appear  in  the  menu

structure,  and no keyboard commands exist. The state of the more  relevant

toggle-able functions and variables is given in a panel near the top of the

screen,  all  of  which may be changed from the Settings box  in  the  menu

(under  Options). The current waveform appears in the center of the  screen

with  the current start, end and loop arrows above and below. At the  lower

right  is a collection of click-boxes which control the pitch of the  sound

when  replayed. The vertical arrows control the octave of the sound,  while

the large and small left/right arrows alter the pitch within an octave. The

larger  arrows  provide  coarse control. The Reset  box  will  restore  the

original default replay pitch, canceling the effect of other boxes. At  the

lower  right  of the screen are a similar set of boxes  which  control  the

period of the auto-timer. The auto-timer determines the rate of "auto sound

repeat" and the speed of songs.

The  left,  right and loop arrows determine which part  of  the  sound

sample  is to be replayed and looped. The mouse is now used to alter  these

arrows by clicking on one and then dragging it. Normally the resolution  of

mouse  control is fairly rough, however the arrows may now be  placed  with

absolute precision by utilising the Zoom feature. After roughly placing  an

arrow, a region box can be marked in the waveform box by pressing the  left

mouse  button and dragging. If a region is marked near the  relevant  arrow

and  the Zoom option is selected, a magnified view is obtained  where  each

screen pixel represents one sample interval. At this point the arrow can be

dragged  to the exact position desired. Thus, it is now possible  to  align

arrows  such  that  start and end levels match,  thereby  avoiding  audible

"clicks" due to mismatched levels when looping.

As  just mentioned, a new feature known as an "active region box"  can

be  marked in the wave box by pressing the left mouse button and  dragging.

The region box describes an area where some function is to be applied, such

as mirroring, envelope scaling, cutting, pasting etc. Most functions  which

apply  an  operation to a specific area require a region box to  be  marked

before they will be visible in the menu. In fact, all but one option  under

the Edit menu are unavailable until a region is marked. Further, the  Paste

and Mix options remain invisible until a region has been marked, cut, and a

destination  region  has  been marked.  This  "invisible"  approach  avoids

confusing  SAMPLER about where operations should be applied when no  region

has been specified, and avoids accidental damage to the waveform (there  is

no Undo function yet).

Also,  when  in Zoomed mode (and no region box is active),  the  right

mouse  button  allows direct editing of a waveform by  drawing  on  screen.

Using this technique, it is possible to remove clicks and pops from  sounds

where  joins  have been made, by drawing over them. I  find  this  function

extremely useful for tidying sounds after mirroring or clearing just before

saving them to disk.

The  computer  keyboard is now permanently in the  musical  instrument

mode,  ie. pressing an alpha key sounds a note at a particular  pitch.  The

arrangement  of keys is set to look similar to a guitar fretboard or  piano

keyboard depending of the Instrument type setting in the Settings menu. For

the guitar fretboard option the Z key corresponds to an open low E  string,

the A key is an open A string, the Q key is an open D string, and the 1 key

is an open G string. Alternatively, the keyboard may be considered to be  4

rows of semitone intervals starting on the above notes.

For  the  piano  layout, the lower two rows  (Z../,A..')  represent  F

natural to A# with the B key representing C natural and replaying the sound

at  the original recorded pitch. The upper two rows (Q-],1-\)  are  similar

but are an octave above the lower two rows with the Y key being C  natural.

Unfortunately,  it  is  difficult to visualise the  black  notes  initially

because  there  are  no missing keys like on a real  piano!  However,  this

arrangement  may  be  preferable  for anyone not  familiar  with  a  guitar

fretboard.

SAMPLER / PWMSAMPL Menu Functions

---------------------------------

The  following  functions  appear  in the  menu  structure  under  the

indicated major headings.

===============

Settings                                ( Options )

===============

Clicking  on fields in the Settings window allows all  adjustable

program parameters to be edited. Boolean type parameters are  adjusted

by  clicking on either the "Y" box or the "N" box. The box  containing

an  uppercase  character  indicates the  current  setting.  All  other

parameters can be edited by clicking on the field and typing in a  new

value.  If  a  blank field is entered,  the  current  setting  remains

unchanged. Fields in the Settings window include:

Current path: allows a new sound files directory to be  selected.

A  new drive prefix may also be included to allow  change

of drive.

Current sound  file: shows the currently active sound file.  This

field  can  not  be  altered (use  the  Read  sound  file

option).

Instrument  type:  allows a guitar fretboard  or  piano  keyboard

layout  to  be selected for the computer  keyboard  (ref.

discussion on keyboard layout above).

System  type: this function selects the type of computer (XT,  XT

turbo, AT, AT turbo) being used. The faster machines will

use  a  faster sample rate to give better  bandwidth  and

sound  quality.  The  speed  of  song  playback  is  also

affected  by  this  because Pianoman's  .MUS  files  will

contain  different duration values for  different  system

speeds.

To determine the correct selection, start with the

appropriate setting for your PC, and if the sound is very

low  in  pitch or badly distorted, try a  lower  setting.

Slow  XT systems may not be fast enough even for  the  XT

selection.  The only solution to this is to use a  faster

computer!  The default machine type can be configured  in

SAMPLER.CNF.   Note   that  PWMSAMPL  chooses   two   PWM

frequencies depending on this setting. If the XT  setting

causes  severe  noise or distortion in the  PWM  version,

then your PC is just too slow for the job. Selecting  the

AT  setting allows a higher PWM carrier frequency  to  be

used which results in slightly better sound quality  (but

requires more processing speed).

Key release: this toggles key release sensitivity. When  enabled,

releasing a key after playing a sound will terminate  the

sound. When disabled, sounds will continue after the  key

is released.

Loop mode: when active, sounds will loop continuously between the

loop  pointer  and right pointer. When  inactive,  sounds

will  be played once from the left pointer to  the  right

pointer.  Loop  mode  can be used  to  simulate  infinite

length sounds using finite length samples.

Auto  timer:  toggles  the use of the timer facility.  The  timer

determines  how long sounds will last. The  main  purpose

for this is to control the speed of songs (although notes

within songs can be different lengths). However the timer

can be useful even when playing notes manually. For  this

situation, selecting auto timer mode means that the  loop

facility  now loops to the start of the sound  after  the

timer  period. This is useful for  repetitive  percussive

sounds  because  the loop rate is fixed  for  all  notes,

unlike  the normal loop mode where the loop rate  depends

on    the    pitch   of   the    note    being    played.

When  the auto timer is enabled, the loop  setting  still

affects the sound produced. If looping is inactive,  then

for sounds shorter than the timer duration, there will be

silence following the end of a sound until the next timer

instance. If loop mode is active then the sound will loop

as described previously, until the next timer instance.

Trigger level: changes the minimum A/D level required to begin  a

sample operation. WARNING: if no converter is  available,

setting   the  trigger  level  above  zero  will   almost

certainly  cause the program to enter an  infinite  loop,

while waiting for a sample. Re-booting is the only way to

recover from this situation.

Sample rate: sets the sample rate for the sampling function to  a

specified  number  of kilohertz. Setting a  low  sampling

rate  allows  a  longer sample to be  recorded,  while  a

faster  rate  provides a wider bandwidth. If  the  sample

rate  is  altered from the default, then  the  pitch  for

replay will not match the original pitch without manually

altering the playback pitch.

D/A  port: allows a different I/O address to be selected for  the

D/A converted (SAMPLER only). See the discussion below on

D/A port address in Editing the Default Configuration.

===============

Help                                    ( Options )

===============

Provides  very  brief general help for using  SAMPLER.  For  more

information, read this file!

===============

Directory                               ( File )

===============

Provides full screen display of all files in the directory on the

default sound files path.

===============

Read sound file                         ( File )

===============

Provides  a directory listing of sound files in the current  path

and allows a new sound file to be selected by clicking on a file name.

Selecting the Specify Input File box allows a file name to be typed in

(optionally including an overriding path) and loaded. Note that  small

files will overlay only part of the sample buffer. If an active region

(see  above)  has been marked, then the sound will be loaded  in  that

space  and truncated to fit if necessary. If no region is marked,  the

sound  file  will  be loaded starting at the extreme  left.  The  Read

process  can be canceled with the right mouse button, or by entering a

blank response in the manual input box.

================

Write sound file                        ( File )

================

Saves  the  sound between the left and right pointers to  a  disk

file in the current path. Note that the file name may be preceded by a

drive and path prefix to override the current drive/path. The  process

may be canceled as described above.

===============

Quit to DOS                             ( File )

===============

Terminates  SAMPLER and returns control to the DOS  command  line

following confirmation

===============

Cut                                     ( Edit )

===============

Once an active region has been marked (see above), the Cut option

becomes visible. Cutting a region writes the sound information  within

the active region to a temporary clipboard file and CLEARS the  marked

region. Data in the clipboard file will remain stored until the end of

a  session,  or  until another Cut/Copy operation  is  performed.  The

clipboard  data  is  available to the Paste  and  Mix  functions  (see

below).  Note  that Cut differs from Copy in that the marked  area  is

cleared  (ie. silence) after the Cut operation. If you wish to move  a

sound  to new position then Cut simplifies the Copy/Clear process.  Be

careful not to Cut when you really want to Copy!

===============

Copy                                    ( Edit )

===============

Copy  is similar to the Cut option, but doesn't clear the  marked

region. Note that Copy indicates data is copied to the clipboard file,

not  to the wave box. To copy an area to other positions in  the  wave

box,  Copy the desired area and then use a series of Pastes  to  place

the data.

===============

Paste                                   ( Edit )

===============

After Cutting or Copying data to the clipboard, that data may  be

placed  in a new position in the wave box with a Paste operation.  You

must  specify  the destination region with the left  mouse  button  as

described  above.  The  clipboard data will be truncated  to  fit  the

active  region  if necessary. You may re-paste the same data  as  many

times  as  desired.  The  clipboard  will  only  change  its  contents

following another Cut/Copy or by Quitting to DOS. Pasting data several

times  after  the  original Copied data can be used  to  simulate echo

or delay effects.

===============

Mix                                     ( Edit )

===============

Mix  is similar to Paste except that the waveform data under  the

destination  region  is mixed with the pasted data rather  than  being

replaced  by it. You will be given choices about which data should  be

scaled before the Mix is performed to avoid exceeding the maximum  and

minimum  amplitudes. The Cut data which is about to be pasted  can  be

halved in amplitude first, or left as it is. The existing data in  the

destination  region  can be halved or left alone. If  the  destination

region  is  scaled, you can have only the data within the  destination

region scaled, or the entire sound sample scaled. If you are mixing  a

low amplitude sound in a region containing low amplitude data, scaling

may not be necessary. If the sum of the cut and destination amplitudes

might  exceed the wave box edges, then scaling will be  necessary.  If

mixing  a region which is isolated from the rest of the sound  sample,

you  may  wish to scale just the data in the  destination  region  and

leave  the  rest  of the sample alone. The circumstances  of  the  Mix

operation  will determine the scaling choices made. If you are  unsure

about which sections need scaling, try experimenting with the  various

options on some rubbish waveforms first.

===============

Mirror segment                          ( Edit )

===============

Data  inside  the  active region will become  mirror  image.  The

result  of this is that samples will sound backwards. This is  one  of

the more interesting effects available.

===============

Scale envelope                          ( Edit )

===============

Data  inside  the  active region will be scaled  by  a  specified

factor. Constants for both ends are required, and points are scaled by

a  straight line plotted between those constants. For example, if  the

current data appears with uniform height inside the active region box,

a  decay effect can be simulated by specifying a factor of 1  for  the

left  side  and 0 for the right side. Note that scale factors  may  be

real  numbers,  and that numbers greater than one may  cause  clipping

distortion of the sound.

===============

Clear                                   ( Edit )

===============

Clears (silences) sound inside the active region. You may  remove

unwanted sound by marking the relevant regions and Clearing them.

===============

Zoom                                    ( Edit )

===============

After  marking  an active region and selecting the  Zoom  option,

data  inside  the left edge of the region will be shown  in  magnified

form. It is then possible to perform all normal operations on data  in

fine  detail. When in Zoom mode the right mouse button  allows  direct

editing of waveform data by drawing over the wave box. You can  remove

unwanted  clicks and pops by drawing over them. Also, if you  zoom  on

the area below or above a pointer, the pointer will be visible in  the

zoomed  window and may be moved with absolute resolution. The data  is

displayed as one sample per pixel, so that pointers can be  positioned

over individual samples if desired. When positioning the loop pointer,

the zoom feature can be used  to position the loop pointer so that the

waveform  level matches the level at the right pointer  thus  avoiding

any clicks when loop mode is active.

===============

Sample                                  ( Edit )

===============

Record sound to memory at current sampling rate. This resets

the  three  pointers.   NOTE: the sampling function  requires  an  A/D

converter  to  be  available (see  previous  discussion  on  converter

requirements).  The sampling rate will determine the available  sample

time  and sampling bandwidth. This function is irrelevant for the  PWM

version.

===============

Play song                               ( Song )

===============

As mentioned above, this package supports Pianoman .MUS files  to

provide an automatic song mode. Pianoman is available from the  PCblue

Vol  216 and PCsig Vol 279 catalogs and is available on many  Bulletin

Boards.

Pianoman allows real time recording of tunes from the keyboard as

well  as  manual  editing  of tunes. I  decided  that  using  Pianoman

externally  was  easier  than  writing  an  equivalent  facility  from

scratch. The only problem with this system (apart from having to start

up  programs  continually)  is  that  Pianoman  in  not  system  speed

independent. Although SAMPLER/PWMSAMPL tries to compensate for  system

speed when playing songs, you will probably find that song speed needs

to be adjusted manually (see Auto timer in the Settings section).

After  specifying a song file using the Read songfile option  and

then  selecting  the Play song option, a prompt will appear.  You  may

play  the  song  once from start to finish or  have  it  automatically

repeat itself upon completion. While a song is playing, mouse activity

is  suspended. If you wish to alter song speed etc., holding down  the

right  shift  key pauses the song. The song speed can  be  altered  by

changing the auto timer duration from the boxes at the lower right  on

screen. The song can be terminated by pressing Ctrl (the mouse  cancel

button is disabled).

===============

Read songfile                           ( Song )

===============

This option allows specification of the current song file in  the

same manner as reading a sound file. The desired song file is selected

by  clicking on its name or selecting the manual entry box and  typing

the  name  (with optional path). Song files are assumed to be  in  the

same directory as sound files.

===============

Concert A                               ( Miscellaneous )

===============

Produces a tone at the standard "concert A" pitch (440 Hz).  This

may be useful when preparing to sample a sound if you require it to be

replayed  at  "concert  pitch". If the sound  is  recorded  using  the

default sample rate and was tuned to the reference tone, then  playing

it  back from the keyboard with default tuning should produce a  sound

which  is in tune with other instruments (assuming they are  tuned  to

concert pitch too).

===============

Monitor                                 ( Miscellaneous )

===============

Causes the input signal (A/D required) to be echoed to the output

(D/A  or  speaker)  so  that the input  level  may  set,  equalisation

adjusted etc., before actually sampling.

===============

Play sound                              ( Play sound )

===============

Immediately  replays  the  current sound sample  at  the  default

pitch.  The pitch may be altered using the tuning boxes  (see  above).

This  option provides a fast way to replay sounds if you just want  to

hear what they sound like.

Editing the Default Configuration

---------------------------------

When SAMPLER/PWMSAMPL is first run, a configuration file (SAMPLER.CNF)

is  read to determine some default settings. All defaults can  be  modified

from  the  Settings  menu  box, but you may like  to  change  the  defaults

permanently.  To  achieve  this, a pure ASCII editor can be  used,  but  be

careful to maintain the correct format. There should be four lines  present

and no leading spaces on any line. The four lines are

1: Default  sound  files path - this  determines  where

sound  files will be when first run. The  string  may

include a drive prefix.

eg. c:\data\sounds

2: Default sound file - this sound sample file will  be

loaded  when first run, and should be in the  default

sound file path.

eg. bass

3: System type - this is a single letter indicating the

type of system being used as follows:

X - XT,  T - XT turbo,  A - AT,  U - AT turbo

Note that this only affects SAMPLER. PWMSAMPL will

override system selection because all timing  must

be fixed for PWM output.

4: Parallel  port  number or address - this  will  determine

which  port SAMPLER will use for D/A output. The port  may

be described as LPTx if a printer port is to be used, or a

decimal  number which names a D/A board port  address.  As

different   sources   of  documentation   give   different

information  about LPT numbering, if no data appears on  a

printer  port at first, try LPT1 to LPT3 until  data  does

appear.  Use  this approach if you aren't sure  which  LPT

number you are connecting to.

Note that  PWMSAMPL will ignore this line.

5:  Default  instrument  (keyboard layout)  -  this  is

either guitar or piano.

(C) Copyright 1989 by Rowan McKenzie

Comments   or   suggestions   for   improvement

are  most welcome.

I   accept  no responsibility for  any   injury

or damage caused by the use of any of these files.

Rowan McKenzie

35 Moore Ave,

Croydon 3136

Victoria,

Australia

Can be contacted through:

Eastcom OPUS (830/312),

Melbourne Data Exchange (832/360),

The Software Bank (632/301),

Eastwood BBS (831/343),

Tardis BBS Vic.

(All Vic., Australia)

(and most other Vic Opus boards)