IMPORTANT NOTE:

---------------

THIS  PROGRAM  IS ONLY DESIGNATED FOR AMATEUR  AND  EDUCATIONAL

PURPOSES.  A COMMERCIAL USAGE IS ALLOWED ONLY WHEN THE AUTHOR'S

WRITTEN PERMISSION IS GIVEN.

THE  PROGRAM MAY BE COPIED AND DISTRIBUTED TO OTHER USERS  ONLY

IN ITS ORIGINAL STATE.  AN EXCEPTION IS MADE FOR THE  SO-CALLED

'TIPS'-FOLDER WHICH MAY BE USED FOR SUGGESTIONS OR COMMENTS.

NO  GARANTEE IS GIVEN FOR THE CORRECTNESS AND ACCURACY  OF  ALL

DATA  WHICH ARE ASSOCIATED WITH THE USAGE OF THE  PROGRAM.  THE

USAGE  IS  WITH  THE USER'S PERSONAL  RISK.  THE  AUTHOR  WOULD

APPRECIATE INFORMATION ABOUT PROGRAM ERRORS AT ALL TIMES.

-/-

WICHTIGER HINWEIS:

------------------

DIES PROGRAMM IST NUR FUER AMATEUR- UND LEHRZWECKE  VORGESEHEN.

EINE GEWERBLICHE NUTZUNG IST NUR MIT SCHRIFTLICHER  GENEHMIGUNG

DES AUTORS ERLAUBT.

DAS  PROGRAMM  DARF  FREI KOPIERT  UND  WEITERGEREICHT  WERDEN,

SOFERN  ES  IN ALLEN SEINEN  BESTANDTEILEN  IM  ORIGINALZUSTAND

BELASSEN WIRD.  DIES BETRIFFT NICHT DEN 'TIPS'-ORDNER,  IN  DEM

DER ANWENDER HINWEISE FUER NACHFOLGENDE BENUTZER ABLEGEN KANN.

ES  WIRD  KEINE GEWAEHR FUER DIE RICHTIGKEIT DER  DATEN  UEBER-

NOMMEN,  DIE  AUS DEM GEBRAUCH DES PROGRAMMS  RESULTIEREN.  DIE

BENUTZUNG  ERFOLGT AUF EIGENE GEFAHR.  DER AUTOR IST  FUER  DIE

MITTEILUNG EVENTUELLER FEHLER JEDER ZEIT DANKBAR.

Volker Grassmann, DF5AI

Hannoversche Str. 103

D-3400 Goettingen-Weende

Fed. Rep. of Germany

-----------------------------------------------------------------

C H I R P

CHARACTERISTICS OF IONOSPHERIC RADIO PROPAGATION

------------------------------------------------

1. INTRODUCTION

CHIRP is a program designated for hf radio operators,  for e.g.

radio amateurs, broadcast listeners or any other users of short

wave  radio propagation.  The program's name is reminiscent  of

the  real chirp-ionosonde.  In fact CHIRP acts as a  monitoring

system  for  hf  propagation  in  the  range  1-30  MHz   which

calculates  the best frequencies,  times and paths  to  distant

radio  stations.  This information may be important when  radio

schedules  have to be established or when best  conditions  for

"dxing" must be known in advance.

The following disk files are part of the program package:

CHIRP.PRG               the program

CHIRP.DOC               this document

TIPS\TIPS.DOC           further user information

CHIRP.DAT\CHIRP.MAP     worldmap

CHIRP.DAT\MINIFTZ.DAT   MINIFTZ's grip point data

2. THE HEART OF THE PROGRAM: MINIFTZ

Th.  Damboldt and P.  Suessmann at the "Forschungsinstitut  der

Deutschen  Bundespost  beim  Fernmeldetechnischen  Zentralamt",

Darmstadt  (Germany),  developed an advanced grid point  method

whiches  makes the "CCIR data of  ionospheric  characteristics"

suitable for micro computer applications [1-3].  This work  was

done  because  of  the low accuracy obtained  by  former  micro

computer  evaluations,  see [1] or [2] for  more  details.  The

results obtained by the new FTZ-method have been tested against

measured  data  published by the CCIR.  In field  strength  the

average  difference was found be  about 0 dB and the RMS  error

was about 11 dB,  [3].  In MUF(3000) the average difference was

less  than 0.1 MHz with a standard deviation of 2.3  MHz,  [2].

Clearly the new method is comparable in accuracy to predictions

calculated on mainframe computers.

These  developments  have been incorporated  into  the  program

MINIFTZ.  For a specific radio link and for a particular season

of  the  year  MINIFTZ calculates  the  diurnal  variations  of

several parameters.

Table: Output parameters obtained with MINIFTZ

----------------------------------------------

-  basic MUF (MUF: maximum usable frequency)

-  field strength at MUF

-  skip mode at MUF (E- and F-region "hops")

-  antenna elevation for skip mode at MUF

-  frequency of optimum traffic (FOT)

-  field  strength,  skip  mode  and elevation  angle  at  user

defined frequencies (max. 11)

The  author  of CHIRP would like to express his thanks  to  Dr.

Thomas Damboldt,  Forschungsinstitut der DBP beim FTZ, who made

the MINIFTZ FORTRAN- and BASIC-source listings available.

The MINIFTZ-source has now been translated for the Atari ST and

is  used  by CHIRP.  It is expected that the  user  is  already

familiar  with the terminology and usage of radio  predictions.

Refer to [4] if further explanation is required.

3. THE WORLD MAP INDICATOR

The  world  map  indicator (WMI) is  a  graphic  display  which

generally works independently from MINIFTZ.  In accordence with

the  mouse cursor setting on the screen map the WMI  calculates

short/long  distance  and antenna bearing in  relation  to  the

location of reference (e.g.  your home city) and indicates  the

actual QTH-locator.  Ray paths may be calculated and plotted on

the screen by clicking the right mouse button or by entering  a

location via keyboard. This gives the user an impression of how

the  radio waves travel around the world.  Instead of  a  radio

beam,  which  is  in fact an  idealization,  the  real  antenna

characteristic  which distributes radio power into  an  angular

section may be taken into account.

Both  MINIFTZ and the WMI are coupled only in a  certain  case:

when clicking the RUN command in the menu.  The very last radio

path  which  was  plotted on the screen map  defines  the  geo-

graphical  input  data for MINIFTZ's purposes.  These  are  the

geographical locations of both the radio terminals and  whether

short or long path is required.

4. INPUT OF GEOGRAPHICAL LOCATIONS VIA KEYBOARD

A certain geographical location (e.g.  your home city) is taken

as  reference  for all calculations performed  by  CHIRP.  This

location  must  be entered by keyboard and is  indicated  by  a

special  symbol  on the map.  A second location is  defined  by

simply  pointing  with the mouse cursor  to  the  corresponding

place on the screen map or by another keyboard input.

Keyboard  entries  may  be  performed by  using  one  of  three

different data formats:

a. Geographical  coordinates  which  are a  pair  of  numerical

values separated by a slash ("/").  The first number is  the

geographical  latitude  and  the second  is  the  longitude.

Geographical coordinates must be entered as decimal  numbers

e.g.  52  degrees  30  minutes must be  entered  as  '52.5'.

Negative  latitudes are used to describe the southern  hemi-

sphere,  a  minus-sign  in front of  longitude  indicates  a

meridian west of Greenwich. Instead of using negative values

the user may also indicate northern and southern  hemisphere

by  'N' or 'S' and western or eastern longitudes by  'W'  or

'E' identifiers respectively. Examples of valid geographical

coordinates:        "52.41/10.2275",        "5.03n/12.987e",

"68.3S/129.0W".

b. Worldwide  locators  which  are  always  given  as  a   six-

character-code.   Examples   of  valid   inputs:   "JO52CJ",

"em57xx".

c. European   locators:   EUlocs  consist  of  at  least   five

characters.  This system which was developed in the  sixties

is  valid  only for the European  area.  On  some  occasions

suffixes  are necessary to avoid ambiguous locators  because

the same locator may occur in different parts of Europe. The

extensions (N,  NE,  E,  SE, S, SW, W, NW which means north,

northeast  etc.)  must  be separated by  a  blank  from  the

original locator.  When no extension is supplied the central

European  locator  alphabet (all the squares AA to  ZZ  from

Spain  to  Russia) is assumed.  An extension  specifies  the

adjacent alphabet in northern,  northeastern etc. direction.

Examples of valid European locators  are:  "FM42F",  "hv12f"

which  is located in Sweden,  "HV12f S" which is located  in

the Mediterranean. The user may refer to the program QTH_LOC

for further explanation.

CHIRP  makes  use of a build-in interface  which  analyses  the

user's input data.  There is no need to tell the program  which

type of data was entered. The user will be alerted when illegal

locations of reference  are entered. In all other cases illegal

input is ignored.

5. MINIFTZ INPUT

The  PARAMETER  menu  is  used to  pass  input  parameters  for

MINIFTZ's purposes. The location of reference is labeled as the

transmitter's  geographical  position while  the  second  radio

terminal  is labeled as receiver.  In fact this distinction  is

introduced  just  for convenience and has no  meaning  for  the

calculation itself.  Transmitter and receiver locations may  be

interchanged without affecting the results.

The  transmitter POWER is entered in watts (W) and  the  trans-

mitter's ANTENNA gain is entered in decibels with respect to an

isotropic radiator (dBi).

Because of ground effects real short wave antennas,  even  when

horizontally mounted, radiate power along a line of sight which

is tilted with respect to the horizontal. The antenna's MINimum

ELEVATION angle is entered in degrees.

The  electic  field strength of the radio wave is  measured  in

uV/m (micro volts per meter).  A threshold may be given as MIN.

FIELD STRENGTH which is used to suppress output data if too low

a level is found to be present.  The electric field strength is

entered  in  decibels above 1 uV/m and should not be  mixed  up

with  the  signal-to-noise  ratio which is  controlled  by  the

receiver's specifications.

Due  to  seasonal variations in the ionosphere  the  user  must

specify the month for which the calculation is to be performed.

The  year  which must also be entered has no  meaning  for  the

calculation.

The  solar-terrestrial  relations are respected by  the  actual

sunspot number which is published by certain bulletin services.

Because  radio predictions have a statistical meaning the  user

must  enter a desired percentage of time.  MINIFTZ data may  be

calculated to be valid for 10%, 50% or 90% of time.

MINIFTZ will calculate the diurnal variation of field  strength

and  skip  mode  on certain frequencies which  are  of  special

interest  to the user.  A maximum of eleven frequencies may  be

specified by the USER DEF. FREQ. function. When HAM FREQUENCIES

is selected MINIFTZ will analyse predefined frequency  settings

according to the international amateur radio bands.

All settings may be stored on diskette.  The SAVE PARMS command

in  the OPTION menu is used to create the disk  file  CHIRP.INF

which holds all user data.  The next time CHIRP is loaded  from

diskette the previous settings will be again available.

6. MINIFTZ-OUTPUT

The calcualtion is initiated by clicking the RUN command in the

MINIFTZ menu.  Under certain conditions the RUN command is  not

available  and MINIFTZ cannot be invoked.  The previous  output

may  be  recalled to the screen by the  RECALL  DATA  function.

Furthermore,  the  output  data  may be redirected  to  a  line

printer when PRINTER is active. A dialog box is used to ask the

user  whether  field strengths or modes  and  elevation  angles

should be displayed.  The data is calculated for each hour  per

day (1h - 24h UTC).

The  FIELD STRENGTH tables shows the estimated  field  strength

levels  at the user frequencies.  In addition to this  the  MUF

(maximum  usable frequency),  the skip mode at MUF and the  FOT

(frequency of optimum traffic) is displayed.

The MODES AND ELEVATION ANGLES table displays which ionospheric

hop  modes are expected at the user frequencies  in  accordence

with  the time of day.  MUF,  field strength at MUF and FOT  is

supplied furthermore.  The modes and elevation angles are given

by a code which is best explained by an example:

2F04

//\\

two hops via F-layer    elevation 4 degrees

7. REFERENCES

[1] Die Berechnung von foF2, M(3000) und  von  Vorhersagen  der

Raumwellenfeldstaerke im Kurzwellenbereich mit Hilfe  eines

"Home Computers"

Th. Damboldt, P. Suessmann

Kleinheubacher Berichte, Band 30, 1987

ISSN 0343-5725

[2] A simple method of estimating foF2 and M(3000) with the aid

of a home computer

Th. Damboldt, P. Suessmann

FTZ Darmstadt, 1988

[3] FTZ  high-frequency  sky-wave  field  strength   prediction

method for use on home computers

Th. Damboldt, P. Suessmann

FTZ Darmstadt, 1988

[4] Wellenausbreitung I und II

J. Grosskopf

BI-Hochschultaschenbuecher Band 141/141a und 539/539a

Mannheim 1970