T O P O   - The 3-D Topographic Map Plotting Program

Version 1.0

(c) 1992 by Donald A. Burrows

Basic Instructions

INTRODUCTION

This program began as a long time dream to be able to put topographic

maps in to my comuter so that I could view any region at any scale and

from any point of view I desired. My opportunity came when I acquired

an AT clone and gained access to an USGS topographic data base of the

United States on a mainframe computer. The entire database was too

large for my 60MB hard disk so I chopped portions of it into 1 degree

latitude by 1 degree longitude blocks and stored them as individual

files on floppy diskettes. At the same time I discovered Lee Adams'

book "Supercharged C Graphics" which gave me the 3-D graphics subroutines

I needed to put together this program.  It's a fairly flexible program

that allows one to depict the data not only as more or less traditional

contour maps (Conic Projection, with standard parallels at 38 and 46

degrees North), but also as 3-D oblique views both with color contours

and black and white shaded relief surfaces.

The program is designed primarily to be used in the VGA 640x480x16

mode, but will operate in the EGA 640x350x16 and 640x200x16 modes as

well but with degraded graphics. While it will run without a math

co-processor, it is highly recommended to have one as otherwise the

plotting procedure can be incredibly slow. It's performance on an AT

with a math co-processor is adequate, but it runs very nicely on 33MHz

486 machine. The program was intended to be used on a hard disk with

the topographic data files on floppy diskettes; however, it can be

run with both program and data on floppys or both on the hard disk.

GETTING STARTED

This section is for those who want to learn the program by doing.

Examples will be given showing how to produce several different types

of maps. It is assumed that the files TP46121.DAT, WESTWASH.PCX and

MTRAINR.PCX are present in the same directory or diskette as TOPO.EXE.

Assuming the files are on the C drive in a directory named TOPO,

the program can be started by entering

C:\TOPO > TOPO

Once the program screen is displayed, then menu selections can be

made by either using a mouse and the left mouse button or by the use of

the arrow and enter keys. Most menu boxes can be exited by using the

escape key or the right mouse button.

To start with here is how to view two completed maps.

Map #1 - WESTWASH.PCX - This is a color contour map of western

Washington state and southwestern British Columbia, and covers all

the area contained in the MAPDATA1.ZIP file. and consists of 22, 1

degree by 1 degree quadrangles.

1. Using mouse and left button or arrow and enter keys select REGION.

2. Next select the item LOAD MAP in the menu box.

3. Enter WESTWASH.PCX followed by <ENTER>. It should be displayed

on the bottom line.

Map #2 - MTRAINR.PCX - This is a black and white shaded relief view

of Mount Rainier from 23 miles to the northwest. To view it repeat steps

1 and 2 above, then enter MTRAINR.PCX followed by <ENTER>.

This next section tells how to create new maps of the region in

the topographic data file TP46121.DAT which shows a section of the

Cascade mountains of southern Washington including two major volcanic

peaks, Mt. Rainier (14,410 ft.) and Mt. Adams (12,326 ft.).

Map #3 - Plot the entire 1 degree quadrangle contained in TP46121.DAT.

1. select REGION

2. select SELECT REGION

3. enter for Northern Boundary  47 <ENTER>

enter for Southern Boundary  46 <ENTER>

enter for Western Boundary  122 <ENTER>

enter for Eastern Boundary  121 <ENTER>

4. select TYPE

5. select HIRES CONTOUR

6. select FEATURES

7. select VIEWPOINT

8. enter for Azimuth Angle   0  <ENTER>  (North at top of screen)

enter for Vertical Angle  0  <ENTER>  (View from directly overhead)

enter for Distance (miles) 0 <ENTER>  (Autoscale to fit area on screen)

9. select FEATURES

10. select SET CONTOURS

11. enter for base elevation (ft)  0  <ENTER>  (sea-level)

enter for contour interval (ft)  1000 <ENTER>

12. select REGION

13. select  CLEAR SCREEN  (assuming an image is already being displayed)

14. select REGION

15. select DISPLAY

Map #4 - Once the whole region is displayed, then particular areas

of interest can be noted and a new map centered on those regions can be

created. In this case the focus shifts to Mt. Rainier in the northwest

corner of the previous map and a black and white shaded relief drawing

is specified.

1. select REGION

2. select SELECT REGION

3. enter for Northern Boundary   47

enter for Southern Boundary   46.75

enter for Western Boundary   122

enter for Eastern Boundary   121.5

4. select TYPE

5. select SHADED (B/W)

6. select FEATURES

7. select VIEWPOINT

8. enter for Azimuth Angle     90     (0 is viewing to the north, 90 to

the east, 180 to the south and 270 to the west)

enter for Vertical Angle    80    (This is 80 degrees from overhead or

10 degrees above the horizon)

enter for Distance          16    (miles)

9. select FEATURES

10. select SET SOLAR ANGLE

11. enter for Solar Azimuth  120   (180 is directly behind observer, 90

to the observers right, 270 to observers left and 0 directly in

front of observer.)

12. enter for Solar Height    35   (90 places sun overhead, 0 at horizon)

13. select REGION

14. select CLEAR SCREEN

15. select REGION

16. select DISPLAY

DESCRIPTION OF MENU OPTIONS

REGION - Under this heading a map region can be specified, the screen cleared,

a completed map viewed, a newly drawn map saved or the current selection

plotted.

SELECT REGION - Under this heading the northern, southern, western and

eastern boundaries of a region to be plotted can be specified. The units

are degrees and decimal fractions of degrees. North latitude and West

longitude are positive values.

CLEAR SCREEN - This removes any currently displayed images from the

screen. In the black and white mode it can also be used to replace

a black background with a white background.

LOAD MAP - This is for viewing saved maps and should work with any

ZSoft PCX files.

SAVE - This is used to save completed maps using the ZSoft PCX format.

Files are automatically named beginning with mapfil00.pcx up through

mapfil99.pcx.

DISPLAY - This causes a map to be plotted using the currently specified

map parameters.

TYPE - Under this heading the map plotting mode i.e contour or shaded, can

be selected.

SHADED (B/W) - This specifies a black and white shaded relief type plot.

If a black background is desired choose CLEAR SCREEN before selecting

SHADED (B/W). If a white background is desired then choose CLEAR SCREEN

after selecting SHADED (B/W). This also specifies a high contrast mode.

A lower contrast mode can be obtained by selecting ALT PALETTE under

FEATURES.

HIRES CONTOUR - This specifies a plot using the color contour mode. It

also specifies a blue background. Using HIRES CONTOUR and ALT PALETTE

one can toggle back and forth between a white background and a blue

background.

ABOUT TOPO - Displays version, author and copyright date.

FEATURES - Under this heading a number of different plotting options can

be specified, such as selecting an alternate color palette, setting the

point from which the map is to be viewed, setting the contour interval,

and setting the direction of the lighting.

ALT PALETTE - In color contour mode this specifies a palette with a

white background and a pink max contour color. In black and white mode

it specifies a lower contrast gray scale than the default palette.

SET VIEWPOINT - This is used to specify the placement of the observer

relative to the map. The default azimuth angle is 0 degrees which places

north at the top of the map. 90 would place east at the top, 180 - south,

and 270 - west. The default vertical angle is 0 degrees which indicates

that the map is being viewed from directly overhead. 90 degrees in this

case would mean that the observer would be looking at the map horizontal-

ly from sea-level. Distance is specified in miles from a point at

sea-level located horizontally in the center of the map. The default

value is 155 miles which is a good value for observing a 1 degree by 1

degree region from directly overhead. Entering a distance of 0,

will cause the program to autoscale to fit the selected region on the

screen assuming that both azimuth angle and vertical angles are also 0.

SET CONTOURS - This option is used to specify the altitude from which

color contours are computed and the altitude interval between successive

colors. In the 16 color mode there are 15 altitude intervals. If the

base elevation is 0 (sea-level) and the contour interval is 500 ft. then

the background (blue or white) color will indicate all regions of

altitude 0; the dark green, altitude 0 to 500; up to white or pink for

all regions over 7500 ft. The default value of the base elevation is 0,

and the default value for the contour interval is 600 ft. Note: In the

topographic data files the elevations are specified to only the nearest

10 meters above sea level, consequently areas that are less than 5

meters above sea level have apparently been rounded off to 0 so that

they will be displayed asocean instead of being displayed as land.

The order of the color contours is as follows:

Sea Blue / White

Dark Green

Green

Light Green

Dark Yellow-Green

Light Yellow-Green

Yellow

Brown

Dark Brown

Tan

Orange

Dark Gray

Light Gray

Light Tan

Light Olive

White / Pink

SET SOLAR ANGLE - This applies to the shaded mode and determines the

direction of the lighting relative to the observer. A solar azimuth

angle of 0 has the lighting coming from directly in front of the observer,

while a value of 90 will have it coming from the observers right, 180 -

from behind the observer and 270  - from the observers left. Solar height

determines the vertical angle from which the lighting will come. In

this case 0 degrees would indicate the sun is on the horizon of the

map and 90 would place it directly overhead. Default values are: Solar

Azimuth - 120 degrees and Solar Height 35 degrees.

MAGNIFY - This allows the vertical scale of the maps to be enhanced. It

has no effect of the contour display. Selecting x1 means that there is no

vertical exageration i.e. the vertical scale is the same as the horizontal

scale. Four different vertical magnifications can be selected. 1.5 times

normal, 2 times normal, 3 times normal and 5 times normal.

QUIT - This exits the program. Pressing <ALT>X has the same effect.

Version 1.0 of TOPO is a first release and the author would appreciate

hearing about your experiences and ideas concerning TOPO. Other

topographic files can also be obtained. Contact the author for more

information at

GE mail address  D.BURROWS2

or  Don Burrows

526 Belmont Road

Grand Forks, ND 58201

(701) 775-3523

--------------------IMPORTANT -- WARRANTY & DISCLAIMER -----------------

TOPO version 1.0 is freely provided for use without charge and may be

freely distributed. The author makes NO warranties, expressed or implied

as to the quality or the performance of this program. The author will

NOT be held liable for any direct, indirect, incidental or consequential

damage resulting from the use of this program. Your use of this program

constitutes your agreement to this disclaimer and your release of the

author from any form of liability or litigation.

----------------------------------------------------------------------------