*

*   Disk Diagnostic program v 1.0

*

*************************

******* Copyright *******

******** Notice *********

*************************

All Materials on this disk are copyrighted by Custom Services 1986.

All items were produced by Custom Services as "No-Frill" Software.

You may redistribute any or all of the items on this disk so long as

you also move this notice(which is included in each source file).

Custom Services does not charge for this software except to ask for

a small donation(no more than $5.00) from anyone using one of these

programs.  Those users who register with a donation will recieve update

information.  Under no instance may any program or part of a program be

copied for commercial use with out a written agreement from us.

To obtain a list of all disks currently available in the Custom Services

library, and ordering information, send a preaddressed mailing label

and 2 loose first class stamps to:

Custom Services

P. O. Box 254

Moorestown, N. J. 08057

*

* Format of Call:

*

*   SYSTEST  -F[ull] -V[erbose] -D[rive] n

*

*   This program will  read the disk sector by sector checking to see

*   if the disk/diskette combination can be accessed.  The options

*   and their meaning are as follows:

*

*   Drive   specifies the drive to use, default is drive 0, the

*           internal drive.

*

*

*   Full    This will do a seek and read test.  The default if this

*           option is not specified is to do a read test.

*

*  Verbose  This will print statistics on the run(not just error info)

*

*

Disk Head Cleaning

You will find that Amiga disk drives do not need much in the way of

cleaning and maintenance.  This program will clean your heads if you follow

the instructions below:

a) Obtain a double sided 3 1/2 inch cleaning disk

b) prepare the disk for the cleaning(add the cleaning fluid ext)

c) enter the command: SYSTEST -D x -F

d) When the program prompts for the disk to be inserted place the

cleaning disk in the drive and then hit return.

c) As soon as the program starts seeking type a Ctrl-C(Hold the

Ctrl and C key down at the same time) to abort the program after

the Seek test is done.

This will move the disk head around on the cleaning surface extending

the life of both your disk heads and the cleaning disk.  A cleaning disk

normally is good until you start to see dirt from the heads on it(about 15

to 20 cleanings).  You should try to clean your heads about once a month if

you make heavy use of your drive(s).  I was going to make a separate cleaner

program but found I didn't need it!  CLEAN HEADS are important!!