HandShake V1.50a Summary of Features.

If you want to see how this version of Handshake has changed from the

previous versions please refer to the Version History section at the end

of this document.

WHAT IS IT?

Handshake is a Full featured VT52/100/102 terminal emulator for those

people who really need it. I have taken great pains to support the full

VT102 spec. (VT102 is a superset of VT100).

Feel free to use and distribute this program as you like, as long as it is

not for commercial use. If you like it, use it and want to see it get

better, please send a contribution of $25.00 to the address listed in the

program's "About.." requester. Commercial users are urged to contact the

author. Please do not alter or rename the distribution ARC file in any

way if you pass it along or post it on a bulletin board. The current

Handshake distribution ARC file is called HDSK150a.arc. This way people

can tell what version of Handshake is on a BBS.

WHAT IS IT NOT?

Handshake doesn't try to be an all round, general purpose, modem program

which supports a multitude of transfer protocols, a script language, etc.

There are plenty of good programs out there which fulfill this need.

Several Handshake users however claim they use it for all their

communications and that handshake does more than many programs that make

greater claims.

WHAT IS THIS DOCUMENT?

This document is a quick description of the features of Handshake paying

particular attention to the less obvious aspects of Handshake so that an

experienced user will be able to use the program. This document assumes

that the user has experience with terminal programs and the Amiga.

Especially the Intuition user interface ( Menus, requesters, etc. )

WHAT IS THIS DOCUMENT NOT?

This is not full Handshake documentation!! That will be coming at a later

date. If you are not familiar with terminal programs or the Amiga user

interface, find a friend who is and ask him/her to help you.

WHAT DO YOU NEED TO RUN HANDSHAKE?

An Amiga with 256K or more. Version 1.2 of Kickstart and Workbench.

THIS PROGRAM WILL NOT WORK WITH VERSION 1.1 OR EARLIER OF AMIGADOS.

Hardware to connect to the other computer. This may consist of only a cable

if you are locally wired, or a cable and a modem if you are accessing the

other computer over the telephone. This program does work with memory

expanded beyond 512K.

WHY SHOULD YOU USE HANDSHAKE?

If you have a requirement for FULL VT100/102 compatability and have not

been satisfied by other programs which only do half the job, this is

probably the program you have been looking for!

Here is a list of some of the features that Handshake offers:

- Full 24 line display with title bar enabled! No more turning the

title bar on and off to see all 24 lines.

- Full VT52 support. Every VT100 has a VT52 inside of it. So does

Handshake -- right down to the VT52 graphics character set.

- VT102 Edit mode. Local insert and deletes.

- Smooth or Jump scrolling.

- Block or Underline see through reverse video cursor. Blinking or

non-blinking

- Full VT100 Special Graphics Character Set.

- Full double height and double width characters.

- 132 character per line mode. It may be necessary to run preferences

and use the screen centering gadget a little to the left in order to

see the last few columns in 132 column mode.

- Double height and width characters in 132 column mode.

- All character modes. Normal, Bold, Reverse video, Underline and

Blinking in any combination.

- Full keyboard support. This includes VT52 mode. Handshake handles the

escape sequences to turn auto repeat on and off and lock the

keyboard. No more run away cursors while in EDT under VMS etc.

- ASCII file transfer mode with user definable end-of-line translation

for both sending and receiving. No more carriage returns in your

received files (unless you want them).

- XMODEM, XMODEM/CRC, YMODEM and YMODEM Batch protocols for binary

files transfers.

- Six user definable function keys.

- Dialing Support with a 20 Phone number Directory. Optional auto

redialing is available. System characteristics ( baud rate, data

bits, parity, stop bits ) can be specified in the dialing directory

entry. Auto redial can be specified in the directory entry. This

dialing mode is geared to Hayes compatible modems, although the user

can specify the dialing prefix ( ATDP or ATE0DT, etc. ).

- Fast! Using a 95K text file captured from a VAX which was then sent

to Handshake from a local microcomputer, Handshake maintained a

throughput of 875 characters per second. This means that Handshake

can take characters as fast as your host can send them at 7200 baud,

and still look good at 9600. A real VT100 can't keep up at 9600

baud either. You can use the shareware BLITZFONTS program to get even

better performance.

- Complete Workbench integration. Handshake can be started from the

Workbench. As you save parameter files, icons are created. These icons

can be used to invoke Handshake later with the same paremeters. You

can even specify a host to dial immediately when the icon is invoked.

This feature can be turned off if you prefer the CLI and do not want

.info files cluttering up your disk.

WHAT DO YOU NEED TO KNOW ABOUT HANDSHAKE TO USE IT NOW?

The follwing things about Handshake are not obvious so you should

familiarize yourself with this section.

* READ THIS NOTE.

* Handshake uses a 660 pixel wide screen instead of the usual 640 pixels.

* This means that the Handshake extends 20 pixels further to the right

* Than most screens. It may not be possible to seew the screen depth

* arrangement gadgets at the right of the title bar or the last 3 or 4

* columns of test in 132 column mode unless you run preferences and move

* the screen centering gadget a little to the left!!

Command line.

Handshake can be invoked from the CLI. The general form of the command

line is illustrated below (parts in square brackets are optional):

Handshake [-p parameter_file_name] [destination_name]

The parameter file name is the parameter file to be used, if none is

specified the default file Handshake.parms is used.

The destination name is the name of the remote computer to be dialed.

This name must be one that appears in dialing directory of the

parameter file being used and must be spelled exactly the same way it

is spelled in the dialing directory entry (Case of characters is not

significant).

If the name in the dialing diretory entry ends in a '*' or the Auto

Redial menu selection is set in the parameter file, Handshake will keep

dialing the number until a connection is made. If the name in the

dialing directory entry ends in a '*', do not include the '*' on the

command line when specifying the destination name.

If this dialing directory entry appeared in the default file

Handshake.parms:

555-1234:1200,8,n,1 Fred's BBS*

The following command line coule be used to call this BBS.

Handshake "Fred's BBS"

Note that the '"'s are necessary in this case because the name of the

BBS has a space in it.

Here is another example.

Handshake -p universities.parms UCLA

This would dial the phone number associated with the descriptor UCLA

in the parameter file universities.parms

For a complete description of dialing directory entries, see the

discussion later in this section

Workbench.

Handshake can also be invoked from the Workbench. There is an icon which

looks like a terminal. Invoking this icon will cause Handshake to be

invoked with the parameters from Handshake.parms, or if this file does

not exist Handshake will use it's built in parameters.

In the project menu there is an item "Create Icons". if you set this to

"Yes" Handshake will create a telephone icon every time that you save

the parameters. While Handshake is saving the parameters it will also

prompt you for a dialing descriptor which if specified will be used as

the initial dialing destination when the icon is invoked.

CLI users will probably want to keep the "Create Icons" option set to

"No" so that their disks will not be cluttered with un-necessary .info

files.

Keyboard.

VT100 Keys                Amiga Keys

----------                ----------

Arrow keys                Arrow keys

F1 through F4             F1 through F4

Keypad 0 through 9        Keypad 0 through 9

Keypad .                  Keypad .

Keypad -                  Keypad -

Keypad ,                  Help key              ** Watch for this **

Keypad Enter              Keypad Enter

The Back space and Del keys can be reversed with a menu option.

User Definable Function Keys.

The function keys F5 through F10 are now user definable keys. There

is now a new menu item in the Porject menu, the "Function keys" item.

Selecting this option will cause the Function Key Edit Requester to

appear. Just type in the strings that you want the fucntion keys to

transmit. When you are finished, just select the ALL DONE gadget.

There are two other gadgets on the Function Key Edit Requester. These

allow the user to specify which characters in the transmit strings should

be used to represent the carriage return and line feed keys. The default

values are the '~' for carriage return and '`' for line feed. Please

note that inorder to change these charcters it is necessary to delete the

character which appears in the gadget before typing in the new one.

The function key definitions are saved along with the phone directory

and other options the the selected Handshake parameter file.

File Transfers.

The "Receive/Transmit CR/LF as ..." menu options are only applicable to

the ASCII file transfer modes. These can be used to translate end of

line characters when sending to the other system. For instance, when

sending to a VAX which is receiving with the CREATE command, you

will want to send a carriage return to the  VAX at the end of each

line. Amiga text files have no carriage returns in them so lines are

terminated with line feed characters. By selecting the Transmit LF as

CR option Handshake will do the conversion for you.

As of version 1.30b you no longer have to terminate ASCII Receive

(capture) mode in order to do a binary file transfer. Handshake

will automatically suspend cpature mode while the transfer is taking

place and will restart it  when the transfer has completed.

If you use the XMODEM, XMODEM/CRC or single file YMODEM binary file

transfer mode, be aware that the received file will probably have to

be truncated. This is because all files received by XMODEM are a

multiple of 128 bytes long. AmigaDos does not like these extra bytes.

There are several good public domain utilities available to strip

these bytes off the end of the file (TAILRID, for one). If you send

your files in ARC format then this is not a problem.

Handshake also supports the YMODEM Batch protocol. Using this protocol

lets you transfer multiple files in one trnasfer. Frees the receiving

user from the necessity of having to enter the name of the file

being received, and results files that have the same length as the

original file (N need to chop extra bytes off the end of hte file).

Handshake does however ask the receiving user to specify the directory

path into which the received files should be placed. This directory

path MUST end in a ":" or a "/". Some exapmles of valid directory

paths are:

df0:

df1:source/

source/

:source/

vd0:source/headers/

RAM:download_files/

my_files:               /* In this case my_files must be a vilume

name or have been assigned with the

assign command.

*/

If you do not specify a directory path the files will be placed in the

current directory.

When sending with YMODEM Batch protocol you can specify a file name as

usual, or use wildcards as well. Here are some sample YMODEM Batch

send file specifications:

df0:source/my_file.c

ram:#?

df1:source/headers/#?.h

You get the idea. YMODEM Batch will transfer all the specified files

to the receiver. It will send the file names, and the file lengths as

well.

Handshake now supports the KERMIT file transfer protocol. It is used in

mush the same way as the YMODEM protocol which also supports multiple

file transfers. There are two KERMIT modes. KERMIT Binary and KERMIT

Text. Kermit Binary requires you to be connected with the other computer

with 8 data bits. It will not work with a seven bit connection. Perhaps

I will put in the binary escaping feature for the next release. KERMIT

Binary does image transfers, the received or sent file is bit for bit

identical with the original. Kermit Text is used for text files, and

will work with either a 7 bit or 8 bit connection. It should be used

for text files only.

For the binary transfers and for the ASCII Send, Handshake pops up

two windows, one in its own screen and one in the workbench screen.

These windows provide information on the progress of the transfer.

The one in the Handshake screen can be used to abort the transfer

just by clicking on the close gadget in the upper left hand corner.

The window in the Workbench screen is provided so that during a long

file transfer you can use your multi-tasking Amiga to do other things

and not have to keep flipping back to handshake to see if the transfer

has completed.

If you have the Audio Bell option enabled, You will hear a rising

sequence of notes for a successfull file transfer, and a falling

sequence of notes for an unsuccessfull file transfer.

Dialing the Phone.

In order to dial a number from the dialing directory ( Menu option

"Dial Phone" ), click using the mouse on the small square gadget

immediately to the left of the number that you want to dial. Clicking

on the text of the number allows you to edit the entry, but not to

dial it.

The dialing directory is saved to disk whenever you save the parameters

It is possible to have several diailing directories by using different

parameter files.

Whenever Handshake goes into auto-dialing mode, it pops up a pair of

windows much like in file transfer mode to keep you informed of progress.

See the description in the file transfer section above.

If the Audio bell feature is enabled, you will hear a four note

sequence to indicate that the connection has been completed.

Format of dialing directory entries.

A dialing directory entry has the following general format. The parts

in the square brackets are optional. The * at the end of the line is

used to indicate auto redialing.

telephone_number[:baudrate[,data_bits[,parity[,stop_bits]]]] [desc][*]

Below are some sample dialing directory entries:

123-4567                                      Simplest case. Dial the

number using the current

serial parameters.

123-4567 Joe's BBS                            Same as above with a

description.

123-4567 *                                    Both of these will dial

123-4567 Joe's BBS*                           Until the other system

Answers.

123-4567:1200                                 Call the number changing

baud rate to 1200 if

necessary.

123-4567:1200,8,n,1 Joe's BBS*                A biggie. Call Joe's

until it answers using

1200 baud, 8 data bits,

no parity and 1 stop

bit.

Interlace.

The Half screen interlace mode is intended to give you a full 24 line

VT100 Emulation in half a screen. This leaves you the other half to do

whatever you like, and still watch your terminal screen.

The full screen interlace mode is useful in 132 column mode.

Readability of characters in interlaced mode may depend on your

selection of colors. (I find that black characters on a gray

background look just super ).

WHAT PROBLEMS ARE THERE WITH HANDSHAKE?

Nothing that I currently Know of. However you may find that you can not see

the last two or three columns of 132 column mode. If this happens to you,

run Preferences and move the screen centering gadget to the left a bit.

This should fix the problem.

WHO DO YOU CALL WITH PROBLEMS, SUGGESTIONS, MAKE CONTRIBUTIONS TO ETC?

Me!

Eric Haberfellner,

57 Marble Arch Crescent

Scarborough, Ontario

CANADA

M1R 1W8

Phone (416) 757-1055 ( After Six P.M. EST please ).

I am on BIX as ehaberfellner.

HANDSHAKE VERSION HISTORY

Version 1.50a

- Added user definable function keys. keys F5 through F10 are now

available as user definable keys. See the documentation above.

- Kermit File transfer protocol added. See documentation above.

- Some characters were getting lost after file transfers. this is now

fixed.

Version 1.42a

- Fixed the horrid "Beep forever" bug.

- Fixed the rename command in EXEC_ME for Handshake.rev.

Version 1.41a

- Fixed bug in YMODEM Batch protocol. It would work with another Amiga

running Handshake, but not with any other YMODEM Batch

imp[lementations.

Version 1.40a

- Fixed the horrible dialing bug which appeared in v1.30b. Boy am I

embarassed. Dialing would not work unless AUTO REDIAL was selected.

- AmigaDos requesters now appear in the Handshake screen instead of

popping the Workbench screen to the front. Big improvement.

- YMODEM and YMODEM Batch file transfer protocols are here. See the

File transfer section of this document and the Binary Protocol

item of the TRANSFER menu.

- Added a bell. Handshake now beeps upon receipt of a BEL character.

Handshake also now gived an audio indication of file transfer

completion and connection in auto redial mode. The user can

select audio bell, visual bell, both or none. See the new Bell Type

item in the TERMINAL menu.

- Added the VT100 Margin bell feature. See the new Margin Bell

item in the TERMINALcmenu.

- Added a blinking/non-blinking option for the cursor. If you are

using a parameter file from a previous version of Handshake,

the cursor will not blink initially. Go to the Cursor item of the

TERMINAL menu and select blinking. Now save your parameters. From

now on the cursor will blink again when you start Handshake.

- Speeded up CRC calculation for file transfers.

- Fixed a minor bug which caused XMODEM sends to always show 1 error

even there had not been one. This did not affect the transfer.

- Fixed a bug which sometimes caused inproper font selection after

going into VT52 mode and back out again.

- Fixed bug in UK Character set support.

- Speeded up CRC calculation for file transfers. I am now doing

bytewise instead of bitwise CRC calculation.

Version 1.30b

- Replaced the abort requesters with windows. The windows are used to

provide progress  information about file transfers and auto-dialing

as well as serving as abort devices. To abort a file transfer or

auto-dialing just click on the close gadget of the window in the

handshake screen.

- The windows described above are also duplicated on the Workbench

screen. This is so that you can do other things while a large file

transfer is in progress and not have to keep flipping back to

Handshake to see if it is finished yet. Same for auto-dialing.

- Relinked with a "tiny" _main.o so that the lattice standard input/

output window no longer pops up in the workbench screen.

- Capture mode (ASCII Receive) can now remain active during binary

file transfers. Handshake will suspend it until the transfer is

over and then start it up again.

- Added XMODEM/CRC file transfer mode.

Version 1.21a

- Local echo mode did not work properly. This has been fixed.

- Reseting the terminal would sometimes leave a double cursor on the

- Screen. This has been fixed.

Version 1.20a

- Improved Workbench interface. Handshake will now optionally create

create Telephone icons for saved parameter files. This allows the

user to select which paramter file to use from the Workbench. The

icons can also be created in such a way that Handshake will

automatically start dialing a specified number when it is invoked

with that icon.

- Fixed a bug which caused XMODEM transmits to sometimes hang.

- Changed autodialing support so that it now works with modems that

report status while they are dialing (Like the Volksmodem 1200).

- Dialing destinations which contained spaces on the command line did

not work, even when enclosed in quotes. This has been fixed.

- Selecting the Reset Terminal option from the menu or typing Amiga-V

would cause two cursors to appear on the screen. This has been fixed.

Version 1.10a

- At last, a real 132 columns instead of 128.

- Added some intelligence to the routines that display the cursor. This

yielded a 35% performance improvement. Handshake now has an average

throughput of about 875 characters per second.

- Cleaned up parsing of command line parameters especially the dialing

directory entry.

- Put error checking into reading of parameter files. If the file does

not exist or is of the wrong size put up a requester advising the

user. If the default file Handshake.parms cannot be found do not

annoy the user with a requester (use the built in defaults).

- The command line dialing direcotry entry is now case insensitive.

- Added some register variables in several places. Handshake is now

about six percent faster because of this.

- Fixed a bug which caused the line attribute for the bottom line to

not be reset when the screen was cleared.

- Fixed a hideous bug which caused the input subtask to lock up

sometimes. This would appear to the user as a blank title bar. This

would only happen if the user interface task sent a "go to sleep"

message to the serial input task at the same instant that an IO

completed, therefore this was more likely to happen at higher baud

rates. Sorry about that one.

Version 1.02a ( not released to the public at large ).

- Added command line parameters to specify parameter file to use, and

to allow the user to specify a remote computer to dial from the

command line.

- Added 20 second dialing timeout to the dial timeout menu.

- In order to abort redialing it was sometimes necessary to select the

abort requester more than once. This is now fixed.

- Moved File transfer and redialing abort requester to upper right

portion of screen.

- General cleanup of the file transfer code. Receiveing to disk is now

much faster. Transfer aborts now work porperly at high baud rates.

Version 1.01a

- Auto redialing added.

- Pulse/Dial menu option replaced with user definable dialing string.

- Added menu item to allow user to specify backspace & del key

operation.

- Fixed bug which caused menus to not be updated after a reset if

no parameter file was present.

Version 1.00a

- Handshake Unleashed on an unsuspecting public. An historic Moment!