FMS.DOC

FILE BASED TRACKDISK SIMULATOR

* Useful for creating a floppy-like partition on your harddisk

(so you can diskcopy to a floppy) WITHOUT actually having to

create a special partition for it.

* Useful for testing new filesystems and such

* Up to 32 Units, OFS or FFS selectable via Mountlist

* THIS IS NOT A RAM DISK.  Operation will be necessarily slower

than your HD, but that isn't the point behind using the device!

* Advantages over RAD / Other ram disks ... doesn't use RAM.

Designed for hard disk users.  By putting the 'partition'

file on the hard disk, it sticks around and you don't loose

the data you wrote to it, so mounting it again after reboot

or power up yields whatever was there before.

The fmsdisk.device simulates a trackdisk.device for fixed media

via a DOS file (one file per unit).  Simply setup your Mountlist,

mount the simulated trackdisk, and format, and you are off.

This has particular advantages for those hard disk users who make

lots of distributions, like me.  I can create a simulated partition

using fmsdisk.device that uses exactly the same parameters as

my favorite floppy device (and run the appropriate file system,

in this case the OFS).  After formatting, I can copy the distribution

onto the simulated partition and then DiskCopy it to the floppy.

This also has advantages to people fooling around with writing their

own file systems, as the disk file used may be examined at any time

through normal programs while the 'filesystem' is running.

INSTALLATION

copy fmsdisk.device devs:

Assign FMS: to some empty directory on your hard disk (doesn't have

to be empty, but might as well be clean, eh?).  Files will be

named FMS:Unit%d

Add appropriate entries to your Mountlist (see example Mountlist).

Mount FF0:

; Only need to format the first time to create the file.

Format drive FF0: name "FF0" NOICONS

Repeat above two lines for each simulated partition you wish to add.

------------------

VERSION HISTORY

3.0	- Seek()'s are done offset from the current position, giving

great speed increases over previous versions.

- Re-compiled with Lattice v5.05, making code less than half

the size of previous versions.

2.0

- doesn't Seek() if the file is already positioned at the

requested next read/write sector.

- default buffers set to 10 instead of 2

- overall performance and efficiency increased.

1.0	- First release