AutoAddRAM v2.03 (c) Copyright 1990 by Jonathan Potter

This program allows you to add multiple non-autoconfiguring memory boards

to the free memory list quickly and easily. It reads from a text file a list

of addresses, and attempts to add memory at those addresses. It will not

attempt to add memory that does not exist, or has already been added.

To use, set up a file in the S: directory, called MemoryList.AAR. This file

contains, on each line, the start and end addresses of each memory chunk you

wish to add. For instance, if you had a 1 meg RAM board at $200000, a 2 meg

RAM board at $400000, and 256K KickRAM at $f80000, your S:MemoryList.AAR would

read :

200000 2ffffe

400000 5ffffe

f80000 fbfffe

You may also specify the priority and name for each chunk of memory. You

may specify neither priority or name, just priority, or priority and name (ie

you must include priority if you specify the name). For example,

200000 2ffffe 10

400000 5ffffe 0 mymemorychunk

f80000 fbfffe

Consult the documentation of your memory boards if you do not know the

start and end addresses of that memory.

Jonathan Potter

P.O. Box 289

Goodwood, SA 5034

Australia

(08) 2932788