****    TapeMon:  a monitor program for BTNtape handler           ****

****       Version 2.0   03/29/91                                 ****

****       Freeware by Bob Rethemeyer   (DrBob@cup.portal.com)    ****

****      (c) Copyright 1990,1991 Robert Rethemeyer               ****

TapeMon is an optional companion program to the BTNtape SCSIdirect

tape handler.  Since the handler cannot directly inform you of what

it is doing,  TapeMon, running in a separate CLI, communicates with

the handler and prints information for you.  Think of it as a window

into the handler.

To run TapeMon, bring up a separate CLI window.  Enter "TapeMon" or

"TapeMon [name]", where [name] is the name of the handler device process.

If you do not enter the name, the monitor will default to using "TAPE:".

The handler must be loaded and running for TapeMon to work, otherwise

it prints a message and immediately quits.  If you have mounted the

tape handler but not accessed the tape yet, the handler is not yet loaded,

and TapeMon will give up.  To avoid this, invoke TapeMon after starting

tape access, or use "Mount = 1" in your mountlist entry.

Once the handler is loaded and running, you may invoke or terminate

the monitor at any time (but only one TapeMon may run at a time).

To terminate the monitor, enter control-C from the TapeMon CLI window,

or send a BREAK command from another CLI.

If the handler encounters a tape error, it remembers the data returned

by SENSE.  When TapeMon is started, it prints the retained sense data.

So if you weren't running TapeMon at the time an error occurs, you can

still see the sense data by starting TapeMon after the fact.

TapeMon accepts a second command line argument as a flag to print all

the sense information when a sense occurs.  The value of the second

argument is ignored, but if it is present, complete sense is printed.

Example:  TAPEMON TAPE: XYZ

The presense of the XYZ will cause the sense message to also print

32 bytes of hex sense information as it is returned from the drive.

TapeMon output is to stdout, so you may redirect it if you like.

Example:   TAPEMON >filename

MESSAGES

The following summarizes the messages from the handler/monitor:

Drive: ............       shows the manufacturer, model, and ROM version

of the drive.  This is the data returned by the

drive with the SCSI INQUIRY command.

Sequential Access         One of the first two messages is printed to show

Direct Access             which type of drive you have.  The third message

BAD LOGICAL UNIT NUMBER   is printed if you use the wrong LU number.

TAPE: Opened at block x   printed when the TAPE: file is opened by DOS.

x is the first block number of the file.

For sequential tapes, block numbers are relative

to the beginning of the file (not necessarily

the beginning of the tape).

Closed at block x         printed when the TAPE: file is closed by DOS.

x is the next available block.

Writing block x * n       printed when a tape write is started by the

handler. x = block number. n = number of blocks

(fixed block mode, VB-0), or number of bytes

(variable block mode, VB-1).

Reading block x * n       printed when a tape read is started by the handler.

Writing filemark          printed when TAPE: is closed in write mode,

sequential drives only.

Unsupported_pkt= x        printed when DOS sends a packet which the handler

rejects.  Example: 25 for the INFO command.

Function/mode error       the application attempted a Read() while opened

in write mode, or a Write() while in read mode.

Rewinding                 printed when handler starts a tape rewind

Skipping n file(s)        printed when TAPE:num form is used to skip over

filemarks (sequential drives only).

Raw command               printed when handler receives a raw command request

Mode Select block size x  printed when handler detects that drive block size

is not equal to BS, and a Mode Select is issued

to correct it.

User Mode-Select          printed when handler passes the user mode-select

request on to the drive.

Sense: ????? other=yy,zz  printed when the handler issues SENSE to the tape

drive due to a "check condition" (error).

????? is the sense key meaning.  Filemark and

end-of-tape are identified separately. yy,zz is the

additional sense code from bytes 12 and 13 of the

sense data (some drives provide this). If complete

sense was selected by TapeMon arg2, 32 additional

bytes will be printed representing the exact data

returned by the drive for the sense.

Last sense= ?????, yy,zz  retained sense data printed at TapeMon startup

ERROR(S) IN STARTUP       printed when the handler detects an unrecognized

parameter in the mountlist Startup statement.

SCSI ????? ERROR          printed when the scsidirect driver detects an

error (io_Error != 0). ????? is the error meaning.

Capacity: xxxx blocks     (for the 3M drive only) printed at Open() time

to show the number of blocks available on the

tape volume.  If this number is huge, your

3M drive may not support the READ_CAPACITY

command- don't worry about it.

Time to insert tape# x    printed when EOT requester is displayed

Can't get memory for tape buffers   printed for Open() attempts when not

enough memory is available for buffers.

Try a smaller number for NB.

HOW IT WORKS

TapeMon finds the tape handler process and locates a common

data structure attached to it.  The monitor waits for signals from the

handler.  The handler detects the presence of the monitor and signals

it when a message needs to be printed.  A break signal causes the monitor

to disconnect itself from the handler and terminate.