Newser.device update information

--------------------------------

V2.10 --- June 27, 1991

This version is very similar to V2.00. It adds a routine for

controlling the RTS and DTR control lines directly. The method is

compatible with that of ASDG's Dual Serial Board. See the source

code for more information. A couple of potential problems were

fixed as well.

This will likely be the final "official" version of newser,

except perhaps for minor bug fixes. HAVE FUN!

Documentation for V2.00:

Please read this completely, since it contains information you'll

probably need to know sometime...

The time has come for another update to the serial device driver

for the I/O expansion board. Previous versions basically worked,

but had various shortcomings. This, the third release, tries to

finally do everything right...(and seems to succeed!!)

What's new?

-----------

* Better performance. The last version increased read performance

to the max; this version addresses write performance as well.

Context-switching overhead has been reduced to a minimum to make

writing much faster than the previous version, and faster than

the regular serial.device too. The overall performance of the

driver is now extremely satisfying!

* CTS handshaking now works. This is a major fix. Previous

versions would mysteriously hang if CTS was ever deasserted. (The

serial chip documentation was inadequate...). It is now safe (and

recommended) to turn on CTS handshaking. Actually there is no way

to turn off CTS handshaking (a limitation of the serial chip) but

if you specify no CTS handshaking, the driver will check the

status of CTS before starting a write operation: if CTS is not

asserted, the write is aborted. On the other hand, if CTS

handshaking is turned on, and CTS is not asserted (e.g. the modem

is turned off), the driver will wait forever for CTS to be

asserted; expect this. Terminal software is supposed to provide a

time-out to avoid "hanging" in this case, but I doubt the

authors' of these programs bothered to do it.

* The "random lock-up" bug reported by some users is fixed.

* IOPatch received a small update: It now calls CacheClearU after

duplicating itself, when running under kickstart 2.0. Also the

gadget text was changed to be slightly more intuitive.

One bug report remains an unsolved mystery. A user reported total

failure when using the I/O board in a system with a GVP Series-II

controller; when the controller was removed, the board functioned

normally. I have checked the driver very carefully and don't

believe that it could be responsible for such a drastic failure.

Instead it's probably a hardware problem. First suggestion: add a

pull-up resister on the reset line, as specified in the assembly

manual.

Tips for using newser.device

----------------------------

* Turn off XON/XOFF handshaking unless you really really really

need it (you probably don't). If XON/XOFF is on, the driver can't

use its "RAD_BOOGIE" mode to speed up reads; instead of doing a

fast bulk copy, the driver is forced to check each byte for

XON/XOFF...this is extremely slow!!!!

* If you are communicating with a modem at a speed higher than

the connect rate, CTS handshaking MUST be turned on for ZModem

transfers to work properly.

* Higher numbered serial "units" have higher interrupt

priorities. If, for example, you are using two ports, one at

19,200bps and the other at 2400bps, then a good arrangement is to

connect the 19,200bps line to unit 1, and the other to unit 0.

The other way around will give poor results.

Compatibility

-------------

This version of the serial driver was tested with the following

programs:

* JRComm 1.02

* Handshake 2.12a/2.20c

* Ncomm 1.9

* C-Net 1.95t (BBS software)

Special thanks go to John Hoyt for his beta-testing and

encouragement. Call his excellent C-Net BBS at 213-633-5002.

Important notice to all I/O board users!

----------------------------------------

Due to the longer cable length that the moved CIA must drive,

many of you are experiencing erratic floppy operation, even after

grounding the I/O board as recommended. Fortunately I seem to

have run into the cure! It turns out that this problem is highly

dependent on the CIA revision. The CIAs that came with my A2000,

labeled 8520A-1, worked absolutely horribly, BUT when I replaced

it with an 8520R2 from my old A1000, the erratic floppy problem

COMPLETELY disappeared even when the board had no extra

ground/power attachments. If you are having this problem, trade

CIAs around!  Note: New Amigas are being shipped with a CIA

revision that promises much better drive characteristics.

However, this new version is not a perfect replacement, requiring

some additional pull-up resisters. The bottom line is this. If

you have the latest CIA revision, you are probably not having any

problems. If you have an old CIA revision, you can't substitute

the latest revision without some hacking; instead, try swapping

with an old A1000 like I did. (The 1000 won't know the

difference).

The Future

----------

I'm still interested in a Zorro-II version of the I/O board,

maybe even with a 6502 on board...(or am I dreaming?). I talked

to one person who expressed interest in designing such a board,

but I haven't contacted him in several months. Don't be shy; give

me a call and get started!!

----------------------------------------------------------------

Since I put so much time into this upgrade (unpaid of course), I

would appreciate receiving a few disks in the mail...I especially

like programmers tools/utilities, megademos, and anything fun.

Thank you for your support!!!

My address...

P.O. Box 1532

Southgate, MI  48195

USA

Phone: (313)-479-1056

Internet: dxb132@psuvm.psu.edu

Bix: danbabcock