A3000 Kickstart ROM Procedure

This document describes how to create your own 2.04 (or later)

ROM-based A3000. It entails procedures & modifications that you may need

to make to your A3000. It requires the A3000 superkickstart disk and

two 200 nanosecond or faster 27C220 EPROMS. For obvious copyright reasons,

I cannot distribute the actual kickstart ROM images, but the program

ROMSPLIT or ROMSPLIT1 will create the images from your A3000 which

most eprom programmers can read.

The next question is why create EPROMS? Well, 68040 cards for the A3000

require 2.0 in ROM. These cards are ready and shipping but A3000s

with 2.0 in ROM aren't shipping yet. With this procedure you can make

copies for yourself in a legal manner. Developers can use their

040 machines to test newer betas (beyond 2.04) without having to pull

out their 040 cards. The reason the superkickstart process fails on

040 cards is that the kickstart ROM image is remapped into fast ram

via the MMU. The 040 MMU is different enough from the 030 MMU that

the bootroms crash.

The programs ROMSPLIT and ROMSPLIT1 are hereby released to the public

domain with no distribution restrictions whatsoever.

WARNING! Performing these procedures will probably take your A3000 out

of warranty. Please carefully read the instructions completely and

do not start until you thoroughly understand the process.

1) A3000s have four ROM sockets. Only two are used in any one machine. Most

A3000s have special bootroms which load kickstart 2.0 and 1.3 from

disk. These ROMs must be pulled. Earlier A3000s used 27C220s, later

ones use masked ROMs, with each tpe using a different pair of sockets on the

A3000 motherboard. The four ROM sockets are located on the motherboard

along the right-hand side of the A3000. The first (front) two are for the

Masked ROMs,the second two are for EPROMS.

2) Unfortunately CBM started to not solder the EPROM sockets into the

most current A3000s although the traces and holes are there. To use these

machines, you must unsolder the holes, put in a 40 pin socket and perform

a decent soldering job. To effectively desolder the motherboard holes for

EPROM sockets, you must remove the motherboard. A temperature controlled

grounded tip desoldering station is strongly suggested. In soldering on your

A3000 motherboard, you will definitely void warranty and you perform this

procedure ENTIRELY AT YOUR OWN RISK!

3) By running the Update2.x program on your A3000 install disk, put the

kickstart image for the version of the ROM which you want onto your

disk. It will be called kickstart and located into your devs: directory.

4) Depending on your EPROM programmer, run ROMSPLIT or ROMSPLIT1. These

programs read the devs:kickstart file and create two binary output files

called Kickrom0 and Kickrom1. ROMSPLIT stores the image in order as read

by the 68030 processor. ROMSPLIT1  performs byte shuffling required by the

Intel processors. My BP Microsystems EP-1140 programmer requires that

ROMSPLIT1 be used. I suspect that most programmers will want Intel

byte ordering, but I included both programs just in case.

5) Copy Kickrom0 and Kickrom1 files to your EPROM programmer. Use

CrossDos, or Lharc the files and use a telecom program, use whatever works.

On the EP-1140, I use CrossDos and a 720K formatted diskette to put the

files on my 386PC. I then load the programmer software, select

INTEL 27C220 and load binary file. Note that some programmers may not

load binary files. Most do.

6) Program the EPROMS.

7) Take the EPROMS and put them into the last two ROM sockets (The furthest

two ROM sockets from the front of the A3000. Note that Kickrom0 corresponds

to ROM0 and Kickrom1 corresponds to ROM1. If your A3000 had EPROMS, read

the label. It usually identifies which ROM goes where. If not, look at the

silkscreen. It identifies ROM0 and ROM1. Be careful of PIN 1 orientation!

Make sure all pins go into the sockets.

8) Make sure that if your A3000 has masked ROMs that you pull them out of

their sockets; there should only be two ROMs or EPROMs in the machine at a

time.

9) Put the A3000 back together. Power up. Note that the A3000 daughtercard

must be installed for the system to work. If the machine doesn't come up,

recheck ROM installation. Try swapping the two ROMs (The silkscreen may

be wrong on Rev 7 motherboards). In worst case, put the original bootroms

back in and see if you can boot. Perhaps you can reprogram using the

opposite program (ROMSPLIT versus ROMSPLIT1), which generates different

EPROM binary files.

10) At this point, the machine is running 2.0 in ROM.( You could possibly

do 1.3 as well!!) This procedure has been tested with 37.175 and newer

beta versions and found to work in these cases.

11) For more performance, you can obtain faster 27C220 EPROMS. 150 nsec

speed Eproms will allow you to play with jumpers (see your A3000 manual)

to cut out a few wait states. Generally, put the ROM speed jumpers in the

16 Mhz position.