SetTDDelay

1991 Sebastiano Vigna

SetTDDelay is a very small and simple 2.04-only utility which sets the

public fields tdu_StepDelay, tdu_SettleDelay and tdu_CalibrateDelay of the

trackdisk.device public unit. You can specify zero or more units (specifying

no unit is equivalent to specifying all units). For instance

SetTDDelay 0 2 StepDelay 5000

will set the step delay of units 0 and 2 to 5000s, while

SetTDDelay CalibrateDelay 3000

will set the calibrate delay for all units to 3000s. If you don't specify

any of the keywords StepDelay, SettleDelay or CalibrateDelay SetTDDelay will

simply print the current settings of trackdisk.device. SetTDDelay's template

will be printed if you specify the standard '?' argument. SetTDDelay is a

pure command and can be made resident.

You should never change the delays of a public unit unless you really know

what you're doing. This command has been distributed because some drives

used with the Amiga are out of specs, and this causes SuperDuper 2.0 to

complain ("Error while recalibrating"). The suggestion is to set the step

and calibrate delay of the guilty unit to 5000s with a

SetTDDelay <unit number> StepDelay 5000 CalibrateDelay 5000

This operation is equivalent to using Leonard Poma's m5afix program (a patch

which sets to 5000 the step delay). m5afix works also under 1.3. SetTDDelay

it's more flexible but works only under 2.04.

SetTDDelay is  1991 Sebastiano Vigna and it's freely distributable as long

as all of its files are included in their original form without additions,

deletions, or modifications of any kind, and only a nominal fee is charged

for its distribution. This software is provided "AS IS" without warranty of

any kind, either expressed or implied. By using SetTDDelay, you agree to

accept the entire risk as to the quality and performance of the program.

Comments, complaints, desiderata are welcome.

Sebastiano Vigna

Via Valparaiso 18

I-20144 Milano MI

BIX: svigna

UUCP:...{uunet|piramid|rutgers}!cbmvax!cbmehq!cbmita!sebamiga!seba