;

;	Commodore VIC-20 ROM Memory Map

;	V1.0	1 Sep 1994

;

; Data types in headers (for reassembler):

;

;	DATA		Misc data

;	TEXT		String terminated with 00

;	WORD		Vectors in LO/HI byte pairs

;	CHIP		I/O Area

;	EMPTY		ROM containing FF's or AA's

;

;

; BASIC v2.0 interpreter ROM ($B000 - $DFFF)

; This is almost identical to the C64 one.

;

c000	-	Basic Restart Vectors			WORD

c004	-	'cbmbasic'				DATA

c00c	stmdsp	BASIC Command Vectors			WORD

c052	fundsp	BASIC Function Vectors			WORD

c080	optab	BASIC Operator Vectors			WORD

c09e	reslst	BASIC Command Keyword Table		DATA

c129	msclst	BASIC Misc. Keyword Table		DATA

c140	oplist	BASIC Operator Keyword Table		DATA

c14d	funlst	BASIC Function Keyword Table		DATA

c19e	errtab	Error Message Table			DATA

c328	errptr	Error Message Pointers			WORD

c364	okk	Misc. Messages				DATA

;c364		'<cr>ok<cr>'				TEXT

;c369		'error'					DATA

c389	-	Unused byte				DATA

c38a	fndfor	Find FOR/GOSUB Entry on Stack

c3b8	bltu	Open Space in Memory

c3fb	getstk	Check Stack Depth

c408	reason	Check Memory Overlap

c435	omerr	Output ?OUT OF MEMORY Error

c437	error	Error Routine

c469	errfin	Break Entry

c474	ready	Restart BASIC

c480	main	Input & Identify BASIC Line

c49c	main1	Get Line Number & Tokenise Text

c4a2	inslin	Insert BASIC Text

c533	linkprg	Rechain Lines

c560	inlin	Input Line Into Buffer

c579	crunch	Tokenise Input Buffer

c613	fndlin	Search for Line Number

c642	scrtch	Perform [new]

c65e	clear	Perform [clr]

c68e	stxpt	Reset TXTPTR

c69c	list	Perform [list]

c717	qplop	Handle LIST Character

c742	for	Perform [for]

c7ae	newstt	BASIC Warm Start

c7c4	ckeol	Check End of Program

c7e1	gone	Prepare to execute statement

c7ed	gone3	Perform BASIC Keyword

c81d	restor	Perform [restore]

c82c	stop	Perform [stop], [end], break

c857	cont	Perform [cont]

c871	run	Perform [run]

c883	gosub	Perform [gosub]

c8a0	goto	Perform [goto]

c8d2	return	Perform [return]

c8f8	data	Perform [data]

c906	datan	Search for Next Statement / Line

c928	if	Perform [if]

c93b	rem	Perform [rem]

c94b	ongoto	Perform [on]

c96b	linget	Fetch linnum From BASIC

c9a5	let	Perform [let]

c9c4	putint	Assign Integer

c9d6	ptflpt	Assign Floating Point

c9d9	putstr	Assign String

c9e3	puttim	Assign TI$

ca2c	getspt	Add Digit to FAC#1

ca80	printn	Perform [print#]

ca86	cmd	Perform [cmd]

ca9a	strdon	Print String From Memory

caa0	print	Perform [print]

cab8	varop	Output Variable

cad7	crdo	Output CR/LF

cae8	comprt	Handle comma, TAB(, SPC(

cb1e	strout	Output String

cb3b	outspc	Output Format Character

cb4d	doagin	Handle Bad Data

cb7b	get	Perform [get]

cba5	inputn	Perform [input#]

cbbf	input	Perform [input]

cbea	bufful	Read Input Buffer

cbf9	qinlin	Do Input Prompt

cc06	read	Perform [read]

cc35	rdget	General Purpose Read Routine

ccfc	exint	Input Error Messages			DATA

;ccfc	'?extra ignored<cr>'

;cd0c	'?redo from start<cr>'

cd1e	next	Perform [next]

cd61	donext	Check Valid Loop

cd8a	frmnum	Confirm Result

cd9e	frmevl	Evaluate Expression in Text

ce83	eval	Evaluate Single Term

cea8	pival	Constant - pi				DATA

cead	qdot	Continue Expression

cef1	parchk	Expression in Brackets

cef7	chkcls	Confirm Character

cef7	-	-test ')'-

cefa	-	-test '('-

cefd	-	-test comma-

cf08	synerr	Output ?SYNTAX Error

cf0d	domin	Set up NOT Function

cf14	rsvvar	Identify Reserved Variable

cf28	isvar	Search for Variable

cf48	tisasc	Convert TI to ASCII String

cfa7	isfun	Identify Function Type

cfb1	strfun	Evaluate String Function

cfd1	numfun	Evaluate Numeric Function

cfe6	orop	Perform [or], [and]

d016	dorel	Perform <, =, >

d01b	numrel	Numeric Comparison

d02e	strrel	String Comparison

d07e	dim	Perform [dim]

d08b	ptrget	Identify Variable

d0e7	ordvar	Locate Ordinary Variable

d11d	notfns	Create New Variable

d128	notevl	Create Variable

d194	aryget	Allocate Array Pointer Space

d1a5	n32768	Constant 32768 in Flpt			DATA

d1aa	facinx	FAC#1 to Integer in (AC/YR)

d1b2	intidx	Evaluate Text for Integer

d1bf	ayint	FAC#1 to Positive Integer

d1d1	isary	Get Array Parameters

d218	fndary	Find Array

d245	bserr	'?bad subscript error'

d248	-	'?illegal quantity error'

d261	notfdd	Create Array

d30e	inlpn2	Locate Element in Array

d34c	umult	Number of Bytes in Subscript

d37d	fre	Perform [fre]

d391	givayf	Convert Integer in (AC/YR) to Flpt

d39e	pos	Perform [pos]

d3a6	errdir	Confirm Program Mode

d3e1	getfnm	Check Syntax of FN

d3f4	fndoer	Perform [fn]

d465	strd	Perform [str$]

d487	strlit	Set Up String

d4d5	putnw1	Save String Descriptor

d4f4	getspa	Allocate Space for String

d526	garbag	Garbage Collection

d5bd	dvars	Search for Next String

d606	grbpas	Collect a String

d63d	cat	Concatenate Two Strings

d67a	movins	Store String in High RAM

d6a3	frestr	Perform String Housekeeping

d6db	frefac	Clean Descriptor Stack

d6ec	chrd	Perform [chr$]

d700	leftd	Perform [left$]

d72c	rightd	Perform [right$]

d737	midd	Perform [mid$]

d761	pream	Pull sTring Parameters

d77c	len	Perform [len]

d782	len1	Exit String Mode

d78b	asc	Perform [asc]

d79b	gtbytc	Evaluate Text to 1 Byte in XR

d7ad	val	Perform [val]

d7b5	strval	Convert ASCII String to Flpt

d7eb	getnum	Get parameters for POKE/WAIT

d7f7	getadr	Convert FAC#1 to Integer in LINNUM

d80d	peek	Perform [peek]

d824	poke	Perform [poke]

d82d	wait	Perform [wait]

d849	faddh	Add 0.5 to FAC#1

d850	fsub	Perform Subtraction

d862	fadd5	Normalise Addition

d867	fadd	Perform Addition

d947	negfac	2's Complement FAC#1

d97e	overr	Output ?OVERFLOW Error

d983	mulshf	Multiply by Zero Byte

d9bc	fone	Table of Flpt Constants			DATA

d9ea	log	Perform [log]

da28	fmult	Perform Multiply

da59	mulply	Multiply by a Byte

da8c	conupk	Load FAC#2 From Memory

dab7	muldiv	Test Both Accumulators

dad4	mldvex	Overflow / Underflow

dae2	mul10	Multiply FAC#1 by 10

daf9	tenc	Constant 10 in Flpt			DATA

dafe	div10	Divide FAC#1 by 10

db07	fdiv	Divide FAC#2 by Flpt at (AC/YR)

db0f	fdivt	Divide FAC#2 by FAC#1

dba2	movfm	Load FAC#1 From Memory

dbc7	mov2f	Store FAC#1 in Memory

dbfc	movfa	Copy FAC#2 into FAC#1

dc0c	movaf	Copy FAC#1 into FAC#2

dc1b	round	Round FAC#1

dc2b	sign	Check Sign of FAC#1

dc39	sgn	Perform [sgn]

dc58	abs	Perform [abs]

dc5b	fcomp	Compare FAC#1 With Memory

dc9b	qint	Convert FAC#1 to Integer

dccc	int	Perform [int]

dcf3	fin	Convert ASCII String to a Number in FAC#1

ddb3	n0999	String Conversion Constants		DATA

ddc2	inprt	Output 'IN' and Line Number

dddd	fout	Convert FAC#1 to ASCII String

de68	foutim	Convert TI to String

df11	fhalf	Table of Constants			DATA

df71	sqr	Perform [sqr]

df7b	fpwrt	Perform power ($)

dfb4	negop	Negate FAC#1

dfbf	logeb2	Table of Constants			DATA

dfed	exp	Perform [exp]

;

;	VIC-20 Kernel ROM

;

;e000	(exp continues) EXP continued From BASIC ROM

e040	polyx	Series Evaluation

e08a	rmulc	Constants for RND			DATA

e094	rnd	Perform [rnd]

e0f6	bioerr	Handle I/O Error in BASIC

e109	bchout	Output Character

e10f	bchin	Input Character

e115	bckout	Set Up For Output

e11b	bckin	Set Up For Input

e121	bgetin	Get One Character

e127	sys	Perform [sys]

e153	savet	Perform [save]

e162	verfyt	Perform [verify / load]

e1bb	opent	Perform [open]

e1c4	closet	Perform [close]

e1d1	slpara	Get Parameters For LOAD/SAVE

e1fd	combyt	Get Next One Byte Parameter

e203	deflt	Check Default Parameters

e20b	cmmerr	Check For Comma

e216	ocpara	Get Parameters For OPEN/CLOSE

e261	cos	Perform [cos]

e268	sin	Perform [sin]

e2b1	tan	Perform [tan]

e2dd	pi2	Table of Trig Constants			DATA

;e2dd	1.570796327	pi/2

;e2e2	6.28318531	pi*2

;e2e7	0.25

;e2ec	#05	(counter)

;e2ed	-14.3813907

;e2f2	42.0077971

;e2f7	-76.7041703

;e2fc	81.6052237

;e301	-41.3417021

;e306	6.28318531

e30b	atn	Perform [atn]

e33b	atncon	Table of ATN Constants			DATA

;e33b	#0b	(counter)

;e3ec	-0.000684793912

;e341	 0.00485094216

;e346	-0.161117018

;e34b	 0.034209638

;e350	-0.0542791328

;e355	 0.0724571965

;e35a	-0.0898023954

;e35f	 0.110932413

;e364	-0.142839808

;e369	 0.19999912

;e36e	-0.333333316

;e373	 1.00

e378	init	BASIC Cold Start

e387	initat	CHRGET For Zero-page

e39f	rndsed	RND Seed For zero-page			DATA

;e39f	0.811635157

e3a4	initcz	Initialize BASIC RAM

e404	initms	Output Power-Up Message

e429	words	Power-Up Message			DATA

;e429	' bytes free<cr>'

;e436	'<clr>**** cbm basic v2 ****<cr>'

e44f	bvtrs	Table of BASIC Vectors (for 0300)	WORD

e45b	initv	Initialize Vectors

e467	bassft	BASIC Warm Restart [RUNSTOP-RESTORE]

e475	?

e47c	-	Unused Bytes For Future Patches		EMPTY

e4a0	-	Serial Output 1

e4a9	-	Serial Output 0

e4b2	-	Get Serial Data And Clock In

e4bc	-	Get Secondary Address patch for Serial LOAD/VERIFY

e4c1	-	Relocated Load patch for Serial LOAD/VERIFY

e4cf	-	Tape Write patch for CLOSE

e4da	-	Unused					EMPTY

e500	iobase	Return I/O Base Address

e505	screen	Return Screen Organization

e50a	plot	Read / Set Cursor X/Y Position

e518	cint1	Initialize I/O

e55f	-	Clear Screen

e581	-	Home Cursor

e587	-	Set Screen Pointers

e5b5	-	Set I/O Defaults (Unused Entry)

e5bb	-	Set I/O Defaults

e5cf	lp2	Get Character From Keyboard Buffer

e5e5	-	Input From Keyboard

e64f	-	Input From Screen or Keyboard

e6b8	-	Quotes Test

e6c5	-	Set Up Screen Print

e6ea	-	Advance Cursor

e719	-	Retreat Cursor

e72d	-	Back on to Previous Line

e742	-	Output to Screen

e756	-	-unshifted characters-

e800	-	-shifted characters-

e8c3	-	Go to Next Line

e8d8	-	Output <CR>

e8e8	-	Check Line Decrement

e8fa	-	Check Line Increment

e912	-	Set Colour Code

e921	-	Colour Code Table			DATA

e929	?						DATA

e975	-	Scroll Screen

e9ee	-	Open A Space On The Screen

ea56	-	Move A Screen Line

ea6e	-	Syncronise Colour Transfer

ea7e	-	Set Start of Line

ea8d	-	Clear Screen Line

eaa1	-	Print To Screen

eab2	-	Syncronise Colour Pointer

eabf	-	Main IRQ Entry Point

eb1e	scnkey	Scan Keyboard

eb71	-	Process Key Image

ec46	-	Pointers to Keyboard decoding tables	WORD

ec5e	-	Keyboard Decoding Table	- Unshifted	DATA

ec9f	-	Keyboard Decoding Table	- Shifted	DATA

ece0	-	Keyboard Decoding Table	- Commodore	DATA

ed21	-	Graphics/Text Control

ed69	-	Keyboard Decoding Table			DATA

eda3	-	Keyboard Decoding Table	- Control	DATA

ede4	-	Video Chip Set Up Table			DATA

edf4	-	Shift-Run Equivalent			DATA

edfd	-	Low Byte Screen Line Addresses		DATA

ee14	talk	Send TALK Command on Serial Bus

ee17	listn	Send LISTEN Command on Serial Bus

ee49	-	Send Data On Serial Bus

eeb4	-	Flag Errors

eeb4	Status #80 - device not present

eeb7	Status #03 - write timeout

eec0	second	Send LISTEN Secondary Address

eec5	-	Clear ATN

eece	tksa	Send TALK Secondary Address

eed3	-	Wait For Clock

eee4	ciout	Send Serial Deferred

eef6	untlk	Send UNTALK on Serial Bus

ef04	unlsn	Send UNLISTEN on Serial Bus

ef19	acptr	Receive From Serial Bus

ef84	-	Serial Clock On

ef8d	-	Serial Clock Off

ef96	-	Delay 1 ms

efa3	-	RS-232 Send

efee	-	Send New RS-232 Byte

f016	-	'No DSR' Error

f019	-	'No CTS' Error

f021	-	Disable Timer

f027	-	Compute Bit Count

f036	-	RS-232 Receive

f05b	-	Set Up To Receive

f068	-	Process RS-232 Byte

f0bc	-	Submit to RS-232

f0ed	-	Send to RS-232 Buffer

f116	-	Input From RS-232

f14f	-	Get From RS-232

f160	-	Serial Bus Idle

f174	-	Table of Kernal I/O Messages		DATA

;f174	'<cr> i/o error #'

; ...

;f1df	'ok<cr>'

f1e2	-	Print Message if Direct

f1e6	-	Print Message

f1f5	getin	Get a byte

f20e	chrin	Input a byte

f250	-	Get From Tape / Serial / RS-232

f27a	chrout	Output One Character

f2c7	chkin	Set Input Device

f309	chkout	Set Output Devic

f34a	close	Close File

f3cf	-	Find File

f3df	-	Set File values

f3ef	clall	Abort All Files

f3f3	clrchn	Restore Default I/O

f40a	open	Open File

f495	-	Send Secondary Address

f4c7	-	Open RS-232

f542	load	Load RAM From Device

f549	LOAD	-load-

f55c	-	Load File From Serial Bus

f5ca	-	Load File From Tape

f647	-	Print "SEARCHING"

f659	-	Print Filename

f66a	-	Print "LOADING / VERIFYING"

f675	save	Save RAM To Device

f685	SAVE	-save-

f692	-	Save to Serial Bus

f6f1	-	Save to Tape

f728	-	Print "SAVING"

f734	udtim	Increment Real-Time Clock

f760	rdtim	Read Real-Time Clock

f767	settim	Set Real-Time Clock

f770	STOP	Check STOP Key

;f77e	-	Output I/O Error Messages

f77e	'too many files'

f781	'file open'

f784	'file not open'

f787	'file not found'

f78a	'device not present'

f78d	'not input file'

f790	'not output file'

f793	'missing filename'

f796	'illegal device number'

f7af	-	Find Any Tape Header

f7e7	-	Write Tape Header

f84d	-	Get Buffer Address

f854	-	Set Buffer Stat / End Pointers

f867	-	Find Specific Tape Header

f88a	-	Bump Tape Pointer

f894	-	Print "PRESS PLAY ON TAPE"

f8ab	-	Check Tape Status

f8b7	-	Print "PRESS RECORD..."

f8c0	-	Initiate Tape Read

f8e3	-	Initiate Tape Write

f8f4	-	Common Tape Code

f94b	-	Check Tape Stop

f95d	-	Set Read Timing

f98e	-	Read Tape Bits

faad	-	Store Tape Characters

fbd2	-	Reset Tape Pointer

fbdb	-	New Character Setup

fbea	-	Send Tone to Tape

fc06	-	Write Data to Tape

fc95	-	Write Tape Leader

fccf	-	Restore Normal IRQ

fcf6	-	Set IRQ Vector

fd08	-	Kill Tape Motor

fd11	-	Check Read / Write Pointer

fd1b	-	Bump Read / Write Pointer

fd22	-	Power-Up RESET Entry

fd3f	-	Check For A-ROM

fd4d	-	ROM Mask 'a0CBM'			DATA

fd52	restor	Restore Kernal Vectors (at 0314)

fd57	vector	Change Vectors For User

fd6d	-	Kernal Reset Vectors			WORD

fd8d	ramtas	Initialise System Constants

fdf1	-	IRQ Vectors For Tape I/O		WORD

fdf9	ioinit	Initialise I/O

fe39	-	Enable Timer

fe49	setnam	Set Filename

fe50	setlfs	Set Logical File Parameters

fe57	readss	Get I/O Status Word

fe66	setmsg	Control OS Messages

fe6f	settmo	Set IEEE Timeout

fe73	memtop	Set / Read Top of Memory

fe82	membot	Set / Read Bottom of Memory

fea9		NMI Transfer Entry

fed2		Warm Start Basic [BRK]

;fed2	User Vector (default BRK)

ff56		Exit Interrupt

ff5b		RS-232 Timing Table			DATA

ff72		IRQ Transfer Entry

;

;ff8a	Vic-20 Kernel Jump Table

;

ff8a	jmp $fd52	restor		Restore Vectors

ff8d	jmp $fd57	vector		Change Vectors For User

ff90	jmp $fe66	setmsg		Control OS Messages

ff93	jmp $eec0	secnd		Send SA After Listen

ff96	jmp $eece	tksa		Send SA After Talk

ff99	jmp $fe73	memtop		Set/Read System RAM Top

ff9c	jmp $fe82	membot		Set/Read System RAM Bottom

ff9f	jmp $eb1e	scnkey		Scan Keyboard

ffa2	jmp $fe6f	settmo		Set Timeout In IEEE

ffa5	jmp $ef19	acptr		Handshake Serial Byte In

ffa8	jmp $eee4	ciout		Handshake Serial Byte Out

ffab	jmp $eef6	untalk		Command Serial Bus UNTALK

ffae	jmp $ef04	unlsn		Command Serial Bus UNLISTEN

ffb1	jmp $ee17	listn		Command Serial Bus LISTEN

ffb4	jmp $ee14	talk		Command Serial Bus TALK

ffb7	jmp $fe57	readss		Read I/O Status Word

ffba	jmp $fe50	setlfs		Set Logical File Parameters

ffbd	jmp $fe49	setnam		Set Filename

ffc0	jmp ($031a)	(iopen)		Open Vector [F40A]

ffc3	jmp ($031c)	(iclose)	Close Vector [F34A]

ffc6	jmp ($031e)	(ichkin)	Set Input [F2C7]

ffc9	jmp ($0320)	(ichkout)	Set Output [F309]

ffcc	jmp ($0322)	(iclrch)	Restore I/O Vector [F353]

ffcf	jmp ($0324)	(ichrin)	Input Vector, chrin [F20E]

ffd2	jmp ($0326)	(ichrout)	Output Vector, chrout [F27A]

ffd5	jmp $f542	load		Load RAM From Device

ffd8	jmp $f675	save		Save RAM To Device

ffdb	jmp $f767	settim		Set Real-Time Clock

ffde	jmp $f760	rdtim		Read Real-Time Clock

ffe1	jmp ($0328)	(istop)		Test-Stop Vector [F770]

ffe4	jmp ($032a)	(igetin)	Get From Keyboad [F1F5]

ffe7	jmp ($032c)	(iclall)	Close All Channels And Files [F3EF]

ffea	jmp $f734	udtim		Increment Real-Time Clock

ffed	jmp $e505	screen		Return Screen Organization

fff0	jmp $e50a	plot		Read / Set Cursor X/Y Position

fff3	jmp $e500	iobase		Return I/O Base Address

;fff6	Unused Vectors

fff6	[ffff]

fff8	[ffff]

;fffa	Transfer Vectors

fffa	[fea9]		NMI

fffc	[fd22]		RESET

fffe	[ff72]		IRQ