DC2000 README File		   Revision 1.00

Enclosed in this diskette are device drivers for DC2000 VL-Bus IDE Disk

Controller.

DC2000.SYS   is the driver for DOS

DC2000.386   is the driver for Microsoft Windows 3.1

P2000310.DSK is for Netware 386 3.10.

P2000311.DSK is for Netware 386 3.11.

+===============================================+

|	DOS Driver for DC2000 VL-Bus IDE Controller  |

+===============================================+

The file DC2000.SYS is the DOS driver for DC2000 disk controller. You

will need it under following situations:

- Your DC2000 has no on-board external BIOS to take the advantage of

the high speed 32-bit VL-Bus.

- Your DC2000 has on-board external BIOS, but it can not work under the

default Turbo mode. You need to select the operating mode every time

when you power up or reboot your system.

Please follow the steps below to install the driver for DOS.

- Follow the installation steps as described in the DC2000 Installation

Guide and bring up your DC2000.

- Copy the driver DC2000.SYS to your system in the appropriate path.

- Add the following statement in your CONFIG.SYS file:

device = [drive:][\path\] DC2000.sys [/F or /T]

where [drive:] and [\path\] point to the directory that contains the

the DC2000.sys file. The parameters F and T define the operating mode

as follows:

F : DC2000 working in the Fast mode

T : DC2000 working in the TURBO mode

Example:

If the DOS driver DC2000.SYS resides in the root directory of the

drive C:, and you want your DC2000 to run under TURBO mode, add the

following statement to the file CONFIG.SYS.

device= c:\DC2000.sys /t

+========================================================+

|	Microsoft Windows version 3.1 disk driver for DC2000  |

+========================================================+

The file DC2000.386 is the Microsoft Windows 3.1 disk driver for

DC2000 controller.

While Microsoft Windows version 3.1 supports 32-bit disk access, the

disk data transfer between host and the IDE adapter is still limited

by the 16-bit ISA bus. If you want to speed up your hard drives under

Windows through the 32-bit VL-Bus and multiple sectors Read/Write,

you will need this driver.

Please follow the steps below to install the driver for DC2000.

1. Copy the driver DC2000.386 to your system in the appropriate path.

It is recommended that the driver DC2000.386 be copied to the Windows

SYSTEM directory.

2. Check if the following lines exist in the [386Enh] section of your

SYSTEM.ini file.

[386Enh]

32BitDiskAccess=ON

device=*int13

If the statements do not exist, add them into the [386Enh] section.

If the statements exist, but 32BitDiskAccess is set to "OFF", you

need to change it to "ON".

3. Install the virtual device "DC2000.386".

Add the following command lines in the [386Enh] section of the

SYSTEM.INI file:

device=[drive:][\path\]DC2000.386

DisksAccessMode=[Fast or Turbo]

The [drive:] and [\path\] point to the directory that contains

DC2000.386 file. Note that the setting must specify the full path

of the device driver DC2000.386.

The disks access mode setting specifies disk access under fast mode

or turbo mode.

4. Delete the "device=*wdctrl" device setting.

Delete the command line "device=*wdctrl" from the [386Enh] section

of your SYSTEM.INI file, if it exists.

+=======================================+

|	Netware 3.xx disk driver for DC2000  |

+=======================================+

Please  follow the  steps below  to install the driver for Netware 3.10.

Use P2000311.DSK instead of P2000310.DSK in the following procedure when

you are installing Netware 3.11. Note that you don't need to load the

ISADISK.DSK when you are installing the Netware driver for DC2000.

1.  After you have installed the DC2000, bring up the Netware server

until the prompt ":" appears on the screen.

2.  Type the following command after the ":" prompt:

:load P2000310 [/F or /T] port=1f0 int=e

The parameters F and T define the operating mode as follows:

F : DC2000 working in the Fast mode

T : DC2000 working in the TURBO mode

+==============================+

|	Trademarks Acknowledgement  |

+==============================+

VESA is a registered trademark of the Video Electronics Standards

Association. VL-Bus is a trademark of the Video Electronics Standards

Association.

MS-DOS and Windows are registered trademarks of Microsoft Corporation.

PC, PC-AT and OS/2 are registered trademarks of IBM Corporation.

Xenix is a registered trademark of Santa Cruz Operation Corporation.

Unix is a registered trademark of American Telephone and Telegraph Corp.

Netware is a registered trademark of Novell Corporation.

#