COMPRESSOR  -  VERSION 1.1

BY TOM WALTON   -   SEPTEMBER 1, 1986

NATURAL GAS PIPELINE COMPRESSOR CALCULATION

WITH

VARIABLE COMPRESSION EFFICIENCY

ANOTHER PRODUCT OF Midnight Magic Software

===========================================

COMPRESSOR  is  a  demonstration program that is  useful  to  the

engineer working with centrifugal compressors to compress natural

gas  or  gas with high CO2(up to 80%) or H2S(up to 80%)  content.

The  program uses data taken from the  compressor  characteristic

curve  to  determine  the actual operational  efficiency  at  the

operating  point.   The  technique for determining the  operating

efficiency, using three data points from the MAX SPEED CURVE, was

published  in the SEPTEMBER 1,  1986 Oil and Gas  Journal.   This

program  uses  a  slightly  more  complex  method  to  fit   four

efficiency data points to a curve.

The program uses the gas gravity and inert analysis (plus methane

mole  fraction  if gravity > 0.75 or if the gas contains H2S)  to

calculate the necessary fluid properties.  These calculations are

based  on the compressability correlation taken from  "VOLUMETRIC

AND  PHASE  BEHAVIOR  OF  OILFIELD  HYDROCARBON  SYSTEMS",   M.B.

Standing, 9th printing, SPE, 1981, Page 121.  This correlation is

a  curve fit of the Standing-Katz  compressability  charts.   Its

applicable  range  is  1.2<Tr<2.4  and  0<Pr<13.   There  are  no

messages  to  indicate that these limits have been  exceeded,  so

check your results if the gravity is high.   The specific gravity

is arbitrarily limited to 1.50.

To use the program, enter;

d:COMPRESS<cr>      -- where d: is the drive designation.

The  first screen tells the user what data is required to run the

program.  To continue, press any key.

The  second  screen  is the fluid  property  input  screen.   The

program contains an example which may be run by pressing the down

arrow on the numeric keypad once for each entry.  This feature is

also  available  for normal use to allow the user to go back  and

modify  one  value  without entering all  the  data  over  again.

Backtracking  through the data is done by pressing the <esc>  key

until the proper value is replaced with the curser bar.  Pressing

a carrage return without entering a value will cause a zero to be

used.

Skipping  to previous screens puts the curser at the first  entry

on  the  screen and the down arrow is used to move the curser  to

the proper entry.   Existing values are replaced by entering  the

new value and pressing the <cr>.   Pressing the down arrow at any

point  in  an entry will cause the entry to be lost and  the  old

value to be used.

1

The third and fourth screens are used to describe the  compressor

envelope.   The  third screen asks for a description of the SURGE

LINE.   The  values entered here do not effect the  calculations,

but are used to tell the user if the selected operating point  is

within  the operation envelope.   If both data points entered are

the same, or describe a vertical line, the program will halt with

a zero divide error ( Run-time error 02).

The  fourth  screen asks for four data points along  the  MAXIMUM

SPEED  CURVE.    The  data  required  are  the  ACFM,   HEAD  and

%  EFFICIENCY at four points  on the curve.   If  the  efficiency

exhibits  a  maximum,  select two points from either side of  the

maximum  if  possible.   Poor  data will  cause  messages  to  be

displayed  on the last line of the screen,  but will normally not

halt processing.

The  fifth  screen  is the operational data  screen.   The  inlet

temperature,  inlet  and  discharge pressures and flow  rate  are

required.   The  final calculated results are also  displayed  on

this screen.   As described above, the program allows the user to

go  back through the data with the <esc> key and to move  forward

through  it  with  the  down arrow key and to  change  one  value

without  disturbing other values.   Pressing the return key  when

the  curser bar is visible will cause the corresponding value  to

be replaced with what has been entered.

BUILT IN SAMPLE PROBLEM

The  sample  problem  built into the program is  taken  from  the

compressor envelope published in the Oil and Gas Journal.  To try

it,  run the program and simply press the down arrow key for each

entry.   When  the  fourth screen has been entered  and  any  key

pressed  to  continue,  a slight pause of one or two  seconds  is

experienced  while  the  final  calculations  are  done  for  the

envelope.   When the fifth screen is displayed, continue to press

the down arrow until the last entry is filled and the bottom line

message is showing.

The  data  displayed  is  a complete  picture  of  the  operating

parameters  of a centrifugal compressor.   The operating point is

displayed as are the limiting parameters.   The SURGE ACFM is the

minimum ACFM that would be permitted at the operating head before

the unit went into surge (or the surge controller became active).

The  MAX  HEAD is the maximum head that can be  obtained  at  the

operating   ACFM  without  overspeeding  the  compressor.     The

calculated operating speed and the operating efficiency are  also

displayed   along   with  the  efficiency-corrected   compression

horsepower.  The fluid parameters are displayed as well.

The user can modify the example to see what the effects of higher

flow  rates,   higher  discharge  pressure,  changes  in  suction

temperature, etc. will have on the operation of the compressor.

2

For example,  press the <esc> to move the curser bar to DISCHARGE

PRESSURE and enter 1200<cr>.   Make sure you press the return and

not the down arrow or the value will be unchanged.   Look at  the

bottom  of  the  right hand column,  the  calculated  speed,  the

calculated efficiency and the discharge temperature.

Change  the discharge pressure back to 1020 and change the  inlet

flow  rate  to 700 MCFD.   Look at the bottom of the  right  hand

column,  the calculated discharge temperature, the efficiency and

the calculated ACFM.

ERROR MESSAGES

The  program has a number of data checks built into  the  program

to  prevent or reduce data errors.   The basic input routine will

accept numbers in the form xxxx.x or x.xxxExx.   Checks are  made

to ensure that letters and other symbols are not entered and  the

form of the numbers is checked.   For example,  "0.-55",  "-.3.6"

and  "34.5-" are not valid data forms.   If these are entered the

computer  beeps and a message is displayed on the bottom line  of

the screen.

Once an input value has been entered,  it is checked for validity

if this is possible.  Some of the checks made are:

- specific gravity   0.5539 < G < 1.50

- mole fraction         0.0 < Z < 1.0  and SUM(Z) <= 1.0

- suction pressure     14.7 < Ps

- discharge pressure     Pd > Ps

In  addition,  a number of data checks are made of  the  envelope

data.  The ACFM - Head data are fitted to quadratic curves in two

sets of three points:  (1,2,4) and (1,3,4).   The coefficients of

the curves are checked as described in the O&GJ article.   If one

curve is concave upward or has a negative constant,  a message is

printed  to indicate that one of the two sets of data have a  bad

point.   The  bad  set  of  data is  discarded  and  the  program

continues.   If both sets of coefficients are bad,  the following

message is printed on the screen:

** ERROR ** IMPROPER VALUES IN ENVELOPE DATA

If you continue at this point,  a fatal error will result and the

program will have to be restared.

After these checks are done, the first curve is used to calculate

the  head  at the third ACFM point and the result is compared  to

the input data.  If the values are different by more than 2%, the

following message is printed:

PROBLEM WITH MAX SPEED DATA - CHECK ACCURACY BEFORE PROCEEDING

If  this  or  no message  is  printed,  the  resulting  quadratic

coefficients are averaged and the program proceeds.

3

OTHER PRODUCTS

The methods used in this program are taken from a larger pipeline

simulation program written by the author.   This product,  called

"PLM",  is commercially available on a licence or timeshare basis

and priced to be competitive.  For further information, write to;

Tom Walton

9 Acadie Road

Aylmer, Quebec

Canada

J9J 1H7

4

###############################################################################################