PROGRAM NAME - NOBLINK Version 1.0, May 1, 1985

AUTHOR    -    Jim Kyle (76703,762)

PURPOSE   -    Patches  IBM's  ANSI.SYS (installed copy) to  eliminate  the

annoying blink that occurs when using an IBM color display.

DESCRIPTION -  This program overlays the text-mode scrolling portion of the

resident copy of ANSI.SYS with a new routine which waits for

retrace,  then scrolls 2 lines,  and repeats this process to

scroll  a  total  of 24 lines of text.  It  then  waits  for

retrace  again,  and  inserts the blank line 25.  The  video

display is never blanked,  thus getting rid of the  blackout

effect.

For use only with the ANSI.SYS driver furnished with version

2.10  of DOS.  Should work with any other,  however,  if the

destination  address is made to match that of  the  existing

code and if registers are set up the same as in 2.10.

1

#

0H	;color display screen segment

JZ	OK		;ok to install

MOV	DX,OFFSET EM2

JMP	SHORT FINI

OK:	MOV	SI,OFFSET NEW	;set up for move

MOV	DI,01C0H	;offset to put patch at

MOV	CX,NEWLEN	;length of patch

PUSHF

CLD

CLI			;disable interrupts while moving!

REPZ	MOVSB

POPF

MOV	DX,OFFSET MSG3	;report success

FINI:	MOV	AH,9

INT	21H

INT