SB-016

TR-8300 Alignment                            09-02-77

The TR08300 is a 440 MHz FM transceiver that is factory tuned to transmit

from 445.0 MHz to 450 MHz and receive from 442.0 MHz to 447.0 MHz.

However, in some areas the operating ranges are different.  The TR-8300 may

be retuned for these other operating ranges.  All adjustments should be

made in the center of the 5 MHz range desired.

RX ALIGNMENT:

Adjust the oscillator coil L14 1/2 to 1/4 turn counterclockwise until the

LED in the channel indicator window turns on.  This indicates that the

oscillator is now working.   Connect a signal generator to the TR-8300 and

adjust L15, L16, TC-6 and TC-7 for maximum "S" meter indication.  (These

should be very small adjustments.)  Adjust TC-5 in the helical resonator

for maximum "S" meter indication.  This is the only adjustment made to the

helical reasonator.

TX ALIGNMENT:

Connect a wattmeter and dummy load to the TR-8300.  Adjust the oscillator

coil L3 for maximum RF output.  No other transmitter alignment should be

necessary.

(c) 090277TKC

Downloaded From P-80 International Information Systems 304-744-2253