SB-820

TR-7625 Final Module Relpacement                        3-20-80

When replacing the output hybrid with a "Red" Mitsubishi logo, and

additional 47pF cap must be installed between teh M57712H output terminal

and the RFO wrap post.

Install the M57712H with white silicone heatsink compound.  Install a 47pF

as shown between the hybird output and the RFO terminal.  Spread L21 as

necessary for at least 30W output at approximately 5.5 A total current

consumption.

Note:  Hybirds not having a red mitsubishi logo do not require the

additional cap or adjustment.

JEB/yn

(c) 32080TKC

Downloaded From P-80 International Information Systems 304-744-2253