SB-827

TS-120S VFO Stability                       6-6-80

(Supercedes 7/9/79 bulletin)

VFO drift characteristics may be improved by changing component values and

location in the VFO.

This change applies to serial numbers before 00100001.

Note:  DO NOT disturb lead dress or component placement while working

inside the VFO.

1.   Remove the VFO unit by four 3mm hex head bolts.

2.   Withdraw out the front panel and unplug the leads.

3.   Remove the VFO cover by 5 phillips head screws.

4.   Remove C17, and C18 from the PCB.  Thes may be cut flush to the board.

5.   Reposition the Ground terminal for the Variabel Capacitor by heating

the post and pushing it through to the opposite side of the PCB.

6.   Install paralleled 10pF green and 18pF yellow caps from the bottom

tank coil terminal (LH) to the modified Ground post.  Position the caps

near the coil, not the PCB.  Make certain the ground lead doesn't touch the

coil or coil mounting.

7.   To reassemble, perform steps 3,2,1.  Snug all VFO case screws.  A

later complaint of frequency jump would indicate a loose VFO shell.

New Parts:  10pf CC45SG1H100J

18pf CC45RG1H180J

Analog Dial Calibration:

1.   Turn the main dial fully CCW.  The red cursor should line up with the

VFO mark on the sub-dial.  If it does not, remove the main knob, loosen set

screws and align the scale start point with the red cursor.

2.   Turn the main knob to 50 kHz analog.  Adjust the aluminum slip sub-

dial to line up with any one of the larger black dial marks.

3.   Note the digital error.  If it is MORE than 2 kHz adjust the VFO

trimmer cap TC1 (center under the seal tape) to exactly 50.0 on the digital

readout.

4.   Turn the main knob to 450 analog.  If the digital error is less than 2

kHz it is in spec.  If the digital error is greate, proceed:

For instance if the digital error is 14.454.0 (plus 4 kHz), multiply the

error times 4 (16 kHz) and adjust the VFO trimmer cap to the desired

frequency (14.450.0) less the error, or 14.434.0.  Next adjust the VFO

inductor L10 (front under the seal tape) back up to the desired frequency

of 14.450.0.

5.   If the error in step 4 was in the minus direction, reverse the

direction of correction adjustment in step 4.

6.   VFO linearity final check:  The digital readout and analog dial should

agree to within +/- kHz at every 100 kHz dial point.

JEB/yn

(c) 6680TKC

Downloaded From P-80 International Information Systems 304-744-2253