SB-847R

TS-830S Low Voltage Supply Optional Improvements                  7-29-81

Power supply drift and stability may be improved by these optional

component changes and additions.

On the AF AVR unit X49-1140-00 (Vicinity of Q27-Q30) change:

R124 from 1K to 1.5K (RD14CB2E152J)*

R127 from 470 to 390 (RD14CB2E391J)*

R130 from 47K to 33K (RD14CB2E333J)#

D25 from WZ-061 to XZ-053 (V11-4101-60)*

*This will improve temperature drift from a maximum of about 100mV to a

maximum of 1mV, and may be applied to any unit before serial # 201xxxx.

Add two 22 k ohm 1/4 resistors (RD14CB2E223J0, one each across Q30, and

Q34, collector to emitter.#

#This will improve overall stability and may be applied to any unit before

serial #105xxxx.

Note:  These changes are at the owners option and may not be performed in-

warranty.

JEB/sh

(c) 041482TKC

Downloaded From P-80 International Information Systems 304-744-2253