SB-853

TR-8400 Transmitter Freq Response, Voice or DTMF                   1-25-82

Operator complaints of insufficient lows in transmit, or faulty Autopatch

operation traceable to incorrect twist (high to low tone group balance) may

be remedied by changing two capacitors at the mic amplifier input.

On the RX/TX unit X44-1340-11, change C62 and C65 from 0.1uf to 1.0uf, 35v

tantalum (CS15E1V010M) to increase low frequency transmitter frequency

response.  Adjust Deviation (VR3) or mic gain (VR2) as necessary.

Installation time for this procedure is 1/2 or less.

JEB/sh

(c) 121589TKC

Downloaded From P-80 International Information Systems 304-744-2253