SB-855

TR-8400 Transmit Delay Time                                  1-25-82

PLL lockup time when switching from receive to transmit may be shortened

for faster transmitter response.  This change applies to units before

serial number 1060455.

On the PLL unit X50-1670-xx, check the values of R31 (390 ohm) and C47, C48

(6.8uf) near Q7 and Q8.  If these component values differ, change as

required.  This will give approximately a 0.12 second delay, which should

be satisfactory for practical use.  However, if shorter delay is required,

replace R31 with an 820 ohm resistor.  This will yield an approximate 0.096

second turn-on delay.

No adjustment is required after this part change.

Note:  When R31 is 820 ohms, CTCSS (Subtone) will be slightly distorted.

Installation time for this procedure is 1/2 or less.

JEB/sh

(c) 121589TKC

Downloaded From P-80 International Information Systems 304-744-2253