SB-884

TS-930S 15 Meter Internal Beat Tone                            1-27-84

Some users have reported an internal beat tone that appears every 10 KHz ,

when operating in the range of 20.000 to 21.500 MHz.  Apparently the RV

terminal of Switch Unit H is being affected by the 1st loop of the PLL

unit.  The cure is relatively simple.  Add a .01uF 50v Disk ceramic

capacitor from the RV terminal to ground.

CLM

Time required for this modification is 1/2 hour or less.

(c) 121589TKC

Downloaded From P-80 International Information Systems 304-744-2253