SB-886

TS-930S Intermittent TX Power Output                            5-11-84

The following procedure should correct any tendency of the TS-930S to

exhibit intermittent TX power output.  Most of the reported cases of thsi

nature have been traced to poor contact of one or more plated thru holes

mentioned below.  Careful adherence to this procedure should prevent

reoccurrance of this symptom.

Procdeure:

1.   Remove the top and bottom covers.

2.   Remove the 10 screws  securing the final unit to the chassis.

3.   Disconnect all cable assemblies from the final assembly, and remove it

from the radio.  The fan motor cable may have to be removed for easy

access.

4.   Remove all Final Unit PC board screws and turn the circuit board foil

side up.

5.   Desolder Q6, D2, and the plated-thru hole connecting the foils from

R20 and Q8 base together (between T2 and VR1).  Do not remove the

components!

6.   Carefully remove the green solder resistant coating from the immediate

areas of these six points, so that there is bare copper foill up to and

surrounding the eyelets.

7.   Carefully resolder these six points, and those listed below:

Q1 Emitter and base

D4 anode and cathode

Q2 Emitter and base

Q3 Emitter and Base

Q7 Emitter and Base

Q4 and Q5 Base and collectors

Plated-thru hole between C36 and C15

(2) Plated-thru hole sby the molex connector.

Check and resolder the input and output coax connectors.

8.   Double check your work to ensure that there are no solder bridges or

splashes.

9.   Check the value of R19.  If it is not 6.8 K ohms, change it so it is.

10.  Reinstall the final assembly, and readjust the idle bias currents of

the driver and final transistors in accordance with Service Bulletin 867.

CAUTION:  ENSURE THAT NO CABLES ARE PINCHED BETWEEN THE FINAL ASSEMBLY AND

CHASSIS WHEN REINSTALLING THE FINAL UNIT.  THE FAN CABLE IS ESPECIALLY

VULNERABLE, SO PAY CLOSE ATTENTION!

Time required for this modification is 1 hour or less.

(c) 121589TKC

Downloaded From P-80 International Information Systems 304-744-2253