SB-904

TM-2550 DCL Call Sign Data                  2-7-86

Some owners of the TM-2530/50/70 have reported a loss of the DCL system

call sign data when the power switch is turned off.  The call sign data

actually remains in memory, but timing prevents its utilization in some

radios.  Adding a resistor will correct this problem.

If you receive a radio that exhibits this symptom add a 47K ohm resistor to

IC3 to the Slave CPU on the control unit (X53-1440-XX) between pins 4 and

20.  Cover the resistor leads with "spaghetti" type insulation.  Adding the

resistor to radios that do not have this symptom will not cause any

problems, so you may wish to add the resistor if you receive a radio for

service for some other problem.

Before soldering unplug the DC power cable and antenna from the radio.

When soldering to this IC you must use a soldering iron that provides

voltage isolation on the tip since IC3 is a CMOS device and can be easily

damaged by static electricity or voltage on your iron.  Use normal CMOS

precautions.  One final precaution:  Don't bridge any of the pins on the IC

with excess solder!

Time required for this modification is 1/2 hour or less.

(c) 121589TKC

Downloaded From P-80 International Information Systems 304-744-2253