SB-908

TS-940S PLL Unlock                 4-8-86

Reports of a PLL unlock on the lower frequencies may be due to low VCO

levels.  The following change will increase the VCO level and should

prevent reoccurrance of this symptom.

Part Required

C168           change to 15 pF 50v (CC45SL1H150J)

Procedure:

1.   On the RF Unit (X44-1660-00) change C168 from a 33 pF capacitor to a

15pf 50v capacitor.

2.   Readjust the VCO BPF (RF unit L74-76) according to the instructions

contained in the TS-940S Service Manual.

This change is applicable to serial numbers prior to 606XXXX.

Time required for this modification is 1 hour or less.

(c) 121589TKC

Downloaded From P-80 International Information Systems 304-744-2253